3GPP TSG-SA3 Meeting #91	S3-181167

# Belgrade, Serbia, 16th Apr 2018 - 20th Apr 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0033 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Deletion of Editor's Notes in clause 6.1.2 and 6.1.3 and restructuring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  | Date: |  |  | 2018-04-04 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Deletion of Editor’s Notes can be done because they are no longer necessary. The restructuring of the failure cases is done because they apply to both EAP AKA’ and 5G AKA and are now under 5G AKA clause. New clause is added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Deletion of Editor’s Note and adding a new clause where the failure cases are put. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification because unnecessary Editor’s Notes are left in and because the structure is unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.1, 6.1.3.2, 6.1.3.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

#### 6.1.3.1	Authentication procedure for EAP-AKA'

EAP-AKA' is specified in RFC 5448 [12]. The 3GPP 5G profile for EAP-AKA' is specified in the normative Annex F.

EAP-AKA' is applied within the 5G authentication framework whenever the UDM/ARPF has received an Nudm_UEAuthentication_Get Requestmessage from the AUSF, and has chosen EAP-AKA' as the authentication method, cf. sub-clause 6.1.2 of the present document.

The authentication procedure for EAP-AKA' works as follows, cf. also Figure 6.1.3.1-1:

0. The AUSF shall send a Nudm_UEAuthentication_Get Request message to the UDM/ARPF, if SUCI was included in the Nudm_UEAuthentication_Get Request, UDM/SIDF shall deconceal the SUPI and then choose EAP-AKA’ as authentication method as described in clause 6.1.2.

1.	The UDM/ARPF shall first generate an authentication vector with Authentication Management Field (AMF) separation bit = 1 as defined in TS 33.102 [9]. The UDM/ARPF shall then compute CK' and IK' as per the normative Annex A and replace CK and IK by CK' and IK'.

2.	The UDM/ARPF shall subsequently send this transformed authentication vector (RAND, AUTN, XRES, CK', IK') to the AUSF from which it received the Nudm_UEAuthentication_Get Request together with an indication that the AV is to be used for EAP-AKA' using a Nudm_UEAuthentication_Get message.

NOTE:	The exchange of a Nudm_UEAuthentication_Get Request message and an Nudm_UEAuthentication_Get Responsemessage between the AUSF and the UDM/ARPF described in the preceding paragraph is the same as for trusted access using EAP-AKA' described in TS 33.402 [11], sub-clause 6.2, step 10, except for the input parameter to the key derivation, which is the value of <network name>. The "network name" is a concept from RFC 5448 [12]; it is carried in the AT_KDF_INPUT attribute in EAP-AKA'. The value of <network name> parameter is not defined in RFC 5448 [12], but rather in 3GPP specifications. For EPS, it is defined as "network access identity" in TS 24.302 [13], and for 5G, it is defined as "serving network name" in sub-clause 6.1.1.4 of the present document.

Editor's Note: The number of the stage 3 specification TS 24.yyy [xx] is FFS.

Figure 6.1.3.1-1: Authentication procedure for EAP-AKA'

In case SUCI was included in the Nudm_UEAuthentication_Get Request, UDM will include the SUPI in the Nudm_UEAuthentication_Get Response.

The AUSF and the UE shall then proceed as described in RFC 5448 [12] until the AUSF is ready to send the EAP-Success.

3.	The AUSF shall send the EAP-Request/AKA'-Challenge message to the SEAF in a Nausf_UEAuthentication_Authenticate Response message.

4.	The SEAF shall transparently forward the EAP-Request/AKA'-Challenge message to the UE in a NAS message Auth-Req message. This message shall also include the ngKSI that will be used by the UE and AMF to identify the partial native security context that is created if the authentication is successful. The ME shall forward the RAND and AUTN received in EAP-Request/AKA'-Challenge message to the USIM.

5.	At receipt of the RAND and AUTN, the USIM shall verify the freshness of the authentication vector by checking whether AUTN can be accepted as described in TS 33.102[4]. If so, the USIM computes a response RES. The USIM shall return RES, CK, IK to the ME. If the USIM computes a Kc (i.e. GPRS Kc) from CK and IK using conversion function c3 as described in TS 33.102 [4], and sends it to the ME, then the ME shall ignore such GPRS Kc and not store the GPRS Kc on USIM or in ME.The ME shall derive CK' and IK' according to TS 33.402 [11], sub-clause 6.2, step 15.

If the verification of the AUTN fails on the USIM, then the USIM and ME shall proceed as described in sub-clause 6.1.3.2.1.

6.	The UE shall send the EAP-Response/AKA'-Challenge message to the SEAF in a NAS message Auth-Resp message.

7.	The SEAF shall transparently forwards the EAP-Response/AKA'-Challenge message to the AUSF in Nausf_UEAuthentication_Authenticate Request message.

8.	The AUSF shall verify the message, and if the AUSF has successfully verified this message it shall continue as follows, otherwise it shall return an error.

9.	The AUSF and the UE may exchange EAP-Request/AKA'-Notification and EAP-Response /AKA'-Notification messages via the SEAF. The SEAF shall transparently forward these messages.

10.	The AUSF uses the first 256 bits of EMSK as the KAUSF and then calculates KSEAF from KAUSF as described in clause A.6. The AUSF shall send an EAP Success message to the SEAF inside Nausf_UEAuthentication_Authenticate Response, which shall forward it transparently to the UE. Nausf_UEAuthentication_Authenticate Response message contains the KSEAF. If the AUSF received a SUCI from the SEAF when the authentication was initiated (see sub-clause 6.1.2 of the present document), then the AUSF shall also include the SUPI in the Nausf_UEAuthentication_Authenticate Response message.

11.	The SEAF shall send the EAP Success message to the UE in the N1 message.

The key received in the Nausf_UEAuthentication_Authenticate Response message shall become the anchor key, KSEAF in the sense of the key hierarchy in sub-clause 6.2 of the present document. The SEAF shall then derive the KAMF from the KSEAF and the SUPI according to Annex A.7 and send it to the AMF On receiving the EAP-Success message, the UE calculates KAUSF and KSEAF in the same way as the AUSF and shall derive the KAMF from the KSEAF and the SUPI according to Annex A.7..

The further steps taken by the AUSF upon receiving a successfully verified EAP-Response/AKA'-Challenge message are described in sub-clause 6.1.4 of the present document.

If the EAP-Response/AKA'-Challenge message is not successfully verified, the subsequent AUSF behaviour is determined according to the home network's policy.

If the AUSF and SEAF determines that the authentication was successful, then the SEAF provides the ngKSI and the KAMF to the AMF.

# *** End of First Change ***

# *** Second Change ***

#### 6.1.3.2	Authentication procedure for 5G AKA

##### 6.1.3.2.0	5G AKA

5G AKA enhances EPS AKA [10] by providing the home network with proof of successful authentication of the UE from the visited network. The proof is sent by the visited network in an Authentication Confirmation message.

5G AKA is applied within the 5G authentication framework whenever the UDM/ARPF has received a Nudm_UEAuthentication_Get Request message from the AUSF, and chosen 5G AKA as the authentication method, cf. sub-clause 6.1.2 of the present document.

NOTE 1:	5G AKA does not support requesting multiple AVs, neither the SEAF pre-fetching AVs from the home network for future use.

In 5G AKA there are two types of authentication vectors, namely the following:

-	5G HE AV: the 5G Home Environment Authentication Vector is the type of authentication vector(s) that is received by the AUSF from the UDM/ARPF in the Auth-info Resp. The 5G HE AV has the following fields: RAND, AUTN, XRES*, and KAUSF.

-	5G AV: the 5G Authentication Vector is the type of authentication vector that the SEAF receives from the AUSF in the 5G-AIA message. The 5G AV has the following fields: RAND, AUTN, HXRES*, and KSEAF. The difference between the 5G HE AV and the 5G AV is that the XRES* and the KAUSF in the 5G HE AV is replaced by the HXRES* and the KSEAF in the 5G AV.

Figure 6.1.3.2-1: Authentication procedure for 5G AKA

The authentication procedure for 5G AKA works as follows, cf. also Figure 6.1.3.2-1:

0.	The AUSF shall sentd a Nudm_UEAuthentication_Get Request message to the UDM/ARPF, if SUCI was included in the Nudm_UEAuthentication_Get Request, UDM/SIDF shall deconceal the SUPI and then chose 5G-AKA as authentication method for the SUPI as described in Clause 6.1.2.

1.	For each Nudm_Authenticate_Get Request,  the UDM/ARPF shall create a 5G HE AV. The UDM/ARPF does this by generating an AV with the Authentication Management Field (AMF) separation bit set to "1"as defined in TS 33.102 [9], The UDM/ARPF shall then derive KAUSF as per Annex A.2, and XRES* as per Annex A.4. Finally, creating the UDM/ARPF shall create a 5G HE AV from RAND, AUTN, XRES*, and KAUSF.

2.	The UDM/ARPF shall then return the 5G HE AV to the AUSF together with an indication that the 5G HE AV is to be used for 5G-AKA in a Nudm_UEAuthentication_Get Response. In case SUCI was included in the Nudm_UEAuthentication_Get Request, UDM will include the SUPI in the Nudm_UEAuthentication_Get Response..

3.	The AUSF may store the XRES* temporarily until it expires. The AUSF may store the KAUSF.

4.	The AUSF shall then generate the 5G AV from the 5G HE AV received from the UDM/ARPF by computing the HXRES* from XRES* according to Normative Annex A.5 5 and KSEAF from KAUSF according to Annex A.6, and replacing the XRES* with the HXRES* and KAUSF with KSEAF in the 5G HE AV.

5.	The AUSF shall then return the 5G AV (RAND, AUTN, HXRES*, KSEAF) to the SEAF in a Nausf_UEAuthentication_Authenticate Response.

6.	The SEAF shall send RAND, AUTN to the UE in a NAS message Auth-Req. This message shall also include the ngKSI that will be used by the UE and AMF to identify the KAMF and the partial native security context that is created if the authentication is successful. The ME shall forward the RAND and AUTN received in NAS message Auth-Req to the USIM.

7.	At receipt of the RAND and AUTN, the USIM shall verify the freshness of the authentication vector by checking whether AUTN can be accepted as described in TS 33.102[4]. If so, the USIM computes a response RES. The USIM shall return RES, CK, IK to the ME. If the USIM computes a Kc (i.e. GPRS Kc) from CK and IK using conversion function c3 as described in TS 33.102 [4], and sends it to the ME, then the ME shall ignore such GPRS Kc and not store the GPRS Kc on USIM or in ME. The ME then shall compute RES* from RES according to Annex A.4. The UE shall calculate KSEAF from KAUSF according to clause A.6.

8.	An ME accessing 5G shall check during authentication that the "separation bit" in the AMF field of AUTN is set to 1. The "separation bit" is bit 0 of the AMF field of AUTN.

NOTE:	This separation bit in the AMF field of AUTN can not be used anymore for operator specific purposes as described by TS 33.102 [9], Annex F.

The UE shall return RES* to the SEAF in a NAS message Auth-Resp.

9.	The SEAF shall then compute HRES* from RES* according to Annex A, and the SEAF shall compare HRES* and HXRES*. If they coincide, the SEAF shall consider the authentication successful. If not, the SEAF proceed as described in sub-clause 6.1.3.2.2.

10.	The SEAF shall send RES*, as received from the UE, in a Nausf_UEAuthentication_Authenticate Request message (containing the SUPI or SUCI and the serving network name) to the AUSF.

11.	When the AUSF receives the Nausf_UEAuthentication_Authenticate Request message including a RES* it may verify whether the AV has expired. If the AV has expired the AUSF may consider the confirmation unsuccessful. AUSF shall compare the received RES* with the stored XRES*. If the RES* and XRES* are equal, the AUSF shall consider the confirmation message as successfully verified.

12.	The AUSF shall indicate to the SEAF in the Nausf_UEAuthentication_Authenticate Response whether the confirmation was successful or not. In case the AUSF received a SUCI from the SEAF when the authentication was initiated (see subclause 6.1.2 of the present document), and if the confirmation was successful, then the AUSF shall also include the SUPI in 5G-ACA.

If the authentication was successful, the key KSEAF received in 5G AV shall become the anchor key in the sense of the key hierarchy in sub-clause 6.2 of the present document. Then the SEAF shall derive the KAMF from the KSEAF and the SUPI according to Annex A.7, and shall provide the ngKSI and the KAMF to the AMF.

If a SUCI was used for this authentication, then the SEAF shall only provide ngKSI and KAMF to the AMF after it receives the 5G-ACA message containing SUPI; no communication services will be provided to the UE until the SUPI is known to the serving network.

The further steps taken by the AUSF upon receiving a successfully verified confirmation message are described in sub-clause 6.1.4 of the present document.

#### 6.1.3.3	Handling of synchronization, MAC, and verification failures

##### 6.1.3.3.1	Synchronization failure or MAC failure on USIM

This clause describes synchronisation failure or MAC failure on USIM.

In step 8 in Figure 6.1.3.2-1 when 5G AKA is used; or in step 5 in Figure 6.1.3.1-1 when EAP-AKA’ is used, at the receipt of the RAND and AUTN, if the verification of the AUTN fails, then the USIM indicates to the ME the reason for failure and in the case of a synchronisation failure passes the AUTS parameter (see TS 33.102 [9]) to the ME.

If 5G AKA is used: The ME shall respond with NAS message Auth-Failure with a CAUSE value indicating the reason for failure. In case of a synchronisation failure of AUTN (as described in TS 33.102 [9]), the UE also includes AUTS that was provided by the USIM. Upon receipt of an authentication failure message, the AMF/SEAF may initiate further identity requests and authentications towards the UE. (see TS 24.501 [35]).

If EAP-AKA’ is used: The ME shall proceed as described in RFC 4187 [21] and RFC 5448 [12] for EAP-AKA’.

##### 6.1.3.3.2	RES* verification failure in SEAF or AUSF or both

This clause describes how RES* verification failure in the SEAF or in the AUSF shall be handled.

In step 10 in Figure 6.1.3.2-1, the SEAF shall compute HRES* from RES* according to Annex A, and the SEAF shall compare HRES* and HXRES*. If they don’t coincide, then the SEAF shall consider the authentication as unsuccessful.

The SEAF shall proceed with step 11 in Figure 6.1.3.2-1 and after receiving the 5G-ACA from the AUSF in step 13 in Figure 6.1.3.2-1, proceed as described below:

-	If the AUSF has indicated in the Authentication Confirmation Answer (5G-ACA) to the SEAF that the confirmation of the RES* was not successful in the AUSF, or

-	if the confirmation of the RES* was not successful in the SEAF,

then the SEAF shall either reject the authentication by sending an Authentication Reject to the UE if the SUCI was used by the UE in the initial NAS message or the SEAF/AMF shall initiate an Identification procedure with the UE if the 5G-GUTI was used by the UE in the initial NAS message to retrieve the SUCI.

Also, if the SEAF does not receive any Authentication Confirmation Answer (5G-ACA) from the AUSF as expected, then the SEAF shall either reject the authentication to the UE or initiate an Identification procedure with the UE.

# *** End of Second Change ***