# 3GPP TSG SA WG3 (Security) Meeting #94	S3-190549

# Kochi (India), 28 January – 1 February 2019                                      revision of S3-190198

# Source:	Huawei, HiSilicon

# Title:	URLLC solution for N3 tunnel redundancy

# Document for:	Approval

Agenda Item:	8.13

1	Decision/action requested

This contribution proposes a pCR for a solution to TR 33.825.

2	References

[1]	3GPP TS 23.725 Study on enhancement of Ultra-Reliable Low-Latency Communcation (URLLC) v0.4.0

3	Rationale

The KI#1 is proposing to address the security policy handling for redundant transmission. Even if redundant transmission is the means to guarantee the high-reliability communication, the added user plane paths for transmission redundancy shall provide equal level of security as is provided to the single path. According to SA2 TR23.725 [1], there is a conclusion for redundant transmission that recommends to support high-reliability via single RAN and single UPF. In this case, there are two redundant N3 tunnels established for transferring the same data.

The contribution proposes a solution addressing the security support for N3 tunnel redundancy.

4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.825.

***	BEGIN CHANGES	***

6.X	Solution <X>: Security support for N3 tunnel redundancy

6.x.1	Introduction

This solution addresses Key Issue#1. If the user data is redundant by means of two duplicated N3 tunnels to fulfill the high-reliability feature, as illustrated in figure 6.x.1-1 below, the NDS/IP framework which is used to secure the network domain interfaces should be reused with the following precautions.

Figure 6.x.1-1 redundant transmission architecture

This data redundancy architecture supports the redundant transmission based on two N3 tunnels between a single NG-RAN node and the UPF. It is required that the RAN node and UPF shall support NDS/IP to ensure the security of the N3 interface. The transport of user data over N3 shall be integrity, confidentiality and replay-protected.

Editor’s Note: More clarifications are required on the redundant N3 tunnels with respect to the SA2 agreed solutions.

6.x.2	Solution details

If the IPsec tunnel is used to protect all N3 tunnels between NG-RAN and UPF, the current mechanism defined in TS33.501[X] clause 9.3 and clause 9.1 shall be reused.

The N3 interface for 5GC and 5G-AN according to NDS/IP is specified in TS 33.210 [Y]. Traffic on interfaces carrying control plane signalling can be both integrity and confidentiality protected according to NDS/IP.

IPsec ESP implementation shall be done according to RFC 4303 [Z] as profiled by TS 33.210 [Y]. For IPsec implementation, tunnel mode is mandatory to support while transport mode is optional.

IKEv2 certificate-based authentication implementation shall be done according to TS 33.310. The certificates shall be supported according to the profile described by TS 33.310 [R]. IKEv2 shall be supported conforming to the IKEv2 profile described in TS 33.310 [R].

In order to protect the traffic on the N3 reference point, it is required to implement IPsec ESP and IKEv2 certificate-based authentication. IPsec is mandatory to implement on the NG-RAN. On the core network side, a SEG may be used to terminate the IPsec tunnel.

Editor’s Note: In relation to KI#1, the relevant threats on the backhaul interfaces are FFS.

6.x.3	Evaluation

Editor’s note: The evaluation of the solution is FFS.

***	END OF CHANGES	***