# 3GPP TSG SA WG3 (Security) Meeting #94	S3-180381

# 28 January – 1 February 2019, Kochi (India)	revision of S3-18xabc

# Source:	Qualcomm Incorporated

# Title: 	Protection against Man-in-the-Middle fake base station attacks

# Document for:	Approval

Agenda Item:	8.9

# 1	Decision/action requested

Approve the key issue

# 2	References

[1]	TR 33.809 Study on 5G security enhancements against false base stations

# 3	Rationale

Typical fake base station attacks result in denial of service to UE. Consequently, UE or user may infer such attacks based on the service unavailability and take an action accordingly. However, a more sophisticated attacker may launch various types of attacks in a stealth manner using fake base stations.  MitM fake gNB transports security protected messages between the UE and the network without any modification while altering unprotected messages.

Without addressing the MitM threats, detection of fake base stations and countermeasures against them have limited effectiveness.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR [1].

***** START OF CHANGES *****

## 5.x Key issue #x: Protection against Man-in-the-Middle fake gNB attacks

### 5.x.1	Issue details

Typical fake base station attacks result in denial of service to UE. Consequently, UE or user may infer such attacks based on the service unavailability and take an action accordingly. However, a more sophisticated attacker may launch various types of attacks in a stealth manner using fake base stations.  MitM fake gNB transports security protected messages between the UE and the network without any modification while altering and/or injecting unprotected messages.

Without addressing the MitM threats, detection of fake base stations and countermeasures against them have limited effectiveness.

### 5.x.2 Security threats

A MitM false base station may force a UE to camp on to it by passing all the message on between the UE and real base station. It may then deny the UE service, e.g. reject or drop service request, not pass on paging messages etc.

### 5.x.3 Potential security requirements

Any solutions against the fake base station attacks should address the issues of the MitM fake base station attacks.

***** END OF CHANGES *****