3GPP TSG-SA3 Meeting #98Bis-e 	S3-200696

# e-meeting, 14-17 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0795 | Rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UP integrity protection for UP Signalling messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI-16 |  |  |  |  |  | Date: |  |  | 01-04-2020 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To address potential attack on the sensitive UP traffic (like DNS, ICMP). |  |  |  |  |  |  |  |  |
| Summary of change: |  | To separate and protect sensitive traffic (like DNS, ICMP) in a dedicated PDU sessions that support UP IP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Possiblity of manipulation of encrypted sensitive UP traffic. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************Start of Change***********

### 6.6.X		UP integrity protection for UP Signalling messages

The network may configure a special DNN to establish a dedicated PDU session for the protection of the UP Signalling messages. If the UE capability does not support UP integrity protection at “Full data rate” and if the special DNN is configured, then the UE shall initiate the PDU session establishment procedure to establish the dedicated PDU session, after successful initial registration. The network shall activate the UP integrity protection for this dedicated PDU session. After successful establishment of the PDU session, the dedicated PDU session shall be used to securely exchange the UP signalling messages (for example, DNS, ICMP protocol messages) between the UE and the 5GC, from any other PDU sessions.

************End of Change***********