3GPP TSG-SA3 Meeting #98e 	draft_S3-200129-r1

- e-meeting, 2 – 6 March 2020

- Source:	ZTE,

- Title:	Adding UDM to the AKMA reference model

- Document for:	Approval

Agenda Item:	3.9

# 1	Decision/action requested

This document proposes to add UDM to AKMA reference model considering different UE’s suscribtion. SA3 is kindly requested to approve this doc.

# 2	References

[1]	3GPP TS 33.535, v 0.2.0

# 3	Rationale

Not all users in operator network use the AKMA. For example, the V2X users of the operator network may use the AKMA. However, other users may not use the AKMA. If all users in operator network excute the AKMA operation, it will increase the load of the AUSF and UDM and the power consumption of non-AKMA UE. Therefore, the AKMA users of the operator involve the AKMA subscription, and only the users of the AKMA subscription can use the AKMA. Otherwise, the AKMA will not be used. This pCR adds UDM to the AKMA reference model.

# 4	Detailed proposal

***	BEGIN CHANGES	***

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AAnF	AKMA Anchor Function

AF	Application Function

AMF	Access and Mobility Management Function

AUSF	AUthentication Server Function

NEF	Network Exposure Function

UDM	Unified Data Management

# 4	Architecture for Authentication and Key Management for Applications (AKMA)

Editor’s Note: Titles and Numberings of sub clauses could be adjusted according to the content.

## 4.1	Reference model

Figure 4.1-1 shows a fundamental network model of AKMA, as well as the interfaces between them.

Figure 4.1-1: Fundamental Network Model for AKMA

NOTE: Figure 4.1-1 shows the case where AAnF is deployed as a standalone function. Deployments can choose to collocate AAnF with AUSF or with NEF according to operators’ deployment scenarios.

The AKMA service requires a new logical entity: AKMA Anchor Function (AAnF).

AAnF is the anchor function in the HPLMN that generates the key material to be used between the UE and the AF and maintains UE AKMA contexts to be used for subsequent bootstrapping requests.

## 4.2	Network elements

Editor’s Note: This clause will describe the functionalities and requirements of each entities in the reference model.

### 4.2.1	AAnF

AAnF enables the AKMA anchor key (KAKMA) derivation for AKMA service. Before invoking AKMA service, UE shall have successfully registered to the 5G core, which results in KAUSF being stored at the AUSF and the UE after a successful 5G primary authentication.

## 4.3	Interface description

The following interfaces are involved in AKMA network architecture:

- Nnef: Service-based interface exhibited by NEF.

- Nausf: Service-based interface exhibited by AUSF.

- Nudm: Service-based interface exhibited by UDM.

- Naanf: Service-based interface exhibited by AAnF.

- Naf: Service-based interface exhibited by AF.

The AAnF interacts with the AUSF and the AF using Service-Based Interfaces.

### 4.3.1	Reference point Ua*

The reference point Ua* carries the application protocol, which is secured using the key material agreed between UE and AAnF as a result of successful AKMA procedures.

Editor’s Note: It is expected that the existing GBA based Ua protocols are reused with necessary adaptations as the Ua* for AKMA. The changes required for Ua* are FFS.

***	END CHANGES	***