3GPP TSG-SA3 Meeting #98e 	draft_S3-200136-r1

- e-meeting, 2 – 6 March 2020

- Source:	ZTE

- Title:	Update the figure in clause 6.2

- Document for:	Approval

Agenda Item:	3.9

# 1	Decision/action requested

This document proposes to update the figure in clause 6.2. SA3 is kindly requested to approve this doc.

# 2	References

[1]	3GPP TS 33.535, v 0.2.0.

# 3	Rationale

Beasue AF does not participate in the procedure of primary authentication and establishment of the KAKMA, this contribution proposes to update the procedure of generation AKMA application keys for a specific AF to clause 6.2.

# 4	Detailed proposal

***	BEGIN CHANGES	***

## 6.2	Deriving AKMA Application key for a specific AF

Figure 6.2-1 AF Key generation from KAKMA

When the UE initiates communication with the AKMA AF, it shall include the derived AKMA key identifier (KAKMA ID) in the message (cf. clause 6.1).

If the AF does not have an active context associated with the key identifier, then the AF sends a request to AAnF with the key identifier to request application function specific AKMA keys for the UE.  The AF also includes its identity (AF Id) in the request.

If the AAnF is in possession of the AF specific key (KAF), it responds to the AF with the KAF key. If not, the AAnF shall check if it has the UE specific KAKMA key identified by the AKMA key identifier.

If KAKMA is available in AAnF, it shall derive the AF specific AKMA key (KAF) from KAKMA and respond to the AF with KAF and lifetime.

If KAKMA is not available, the AAnF shall send a request to the AUSF to obtain the KAKMA key specific to the UE. It includes the AKMA key identifier in the request. The AUSF shall respond with the KAKMA key identified by the key identifier. The AAnF derives the AF specific key (KAF) from KAKMA and responds to the AF with KAF and lifetime.

Editor’s Note: Derivation of the AF specific KAF is FFS.

***	END CHANGES	***