- 3GPP TSG-SA3 Meeting #98e 																		S3-200216

- e-meeting, 2 – 6 March 2020

- Source:	Huawei, Hisilicon

- Title:	AKMA: Key ID generation

- Document for:	Approval

Agenda Item:	3.9

# 1	Decision/action requested

This contribution proposes the key ID generation. SA3 is suggested to approve this contribution.

# 2	References

NA

# 3	Rationale

It is proposed that the AKMA Key ID and AKMA key are generated separately by the UE and the AUSF during the primary authentication, in order to avoid the impact on the primary authentication.

On the other hand, the KAKMA key identifier value is proposed to reuse the format of NAI by taking the base64 encoded RAND value from the primary authentication, and the MCC, MNC and Routing ID, i.e. base64encode(RAND)@MCC.MNC.RI.3gppnetwork.org.

Here, the RAND is a randomly parameter used to identify the UE. MCC, MNC and routing ID are used to discovery the AUSF.

Compared with the AKMA key ID generated from the Kausf as illustrated in the TR 33.835, using RAND would be much easily to generate the AKMA key ID.

Therefore, the first part of the following editor’s note will be removed, i.e.

Editor’s Note: Format and derivation of KAKMA key identifier and its association with UE identifier is FFS. Whether the key identifier is generated during the primary authentication or as needed, is FFS.

# 4	Detailed proposal

It is proposed to approve the following change.

***	START OF THE 1st CHANGE	***

# 6	AKMA Procedures

## 6.1	Deriving AKMA key during UE registration

There is no separate authentication of the UE to support AKMA functionality. Instead, it reuses the 5G primary authentication procedure executed during the UE Registration to authenticate the UE. A successful 5G primary authentication results in KAUSF being stored at the AUSF and the UE.

Figure 6.1-1 Deriving AKMA root key during UE registration

The UE and the AUSF shall generate the AKMA Anchor Key (KAKMA) and the associated key Identifier from KAUSF as part of the UE Registration procedure.

The KAKMA key identifier identifies the KAKMA key of the UE from which other AKMA keys are derived.

Editor’s Note: Derivation of KAKMA is FFS.

Editor’s Note: Format and derivation of KAKMA key identifier and its association with UE identifier is FFS. Whether the key identifier is generated during the primary authentication or as needed, is FFS.

The KAKMA key identifier shall be NAI

Since AKMA keys are based on KAUSF from primary authentication run, the AKMA keys can only be refreshed by running a fresh primary authentication.

***	END OF THE 1st CHANGE	***