3GPP TSG-SA WG3 Meeting #98e 	S3-200228

- e-meeting, 2 – 6 March 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | DraftCR | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Address the ENs on Security handling in User Plane CIoT 5GS Optimisation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  |  | Date: |  | 2020-3-2 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are five Editor’s Notes which can be easily addressed in clause 6.x.2: EN1: “This clause depends on SA3’s progress in TR 33.861” Since the text is captured from TR 33.861, this EN can be deleted. EN2: “The sub-clauses needs to be revisited when RAN 2 has finalized the work on the RRC Connection Suspend and Resume in 5GS CIoT” According to RAN2’s living CR R2-1916567 and R2-1916566, RRC Connection Suspend and Resume procedure have been specified, the contribution made some clarification accordingly. EN3 and EN4: “It’s FFS in RAN2 whether Resume ID or I-RNTI will be used in 5GS.” According to RAN2’s living CR R2-1916567, I-RNTI is used in 5GS, the contribution modifies the IE accordingly. EN5: “The length of the ShortResumeMAC-I is FFS.” According to RAN2’s living CR R2-1916567, there is an EN in clause 7.3y.4 for PUR handling: “FFS whether the (ng-)eNB authenticate the UE using the short resume MAC-I when receiving RRCConnectionResumeRequest message”. Since RAN2 is discussing the length value and there is no text to describe the length of resume MAC-I in current TS, the EN can be deleted, and the final decision can be made in RAN2. |  |  |  |  |  |  |  |  |
| qu |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete ENs and make some clarification according to RAN2 progress. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not complete for Security handling in User Plane CIoT 5GS Optimization. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x.2, 6.x.2.2, 6.x.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

- 6.x.2	Security handling in User Plane CIoT 5GS Optimization

Editor note: EDT is FFS until RAN2 has finalized the work on EDT.

********** END OF 1st CHANGE **********

********** START OF 2nd CHANGE **********

#### 6.x.2.2			Connection Suspend

When the ng-eNB initiates the Connection Suspend procedure, it sends N2 Suspend Request message to the AMF.  Upon reception of the N2 Suspend Request message, the AMF shall check its local policy. If the local policy indicates that a new NH derivation is needed, the AMF shall increase its locally kept NCC value by one and compute a fresh NH from its stored data using the function defined in Annex A.10. The AMF shall store that fresh {NH, NCC} pair and send it to the ng-eNB in the N2 Suspend Response message.

Upon receipt of the N2 Suspend Response message from the AMF and if the message includes a {NH, NCC} pair, the ng-eNB shall store the fresh {NH, NCC} pair in the N2 Suspend Response message and remove any existing unused stored {NH, NCC} pairs.

The ng-eNB shall send to the UE an RRC Release with releaseCause set to rrc-suspend message that is ciphered and integrity protected in PDCP layer using current AS security context. The ng-eNB shall include a fresh I-RNTI, and an NCC in that RRC Release message. The I-RNTI is used for context identification, and the UE ID part of the I-RNTI assigned by the ng-eNB shall be different in consecutive suspends of the same UE. This is to avoid tracking of UEs based on the I-RNTI. If the ng-eNB has a fresh and unused pair of {NCC, NH}, the ng-eNB shall include the NCC in the RRC Release message. Otherwise, the ng-eNB shall include the same NCC associated with the current KgNB in the RRC Release message. The NCC is used for AS security.

The ng-eNB shall delete the current AS keys KRRCenc, KUPenc (if available), and KUPint (if available) after sending the RRC Release message to the UE, but shall keep the current AS key KRRCint. If the sent NCC value is fresh and belongs to an unused pair of {NCC, NH}, the ng-eNB shall save the pair of {NCC, NH} in the current UE AS security context and shall delete the current AS key KgNB. If the sent NCC value is equal to the NCC value associated with the current KgNB, the ng-eNB shall keep the current AS key KgNB and NCC. The ng-eNB shall store the sent I-RNTI together with the current UE context including the remainder of the AS security context.

Upon receiving the RRC Release with releaseCause set to rrc-suspend message from the ng-eNB, the UE shall decrypt the RRC Release message using the KUPenc key and verify that the integrity of the received the RRC Release message is correct by checking the PDCP MAC-I. If this verification is successful, then the UE shall take the received NCC value and save it as stored NCC with the current UE context.

********** END OF 2nd CHANGE **********

********** START OF 3rd CHANGE **********

#### 6.x.2.3	Connection Resume in CM-IDLE with Suspend to a new ng-eNB

When the UE using user plane CIoT 5GS Optimization decides to resume the RRC connection in CM-IDLE with suspend, the UE sends the RRC Resume Request message on SRB0 (i.e. it is not integrity protected). The UE shall include I-RNTI and a ShortResumeMAC-I in RRC Resume Request message. The I-RNTI is used for context identification and its value shall be the same as the I-RNTI that the UE had received from the source ng-eNB in the RRC Release with releaseCause set to rrc-suspend message in the source cell. The ShortResumeMAC-I is a 16-bit message authentication token, the UE shall calculate it using the integrity algorithm (EIA) in the stored AS security context, which was negotiated between the UE and the source ng-eNB and the current KRRCint with the following inputs:

- 	KEY			: it shall be set to current KRRCint;

-	BEARER		: all its bits shall be set to 1.

-	DIRECTION	: its bit shall be set to 1;

-	COUNT		: all its bits shall be set to 1;

-	MESSAGE	: it shall be set to the following inputs:

Editor's Note: the inputs to the MESSAGE will be updated after FS_5GFBS concludes on this topic.

The source PCI and source C-RNTI are associated with the cell where the UE was suspended. The target Cell-ID is the identity of the target cell where the UE sends the RRC Resume Request message.

For protection of all RRC messages except RRC Reject message following the sent RRC Resume Request message, the UE shall derive a KNG-RAN* using the target PCI, target EARFCN-DL and the KgNB/NH based on either a horizontal key derivation or a vertical key derivation as defined in clause 6.9.2.1.1 and Annex A.12. The UE shall further derive KRRCint, KRRCenc, KUPenc (optionally), and KUPint (optionally) from the newly derived KNG-RAN*. Then the UE resets all PDCP COUNTs to 0 and activates the new AS keys in PDCP layer.

When the target ng-eNB receives the RRC Resume Request message from the UE, the target ng-eNB extracts the I-RNTI from the RRC Resume Request message. The target ng-eNB contacts the source ng-eNB based on the information in the I-RNTI by sending an Xn-AP Retrieve UE Context Request message with the following included: the whole RRC Resume Request message (including resume cause, I-RNTI, spare IE, ShortResumeMAC-I) and target Cell-ID, in order to allow the source ng-eNB to validate the UE request and to retrieve the UE context including the UE 5G AS security context.

The source ng-eNB retrieves the stored UE context including the UE 5G AS security context from its database using the I-RNTI in the RRC Resume Request message. The source ng-eNB verifies the shortResumeMAC-I using the current KRRCint key stored in the retrieved UE 5G AS security context (calculating the shortResumeMAC-I in the same way as described above). If the verification of the shortResumeMAC-I is successful, then the source ng-eNB calculates KNG-RAN* using the target cell PCI, target EARFCN-DL and the KgNB/NH in the current UE 5G AS security context based on either a horizontal key derivation or a vertical key derivation according to whether the source ng-eNB has an unused pair of {NCC, NH} as described in Annex A.12. The source ng-eNB can obtain the target PCI and target EARFCN-DL from a cell configuration database by means of the target Cell-ID which was received from the target ng-eNB. Then the source ng-eNB shall respond with an Xn-AP Retrieve UE Context Response message to the target ng-eNB including the UE context that contains the UE 5G AS security context. The UE 5G AS security context sent to the target ng-eNB shall include the newly derived KNG-RAN*, the NCC associated to the KNG-RAN*, the UE EPS security capabilities, and the ciphering and integrity algorithms used by the UE with the source cell.

The target ng-eNB shall check if it supports the ciphering and integrity algorithms the UE used with the last source cell. If the target ng-eNB does not support the ciphering and integrity algorithms used in the last source cell or if the target ng-eNB prefers to use different algorithms than the source ng-eNB, then the target ng-eNB shall send an RRC Setup message on SRB0 to the UE in order to proceed with RRC connection establishment as if the UE was in RRC_IDLE (i.e., a fallback procedure).

If the target ng-eNB supports the ciphering and integrity algorithms used with the last source cell and these algorithms are the chosen algorithms by the target ng-eNB, the target ng-eNB shall derive new AS keys (RRC integrity key, RRC encryption key and UP keys) using the algorithms the UE used with the source cell and the received KNG-RAN*. The target ng-eNB shall reset all PDCP COUNTs to 0 and activate the new keys in PDCP layer. The target ng-eNB shall respond to the UE with an RRC Resume message on SRB1 which is integrity protected and ciphered in PDCP layer using the new RRC keys.

When the UE receives the RRC Resume message, the UE shall decrypt the message using the KRRCenc that was derived based on the newly derived KNG-RAN*. The UE shall also verify the RRC Resume message by verifying the PDCP MAC-I using the KRRCint that was derived from the newly derived KNG-RAN* If verification of the RRC Resume message is successful, the UE shall delete the current KRRCint key and the UE shall save the KRRCint, KRRCenc, KUPenc (optionally), and KUPint (optionally) from the newly derived KNG-RAN* as part of the UE current AS security context. In this case, the UE shall send the RRC Resume Complete message both integrity protected and ciphered to the target ng-eNB on SRB1 using the current KRRCint and KRRCenc.

If the UE receives RRC Reject message from the target ng-eNB in response to the RRC Resume Request message, the UE shall delete newly derived AS keys used for connection resumption attempt, including newly derived KNG-RAN*, newly derived RRC integrity key, RRC encryption key and UP keys, and keep the current KRRCint and the KgNB/NH in its current AS context. In that case, for the next resume to any target ng-eNB, the UE shall start with the same AS security context as it had when it was suspended originally, i.e., same KgNB/NH shall act as base key for derivation of new KNG-RAN*.

After a successful resume, the target ng-eNB shall perform Path Switch procedure with the AMF as is done in case of X2-handover. The AMF shall verify the UE security capability as described in the clause 6.7.3.1.

********** END OF 3rd CHANGE **********