3GPP TSG-SA WG3 Meeting #98e 	S3-200230

- e-meeting, 2 – 6 March 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0745 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE handling on CHO key derivation for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  |  | Date: |  | 2020-3-2 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN2 defined CHO for NR R16, the security handling on the network part is the same with current text. The main different security handling is on the UE part that the UE may delay derivation of AS key. So, the contribution proposes to add some details on the UE handling for CHO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add details for UE handling for CHO. Updated abbreviations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 6.9.2.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

********** START OF 2nd CHANGE **********

##### 6.9.2.3.4	UE handling

The UE behaviour is the same regardless if the handover is intra-gNB-CU, intra ng-eNB, Xn, or N2 with the exception that during intra-gNB-CU handover, the UE may retain the same key based on an indication from the gNB. The UE behaviour is also same in case of Conditional Handover, as specified in TS 38.300 [52], i.e., the UE shall use the parameters of the selected target cell (PCI, EARFCN-DL/ARFCN-DL) in KNG-RAN* derivations.

If the UE also receives a NASC (NAS Container) in the HO Command message, the UE shall update its NAS security context as follows:

NOTE 1: The purpose of this NASC could be compared to a NAS SMC message.

-	The UE shall verify the freshness of the downlink NAS COUNT in the NASC.

-	If the NASC indicates a new KAMF has been calculated (i.e., K_AMF_change_flag is one),

-	The UE shall compute the horizontally derived KAMF using the KAMF from the current 5G NAS security context identified by the ngKSI included in the NASC and the downlink NAS COUNT in the NASC, as specified in Annex A.13.

-	The UE shall assign the ngKSI included in the NASC to the ngKSI of the new derived KAMF. The UE shall further configure NAS security based on the horizontally derived KAMF and the selected NAS security algorithms in the NASC.

-	The UE shall further verify the NAS MAC in the NASC as described in Clause 6.9.2.3.3 and if the verification is successful, the UE shall further set the NAS COUNTs to zero.

-	If KAMF change is not indicated,

-	If the verification is successful, the UE shall configure the NAS security based on the parameters included in the NASC but shall not set the NAS COUNTs to zero.

-	The UE shall verify the NAS MAC in the NASC.

-	The UE shall further set the downlink NAS COUNT value of the currently active NAS security context to the received downlink NAS COUNT value in the NASC.

If verification of the NASC fails, the UE shall abort the handover procedure. Furthermore, the UE shall discard the new NAS security context if it was derived and continue to use the existing NAS and AS security contexts.

If keySetChangeIndicator in the HO command is true

-	If the HO Command message contained a NASC parameter with the K_AMF_change_flag set to one:

-	The UE shall use the horizontally derived KAMF and the NAS COUNT value of 232-1 in the derivation of the temporary KgNB. The UE shall further process this temporary key as described in subclause 6.9.4.4.

-	Else:

-	The UE handling related to key derivation shall be done as defined in clause 6.9.4.4.

Else

-	If the NCC value the UE received in the HO Command message from target ng-eNB/gNB via source ng-eNB/gNB is equal to the NCC value associated with the currently active KgNB/KeNB, the UE shall derive the KNG-RAN* from the currently active KgNB/KeNB and the target PCI and its frequency ARFCN-DL/EARFCN-DL using the function defined in Annex A.11 and A.12.

-	If the UE received an NCC value that was different from the NCC associated with the currently active KgNB/KeNB, the UE shall first synchronize the locally kept NH parameter by computing the function defined in Annex A.10 iteratively (and increasing the NCC value until it matches the NCC value received from the source ng-eNB/gNB via the HO command message. When the NCC values match, the UE shall compute the KNG-RAN* from the synchronized NH parameter and the target PCI and its frequency ARFCN-DL/EARFCN-DL using the function defined in Annex A.11 and A.12.

The UE shall use the KNG-RAN* as the KgNB when communicating with the target gNB and as the KeNB when communicating with the target ng-eNB.

********** END OF 2nd CHANGE **********