3GPP TSG-SA3 Meeting #98e 	S3-200257

- e-meeting, 2 – 6 March 2020													Revision of S3-20xxxx

- Source:	Huawei, Hisilicon

- Title:	AAnF checks the authorization information of AF

- Document for:	Approval

Agenda Item:	3.9

# 1	Decision/action requested

It is proposed to approve this contribution.

# 2	References

[1] 	NA

# 3	Rationale

The current text lacks the details that AAnF shall check whether the AF is authorized to get the keys. So a sentence is added to let AAnF check the authorization information.

# 4	Detailed proposal

***	BEGIN CHANGES	***

# 6	AKMA Procedures

## 6.2	Deriving AKMA Application key for a specific AF

Figure 6.2-1 AF Key generation from KAKMA

When the UE initiates communication with the AKMA AF, it shall include the derived AKMA key identifier in the message (cf. clause 6.1).

If the AApF does not have an active context associated with the key identifier, then the AF sends a request to AAnF with the key identifier to request application function specific AKMA keys for the UE.  The AF also includes its identity (AF Id) in the request. The AAnF shall check whether the AAnF can provide the service to the AF by checking the AF Id. If succeeds, the following procedures is excuted. Otherwise, the AAnF shall reject the procedure.

Editor’s Note: It is FFS how the AAnF knows whether it can provide the service to a specific AF

If the AAnF is in possession of the AF specific key (KAF), it responds to the AF with the KAF key. If not, the AAnF shall check if it has the UE specific KAKMA key identified by the AKMA key identifier.

If KAKMA is available in AAnF, it shall derive the AF specific AKMA key (KAF) from KAKMA and respond to the AF with KAF and lifetime.

If KAKMA is not available, the AAnF shall send a request to the AUSF to obtain the KAKMA key specific to the UE. It includes the AKMA key identifier in the request. The AUSF shall respond with the KAKMA key identified by the key identifier. The AAnF derives the AF specific key (KAF) from KAKMA and responds to the AF with KAF and lifetime.

Editor’s Note: Derivation of the AF specific KAF is FFS.

**	END OF CHANGES	***