3GPP TSG-SA WG3 Meeting #98e 	S3-200269Draft4 (merging S3-200402)

- e-meeting, 2 – 6 March 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | DRAFT_CR | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add clarifications to the living CR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei ?, Hisilicon?, NTT DOCOMO, Ericsson? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS |  |  |  |  |  |  | Date: |  | 2020-02-18 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The Editor’s note about enabling the use of Namespace patterns has to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifications to current text and Editor’sNotes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of information about enabling patterns in the list may lead to interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X.2, X.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGES ****

- 2	References

[x]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[y]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[z]	Bluetooth Special Interest Group: "Bluetooth Core Specification v4.2", December 2014.

**** NEXT CHANGES ****

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5G-AN	5G Access Network

NG-RAN	5G Radio Access Network

5G AV	5G Authentication Vector

5G HE AV	5G Home Environment Authentication Vector

5G SE AV	5G Serving Environment Authentication Vector

ABBA	Anti-Bidding down Between Architectures

AEAD	Authenticated Encryption with Associated Data

AES	Advanced Encryption Standard

AKA	Authentication and Key Agreement

AMF	Access and Mobility Management Function

AMF	Authentication Management Field

NOTE:	If necessary, the full word is spelled out to disambiguate the abbreviation.

ARPF	Authentication credential Repository and Processing Function

AUSF	Authentication Server Function

AUTN	AUthentication TokeN

AV	Authentication Vector

AV'	transformed Authentication Vector

Cell-ID	Cell Identity as used in TS 38.331 [22]

cIPX	consumer's IPX

CKSRVCC	Cipher Key for Single Radio Voice Continuity

CP	Control Plane

cSEPP	consumer's SEPP

CTR	Counter (mode)

CU	Central Unit

DN	Data Network

DNN	Data Network Name

DU	Distributed Unit

EAP	Extensible Authentication Protocol

EMSK	Extended Master Session Key

EPS	Evolved Packet System

gNB	NR Node B

GUTI	Globally Unique Temporary UE Identity

HRES	Hash RESponse

HXRES	Hash eXpected RESponse

IKE	Internet Key Exchange

IKSRVCC	Integrity Key for Single Radio Voice Continuity

IPX	IP exchange service

KSI	Key Set Identifier

KSISRVCC	Key Set Identifier for Single Radio Voice Continuity

LCS	LoCation Services

LI	Lawful Intercept

MN	Master Node

MR-DC	Multi-Radio Dual Connectivity

MSK	Master Session Key

N3IWF	Non-3GPP access InterWorking Function

NAI	Network Access Identifier

NAS	Non Access Stratum

NDS	Network Domain Security

NEA	Encryption Algorithm for 5G

NF	Network Function

NG	Next Generation

ng-eNB	Next Generation Evolved Node-B

ngKSI	Key Set Identifier in 5G

NIA	Integrity Algorithm for 5G

NR	New Radio

NR-DC	NR-NR Dual Connectivity

NSSAI	Network Slice Selection Assistance Information

PDN	Packet Data Network

PEI	Permanent Equipment Identifier

pIPX	producer's IPX

PRINS	PRotocol for N32 INterconnect Security

pSEPP	producer's SEPP

QoS	Quality of Service

RES	RESponse

SCG	Secondary Cell Group

SEAF	SEcurity Anchor Function

SEG	Security Gateway

SEPP	Security Edge Protection Proxy

SIDF	Subscription Identifier De-concealing Function

SMC	Security Mode Command

SMF	Session Management Function

SN	Secondary Node

SN Id	Serving Network Identifier

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

TLS	Transport Layer Security

UE	User Equipment

UEA	UMTS Encryption Algorithm

UDM	Unified Data Management

UDR	Unified Data Repository

UIA	UMTS Integrity Algorithm

ULR	Update Location Request

UP	User Plane

UPF	User Plane Function

USIM	Universal Subscriber Identity Module

XRES	eXpected RESponse

**** NEXT CHANGES ****

Annex X (normative):
Security and privacy in 5G system location services

- X.1	General

For security and privacy in 5GS LCS (5G System Location Services), the mechanisms defined in 3GPP TS 23.273 [x], 3GPP TS 38.305 [y], and the present document apply.

## X.2	WLAN positioning

Editror’s Note: Reference to proper specification for expressing patterns in SSID has to be added.

This clause applies only to UEs supporting the WLAN positioning.

For the positioning modes that require the UE to send WLAN measurements to the network (e.g., UE-assisted), the network should provide a list of the Basic Service Set Identifier (BSSID) and/or the Service Set Identifier (SSID) to UE.  The BSSIDs and SSIDs in this list may be exact values or patterns. The network shall provide ti the UE a distinctive string with a minimum length of 4 non-null characters, for example "optinLCS".

If the UE receives this list from the network, the UE shall send only those WLAN measurements to the network whose BSSIDs and/or SSIDs match the ones (values or patterns) indicated in the received list, and whose SSID contain the distinctive string.

NOTE 1:	It is up to the network deployment to ensure that the reference WLAN APs are physically secure and tamper resistant.

NOTE 2:	In the present document, it is up to the network implementation to ensure that potentially spoofed BSSIDs and SSIDs received from the UE are detected and excluded.

## X.3	Bluetooth positioning

Editror’s Note: Reference to proper specification for expressing patterns in Bluetooth device-name (Local Name) has to be added.

This clause applies only to UEs supporting the Bluetooth [z] positioning.

For the positioning modes that require the UE to send Bluetooth measurements to the network (e.g., UE-assisted), the network should provide a list of Bluetooth public-address (MAC address) and/or Bluetooth device-name (Local Name) to UE. The Bluetooth public-addresses and Bluetooth device-names in this list may be exact values or patterns. The network shall provide to the UE a distinctive string with a minimum length of 4 non-null characters, for example "optinLCS".

If the UE receives this list from the network, the UE shall send only those Bluetooth measurements to the network whose Bluetooth public-addresses and/or Bluetooth device-names match the ones (values or patterns) indicated in the received list, and whose Bluetooth device-names contain the distinctive string.

NOTE 1:	It is up to the network deployment to ensure that the reference Bluetooth devices are physically secure and tamper resistant.

NOTE 2:	In the present document, it is upto the network implementation to ensure that potentially spoofed Bluetooth public-addresses and Bluetooth device-names received from the UE are detected and excluded.

**** END OF CHANGES ****