3GPP TSG-SA3 Meeting #98e 	S3-200312

- e-meeting, 2 – 6 March 2020													Revision of S3-20xxxx

- Source:	Ericsson

- Title:	[Draft CR] EDT in User Plane CIoT 5GS Optimization

- Document for:	Approval

Agenda Item:	3.10

# 1	Decision/action requested

Approve the proposed clause to the draft CR in [1].

# 2	References

[1]	3GPP, S3-194483 "DraftCR – Living document for supporting 5G CIoT security"

[2]	3GPP TR 33.501 "Security architecture and procedures for 5G system".

# 3	Rationale

It is proposed to introduce MO EDT (Early Data Transmission) in the procedures for RRC Connection Resume/Suspend procedure in clause 6.x.2 for the NB-IoT radio access network connected to 5GC to TS 33.501 [3].

# 4	Detailed proposal

It is proposed to approve the pCR to the living document for 5G CIoT in S3-194483 [1].

**** START OF CHANGES ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[xx]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRAN); Overall description; Stage 2".

**** NEXT CHANGE ****

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

CIoT			Cellular Internet of Things

EDT			Early Data Transmission

MO-EDT			Mobile Originated Early Data Transmission

MT-EDT			Mobile Terminated Early Data Transmission

**** NEXT CHANGE ****

- 6.x.2	Security handling in User Plane CIoT 5GS Optimization

Editor’s Note: This clause depends on SA3’s progress in TR 33.861.

Editor note: The sub-clauses needs to be revisited when RAN 2 has finalized the work on the RRC Connection Suspend and Resume in 5GS CIoT.

6.x.2.1	General

The purpose of this procedure is to allow the ng-eNB to suspend an RRC connection to be resumed by the UE using User Plane CIoT 5GS Optimisation at a later time. The UE may resume the RRC connection in the same or different ng-eNB where the suspend took place. The UE and ng-eNB store the AS security context at suspend and reactivate the AS security context at resume.

The UE and the ng-eNB may also use EDT (Early Data Transmission) feature in this procedure which could comprise of MO-EDT and MT-EDT, as defined in TS 36.300 [xx] and TS 36.331 [21].

#### 6.x.2.2			Connection Suspend

When the ng-eNB initiates the Connection Suspend procedure, it sends N2 Suspend Request message to the AMF.  Upon reception of the N2 Suspend Request message, the AMF shall check its local policy. If the local policy indicates that a new NH derivation is needed, the AMF shall increase its locally kept NCC value by one and compute a fresh NH from its stored data using the function defined in Annex A.10. The AMF shall store that fresh {NH, NCC} pair and send it to the ng-eNB in the N2 Suspend Response message.

Upon receipt of the N2 Suspend Response message from the AMF and if the message includes a {NH, NCC} pair, the ng-eNB shall store the fresh {NH, NCC} pair in the N2 Suspend Response message and remove any existing unused stored {NH, NCC} pairs.

The ng-eNB shall send to the UE an RRC Release with suspendConfig message that is ciphered and integrity protected in PDCP layer using current AS security context. The ng-eNB shall include a fresh Resume ID, and an NCC in that RRC Release with suspendConfig message. The Resume ID is used for context identification, and the UE ID part of the Resume ID assigned by the ng-eNB shall be different in consecutive suspends of the same UE. This is to avoid tracking of UEs based on the Resume ID. If the ng-eNB has a fresh and unused pair of {NCC, NH}, the ng-eNB shall include the NCC in the RRC Release with suspendConfig message. Otherwise, the ng-eNB shall include the same NCC associated with the current KgNB in the RRC Release with suspendConfig message. The NCC is used for AS security.

When the EDT feature is used, and the ng-eNB includes EDT data in the RRC Release with suspendConfig message then the EDT data shall be ciphered in the PDCP layer using the KUPenc.

The ng-eNB shall delete the current AS keys KRRCenc, KUPenc (if available), and KUPint (if available) after sending the RRC Release with suspendConfig message to the UE, but shall keep the current AS key KRRCint. If the sent NCC value is fresh and belongs to an unused pair of {NCC, NH}, the ng-eNB shall save the pair of {NCC, NH} in the current UE AS security context and shall delete the current AS key KgNB. If the sent NCC value is equal to the NCC value associated with the current KgNB, the ng-eNB shall keep the current AS key KgNB and NCC. The ng-eNB shall store the sent Resume ID together with the current UE context including the remainder of the AS security context.

Upon receiving the RRC Release with suspendConfig message from the ng-eNB, the UE shall decrypt the RRC Release message using the KRRCenc key and verify that the integrity of the received RRC Release with suspendConfig message is correct by checking the PDCP MAC-I. If this verification is successful, then the UE shall take the received NCC value and save it as stored NCC with the current UE context.

When the EDT feature is used, and the UE receives EDT data in the RRC Release with suspendConfig message then the EDT data shall be deciphered in the PDCP layer using the KUPenc.

Editor note: It’s FFS in RAN2 whether Resume ID or I-RNTI will be used in 5GS.

#### 6.x.2.3	Connection Resume in CM-IDLE with Suspend to a new ng-eNB

When the UE using user plane CIoT 5GS Optimization decides to resume the RRC connection in CM-IDLE with suspend, the UE sends the RRC Resume Request message on SRB0 (i.e. it is not integrity protected). The UE shall include Resume ID and a ShortResumeMAC-I in RRC Resume Request message. The Resume ID is used for context identification and its value shall be the same as the Resume ID that the UE had received from the source ng-eNB in the RRC Release with suspendConfig message in the source cell. The ShortResumeMAC-I is a message authentication token, the UE shall calculate it using the integrity algorithm (EIA) in the stored AS security context, which was negotiated between the UE and the source ng-eNB and the current KRRCint with the following inputs:

- 	KEY			: it shall be set to current KRRCint;

-	BEARER		: all its bits shall be set to 1.

-	DIRECTION	: its bit shall be set to 1;

-	COUNT		: all its bits shall be set to 1;

-	MESSAGE	: it shall be set to the following inputs:

Editor's Note: the inputs to the MESSAGE will be updated after FS_5GFBS concludes on this topic.

The source PCI and source C-RNTI are associated with the cell where the UE was suspended. The target Cell-ID is the identity of the target cell where the UE sends the RRC Resume Request message.

Editor note: The length of the ShortResumeMAC-I is FFS.

For protection of all RRC messages except RRC Reject message following the sent RRC Resume Request message, the UE shall derive a KNG-RAN* using the target PCI, target EARFCN-DL and the KgNB/NH based on either a horizontal key derivation or a vertical key derivation as defined in clause 6.9.2.1.1 and Annex A.12. The UE shall further derive KRRCint, KRRCenc, KUPenc (optionally), and KUPint (optionally) from the newly derived KNG-RAN*. Then the UE resets all PDCP COUNTs to 0 and activates the new AS keys in PDCP layer.

If the MO EDT feature is used, then the UE shall use the newly derived KUPenc for ciphering of the EDT data in the PDCP layer in the RRC Resume Request message, and for deciphering of the EDT data (if included) in the PDCP layer in the RRC Release message or in the RRC Resume message.

When the target ng-eNB receives the RRC Resume Request message from the UE, the target ng-eNB extracts the Resume ID from the RRC Resume Request message. The target ng-eNB contacts the source ng-eNB based on the information in the Resume ID by sending an Xn-AP Retrieve UE Context Request message with the following included: the whole RRC Resume Request message (including resume cause, Resume ID, spare IE, ShortResumeMAC-I) and target Cell-ID, in order to allow the source ng-eNB to validate the UE request and to retrieve the UE context including the UE 5G AS security context.

The source ng-eNB retrieves the stored UE context including the UE 5G AS security context from its database using the Resume ID in the RRC Resume Request message. The source ng-eNB verifies the shortResumeMAC-I using the current KRRCint key stored in the retrieved UE 5G AS security context (calculating the shortResumeMAC-I in the same way as described above). If the verification of the shortResumeMAC-I is successful, then the source ng-eNB calculates KNG-RAN* using the target cell PCI, target EARFCN-DL and the KgNB/NH in the current UE 5G AS security context based on either a horizontal key derivation or a vertical key derivation according to whether the source ng-eNB has an unused pair of {NCC, NH} as described in Annex A.12. The source ng-eNB can obtain the target PCI and target EARFCN-DL from a cell configuration database by means of the target Cell-ID which was received from the target ng-eNB. Then the source ng-eNB shall respond with an Xn-AP Retrieve UE Context Response message to the target ng-eNB including the UE context that contains the UE 5G AS security context. The UE 5G AS security context sent to the target ng-eNB shall include the newly derived KNG-RAN*, the NCC associated to the KNG-RAN*, the UE EPS security capabilities, and the ciphering and integrity algorithms used by the UE with the source cell.

The target ng-eNB shall check if it supports the ciphering and integrity algorithms the UE used with the last source cell. If the target ng-eNB does not support the ciphering and integrity algorithms used in the last source cell or if the target ng-eNB prefers to use different algorithms than the source ng-eNB, then the target ng-eNB shall send an RRC Setup message on SRB0 to the UE in order to proceed with RRC connection establishment as if the UE was in RRC_IDLE (i.e., a fallback procedure).

If the target ng-eNB supports the ciphering and integrity algorithms used with the last source cell and these algorithms are the chosen algorithms by the target ng-eNB, the target ng-eNB shall derive new AS keys (RRC integrity key, RRC encryption key and UP keys) using the algorithms the UE used with the source cell and the received KNG-RAN*. The target ng-eNB shall reset all PDCP COUNTs to 0 and activate the new keys in PDCP layer. The target ng-eNB shall respond to the UE with an RRC Resume message on SRB1 which is integrity protected and ciphered in PDCP layer using the new RRC keys.

In addition, if the MO EDT feature is used, then the target ng-eNB shall use the newly derived KUPenc for deciphering of the EDT data in the PDCP layer in the RRC Resume Request message, and for ciphering of the EDT data (if included) in the PDCP layer in the RRC Release message or in the RRC Resume message.

When the UE receives the RRC Resume message, the UE shall decrypt the message using the KRRCenc that was derived based on the newly derived KNG-RAN*. The UE shall also verify the RRC Resume message by verifying the PDCP MAC-I using the KRRCint that was derived from the newly derived KNG-RAN* If verification of the RRC Resume message is successful, the UE shall delete the current KRRCint key and the UE shall save the KRRCint, KRRCenc, KUPenc (optionally), and KUPint (optionally) from the newly derived KNG-RAN* as part of the UE current AS security context. In this case, the UE shall send the RRC Resume Complete message both integrity protected and ciphered to the target ng-eNB on SRB1 using the current KRRCint and KRRCenc.

If the UE receives RRC Reject message from the target ng-eNB in response to the RRC Resume Request message, the UE shall delete newly derived AS keys used for connection resumption attempt, including newly derived KNG-RAN*, newly derived RRC integrity key, RRC encryption key and UP keys, and keep the current KRRCint and the KgNB/NH in its current AS context. In that case, for the next resume to any target ng-eNB, the UE shall start with the same AS security context as it had when it was suspended originally, i.e., same KgNB/NH shall act as base key for derivation of new KNG-RAN*.

After a successful resume, the target ng-eNB shall perform Path Switch procedure with the AMF as is done in case of X2-handover. The AMF shall verify the UE security capability as described in the clause 6.7.3.1.

Editor note: It’s FFS in RAN2 whether Resume ID or I-RNTI will be used in 5GS.

#### 6.x.2.4	Connection Resume in CM-IDLE with Suspend to the same ng-eNB

The target ng-eNB may be the same as the source ng-eNB in the description in the previous subclause. If so the single ng-eNB performs the roles of both the source and target ng-eNB. In particular, a new KNG-RAN* shall be derived even if the UE is resuming to the same cell from where it was suspended. However, there is the following difference.

After a successful resume, the ng-eNB shall send N2 Resume Request message to the AMF.  Upon reception of the N2 Resume Request message, the AMF shall check its local policy. If the local policy in the AMF indicates that a new NH derivation is needed, the AMF shall increase its locally kept NCC value by one and compute a fresh NH from its stored data using the function defined in Annex A.10. The AMF shall store that fresh pair and send it to the ng-eNB in the N2 Resume Response message.

Upon receipt of the N2 Resume Response message from the AMF and if the message includes a {NH, NCC} pair, the ng-eNB shall store the {NH, NCC} pair in the N2 Resume Response message and remove any existing unused stored {NH, NCC} pairs. The {NH, NCC} pair may be used in the next suspend/resume or Xn handover procedures.

When the EDT feature is used, the single ng-eNB performs the roles of both the source and target ng-eNB as described in clause 6.x.2.3.

**** END OF CHANGES ****