3GPP TSG-SA3 Meeting #98e 	S3-200317-r1

- e-meeting, 2 – 6 March 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.310 | CR | 0104 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IKEv2 profile update 33.310 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CryptPr |  |  |  |  |  |  | Date: |  | 2020-02-03 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The 3GPP security protocol profiles need to be regularly updated to stay up to date and remain secure. - IETF and other organisations are working on phasing out PKCS#1v1.5 padding. Examples: Public Key Cryptography Standard (PKCS) #1: RSA Cryptography Specifications Version 2.2 (also published in RFC 8017) states that “RSASSA-PKCS1-v1_5 is included only for compatibility with existing applications” https://tools.ietf.org/html/rfc8017 NIST refers to RFC 8017 for the specification of PKCS#1 with additional constraints. https://nvlpubs.nist.gov/nistpubs/FIPS/NIST.FIPS.186-5-draft.pdf IETF has completely removed PKCS#1v1.5 padding in TLS 1.3 (RFC 8466) except for use in legacy certificates. https://tools.ietf.org/html/rfc8446 ENISA writes about PKCS#1v1.5 padding in signatures that “we do not propose that it is used anywhere but in legacy systems”. https://www.enisa.europa.eu/publications/algorithms-key-size-and-parameters-report-2014 BSI states that “Usage of the older PKCS#1v1.5 paddings is not recommended”. BSI does not allow authentication method 1 to be used in IKEv2 at all. For TLS BSI requires that signatures with PKCS#1v1.5 padding is phased out no later than 2026. https://www.bsi.bund.de/EN/Publications/TechnicalGuidelines/tr02102/index_htm.html Critical infrastructure like 3GPP should start to phase out PKCS#1v1.5 padding. ECDSA signature are already mandatory to support in X.509, IKEv2 and TLS. RSA certificates with public key algorithm rsaEncryption can be used with authentication method 14 and RSASSA-PSS. In the future, 3GPP will likely introduce PQC signature algorithms. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - PKCS#1v1.5 padding is not recommended. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Critical 3GPP systems might support weak options and not fulfill minimum requirements and best practices leading to weaknesses. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***	BEGIN CHANGES	***

### 6.2.1b	IKEv2 profile

The following requirements on certificate based IKEv2 authentication in addition to those specified in NDS/IP [1] shall be applied:

For the IKE_INIT_SA and IKE_AUTH exchanges:

-	Following algorithms shall be supported:

-	Authentication: Method 1 - RSA Digital Signature [42];

-	Implementations shall support signatures that use SHA-256, should support signatures that use SHA-384, and shall not support signatures that use SHA-1. Implementations should use SHA-256 as the default hash function when generating signatures.

-	Usage of Method 1 is not recommended as it uses PKCS#1v1.5 padding.

-	Hash Algorithm Notification [43]

-	Implementations shall support SHA2-256, should support SHA2-384, and shall not support SHA1.

-	Authentication: Method 14 - Digital Signature [43].

-	Implementations shall support ecdsa-with-sha256 and should support ecdsa-with-sha384, and should support RSASSA-PSS with SHA-256. Implementations shall not support sha1WithRSAEncryption, dsa-with-sha1, ecdsa-with-sha1, RSASSA-PSS with Empty Parameters, and RSASSA-PSS with Default Parameters.

-	The identity of the CERT payload (including the end entity certificate) shall be used for policy checks;

-	Initiating/responding end entities are required to send certificate requests in the IKE_INIT_SA exchange for the responder and in the IKE_AUTH exchange for the initiator;

-	Cross-certificates shall not be sent by the peer end entity as they are pre-configured in the end entity;

-	The certificates in the certificate payload shall be encoded as type 4 (X.509 Certificate – Signature);

-	An end entity shall rekey the IKE SA when any used end entity certificate expires.

NOTE 2:	Depending on the availability of DNS between peer end entities, the following rule is applied:

-	subjectAltName and IKEv2 policy should both contain IP address (in case DNS is not available);

-	subjectAltName and IKEv2 policy should both contain FQDN (in case DNS is available).

***	END OF CHANGES	***