3GPP TSG-SA3 Meeting #98e 	S3-200331-r1

- e-meeting, 2 – 6 March 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.310 | CR | 0105 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Certificate and CRL profile update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CryptPr |  |  |  |  |  |  | Date: |  | 2020-02-03 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The 3GPP security protocol profiles needs to be regularlarly updated to stay up to date and remain secure. - RSA key lengths less than 2048 have been forbidden since Rel-14, but a note stating that key lengths less than 2048 is likely to be forbidden in a future release was forgotten and therefore still remains in clause 6.1.1. - After the requirements for all RSA certificates have been increased, the bullet regarding RSA CA certificates does not add anything. - CRL profile is missing minimum key lengths. CRL profile states “shall support RSA-4096” while Certificate profile states “should support RSA-4096”. - Certificates with Signature Algorithm RSAEncryption always uses PKCS#1v1.5 padding (note that certificates with public key algorithm rsaEncryption can be used with any RSA algorithm). IETF and other organisations is working on phasing out PKCS#1v1.5 padding. Examples: Public Key Cryptography Standard (PKCS) #1: RSA Cryptography Specifications Version 2.2 (also published in RFC 8017) states that “RSASSA-PKCS1-v1_5 is included only for compatibility with existing applications” https://tools.ietf.org/html/rfc8017 NIST refers to RFC 8017 for the specification of PKCS#1 and with additional constraints. https://nvlpubs.nist.gov/nistpubs/FIPS/NIST.FIPS.186-5-draft.pdf IETF has completely removed PKCS#1v1.5 padding in TLS 1.3 (RFC 8466) except for use in legacy certificates. https://tools.ietf.org/html/rfc8446 ENISA writes about PKCS#1v1.5 padding in signatures that “we do not propose that it is used anywhere but in legacy systems”. https://www.enisa.europa.eu/publications/algorithms-key-size-and-parameters-report-2014 BSI states that “Usage of the older PKCS#1v1.5 paddings is not recommended”. BSI requires that signatures with PKCS#1v1.5 padding is phased out no later than 2026. https://www.bsi.bund.de/EN/Publications/TechnicalGuidelines/tr02102/index_htm.html Critical infrastructure like 3GPP should start to phase out PKCS#1v1.5 padding, otherwise there is a risk that 3GPP systems are deployed with long term RSA certificates with PKCS#1v1.5 padding that cannot be replaced in time. ECDSA signatures are already mandatory to support and recommended for new certificates. Introducing id-RSASSA-PSS as a third certificates signature algorithms do not seem worthwhile at this point. - It is time to start planning deprecation of RSA-2048, otherwise there is a risk that 3GPP systems are deployed with long term RSA-2048 certificates that cannot be replaced in time. Best practice will in some years be to not use RSA-2048. As examples, NIST (USA) and ANSSI (France) forbids use of RSA-2048 from 2031. BSI (Germany) forbids use of RSA-2048 and 2048-bit Diffie-Hellman already from 2023. Critical infrastructure like 3GPP should start to phase out use of RSA-2048. - SHA-1 has been forbidden in signatures since Rel-14, but an example using sha1WithRSAEncrypt still exists in Annex G. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The note is voided. - The bullet regarding RSA CA certificates is deleted. - Key length requirements in CRL and Certificate profiles are harmonised. - PKCS#1v1.5 padding is not recommended. - Note that PKCS#1v1.5 paddign and RSA-2048 will likely be forbidden in a future release. - The example with SHA-1 is deleted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The note and the example are conflicting with the actual specification and may confuse implementors or security researchers analysing 3GPP specifications, leding to implementation weaknesses. Risk that 3GPP systems are deployed with long term RSA-2048 or PKCS #1 v1.5 certificates that cannot be replaced in time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1, 6.1.a, Annex G |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***	BEGIN CHANGES	***

### 6.1.1	Common rules to all certificates

-	Version 3 certificate according to RFC5280 [14].

-	Hash algorithm for use before signing certificate: SHA-256 shall be supported, SHA-384 should be supported, MD5, MD2, and SHA-1 shall not be supported.

NOTE 1:	Void.

-	Signature algorithm: RSAEncryption and ecdsa shall be supported. RSAEncryption is not recommended as it uses PKCS#1v1.5 padding.

-	Public key algorithm: rsaEncryption and id-ecPublicKey shall be supported.

-	Parameters: For ecdsa and id-ecPublicKey, secp256r1 shall be supported.

-	ECDSA is recommended for newly created certificates.

-	For RSA certificates: The public key length shall be at least 2048-bit. A public key length of at least 4096-bit shall be supported. Public key lengths of less than 2048-bit shall not be supported. PKCS#1v1.5 padding and key lengths less than 3072-bits should not be used in certificates that expire after 2030.

-	For ECDSA certificates: The public key length shall be at least 255-bit. A public key length of at least 384-bit shall be supported. Public key lengths of less than 255-bit shall not be supported.

NOTE 2:	Void.

NOTE X:	In practice, certificates often have a long lifetime, for example about ten years. The use of RSA with PKCS#1v1.5 padding and key lengths less than 3072-bits is planned to be prohibited by several organisations no later than 2030.

-	The security level of the public key used to sign the certificate shall be at least the same as the public keys in the certificate.

-	Subject and issuer name format.

-	(C=<country>), O=<Organization Name>, CN=<Some distinguishing name>. Organization and CN shall be in UTF8 format. Note that C is optional element.

or

-	cn=<hostname>, (ou=<servers>), dc=<domain>, dc=<domain>. Note that ou is optional element.

-	CRLs as specified in subclause 6.1a shall be supported for certificate revocation verification.

-	Certificate extensions which are not mandated by this specification but which are mentioned within RFC5280 [14] are optional for implementation. If present, such optional extensions shall be marked as “non critical“.

NOTE 3:	The above requirement implies that an NE, SEG or TLS entity receiving such optional extensions marked as “critical” will react with an error because, according to the introduction to clause 6.1 of the present document, NEs, SEGs and TLS entities shall only accept compliant certificates.

***	NEXT CHANGE 	***

## 6.1a	CRL profile

-	Version 2 CRL according to RFC5280 [14].

-	Hash algorithm for use before signing CRL: SHA-256 shall be supported SHA-384 should be supported, MD5 MD2, and SHA-1 shall not be supported.

NOTE:	Void.

-	Signature algorithm: RSAEncryption and ecdsa shall be supported. RSAEncryption is not recommended as it uses PKCS#1v1.5 padding.

-	Parameters: For ecdsa, secp256r1 shall be supported, secp384r1 should be supported.

-	ECDSA is recommended for newly created CRLs.

-	The security level of the public key used to sign the CRL shall be at least the same as the public keys used to sign the revoked certificates.

-	For RSA: The key length shall be at least 2048-bit. A key length of at least 4096-bit shall be supported. Key lengths of less than 2048-bit shall not be supported.PPKCS#1v1.5 padding and key lengths less than 3072-bits should not be used in certificates  hat expire after 2030w

-	For ECDSA: The key length shall be at least 255-bit. A key length of at least 384-bit shall be supported. Key lengths of less than 255-bit shall not be supported.

NOTE X:	In practice, certificates often have a long lifetime, for example about ten years. The use of RSA with PKCS#1v1.5 padding and key lengths less than 3072-bits is planned to be prohibited by several organisations no later than 2030.

CRL retrieval with LDAPv3 [5] shall be supported as the primary method. HTTP may be used for checking the revocation status of TLS and NE certificates.

***	NEXT CHANGE 	***

Annex G (informative):
Example CMPv2 Message Flow for Initial Enrolment

The purpose of this annex is to provide an overview how the initial enrolment of a base station may be executed.

The message flow for an initial enrolment of a base station to the RA/CA is shown in Figure 8 below. The text below the figure gives a description of this message flow. Precondition for this message flow is that the base station contains the vendor provided private/public key pair and is pre-provisioned with the related base station certificate signed by a vendor CA. If there is a certificate chain up to the vendor root CA, also the intermediate certificates must be pre-provisioned to the base station. The RA/CA is configured with the root certificate of the vendor and its own certificate(s). The exchanged messages are protected by setting the PKIHeader fields “protection” and “protectionAlg”. Example of protectionAlg is set to the value {1 2 840 11359 1 1 11} (sha256With RSAEncrypt) when RSA and SHA-256 is used.

Figure 8: Example message flow for initial base station enrolment

1.	The base station discovers the RA/CA address.

2.	The base station generates the private/public key pair to be enrolled in the operator CA, if this is not pre-provisioned.

3.	The base station generates the Initialization Request (ir). The CertReqMsg inside ir specifies the requested certificate. If the suggested identity is known to the base station, it includes this in the subject field. To provide proof of possession the base station generates the signature for the POPOSigningKey field of the CertReqMsg using the private key related to the public key to be certified by the RA/CA. The base station signs the ir using the vendor provided private key, and includes the digital signature in the PKIMessage. Its own vendor signed certificate and any intermediate certificates are included in the extraCerts field of the PKIMessage carrying the ir.

4.	The base station sends the signed ir message to the RA/CA.

5.	The RA/CA verifies the digital signature on the ir message against the vendor root certificate using the certificate(s) sent by the base station. The RA/CA also verifies the proof of the possession of the private key for the requested certificate.

6.	The RA/CA generates the certificate for base station. If the suggested identity of the base station is not included in the ir message, the RA/CA determines the suggested identity of the base station, e.g. based on the vendor provided identity of the base station contained in the base station certificate.

NOTE: The procedures for determination of the base station identity used by the operator are not in scope of the present document. According to [4], the RA/CA may replace a suggested identity sent by the base station with another identity based on local information.

7.	The RA/CA generates an Initialization Response (ip) which includes the issued certificate and uses the same certReqId value as in the Initialization Request. The RA/CA signs the ip with the RA/CA private key (or the private key for signing CMP messages, if separate), and includes the signature, the RA/CA certificate(s) and the operator root certificate in the PKIMessage. The appropriate certificate chains for authenticating the RA/CA certificate(s) are included in the PKIMessage.

8.	The RA/CA sends the signed ip to the base station.

9.	If the operator root certificate is not pre-provisioned to the base station, the base station extracts the operator root certificate from the PKIMessage. The base station authenticates the PKIMessage using the RA/CA certificate and installs the base station certificate on success.

10.	The base station creates and signs the CertificateConfirm (certconf) message. The CertficateConfirm message uses the same certReqId value as in the Initialization Request.

11. The base station sends the PKIMessage that includes the signed CertificateConfirm to the RA/CA.

12.	 The RA/CA authenticates the PKI Message that includes the CertificateConfirm.

13.	The RA/CA creates and signs a Confirmation message (pkiconf).

14.	The RA/CA sends the signed PKIMessage including the pkiconf message to the base station.

15.	The base station authenticates the pkiconf message.

***	END OF CHANGES	***