3GPP TSG-SA WG3 Meeting #98e	S3-200337

- E-meeting, 2-6 March 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.401 | CR | 0690 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Key derivation for CHO(LTE R16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  |  | Date: |  | 2020-03-02 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN2 defined CHO for both LTE R16 and NR R16, however, the key derivation for CHO is not clear, this CR proposed to add AS key derivation procedure of CHO in LTE R16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update clause 7.2.8.4.1, 7.2.8.4.2 of TS 33.401 with normative text for key derivation in CHO. Updated abbreviations in clause 3.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 7.2.8.4.1, 7.2.8.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGES ****

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AES	Advanced Encryption Standard

AK	Anonymity Key

AKA	Authentication and Key Agreement

AMF	Authentication Management Field

AN	Access Network

AS	Access Stratum

AUTN	Authentication token

AV	Authentication Vector

ASME	Access Security Management Entity

Cell-ID	Cell Identity as used in TS 36.331 [21]

CHO	Conditional Handover

CK	Cipher Key

CKSN	Cipher Key Sequence Number

C-RNTI	Cell RNTI as used in TS 36.331 [21]

CRL	Certificate Revocation List

DeNB	Donor eNB

DoS	Denial of Service

DSCP	Differentiated Services Code Point

EARFCN-DL	E-UTRA Absolute Radio Frequency Channel Number-Down Link

ECM	EPS Connection Management

EDT	Early Data Transmission

EEA	EPS Encryption Algorithm

EIA	EPS Integrity Algorithm

eKSI	Key Set Identifier in E-UTRAN

EMM	EPS Mobility Management

eNB	Evolved Node-B

EPC	Evolved Packet Core

EPS	Evolved Packet System

EPS-AV	EPS authentication vector

E-UTRAN	Evolved UTRAN

gNB	Next Generation Node-B

GERAN	GSM EDGE Radio Access Network

GUTI	Globally Unique Temporary Identity

HE	Home Environment

HFN	Hyper Frame Number

HO	Hand Over

HSS	Home Subscriber Server

IK	Integrity Key

IKE	Internet Key Exchange

IMEI	International Mobile Station Equipment Identity

IMEISV	International Mobile Station Equipment Identity and Software Version number

IMSI	International Mobile Subscriber Identity

IOPS	Isolated E-UTRAN Operation for Public Safety

IRAT	Inter-Radio Access Technology

ISR	Idle Mode Signaling Reduction

KDF	Key Derivation Function

KSI	Key Set Identifier

LWIP	LTE WLAN RAN Level Integration using IPSec

LSB	Least Significant Bit

LSM	Limited Service Mode

LWA	LTE-WLAN Aggregation

MAC-I	Message Authentication Code for Integrity (terminology of TS36.323 [12])

MACT	Message Authentication Code T used in AES CMAC calculation

MeNB	Master eNB

ME	Mobile Equipment

MME	Mobility Management Entity

MME-RN	MME serving the RN

MS	Mobile Station

MSC	Mobile Switching Center

MSIN	Mobile Station Identification Number

NAS	Non Access Stratum

NAS-MAC	Message Authentication Code for NAS for Integrity (called MAC in TS24.301 [9])

NASDVM	Non Access Stratum - Data via MME

NCC	Next hop Chaining Counter

NH	Next Hop

OCSP	Online Certificate Status Protocol

OTA	Over-The-Air (update of UICCs)

PCI	Physical Cell Identity as used in TS 36.331 [21]

PDCP	Packet Data Convergence Protocol

PLMN	Public Land Mobile Network

PRNG	Pseudo Random Number Generator

PSK	Pre-shared Key

P-TMSI	Packet- Temporary Mobile Subscriber Identity

RAND	RANDom number

RAU	Routing Area Update

RN	Relay Node

RRC	Radio Resource Control

SCG	Secondary Cell Group

SEG	Security Gateway

SGSN	Serving GPRS Support Node

SIM	Subscriber Identity Module

SMC	Security Mode Command

SeNB	Secondary eNB

SgNB	Secondary gNB

SN	Serving Network

SN id	Serving Network identity

SQN	Sequence Number

SRB	Source Route Bridge

SRVCC	Single Radio Voice Call Continuity

S-TMSI	S-Temporary Mobile Subscriber Identity

TAI	Tracking Area Identity

TAU	Tracking Area Update

UE	User Equipment

UEA	UMTS Encryption Algorithm

UIA	UMTS Integrity Algorithm

UICC	Universal Integrated Circuit Card

UMTS	Universal Mobile Telecommunication System

UP	User Plane

USIM	Universal Subscriber Identity Module

UTRAN	Universal Terrestrial Radio Access Network

WT	WLAN Termination as used in TS 36.300 [30]

XRES	Expected Response

****** END FIRST CHANGE ******

##### 7.2.8.4.1	Intra-eNB Handover

When the eNB decides to perform an intra-eNB handover it shall derive KeNB* as in Annex A.5 using target PCI, its frequency EARFCN-DL, and either NH or the current KeNB depending on the following criteria: the eNB shall use the NH for deriving KeNB* if an unused {NH, NCC} pair is available in the eNB (this is referred to as a vertical key derivation), otherwise if no unused {NH, NCC} pair is available in the eNB, the eNB shall derive KeNB* from the current KeNB (this is referred to as a horizontal key derivation).

The eNB shall use the KeNB* as the KeNB after handover. The eNB shall send the NCC used for KeNB* derivation to UE in HO Command message.

This clause is also applicable for Conditional Hand Over (CHO).

NOTE: The key derivation mechanism described in this clause is also applicable to CHO defined in TS36.300[30]

****** END SECOND CHANGE ******

##### 7.2.8.4.2	X2-handover

As in intra-eNB handovers, for X2 handovers the source eNB shall perform a vertical key derivation in case it has an unused {NH, NCC} pair. The source eNB shall first compute KeNB* from target PCI, its frequency EARFCN-DL, and either from currently active KeNB in case of horizontal key derivation or from the NH in case of vertical key derivation as described in Annex A.5.

Next the source eNB shall forward the {KeNB*, NCC} pair to the target eNB. The target eNB shall use the received KeNB* directly as KeNB to be used with the UE. The target eNB shall associate the NCC value received from source eNB with the KeNB. The target eNB shall include the received NCC into the prepared HO Command message, which is sent back to the source eNB in a transparent container and forwarded to the UE by source eNB.

When the target eNB has completed the handover signaling with the UE, it shall send a S1 PATH SWITCH REQUEST to the MME. Upon reception of the S1 PATH SWITCH REQUEST, the MME shall increase its locally kept NCC value by one and compute a new fresh NH by using the KASME and its locally kept NH value as input to the function defined in Annex A.4. The MME shall then send the newly computed {NH, NCC} pair to the target eNB in the S1 PATH SWITCH REQUEST ACKNOWLEDGE message. The target eNB shall store the received {NH, NCC} pair for further handovers and remove other existing unused stored {NH, NCC} pairs if any.

NOTE 1:	Because the path switch message is transmitted after the radio link handover, it can only be used to provide keying material for the next handover procedure and target eNB. Thus, for X2-handovers key separation happens only after two hops because the source eNB knows the target eNB keys. The target eNB can immediately initiate an intra-cell handover to take the new NH into use once the new NH has arrived in the S1 PATH SWITCH REQUEST ACKNOWLEDGE.

This clause is also applicable for Conditional Hand Over (CHO).

NOTE 2: The key derivation mechanism described in this clause is also applicable to CHO defined in TS36.300[30]

**** END OF CHANGES ****