3GPP TSG-SA3 Meeting #98e 	S3-200347r2

- e-meeting, 2 – 6 March 2020															Revision of S3-20xxxx

- Source:	Qualcomm Incorporated, Interdigital

- Title:	Proposed text for security establishment clause of NR PC5 unicast security

- Document for:	Approval

Agenda Item:	3.15

# 1	Decision/action requested

This contribution proposes text for the security establishment clause of the NR PC5 unicast security

# 2	References

[1]	TS 33.536v0.2.0

[2]	TS 33.303

# 3	Rationale

This contribution proposes text for the keying clause of the NR PC5 unicast security clause. The text is written to fit the skeleton proposed in S3-200344.

The text is based on the ProSe text [2] and the conclusions from the TR but includes at least the following changes:

- Security policy details (see S3-200343 for discussion on details of this)

- Renaming the KD, KD-sess, PEK and PIK are renamed to KNRP, KNRP-sess, NRPEK and NRPIK respectively where NRP is short for NR PC5.

This document should be reviewed in conjunction with S3-200345, S3-200346, S3-200348 and S3-200349. It was chosen to present this way as it felt easier to keep track of proposed revisions to smaller sets of changes rather than one large set of changes.

The green highlighted text shows the changes based on the security policy solution proposed in S3-200343.

# 4	Detailed proposal

It is proposed to include the below pCR in the TS [1].

**** START OF CHANGES ****

- 5.3.3	Procedures

5.3.3.1 	Securing the PC5 unicast bearer

##### 5.3.3.1.1	General

##### 5.3.3.1.2	Overview

- 5.3.3.1.3	Key establishment procedures

##### 5.3.3.1.4	Security establishment procedures

###### 5.3.3.1.4.1 	General

Clause 5.3.3.1.4.2 describes the security policy and how the UEs handle the policy. There are two different cases when an overall security context may be established; to set up a new connection and to re-key an ongoing connection. These cases are described in clauses 5.3.3.1.4.3 and 5.3.3.1.4.4 respectively. Clause 5.3.3.1.4.5 describes the establishment of security for a user plane bearer.

###### 5.3.3.1.4.2 	Security policy handling

As part of the provisioning process for NR PC5 Unicast (see clause 5.1.2.1 of TS 23.287 [2]), the UE shall be provisioned with the following:

The list of V2X services, e.g. PSIDs or ITS-AIDs of the V2X applications, with Geographical Area(s)  and their security policy which indicates the following:

•	Signalling integrity protection: ON/OFF

•	Signalling confidentiality protection: ON/PREFERRED/OFF

•	User plane integrity protection: ON/PREFERRED/OFF

•	User plane confidentiality protection: ON/PREFERRED/OFF

NOTE 1: No integrity protection on signalling traffic enables services that do not require security, e.g. emergency services.

The signalling integrity protection security policy being OFF means that the UE may accept the connection with no security but may try to establish security anyway. With the integrity protection security policy set to ON, the UE may only accept the connection if a non-NULL integrity algorithm is used for protection of the signalling traffic.

For the other cases, a setting of OFF means that the UE shall only use NULL confidentiality algorithm for that traffic or apply no integrity protection, while a ON setting means that the UE shall use a non-NULL algorithm. If the security policy is PREFERRED, then the UE may accept any algorithm for that particular protection. One use of PREFERRED is to enable a security policy to be changed without updating all UEs at once.

At initial connection, the UE include its signalling security policy in the Direct Communication Request message. The UE(s) responding to this take account of this when choosing the algorithms in the Direct Security Mode Command message. The initiating UE can reject the Direct Security Mode Command if the algorithm choice does not match its policy.

When adding a V2X service to an existing connection, the UE responding to the request shall reject the request if signalling security in use does not match the policy for the new application.

###### 5.3.3.1.4.3 	Security establishment during connection set-up

The clause describes how security is established during connection set-up. The signalling flow is shown in figure 5.3.3.1.4.3-1.

Figure 5.3.3.1.4.3-1: Security establishment at connection set-up

1.	UE_1 has sent a Direct Communication Request to UE_2. This message shall include Nonce_1 (for session key KNRP-sess generation), UE_1 security capabilities (the list of algorithms that UE_1 will accept for this connection), UE_1’s signalling security policy and the most significant 8-bits of the KNRP-sess ID. These bits shall be chosen such that UE_1 will be able to locally identify a security context that is created by this procedure. The message may also include a KNRP ID if the UE_1 has an exisiting KNRP for the UE that it trying to communicate with. The absence of the KNRP ID parameter indicates that UE_1 does not have a KNRP for UE_2. The message also contains Key_Est_Info (see subclause 5.3.3.1.3.2).

2.	UE_2 may initiate a Direct Auth and Key Establish procedure with UE_1. This is mandatory if the UE_2 does not have the KNRP and KNRP ID pair indicated in step 1, and signalling is needed to establish the keys for the particular use case.

3.	UE_2 shall send the Direct Security Mode Command message to UE_1. This message shall only contain the MSB and optionally Key_Est_Info of KNRP ID and optionally Key_Est_Info if a fresh KNRP is to be generated (see clause 5.3.3.1.3).  UE_2 shall include Nonce_2 to allow a session key to be calculated and the Chosen_algs parameter to indicate which security algorithms the UEs will use to protect the data in the message. The Chosen-algs may only indicate the use of the NULL integrity algorithm if UE_2’s signalling security policy has integrity as OFF. UE_2 shall also return the UE_1 security capabilities and UE_1’s signalling security policy to provide protection against bidding down attacks. UE_2 shall also includes the least significant 8-bits of KNRP-sess ID in the messages. These bits are chosen so that UE_2 will be able to locally identify a security context that is created by this procedure. UE_2 shall calculate KNRP-Sess from KNRP and both Nonce_1 and Nonce_2 (see Annex X.Y) and then derive the confidentiality and integrity keys based on the chosen algorithms (Annex X.Y). UE_2 shall integrity protect the Direct Security Mode Command before sending it to UE_1. UE_2 is then ready to receive both signalling and user plane traffic protected with the new security context. UE_2 shall form the KNRP-sess ID from the most significant bits it received in message 1 and least significant bits it sent in message 3.

4. 	On receiving the Direct Security Mode Command, UE_1 shall calculate KNRP-sess and the confidentiality and integrity keys in the same way as UE_2. UE_1 shall check that the returned UE_1 security capabilities and UE_1’s signalling security policy are the same as those it sent in step 1. UE_1 shall also check the integrity protection on the message. UE_1 shall only accept the NULL integrity algorithm if its security policy for signalling indicates that integrity protection is OFF. If both these checks pass, then UE_1 is ready to send and receive signalling and user planetraffic with the new security context. UE_1 shall send an integrity protected and confidentiality protected (with the chosen algorithm which may be the null algorithm) Direct Security Mode Complete message to UE_2. UE_1 shall form the KNRP-sess ID from the most significant bits it sent in message 1 and least significant bits it received in message 3.

5.	UE_2 checks the integrity protection on the received Direct Security Mode Complete.  If this passes, UE_2 is now ready to send user plane data and control signalling protected with the new security context. UE_2 deletes any old security context it has for UE_1.

###### 5.3.3.1.4.4 	Security establishment during re-keying

By rekeying, the UEs ensure fresh session keys KNRP-sess are used. Optionally the rekeying can also enforce refresh of KNRP. Either UE may rekey the connection at any time. This shall be done before the counter for a PDCP bearer repeats with the current keys. A rekeying operation shall refresh the KNRP-sess  and NRPEK and NRPIK, and may refresh KNRP. A rekeying operation follows the flows given in figure 5.3.3.1.4.4-1.

Figure 5.3.3.1.4.4-1: Security establishment during rekeying

1.	UE_1 sends a Direct Rekey Request to UE_2. This message shall include Nonce_1 (for session key generation), UE_1 security capabilities (the list of algorithms that UE_1 will accept for this connection) and the most significant 8-bits of the KNRP-sess ID. These bits are chosen such that UE_1 will be able to locally identify a security context that is created by this procedure. The message may also include a Re-auth Flag, if UE_1 wants to rekey KNRP. The message also contains Key_Est_Info (see subclause 6.5.4).

2.	UE_2 may initiate a Direct Auth Key Establish procedure with UE_1. This is mandatory if  UE_1 included the Re-auth Flag and signalling is needed to establish KNRP.

3.	This step is the same as step 3 in 5.3.3.1.4.3 except that the chosen integrity algorithm shall only be NULL if the NULL integrity algorithm is currently in use and UE_1’s signalling security policy is not included in this message.

4.	This step is the same as step 4 in 5.3.3.1.4.3 except that UE_1 shall only accept the NULL integrity algorithm if the NULL integrity algorithm is currently in use and UE_1 does not check the returned siganlling security policy (as it is not sent in this case).

5.	This step is the same as step 5 in 5.3.3.1.4.3.

6. When UE_1 receives a message integrity protected with the new security context, it shall delete any old security context is has still stored for UE_2.

###### 5.3.3.1.4.5 	Security establishment for user plane bearers

At initial connection or adding a V2X service, the initiating UE includes its user plane security policy in the Direct Security Mode Complete or Link Modification request message respectively. The other UE sends its user plane security policy for the V2X service to the initiating UE in the Direct Communication Accept or Link Modification Accept procedure provided the policies are not incompatible, i.e. on UE’s policy ask for confidentiality/integrity to be OFF while the other UE’s policy asks for it to be ON. If the policies are incompatiable, then the other shall reject the connection setup or Link Modification Request with an appropriate error.

The UE initiating the establishment of a user plane bearer shall select an LCID whose associated value of Bearer for input to the security algorithms (see clauses 5.3.3.1.5.2 and 5.3.3.1.5.3) has not been used with the current keys, NRPEK and NRPIK. If this is not possible the UE shall initiate a re-keying (see clause 5.3.3.1.4.3) before establishing the user plane bearer.

When establishing the user plane bearer the initiating UE shall indicate the configuration of confidentiality and integrity protection in the PC5-RRC message. The confidentiality and integrity protection algorithms are same as those selected for protecting the signalling bearers.

Both UEs shall ensure that user plane for each V2X service is only sent or received (e.g. dropped if received on a bearer with incorrect security) on user plane bearers with the necessary security.

**** END OF CHANGES ****