3GPP TSG-SA3 Meeting #98e 	S3-200352r1

- e-meeting, 2 – 6 March 2020															Revision of S3-20xxxx

- Source:	Qualcomm Incorporated, LG

- Title:	Proposed procedures for the privacy of NR PC5 unicast connections

- Document for:	Approval

Agenda Item:	3.15

# 1	Decision/action requested

This contribution proposes to capture procedures for the privacy of NR PC5 unicast connections

# 2	References

[1]	TS 33.536 v0.2.0

# 3	Rationale

This contribution proposes a pCR for the procedures for the privacy of NR PC5 unicast connections.

# 4	Detailed proposal

It is proposed to include the below pCR in the TS [1].

**** START OF CHANGES ****

- 5.3.3	Procedures

- 5.3.3.1 	Securing the PC5 unicast bearer

Editor’s note: These are FFS

- 5.3.3.2 	Identity privacy for the PC5 unicast bearer

- 5.3.3.2.1	General

The link identifier update procedure given in TS 23.287 [2] is used to provide privacy for the identities in the unicast link. These messages in this procedure are always sent protected and hence the new identities agreed by the UEs are only known to the involved UEs. A three-way message exchange procedure is required with this procedure since both UEs need to change their identifiers during the same procedure and to allow these new values to be acknowledged prior to them being used.

5.3.3.2.2	Procedures

Figure 5.3.3.2.2-1 shows the flows for changing the identities of the UEs involved in a unicast communication. The figure and the following steps only show the security parameters (KNRP ID and KNRP-sess ID) that are changed and the Layer-2 ID but not the other parameters described in TS 23.287 [2].

Figure 5.3.3.2.2-1: Link identifier update procedure

The procedures proceeds with the following steps and give the additional processing on top of that given in TS 23.287 [2].

0. UE_1 and UE_2 are communicating via a unicast and have established the security for the link.

1. UE_1 decides to change the identifiers and sends a Link Identifier Update Request message to UE_2 (see TS 23.287 [2]). UE_1 shall include the MSB of KNRP ID and the MSB of KNRP-sess ID. These bits shall be chosen so that they uniquely identify KNRP and KNRP-sess respectively at UE_1.

2. UE_2 shall choose LSB of KNRP ID and the LSB of KNRP-sess ID so that they uniquely identify KNRP and KNRP-sess respectively at UE_2. UE_2 shall form the new KNRP ID and KNRP-sess ID from the MSB received from UE_1 and the LSB that UE_2 chose. UE_2 shall replace the existing KNRP ID with the new one. UE_2 shall associate the new KNRP-sess ID with the updated Layer-2 IDs (see TS 23.287 [2]) and shall use this new KNRP-sess ID when it uses the updated Layer-2 IDs. UE_2 shall send the LSB of KNRP ID and the LSB of KNRP-sess ID to UE_1 along with the received MSB of KNRP ID and the MSB of KNRP-sess ID in the Link Identifier Update Response message. UE_1 shall check that the returned MSB of KNRP ID and the MSB of KNRP-sess ID are the same as it sent in step 1.

3. UE_1 shall form the new KNRP ID and KNRP-sess ID from the LSB received from UE_2 and the MSB chosen by UE_1 LSB (in step 1). UE_1 shall replace the existing KNRP ID with the new one. UE_1 shall associate the new KNRP-sess ID with the updated Layer-2 IDs (see TS 23.287 [2]) and shall use this new KNRP-sess ID when it uses the updated Layer-2 IDs. UE_1 shall send the Link Identifier Update Ack message to UE_2 including the LSB of KNRP ID and the LSB of KNRP-sess ID. UE_1 shall check that the returned LSB of KNRP ID and the MSB of KNRP-sess ID are the same as it sent in step 2.

**** END OF CHANGES ****