3GPP TSG-SA3 Meeting #99e 	S3-201021

# e-meeting, 11 – 15 March 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | - | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Token-based authorization using consumer-signed authorization codes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2020-04-30 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This draft-CR shows how consumer-signed authorization codes, as described in the discussion paper S3-201018, can be implemented in normative text. The changes in this contribution are on top of the approved draft-CR S3-200285. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.4.1.X (new), 13.4.1.X.1 (new), 13.4.1.X.2 (new), 13.4.1.X.Y (new), 13.4.1.X.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGES ****

#### 13.4.1.X	Service access authorization in indirect communication scenarios

##### 13.4.1.X.1	General

Editor's Note: General introduction to be added.

##### 13.4.1.X.2	Consumer-signed authorization codes

Consumer-signed authorization codes are applicable in the following cases:

-	Scenario D, where the SCP requests token on behalf of the consumer, and

-	Scenario C without TLS between consumer and NRF. The consumer sends the access token request to the NRF via the SCP. From the NRF's point of view, it looks like the SCP sent the access token request on behalf of the consumer.

Authorization codes are generated by the NF consumer in a pre-step. In the main step, the authorization code is used in the actual access token request.

Pre-step:

As pre-condition, the consumer needs to have a certificate and private/public key pair that can be used for signing JWSs.

The consumer generates an authorization code as follows:

An authorization code is a JWT (JSON web token, RFC 7519 [44]) used as payload for a JWS (JSON Web Signature, RFC 7515 [45]). It also is an assertion, following the RFCs 7521 [x] and 7523 [y]. The fields according to RFCs 7521, 7523 and 7515 are determined in the following way:

| Field according to RFC 7521 (assertion framework) and RFC 7523 (OAuth JWT Assertion Profiles) | Value |
| --- | --- |
| Issuer ("iss") | NF consumer |
| Subject ("sub") | NF consumer |
| Audience ("aud") | NRF |
| Issued At ("iat") | The time at which the assertion was issued. |
| Expiry time ("exp") | The time at which the assertion expires. The authorization code should be rather short-lived, as it is only used for one access token request. |
| Scope ("scope") | The consumer may use the scope field to restrict the scope for which the SCP may request access tokens on behalf of the consumer. |
| Grant type ("grant_type") | "urn:ietf:params:oauth:grant-type:jwt-bearer", according to RFC 7523, section 2.1 |

| Field according to RFC 7515 (JSON Web Signature (JWS)) | Value |
| --- | --- |
| "x5u" (X.509 URL) or "x5c" (X.509 Certificate Chain) | The consumer may use this URI to refer to a resource for the X.509 public key certificate or certificate chain. |

As a JWS, the authorization code also needs to follow the 3GPP JWS profile in TS 33.210 [3], clause 6.3.3.

The NF consumer signs the authorization code with its private key.

##### 13.4.1.X.Y		Authorization for indirect communication without delegated discovery procedure

Figure 13.4.1.X.Y-1: Authorization and service invocation procedure

Discovery of the NF Service Producer:

0.	Optionally, the NF Service Consumer may discover the NF Service Producer before requesting authorization to invoke the services of the NF Service Producer.

NF Service Consumer authorization:

The NF Service Consumer and NRF perform the "Access token request before service access" procedure as described in clause 13.4.1.1. If the NF Service Consumer has already discovered the NF Service Producer (Step 0), it can also perform the "Access token request for a specific NF Producer/NF Producer service instance" procedure as described in clause 13.4.1.1.

If there is no mutual TLS between NF Service Consumer and NRF, the NF Service Consumer shall include a valid authorization code. The SCP shall forward the authorization code to the NRF. The NRF shall check that the authorization code is valid, and only issue the access token if the authorization code is valid.

Service request:

The NF Service Consumer, SCP, NRF and NF Service Producer perform the procedure "Indirect Communication without delegated discovery Procedure" described in clause 4.17.11 of TS 23.502 [8]. The following steps describe how the access token received in steps 1 and 2 is used in this procedure.

3.	If no cached data is available, the NF Service Consumer discovers the NF Service Producer via the SCP.

4.	The NF Service Consumer sends a service request for the specific service to the SCP. The service request includes the access token for the NF Service Producer providing the service as received in step 5.

5.	The SCP selects a NF Service Producer instance, performs the API root modifications and forwards the received request to the selected NF Service Producer instance. The request contains the token as received in 6. and valid for the NF Service Consumer.

6.	To authorize the access the NF Service Producer validates the token by verifying the signature and checking if the requested service is part of the token's scope. If the checks are ok the NF Service Producer processes the request and provides a response.

7.	The SCP performs revers API root modifications and forwards the response.

##### 13.4.1.X.Z		Authorization for indirect communication with delegated discovery procedure

Service Request, discovery and access token request: These steps shall be performed over mutually authenticated TLS connections between NF Service Consumer and SCP, SCP and NRF, and SCP and NF Service Producer.

NF consumer

NRF

SCP

NF producer

1. Service request (authorization code)

2. Discovery

3. Nnrf_AccessToken_Get Request (authorization code)

4. Nnrf_AccessToken_Get Response (access token)

5. Service Request (access token)

6. Service Response

7. Service Response

Figure 13.4.1.X.Z.1-2: Service Request, delegated discovery and access token request

1. 	The NF Service Consumer sends a service request to the SCP, it includes a valid authorization code generated in the pre-step.

2.	If necessary, the SCP performs service discovery with the NRF.

3. 	Nnrf_AccessToken_Get Request: The SCP sends an access token request to the NRF according to clause 13.4.1.1, with the following differences: The SCP takes the role of the NF Service Consumer. The SCP includes the authorization code received from the NF Service Consumer in Step 1.

4.	Nnrf_AccessToken_Get Response: The NRF validates the authorization code received in the access token request and retrieves the NF instance ID of the consumer from the authorization code. If the NRF determines that the authorization code is valid and the NF consumer is authorized, the NRF issues an access token with the NF instance ID of the NF Service Consumer retrived from the authorization code as subject. The NRF then sends the access token to the SCP.

5.	The SCP sends the service request to the producer, including the access token received in Step 4.

6.	The producer validates the access token. If the token is valid, the producer sends the service response to the SCP.

7.	The SCP forwards the service response to the NF consumer.

**** END OF CHANGES ****