3GPP TSG-SA3 Meeting #99e 	S3-201081

# e-meeting, 11 – 15 May 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | - | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Client authentications in token-based authorization for indirect communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CableLabs, Mavenir, Nokia, Nokia Bell Labs |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2020-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This draft-CR specifies how the NRF authenticates an NF Service consumer when processing the access token request from the NF Service consumer. The authenticaiton of NF Service consumer are based on the client credentials. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define a mechanism for end to end authentication between the consumer NF and the NRF using client credentials as defined in this CR. This changes is applicable to indirect communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Indirect communication lacks proper security measures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.4.1.X (new), 13.4.1.X.1 (new), 13.4.1.X.2 (new), 13.4.1.X.Y (new), 13.4.1.X.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGES ****

#### 13.4.1.X	Service access authorization in indirect communication scenarios

##### 13.4.1.X.1	General

Editor's Note: General introduction to be added.

##### 13.4.1.X.2	Client Credentials

In indirect communication, NF Service comsumer (cNF) authenticates to the NRF using client credentials which are signed by the cNF using its private key associated with the cNF public key certificate.

The client credentials shall include:

- cNF public key certificate and the certificate chain represented within the JSON Web Key (RFC 7517);

- cNF identifier, i.e., the cNF instance ID;

- a timestamp; and

- the digital signature over the above data using the private key associated with the public key in the cNF certificate.

As an example, the client credentials can be represented using the form of JWT token (RFC 7523) and are used for the client authentication as specified in RFC 6749 and RFC 7521. If the client credentials are encoded using JWT, the JWT shall not include an expiration time field.

##### Note: The encoding details of client credentials is to be decided by CT4.

##### 13.4.1.X.3		Authorization for indirect communication without delegated discovery procedure

When there is mutual TLS based connection between NF Service Consumer and NRF, the NF Service Consumer and NRF perform the "Access token request before service access" procedure as described in clause 13.4.1.1. If the NF Service Consumer has already discovered the NF Service Producer, it can also perform the "Access token request for a specific NF Producer/NF Producer service instance" procedure as described in clause 13.4.1.1.

When there is no mutual TLS between NF Service Consumer and NRF, the NF Service Consumer performs the following procedure to obtain the access token from NRF and use it for service access at the NF Service Producer.

Figure 13.4.1.X.3-1: Authorization and service invocation procedure in indirect communication scenario

0.	Optionally, the NF Service Consumer may discover the NF Service Producer before requesting authorization to invoke the services of the NF Service Producer.

1. 	The NF Service Consumer (cNF) sends an access token request (Nnrf_AccessToken_Get Request) to the SCP. The access token request includes the cNF client credentials as defined in 13.4.1.X.2.

2. 	The SCP forwards the access token request (Nnrf_AccessToken_Get Request) to the NRF including the cNF client credentials.

3.	The NRF validates the client credentials received in the access token request in Step 2 by verifying the cNF certificate, the timestamp, and the digital signature of the client credentials. In addition, the NRF may validate the association of cNF instance ID with the cNF certificate. If those validations are successful and the access request is authorized based on the NRF policy, the NRF issues an access token which includes, among other mandatory fields, the validated cNF instance ID as in the client credentials.

4.   The NRF sends the access token to the SCP in an access token response (Nnrf_AccessToken_Get Response).

5.	The SCP forwards the access token response (Nnrf_AccessToken_Get Response) to the cNF, including the access token.

6.   The cNF sends the service request to the SCP. The service request includes the access token received in Step 5.

7.   The SCP forwards the service request to the NF Service Producer (pNF), including the access token.

8.	The pNF validates the access token. If the validation of the access token succeeds, the pNF sends the service response to the SCP.

9.	The SCP forwards the service response to the cNF.

##### 13.4.1.X.4		Authorization for indirect communication with delegated discovery procedure

Figure 13.4.1.X.4.1: Service Request with delegated discovery and delegated access token request

1. 	The NF Service Consumer (cNF) sends a service request to the SCP. The service request includes the cNF client credentials as defined in 13.4.1.X.2.

2.	The SCP may perform a service discovery with the NRF.

3. 	The SCP sends an access token request (Nnrf_AccessToken_Get Request) to the NRF. The access token request includes the client credentials received from the cNF in Step 1.

4.	The NRF validates the client credentials received in the access token request in Step 3 by verifying the cNF certificate, the timestamp, and the digital signature of the client credentials. In addition, the NRF may validate the association of cNF instance ID with the cNF certificate. If those validations are successful and the access request is authorized based on the NRF policy, the NRF issues an access token which includes, among other mandatory fields, the validated cNF instance ID as in the client credentials.

5.  The NRF sends the access token to the SCP in an access token response (Nnrf_AccessToken_Get Response).

6.	The SCP sends the service request to the NF Service Producer (pNF). The service request includes the access token received in Step 5.

7.	The pNF validates the access token. If the validation of the access token succeeds, the pNF sends the service response to the SCP.

8.	The SCP forwards the service response to the cNF.

**** END OF CHANGES ****