# 3GPP TSG SA WG3 (Security) Meeting #99e	S3-201126

# e-meeting, 11 – 15 May 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | Draft CR | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security of RRC UE capability transfer procedure in 5G for CIoT optimisation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_sec_5G |  |  |  |  |  |  | Date: |  | 2020-04-14 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | GSMA sent an LS to SA3 and RAN2 describing the vulnerability in current 3GPP RRC specifications for the UECapabilityEnquiry procedure. Tampering of UE capabilities by MITM attacker on the radio surface was possible as Rel 15 TS 33.501 specification was allowing UE capability Enquiry to occur before RRC security establishment. MITM attack can cause a denial of service attack, as described in the 33.861 Key issue 15. To address the issue, SA3 specified security protection of the RRC UE capability transfer procedure in 33.501. The fundamental requirement of the protection is that UE supports AS security. However, there is a type of CIoT devices that only support control-plane optimization. Such devices do not support AS security for UE capability transfer. For those devices, the following is the implementation of a security solution that enables the network to verify the UE radio capabilities provided by the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause 6.X.5 is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE capabilities may be tamped over the air, and tampered UE capabilities may persist for UEs without AS security. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other coAMFnts: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGES ***

### 6.X.5	RRC UE capability transfer procedure For Control Plane CIoT optimisation

In the case of Control Plane CIoT EPS optimization, if the network needs (additional) UE radio access capability information, the network initiates the UE capability enquiry procedure to a UE in RRC_CONNECTED state.

Upon receiving the UE capability enquiry when using Control Plane CIoT EPS optimisation only, the AS layer of the UE may trigger a UE capability enquiry procedure. As there is no AS security available, this procedure can not be protected as described in subclause 6.5.3.

To protect the UE capability information, the AS part of the UE triggers the NAS part of the UE to provide the UL_NAS_MAC. UL_NAS_MAC is used to show that the UE capability information is genuine. The UE calculates a UL_NAS_MAC by using the currently used NAS integrity algorithm with the following inputs, KNASint as the key, the uplink NAS COUNT that would be used for the next uplink NAS message, the DIRECTION bit set to 0 and the UL capability information as the message to be protected to calculate NAS-MAC (see AnnexD.3.1.1).

The uplink NAS COUNT is increased by the UE in the same way as if it had sent a NAS message.

The gNB shall send the UE Capability Information, NAS COUNT, UL_NAS_MAC in UE Capability Info Indication message to the AMF that is serving the UE.

The AMF calculates XUL-NAS-MAC (see Annex D.3.1.1) using the same inputs (i.e., uplink NAS COUNT, DIRECTION bit set to 0, and the UE capability information) as the UE used for calculating UL-NAS-MAC. The AMF then compares the received UL_NAS_MAC with XUL-NAS-MAC, and if these are equal, the network is sure that the genuine UE capability information is sent without tampering. Upon successful verification, AMF stores the verified radio capability information; otherwise, it discards the capability and releases the connection.

*** END OF CHANGES ***