3GPP TSG-SA3 Meeting #99e 	S3-201243

# e-meeting, 11 – 15 May 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.819 | CR | 0008 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution of editor's note in solution 15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_VERTICAL_LAN_SEC |  |  |  |  |  | Date: |  |  | 15.5.2020 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Resolving ed note on compatiblity by adding evaluation text. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Deleting ed.notes and adding equivalent evaluation text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ed note in spec unresolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.15.2, 6.15.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3-200382 |  |  |  |  |  |  |  |  |

## ******** START OF CHANGES

## 6.15	Solution #15: CAG ID privacy by indication in RRC layer and providing CAG ID only after NAS security establishment

### 6.15.1	Introduction

This solution addresses key issue #6.2 and proposes a new mechanism to confidentially protect the CAG ID during the CAG access control using the NAS security protection.

### 6.15.2	Solution details

During the registration procedure, the AMF checks that whether the requested CAG ID protected by NAS security sending from UE is within both the NG-RAN's allowed CAG Lists, and subscribed CAG retrieved from the UDM. If the verification successes, the AMF sends the allowed CAG ID to the NG-RAN.

Details procedure during the initial registration is as follows.

Figure 6.15.2-1 CAG ID access control in initial registration scenario

0. UE is configured with Allowed CAG ID list.

1. NG-RAN broadcasts the system information including the CAG ID list supported by the NG-RAN.

2. UE checks there is as least one match between allowed CAG list and NG-RAN-s CAG ID(s).

3. UE to NG-RAN: AN message (AN parameters, Registration Request (Registration type, SUCI, etc.), where the AN parameter includes an indicator, indicating that the UE requires to access the CAG ID(s) broadcasted by the NG-RAN.

4. NG-RAN to AMF: N2 message (N2 parameters, Registration Request (as described in step 1) where the N2 parameter include NG-RAN’s CAG ID(s). If the indicator is received from the UE, the CAG ID(s) supported by the NG-RAN (i.e. NG-RAN’ CAG ID(s)) shall be forwarded to the AMF within the N2 parameters.

5-8. The AMF initiates the following UE authentication.

9. AMF initiates NAS SMC procedure, and sends the NAS security mode command to the UE.

10. UE sends the requested CAG ID to the AMF within the NAS security mode command complete message, which is confidentiality and integrity protected. Here the requested CAG ID belongs to the match CAG IDs in step 2.

11. The AMF retrieves the subscription data from the UDM, which includes the subscribed CAG lists of the UE.

12. The AMF checks whether the requested CAG ID sending from UE is within both the NG-RAN’s allowed CAG Lists, and subscribed CAG lists in the UDM. If the checking does not pass, the AMF sends the registration reject message to the UE. Otherwise, the AMF does the following steps. Here denoting the requested CAG ID as the allowed CAG ID, if the above checking passes.

13. The AMF sends the allowed CAG ID to the NG-RAN within the N2 message.

14. The AMF sends the registration accept to the UE.

For the other registration scenarios, initial NAS protection shall be used here for the requested CAG ID protection while transferred from UE to AMF.

### 6.15.3	Evaluation

The above solution addresses the requirements of key issue #6.2: CAG ID Privacy.

The solution reuses the NAS security context to confidentially protect the CAG ID during the CAG access control, without introducing new security mechanism in UDM or AUSF.

The impact of the solution is to require the UE to send the requested CAG ID after NAS security is established. And then the AMF could check whether the requested CAG ID sending from UE is within both the NG-RAN's allowed CAG lists, and subscribed CAG lists in the UDM.

The compatibility of this security solution with the existing architecture solution would need more evaluation.

## ******** END OF CHANGES