3GPP TSG-SA3LI Meeting #63	S3i160538

# San Antonio, Texas, The USA, October 25-28, 2016		revised version of S3i160528

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.107 | CR | 0232 | rev | 2 | Current version: | 13.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Targeting of Non-Local ID in case of SMS usages: correction of existing texts |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Ministére de l’Economie et des Finances (France)) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI13 |  |  |  |  |  | Date: |  |  | 26/10/2016 |
| Category: | F |  |  |  |  |  | Release: |  |  | Release13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The texts of clauses 6.2, 7.1 and 7.4.7 may induce misunderstandings on how the LI function should intercept SMS MO or MT in case of Non-Local ID targeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | functionnal and grammatical changes on how LI function should intercept SMS in case of Non-Local ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk of not intercepting the SMS of Non-Local ID in CS domain |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2; 7.1 and 7.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

****** FIRST CHANGE ******

## 6.2	Provision of CC - Short Message Service

Figure 14 shows an SMS transfer from the 3G MSC Server to the LEMF. Quasi-parallel to the delivery from / to the mobile subscriber a message, which contains the contents of the SMS with the header, is generated and sent via the Delivery Function 2 to the LEMF in the same way as the Intercept Related Information.

The IRI will be delivered to the LEMF:

-	for a SMS-MO. Dependent on national requirements, delivery shall occur  in the following cases:

- when the 3G MSC receives the SMS from the target MS, or when the 3G MSC detects that an SMS is  to the Non-Local ID target.

- when the 3G MSC receives notification that the SMS-Centre successfully received the SMS that was originated from the target MS, or sent to the Non-Local ID target.

-	for a SMS-MT. Dependent on national requirements, delivery shall occur  in the following case:

-  when the 3G MSC receives the SMS from the SMS-Centre when the SMS was originated from a Non-Local ID target, or will have to be sent to a target MS.

- when the 3G MSC receives notification that recipient MS has received the SMS successfully.The recipient MS is the target MS when the SMS is sent to the target. The recipient MS may not be the target when the SMS was originating from a Non-Local ID target.

Figure 14: Provision of Content of Communication - Short Message Service

Figure 14: Provision of Content of Communication - Short Message Service

****** END OF FIRST CHANGE ******

****** SECOND CHANGE ******

## 7.1	Provision of Intercept Product - Short Message Service

Figure 19 shows an SMS transfer from the 3G SGSN node to the LEA. Quasi-parallel to the delivery from / to the mobile subscriber a SMS event, which contains the content and header of the SMS, is generated and sent via the Delivery Function 2 to the LEA in the same way as the Intercept Related Information. National regulations and warrant type determine if a SMS event shall contain only SMS header, or SMS header and SMS content. Non-Local ID targeting is optional and may require traffic analysis.

The IRI will be delivered to the LEA:

-	for a MO-SMS. Dependent on national requirements, delivery shall occur  in the following cases:

- when the 3G SGSN receives the SMS from the target MS, or when the 3G SGSN detects that an SMS is  to the Non-Local ID target.

- when the 3G SGSN receives notification that the SMS-Centre successfully received the SMS that was originated from the target MS, or sent to the Non-Local ID target;

-	for a MT-SMS. Dependent on national requirements, delivery shall occur in the following cases:

-  when the 3G SGSN receives the SMS from the SMS-Centre when the SMS was originated from a Non-Local ID target, or will have to be sent to a target MS.

- when the 3G SGSN receives notification that recipient MS has received the SMS successfully.The recipient MS is the target MS when the SMS is sent to the target. The recipient MS may not be the target when the SMS was originating from a Non-Local ID target.

3G GSN

SMS

service

center

LEA

delivery

function 2

Target

Figure 19: Provision of Intercept Product - Short Message Service

****** END OF SECOND CHANGE ******

****** THIRD CHANGE ******

### 7.4.7	SMS

For SMS-MO, the event is generated in the 3G SGSN. Dependent on national requirements, event generation shall occur  in the following cases:

- when the 3G SGSN receives the SMS from the target MS, or when the 3G SGSN detects that an SMS is  to the Non-Local ID target.

- when the 3G SGSN receives notification that the SMS-Centre successfully received the SMS that was originated from the target MS, or sent to the Non-Local ID target.

For SMS-MT, the event is generated in the 3G SGSN. Dependent on national requirements, event generation shall occur in the following cases:

-  when the 3G SGSN receives the SMS from the SMS-Centre when the SMS was originated from a Non-Local ID target, or will have to be sent to a target MS.

- when the 3G SGSN receives notification that recipient MS has received the SMS successfully.The recipient MS is the target MS when the SMS is sent to the target. The recipient MS may not be the target when the SMS was originating from a Non-Local ID target.

These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Observed Non-Local ID

Event Type

Event Time

Event Date

Network Element Identifier

Location Information

SMS

SMS Initiator

IAs (if applicable)

****** END OF THIRD CHANGE ******

****** END OF ALL CHANGES ******