3GPP TSG-SA3 Meeting #64	s3i170051

# Sophia-Antipolis, France, 24th Jan 2017 - 27th Jan 2017          (revision of S3i170012)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.108 | CR | 0345 | rev | 1 | Current version: | 13.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Phase 2 of the LALS development - Stage 3 description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rogers Communications Canada (SA3-LI) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI14 |  |  |  |  |  | Date: |  |  | 2017-01-27 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Phase 2 of LALS Development. |  |  |  |  |  |  |  |  |
| Summary of change: |  | LALS delivery interface is enhanced with support of event-triggered location reporting and positioning error reporting.positioning error reporting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Event-triggered location is not supported, positioning error reporting is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.5.3, 5.1.2, 5.2.3, 6.1.3, 6.5.0, 6.5.1.1, 10.5.0, 10.5.1.1, Annexes B.3, B.3a, B.9, O |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

************  CHANGE 1  **********************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ETSI TS 101 331: "Lawful Interception (LI); Requirements of Law Enforcement Agencies".

[2]	ETSI ES 201 158: "Telecommunications security; Lawful Interception (LI); Requirements for network functions".

[3]	ETSI ETR 330: "Security Techniques Advisory Group (STAG); A guide to legislative and regulatory environment".

[4]	3GPP TS 29.002: "3rd Generation Partnership Project; Technical Specification Group Core Network; Mobile Application Part (MAP) specification".

[5A]	ITU-T Recommendation X.680: "Abstract Syntax Notation One (ASN.1): Specification of Basic Notation".

[5B]	ITU-T Recommendation X.681: "Abstract Syntax Notation One (ASN.1): Information Object Specification".

[5C]	ITU-T Recommendation X.681: "Abstract Syntax Notation One (ASN.1): Constraint Specification".

[5D]	ITU-T Recommendation X.681: "Abstract Syntax Notation One (ASN.1): Parameterization of ASN.1 Specifications".

[6]	ITU-T Recommendation X.690: "ASN.1 encoding rules: Specification of Basic Encoding Rules (BER), Canonical Encoding Rules (CER) and Distinguished Encoding Rules (DER)".

NOTE 1:	It is recommended that for [5A], [5B], [5C], [5D] and [6] the 2002 specific versions should be used.

[7]	ITU-T Recommendation X.880: "Information technology - Remote Operations: Concepts, model and notation".

[8]	ITU-T Recommendation X.882: "Information technology - Remote Operations: OSI realizations - Remote Operations Service Element (ROSE) protocol specification".

NOTE 2:	It is recommended that for [8] the 1994 specific versions should be used.

[9]	3GPP TS 24.008: "3GPP Technical Specification Group Core Network; Mobile radio interface Layer 3 specification, Core network protocol; Stage 3".

[10] - [12]	Void.

[13]	IETF STD 9 (RFC 0959): "File Transfer Protocol (FTP)".

[14]	3GPP TS 32.215: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication Management; Charging Management; Charging data description for the Packet Switched (PS) domain)".

[15]	IETF STD0005 (RFC 0791: "Internet Protocol".

[16]	IETF STD0007 (RFC 0793): "Transmission Control Protocol".

[17]	3GPP TS 29.060: "3rd Generation Partnership Project; Technical Specification Group Core Network; General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[18]	3GPP TS 33.106: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Lawful Interception Requirements".

[19]	3GPP TS 33.107: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Lawful interception architecture and functions".

[20]	3GPP TS 23.107: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Quality of Service QoS concepts and architecture".

[21] – [22]	Void.

[23]	ANSI/J-STD-025-A: "Lawfully Authorized Electronic Surveillance".

[24]	ETSI TS 101 671: "Handover Interface for the lawful interception of telecommunications traffic".

[25]	3GPP TS 23.003: "3rd Generation Partnership Project; Technical Specification Group Core Network; Numbering, addressing, and identification".

[26]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[27]	IETF RFC 1006: "ISO Transport Service on top of the TCP".

[28]	IETF RFC 2126: "ISO Transport Service on top of TCP (ITOT)".

[29]	ITU-T Recommendation Q.763: "Signalling System No. 7 - ISDN User Part formats and codes".

[30]	ETSI EN 300 356 (all parts): "Integrated Services Digital Network (ISDN); Signalling System No.7; ISDN User Part (ISUP) version 3 for the international interface".

[31]	ETSI EN 300 403-1 (V1.3.2): "Integrated Services Digital Network (ISDN); Digital Subscriber Signalling System No. one (DSS1) protocol; Signalling network layer for circuit-mode basic call control; Part 1: Protocol specification [ITU-T Recommendation Q.931 (1993), modified]".

NOTE 3:	Reference [31] is specific, because ASN.1 parameter "release-Reason-Of-Intercepted-Call" has the following comment: "Release cause coded in [31] format". In case later version than the given one indicated for ISDN specification ETSI EN 300 403-1 has modified format of the "release cause", keeping the reference version specific allows to take proper actions in later versions of this specification.

[32] - [33]	Void

[34]	ITU-T Recommendation Q.931: "ISDN user-network interface layer 3 specification for basic call control".

[35]	Void.

[36]	Void.

[37]	3GPP TS 23.032: "3rd Generation Partnership Project; Technical Specification Group Core Network; Universal Geographical Area Description (GAD)".

[38]	3GPP TR 21.905: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Vocabulary for 3GPP Specifications".

[39]	ISO 3166-1: "Codes for the representation of names of countries and their subdivisions - Part 1: Country codes".

[40]	3GPP TS 23.228: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS); Stage 2".

[41]	3GPP TS 29.234: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals: 3GPP System to Wireless Local Area Network (WLAN) interworking; Stage 3".

[42]	3GPP TS 23.060: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; General Packet Radio Service (GPRS); Service description".

[43]	3GPP TS 23.234: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3GPP system to Wireless Local Area Network (WLAN) Interworking; System Description".

[44]	3GPP TS 23.401: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[45]	3GPP TS 23.402: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Architecture enhancements for non-3GPP accesses".

[46]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Access (GPRS) Tunneling Protocol for Control Plane (GTPv2-C); Stage 3".

[47]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[48]	3GPP TS 29.275: "Proxy Mobile IPv6 (PMIPv6) based Mobility and Tunneling protocols; Stage 3".

[49]	3GPP TS 24.303: "Mobility management based on Dual-Stack Mobile IPv6; Stage 3".

[50]	(void)

[51]	(void)

[52]	3GPP TS 24.147: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Conferencing Using the IP Multimedia (IM) Core Network (CN) subsystem 3GPP Stage 3".

[53]	3GPP TS 29.273: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[54]	3GPP TS 33.328: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS) media plane security".

[55]	ATIS-0700005 "Lawfully Authorized Electronic Surveillance (LAES) for 3GPP IMS-based VoIP and other Multimedia Services".

[56]	3GPP TS 29.212: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Policy and Charging Control(PCC); Reference points".

[57]	Void.

[58]	IETF RFC 4217: "Securing FTP with TLS".

[59]	3GPP TS 29.272: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Evolved Packet System (EPS); Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[60]	3GPP TS 33.310: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Network Domain Security (NDS); Authentication Framework (AF)".

[61]	IETF RFC 6043: "MIKEY-TICKET: Ticket-Based Modes of Key Distribution in Multimedia Internet KEYing (MIKEY)", available at www.ietf.org

[62]	3GPP TS 25.413: "UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling".

[63]	3GPP TS 29.279: "Mobile IPv4 (MIPv4) based mobility protocols; Stage 3".

[64]	3GPP TS 29.118: "Mobility Management Entity (MME) –Visitor Location Register (VLR) SGs interface specification"

[65]	ANSI/J-STD-025-B: "Lawfully Authorized Electronic Surveillance", July 17, 2006.

[66]	3GPP TS 24.007: "Mobile Radio Interface Signalling Layer 3; General Aspects".

[67]	IETF RFC 3966: "The Tel URIs for Telephone Numbers", December, 2004.

[68]	IETF RFC 791: "Internet Protocol"

[69]	IETF RFC 2460: "Internet Protocol, Version 6 (IPv6) Specification".

[70]	IEFT RFC 3697: "IPv6 Flow Label Specification".

[71]	IETF RFC 4776: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information".

[72]	IETF RFC 5139: "Revised Civic Location Format for Presence Information Data Format Location Object (PIDF-LO)".

[73]	ISO.3166-2: International Organization for Standardization, "Codes for the representation of names of countries and their subdivisions - Part 2: Country subdivision code".

[74]	UPS SB42-4: Universal Postal Union (UPU), "International Postal Address Components and Templates".

[75]	ISO 639-1:2002: "Codes for the representation of names of languages -- Part 1: Alpha-2 code".

[76]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[77]	3GPP TS 24.623: "Technical Specification Group Core Network and Terminals; Extensible Markup Language (XML) Configuration Access Protocol (XCAP) over the Ut interface for Manipulating Supplementary Services".

[78]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[79]	3GPP TS 24.109: "Universal Mobile Telecommunications System (UMTS); Bootstrapping interface (Ub) and network application function interface (Ua); Protocol details".

[80]	IETF RFC 4825:"The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[81]	IETF RFC 7254: "A Uniform Resource Name Namespace for the Global System for Mobile Communications Association (GSMA) and the International Mobile station Equipment Identity (IMEI)"

[82]	IETF RFC 7255: "Using the International Mobile station Equipment Identity (IMEI) Uniform Resource Name (URN) as an Instance ID".

[83]	3GPP TS 22.468: "Group Communication System Enablers for LTE (GCSE_LTE)".

[84]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[85]	3GPP TS 25.321: "Medium Access Control (MAC) protocol specification".

[86]	3GPP TS 24.371: " Web Real-Time Communications (WebRTC) access to the IP Multimedia (IM) Core Network (CN) subsystem (IMS); Stage 3".

[87]	ITU-T Recommendation E.212: "The international identification plan for public networks and subscriptions".

[xx]	OMA MLP TS: "Mobile Location Protocol", OMA-TS-MLP-V3_4-20150512-A

*****  END OF CHANGE 1  *********************

************  CHANGE 2  **********************

### 4.5.3	Types of IRI records

Intercept related information shall be conveyed to the LEMF in messages, or IRI data records, respectively. Four types of IRI records are defined:

1)	IRI-Begin record 			at the first event of a communication attempt,
								opening the IRI transaction.

2)	IRI-End record 				at the end of a communication attempt,
								closing the IRI transaction.

3)	IRI-Continue record 		at any time during a communication attempt
								within the IRI transaction.

4)	IRI-REPORT record 		used in general for non-communication related events.

For information related to an existing communication case, the record types 1 to 3 shall be used. They form an IRI transaction for each communication case or communication attempt, which corresponds directly to the communication phase (set-up, active or release).

For packet oriented data services, the first event of a communication attempt shall be the PDP context activation or a similar event and an IRI-BEGIN record shall be issued. The end of the communication attempt shall be the PDP context deactivation and an IRI-END record shall be issued. While a PDP context is active, IRI-CONTINUE records shall be used for CC relevant IRI data records, IRI-REPORT records otherwise.

Record type 4 is used for non-communication related subscriber action, like subscriber controlled input (SCI) for service activation. For simple cases, it can also be applicable for reporting unsuccessful communication attempts. It can also be applicable to report some subscriber actions which may trigger communication attempts or modifications of an existing communication, when the communication attempt or the change of the existing communication itself is reported separately.

Record type 4 is also used to convey the LALS reports.

For the IMS domain the IRI record types are used in a different way than described in this clause. Details on the IRI type usage in the IMS domain are defined in clause 7.5.

The record type is an explicit part of the record. The 4 record types are defined independently of target communication events. The actual indication of one or several communication events, which caused the generation of an IRI record, is part of further parameters within the record's information content. Consequently, the record types of the IRI transactions are not related to specific messages of the signalling protocols of a communication case, and are therefore independent of future enhancements of the intercepted services, of network specific features, etc. Any transport level information (i.e. higher-level services) on the target communication-state or other target communication related information is contained within the information content of the IRI records.

For packet oriented data services, if LI is being activated during an already established PDP context or similar, an IRI-BEGIN record will mark the start of the interception. If LI is being deactivated during an established PDP context or similar, no IRI-END record will be transmitted. The end of interception can be communicated to the LEA by other means (e.g. HI1).

*****  END OF CHANGE 2  *********************

************  CHANGE 3  **********************

### 5.1.2	Communication IDentifier (CID)

#### 5.1.2.0	General

For each activity relating to a target identity, a CID is generated by the relevant network element. The CID consists of the following two identifiers:

-	Network IDentifier (NID);

-	Communication Identity Number (CIN) - optional.

NOTE 1:	For all non CC related records like SMS, SCI etc. no correlation to a CC could be made.

The CID distinguishes between the different activities of the target identity. It is also used for correlation between IRI records and CC connections, as well as for correlation between LALS reports and IRI records of the triggering events.  It is used at the interface ports HI2 and HI3.

The Communication IDentifier is specified in the subsequent subclauses of 5.1.2. For ASN.1 coding details, see annex B.

*****  END OF CHANGE 3  *********************

************  CHANGE 4  **********************

### 5.2.3	Delivery of IRI

The events defined in TS 33.107 [19] are used to generate Records for the delivery via HI2. The LALS reports defined in [19] are delivered via HI2, as well.

There are thirteen different events type received at DF2 level. According to each event, a Record is sent to the LEMF if this is required. In the case of LALS reports, which are not associated with an event, a Record is sent to the LEMF without the event parameter.

The following table gives the mapping between event type received at DF2 level and record type sent to the LEMF.

It is an implementation option if the redundant information will be sent for each further event.

Table 5.4: Structure of the records for UMTS (CS)

| Event | IRI Record Type |
| --- | --- |
| Call establishment | BEGIN |
| Answer | CONTINUE |
| Supplementary service | CONTINUE |
| Handover | CONTINUE |
| Release | END |
| Location update | REPORT |
| Subscriber controlled input | REPORT |
| SMS | REPORT |
| Serving system | REPORT |
| HLR subscriber record change | REPORT |
| Cancel location | REPORT |
| Register location | REPORT |
| Location information request | REPORT |

The LALS report records are sent to the LEMF with the REPORT IRI Record Type.

NOTE 1:	VOID.

A set of information is used to generate the records. The records used transmit the information from mediation function to LEMF. This set of information can be extended in 3G MSC server or 3G GMSC server or DF2/MF, if this is necessary in a specific country. The following table gives the mapping between information received per event or report and information sent in records.

Table 5.5: Description of parameters

| Parameter | Definition | ASN.1 parameter |
| --- | --- | --- |
| observed MSISDN | Target Identifier with the MSISDN of the target | PartyInformation/msISDN |
| observed IMSI | Target Identifier with the IMSI of the target | PartyInformation/imsi |
| observed IMEI | Target Identifier with the IMEI of the target, it must be checked for each call over the radio interface | PartyInformation/imei |
| event type | Description of which type of event is delivered: Establishment, Answer, Supplementary service, Handover, Release, SMS, Location update, Subscriber controlled input, HLR subscriber record change, Serving system, Cancel location, Register location, Location information request | Umts-CS-Event. In case this parameter is not sent over the HI2 interface, the presence of other parameters on HI2 indicates the event type (e.g. sMS or sciData parameter presence) |
| event date | Date of the event generation in the 3G MSC server or 3G GMSC server or in the HLR | timestamp |
| event time | Time of the event generation in the 3G MSC server or 3G GMSC server or in the HLR |  |
| dialled number | Dialled number before digit modification, IN-modification, etc. | PartyInformation (= originating)/DSS1-parameters/calledpartynumber |
| connected number | Number of the answering party | PartyInformation/supplementary-Services-Info |
| other party address | Directory number of the other party for originating calls Calling party for terminating calls | PartyInformation (= terminating)/calledpartynumber PartyInformation/callingpartynumber |
| call direction | Information if the target is calling or called e.g. MOC/MTC or originating/terminating in or/out | intercepted-Call-Direct |
| CID | Unique number for each call sent to the DF, to help the LEA, to have a correlation between each call and the IRI (combination of Interception Node ID and the correlation number) | communicationIdentifier |
| lawful interception identifier | Unique number for each surveillance lawful authorization | LawfulInterceptionIdentifier |
| CGI/SAI | CGI or SAI of the target; for the location information | locationOfTheTarget |
| location area code | Location-area-code of the target defines the Location Area in a PLMN |  |
| location Information | LALS location information |  |
| serving system identifier | VPLMN ID of the serving system or of the third party network interworking with the HLR | serving-System-Identifier |
| basic service | Information about Tele service or bearer service | PartyInformation/DSS1-parameters-codeset-0 |
| supplementary service | Supplementary services used by the target e.g. CF, CW, ECT | PartyInformation/Supplementary-Services |
| forwarded to number | Forwarded to number at CF | PartyInformation/calledPartyNumber (party-Qualifier indicating forwarded-to-party) |
| call release reason | Call release reason of the target call | Release-Reason-Of-intercepted-Call |
| SMS | The SMS content with header which is sent with the SMS-service | SMS |
| SCI | Non-call related Subscriber Controlled Input (SCI) which the 3G MSC server receives from the ME | PartyInformation/sciData |
| other update | Carrier specific information related to its implementation or subscription process on its HLR | carrierSpecificData |
| NOTE: LIID parameter must be present in each record sent to the LEMF. |  |  |

Table 5.5A: Serving System REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| event type | C | Provide Serving System event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Network identifier of the HLR reporting the event (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| serving system identifier | C | Provide the VPLMN id (Mobile Country Code and Mobile Network Country, E. 212 number [87]). |
| serving MSC number | C | Provide to identify the E.164 number of the serving MSC. |
| serving MSC address | C | Provide to identify the IP address of the serving MSC. |

Table 5.5B: HLR subscriber record change REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| new observed MSISDN | C | Provide at least one and others when available. |
| new observed IMSI |  |  |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| event type | C | Provide HLR subscriber record change event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Network identifier of the HLR reporting the event (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| carrier specific data | C | Provide to raw data of this specific update related to HLR. |

Table 5.5C: Cancel location REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| event type | C | Provide cancel Location change event type. (purge from HLR sent to SGSN included). |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Network identifier of the HLR reporting the event (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| previous serving system identifier | C | Provide the previous VPLMN id (Mobile Country Code and Mobile Network Country, defined in E212 [87])). |
| previous serving MSC-number | C | Provide to identify the E.164 number of the previous serving MSC. |
| previous serving MSC-address | C | Provide to identify the IP address of the previous serving MSC. |

Table 5.5D: Register location REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| event type | C | Provide register location event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Network identifier of the HLR reporting the event (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| previous serving system identifier | C | Provide the previous VPLMN id (Mobile Country Code and Mobile Network Country; defined in E212 [87]) ). |
| previous serving MSC number | C | Provide to identify the E.164 number of the previous serving MSC. |
| previous serving MSC address | C | Provide to identify the IP address of the previous serving MSC. |
| current serving system identifier | C | Provide the previous VPLMN id (Mobile Country Code and Mobile Network Country, defined in E212 [87])). |
| current serving MSC number | C | Provide to identify the E.164 number of the current serving MSC. |
| current serving MSC address | C | Provide to identify the IP address of the current serving MSC. |

Table 5.5E: Location information request REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| event type | C | Provide location information request event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Network identifier of the HLR reporting the event (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| requesting network identifier | C | Provide the requesting network identifier PLMN id (Mobile Country Code and Mobile Network Country, defined in E212 [87]). |
| requesting node type | C | Provide the requesting node type (GMSC; SMS Centre; GMLC, MME, SGSN) |

Table 5.5F: LALS Target Positioning REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed IMEI |  |  |
| event date | M | Provide the date and time the LCS Report is available at LI LCS Client. |
| event time |  |  |
| network identifier | M | Network identifier of the LI LCS Client (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide the LALS location information, if the positioning is successful |
| extended location parameters | O | If available, additional location information and associated QoS information. |
| LALS error code | C | Provide the error identification code if the positioning is not successful. |

Table 5.5x: LALS Enhanced Location for IRI REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed IMEI |  |  |
| event date | M | Provide the date and time the LCS Report is available at LI LCS Client. |
| event time |  |  |
| network identifier | M | Network identifier of the LI LCS Client (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| communication identity number | C | Provided for correlation with the IRI records of the call, if available in the corresponding LALS triggering event. |
| location information | C | Provide the LALS location information, if the positioning is successful. |
| extended location parameters | O | If available, additional location information and associated QoS information. |
| LALS error code | C | Provide the error identification code if the positioning is not successful. |

NOTE 2: 	See the TS 33.107 [19] for a detailed description of LALS. See Annex O for information on using of the CS ASN.1 information object for the LALS reporting.

NOTE 3: 	In some specific scenarios the amount of Enhanced Location for IRI reports data may overload the X2 and/or HI2 interfaces. To prevent the overload, a flow control for Enhanced Location for IRI Reports may be implemented, e.g. by limiting the frequency of the reports for individual target.

*****  END OF CHANGE 4  *********************

************  CHANGE 5  **********************

### 6.1.3	Correlation number

The Correlation Number is unique per PDP context and used for the following purposes:

-	correlate CC with IRI,

-	correlate different IRI records within one PDP context,

-	correlate LALS reports with the IRI records of the triggering events.

As an example, in the UMTS system, the Correlation Number may be the combination of GGSN address and charging ID.

NOTE:	The Correlation Number is at a minimum unique for each concurrent communication (e.g. PDP context) of a target within a lawful authorization.

*****  END OF CHANGE 5  *********************

************  CHANGE 6  **********************

## 6.5	IRI for packet domain

### 6.5.0	Introduction

The IRI will in principle be available in the following phases of a data transmission:

1.	At connection attempt when the target identity becomes active, at which time packet transmission may or may not occur (set up of a data context, target may be the originating or terminating party);

2.	At the end of a connection, when the target identity becomes inactive (removal of a data context);

3.	At certain times when relevant information are available.

In addition, information on non-transmission related actions of a target constitute IRI and is sent via HI2, e.g. information on subscriber controlled input. Also, the LALS reports constitute IRI and are sent via HI2.

The IRI may be subdivided into the following categories:

1.	Control information for HI2 (e.g. correlation information);

2.	Basic data context information, for standard data transmission between two parties.

# The events defined in TS 33.107 [19] are used to generate records for the delivery via HI2.

Unless otherwise noted, the following terminology applies to both GPRS and 3G GSN nodes:

GPRS attach					- also applies to Mobile Station attach

GPRS detach					- also applies to Mobile Station detach

gPRSEvent					- also applies to PDP Context events and Mobile Station events

gPRSCorrelationNumber	- also applies to PDP Context Correlation

gPRSOperationErrorCode	- also applies to PDP Context Operation Error Codes

There are several different event types received at DF2 level. According to each event, a Record is sent to the LEMF if this is required. In the case of LALS reports, which are not associated with an event, a Record is sent to the LEMF without the event parameter.

The following table gives the mapping between event type received at DF2 level and record type sent to the LEMF.

Table 6.1: Mapping between UMTS Data Events and HI2 records type

| Event | IRI Record Type |
| --- | --- |
| GPRS attach | REPORT |
| GPRS detach | REPORT |
| PDP context activation (successful) | BEGIN |
| PDP context modification | CONTINUE |
| PDP context activation (unsuccessful) | REPORT |
| Start of interception with mobile station attached (national option) | REPORT |
| Start of interception with PDP context active | BEGIN or optionally CONTINUE |
| PDP context deactivation | END |
| Location update | REPORT |
| SMS | REPORT |
| ServingSystem | REPORT |
| Packet Data Header Information | REPORT |
| HLR subscriber record change | REPORT |
| Cancel location | REPORT |
| Register location | REPORT |
| Location information request | REPORT |

# The UMTS PS LALS reports are sent to the LEMF in the REPORT IRI records.

A set of information is used to generate the records. The records used transmit the information from mediation function to LEMF. This set of information can be extended in the GSN or DF2 MF, if this is necessary in a specific country. The following table gives the mapping between information received per event and information sent in records.

Table 6.2: Mapping between Events information and IRI information

| parameter | description | HI2 ASN.1 parameter |  |
| --- | --- | --- | --- |
| observed MSISDN | Target Identifier with the MSISDN of the target. | partyInformation (party-identiity) |  |
| observed IMSI | Target Identifier with the IMSI of the target. | partyInformation (party-identity) |  |
| observed IMEI | Target Identifier with the IMEI of the target. | partyInformation (party-identity) |  |
| observed PDP address | PDP address(es) used by the target. In case of IPv4v6 two addresses may be carried. | partyInformation (services-data-information) |  |
| event type | Description which type of event is delivered: PDP Context Activation, PDP Context Deactivation,GPRS Attach, HLR subscriber record change, Cancel location, Register location, Location information request, etc. | gPRSevent (when using Annex B.3) or ePSevent (when using Annex B.9) |  |
| event date | Date of the event generation in the xGSN or in the HLR | Timestamp |  |
| event time | Time of the event generation in the xGSN or in the HLR |  |  |
| access point name | The Access Point Name contains a logical name (see 3GPP TS 23.060 [---TBD---]) | partyInformation (services-data-information) |  |
| PDP type | This field describes the PDP type as defined in 3GPP TS 29.060 [17], TS 24.008 [9], TS 29.002 [4] | partyInformation (services-data-information) |  |
| initiator | This field indicates whether the PDP context activation, deactivation, or modification is MS directed or network initiated. | initiator |  |
| correlation number | Unique number for each PDP context delivered to the LEMF, to help the LEA, to have a correlation between each PDP Context and the IRI. | gPRSCorrelationNumber |  |
| lawful interception identifier | Unique number for each lawful authorization. | lawfulInterceptionIdentifier |  |
| location information | When authorized, this field provides the location information of the target that is present at the SGSN or LI LCS Client at the time of event or LALS report record production. | locationOfTheTarget |  |
| extended location parameters | Additional location information and QoS information | extendedLocParameters |  |
| LALS error code | Positioning error identification code | locationErrorCode |  |
| SMS | The SMS content with header which is sent with the SMS-service | sMS |  |
| failed context activation reason | This field gives information about the reason for a failed context activation of the target. | gPRSOperationErrorCode |  |
| failed attach reason | This field gives information about the reason for a failed attach attempt of the target. | gPRSOperationErrorCode |  |
| service center address | This field identifies the address of the relevant server within the calling (if server is originating) or called (if server is terminating) party address parameters for SMS-MO or SMS-MT. | serviceCenterAddress |  |
| umts QOS | This field indicates the Quality of Service associated with the PDP Context procedure. | qOS |  |
| context deactivation reason | This field gives information about the reason for context deactivation of the target. | gPRSOperationErrorCode |  |
| network identifier | Operator ID plus SGSN, GGSN, or HLR address. | networkIdentifier |  |
| serving system identifier | VPLMN ID of the serving system or of the third party network interworking with the HLR | serving-System-Identifier |  |
| iP assignment | Observed PDP address is statically or dynamically assigned. | iP-assignment |  |
| SMS originating address | Identifies the originator of the SMS message. | DataNodeAddress |  |
| SMS terminating address | Identifies the intended recipient of the SMS message. | DataNodeAddress |  |
| SMS initiator | Indicates whether the SMS is MO, MT, or Undefined | sms-initiator |  |
| serving SGSN number | An E.164 number of the serving SGSN. | servingSGSN-Number |  |
| serving SGSN address | An IP address of the serving SGSN. In case of S4-SGSN, this may be provided as Diameter id and realm of the serving S4-SGSN connected via S6d interface to the HSS. | servingSGSN-Address servingS4-SGSN-address |  |
| NSAPI | Network layer Service Access Point Identifier information element contains an NSAPI identifying a PDP Context in a mobility management context specified by the Tunnel Endpoint Identifier Control Plane This is an optional parameter to help DF/MF and LEA's to distinguish between the sending mobile access networks when the GGSN is used as element of the PDG according TS 23.234 [43]. | nSAPI |  |
| ULI Timestamp | Indicates the time when the User Location Information was acquired. | uLITimestamp |
| destination IP address | Identifies the destination IP address of a packet. | destinationIPAddress |  |
| destination port number | Identifies the destination port number of a packet | destinationPortNumber |  |
| source IP address | Identifies the source IP address of a packet. | sourceIPAddress |  |
| source port number | Identifies the source port number of a packet. | sourcePortNumber |  |
| transport protocol | Identifies the transport protocol (i.e., Protocol Field in IPv4 or Next Header Field in IPv6. | transportProtocol |  |
| flow label | The field in the IPv6 header that is used by a source to label packets of a flow (see RFC 3697 [70]) | flowLabel |  |
| packet count | The number of packets detected and reported in a particular packet data summary report. | packetCount |  |
| packet size | The size of a packet (i.e., Total Length Field in IPv4 [68] or Payload Length Field in IPv6 [69]) | packetsize |  |
| packet direction | Identifies the direction of the intercepted packet (from target or to target) | packetDirection |  |
| packet data header copy | Provides a copy of the packet headers including IP layer and next layer, and extensions, but excluding content. | packetDataHeaderCopy |  |
| summary period | Provides the period of time during which the packets of the summary report were sent or received by the target. | summaryPeriod |  |
| sum of packet sizes | Sum of values in Total Length Fields in IPv4 packets or Payload Length Field in IPv6 packets. | sumOfPacketSizes |  |
| packet data summary reason | Provides the reason for a summary report. | packetDataSummaryReason |  |
| packet data summary | For each particular packet flow, identifies pertinent reporting information (e.g., source IP address, destination IP address, source port, destination port, transport protocol, packet count, time interval, sum of packet sizes) associated with the particular packet flow. | packetDataSummary |  |
| current serving system identifier | Current VPLMN id of the target | current-Serving-System-Identifier |  |
| current serving SGSN-Number | An E.164 number of the current serving SGSN, that comes from the intercepted MAP message . | current-Serving-SGSN-Number |  |
| current serving SGSN-Address | An IP address of the current serving SGSN, that comes from the intercepted MAP message | current-Serving-SGSN- Address |  |
| current serving S4-SGSN-address | In case of S4-SGSN, this may be provided as Diameter id and realm of the serving S4-SGSN connected via S6d interface to the HSS. It is in the intercepted Diameter message. | current-Serving-SGSN- Address |  |
| previous serving system identifier | Previous VPLMN id of the target | previous-Serving-System-Identifier |  |
| previous serving SGSN-Number | An E.164 number of the previous serving SGSN, included in the intercepted MAP message. | previous-Serving-SGSN-Number |  |
| previous serving SGSN-Address | An IP address of the previous serving SGSN, included in the intercepted MAP message. | previous-Serving-SGSN- Address |  |
| previous serving S4-SGSN-address | In case of S4-SGSN, this may be provided as Diameter id and realm of the serving S4-SGSN connected via S6d interface to the HSS. | previous-Serving-SGSN- Address |  |
| requesting network identifier | The requesting network identifier PLMN id (Mobile Country Code and Mobile Network Country, -- defined in E212 [87]). | requesting-Network-Identifier |  |
| requesting node type | Type of requesting node such as MSC, SMS Centre, GMLC, MME, SGSN. | requesting-Node-Type |  |
| other update | Carrier specific information related to its implementation or subscription process on its HLR that will be transmit to LEMF in raw format. CSP will provide to LEMF elements to understand such data. | carrierSpecificData |  |

NOTE:	LIID parameter must be present in each record sent to the LEMF.

*****  END OF CHANGE 6 *********************

***** ******** CHANGE 7 *********************

<<< The following tables and subsequent text belong to the subclause 6.5.1.1>>>

Table 6.9H: LALS Target Positioning REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed IMEI |  |  |
| event date | M | Shall provide the date and time the report is created. |
| event time |  |  |
| network identifier | M | Network identifier of the LI LCS Client (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide the LALS location information, if the positioning is successful. |
| extended location parameters | O | If available, additional location information and associated QoS information. |
| LALS error code | C | Provide the error identification code, if the positioning is not successful. |

Table 6.9x: LALS Enhanced Location for IRI REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed IMEI |  |  |
| event date | M | Provide the date and time the LCS Report is available at LI LCS Client. |
| event time |  |  |
| network identifier | M | Network identifier of the LI LCS Client (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| correlation number | C | Provided for correlation with the IRI records of the call, if available in the corresponding LALS triggering event. |
| location information | C | Provide the LALS location information, if the positioning is successful. |
| extended location parameters | O | If available, additional location information and associated QoS information. |
| LALS error code | C | Provide the error identification code if the positioning is not successful. |

NOTE 1: 	See the TS 33.107 [19] for a detailed description of LALS. See Annex O for information on using of the PS ASN.1 information object for the LALS reporting.

NOTE 2: 	In some specific scenarios the amount of Enhanced Location for IRI reports data may overload the X2 and/or HI2 interfaces. To prevent the overload, a flow control for Enhanced Location for IRI Reports may be implemented, e.g. by limiting the frequency of the reports for individual target.

*****  END OF CHANGE 7  *********************

************  CHANGE 8  **********************

### 10.1.3	Correlation number

The Correlation Number is unique per EPS bearer/tunnel and is used for the following purposes:

-	correlate CC with IRI,

-	correlate different IRI records within one EPS bearer/tunnel,

- 	correlate LALS reports with the IRI records of the triggering events.

NOTE:	The Correlation Number is at a minimum unique for each concurrent communication (e.g. EPS bearer/tunnel) of a target within a lawful authorization. However when different protocols (i.e. GTP and PMIP) are used in the networks, different values for the correlation number can be generated by different nodes for the same communication.

In case of handover between different accesses involving a Gn/Gp interface (i.e. from E-UTRAN to 2G/3G and vice versa), the same correlation number for the PDP context/bearer shall be used before and after the handover during the same IRI transaction.

*****  END OF CHANGE 8  *********************

************  CHANGE 9  **********************

## 10.5	IRI for evolved packet domain

### 10.5.0	Introduction

The IRI will in principle be available in the following phases of a data transmission:

1.	At connection attempt when the target identity becomes active, at which time packet transmission may or may not occur (set up of a bearer/tunnel, target may be the originating or terminating party);

2.	At the end of a connection, when the target identity becomes inactive (removal of a bearer/tunnel);

3.	At certain times when relevant information are available.

In addition, information on non-transmission related actions of a target constitute IRI and is sent via HI2. Also, the EPS LALS reports convey via HI2 as IRI.

The IRI may be subdivided into the following categories:

1.	Control information for HI2 (e.g. correlation information);

2.	Basic data context information, for standard data transmission between two parties.

# The events defined in TS 33.107 [19] are used to generate records for the delivery via HI2.

There are several different event types received at DF2 level. According to each event, a Record is sent to the LEMF if this is required. In the case of LALS reports which are not associated with an event, a Record is sent to the LEMF without the event parameter.

The following table gives the mapping between event type received at DF2 level and record type sent to the LEMF. The applicability of the events to specific access (E-UTRAN, trusted non-3GPP access, untrusted non-3GPP access) and network protocols (GTP/PMIP S5/S8 interface) is specified in [19]. Additional events and mapping with IRI Record type are applicable to EPS in case of interworking between SGSN and PDN-GW over Gn/Gp interface, as specified in this document for PS interception.

*****  END OF CHANGE 9  *********************

************  CHANGE 10  **********************

<<< The following table belongs to the subclause 10.5.0 >>>

Table 10.5.2: Mapping between Events information and IRI information

| parameter | description | HI2 ASN.1 parameter |  |
| --- | --- | --- | --- |
| observed MSISDN | Target Identifier with the MSISDN of the target. | partyInformation (party-identity) |  |
| observed IMSI | Target Identifier with the IMSI of the target. | partyInformation (party-identity) |  |
| observed ME Id | Target Identifier with the ME Id of the target. | partyInformation (party-identity) |  |
| observed MN NAI | Target Identifier with the NAI of the target. | partyInformation (party-identity) |  |
| event type | Description which type of event is delivered | ePSevent |  |
| event date | Date of the event generation in the node | Timestamp |  |
| event time | Time of the event generation in the node |  |  |
| access point name | When provided by the MME, the parameter carries the Access Point Name provided by the UE. When provided by the S-GW/PDN-GW, it is the APN used for the PDN connection | aPN |  |
| APN-AMBR | Contains the Aggregate Maximum Bit Rate for the APN | aPN-AMBR |  |
| PDN type | Indicated the used IP version (IPv4, IPv6, IPv4/IPv6) | pDNType |  |
| PDN address allocation | Provides the IP version (IPv4, IPv6, IPv4/IPv6) and the IP address(es) allocated for the UE. | pDNAddressAllocation |  |
| Protocol Configuration Options | Are used to transfer parameters between the UE and the PDN-GW (e.g. address allocation preference by DHCP) | protConfigOptions |  |
| Attach type | Indicates the type of attach and may carry indication of handover in case of mobility with non-3GPP access. | attachType |  |
| RAT type | Radio Access Type | rATType |  |
| initiator | This field indicates whether the procedure is UE or network initiated. | initiator |  |
| Handover indication | Provides information that the procedure is triggered as part of a handover | handoverIndication, extendedHandoverIndication |  |
| Procedure Transaction Identifier | Identifies a set of messages belonging to the same procedure; the parameter is dynamically allocated by the UE | procedureTransactionId |  |
| EPS bearer identity | Identifies an EPS bearer for one UE accessing via E-UTRAN. It is allocated by the MME. | ePSBearerIdentity |  |
| Bearer activation/ deactivation type | Indicates the type of bearer being activated/deactivated, i.e. default or dedicated. | bearerActivationType, bearerDeactivationType |  |
| Linked EPS bearer identity | Indicates, in case of dedicated bearer, the EPS bearer identity of the default bearer. | linkedEPSBearerId |  |
| Switch off indicator | Indicates whether a detach procedure is due to a switch off situation or not. | detachType |  |
| Detach type | Parameter sent by the network to the UE to indicate the type of detach. | detachType |  |
| Traffic Flow Template (TFT) | Collection of all packet filters associated with the EPS bearer. | tFT |  |
| Traffic Aggregate Description (TAD) | Consists of the description of the packet filter(s) for the traffic flow aggregate. | trafficAggregateDescription |  |
| correlation number | Unique number for each target connection delivered to the LEMF, to help the LEA, to have a correlation between each target connection and the IRI. | ePSCorrelationNumber |  |
| lawful interception identifier | Unique number for each lawful authorization. | lawfulInterceptionIdentifier |  |
| location information | When authorized, this field provides the location information of the target that is present at the node at the time of event record production. | ePSlocationOfTheTarget |  |
| Old location information | Location information of the target before Tracking Area Update. | ePSlocationOfTheTarget |  |
| Failure reason | The reason for the failure or rejection of the Tracking Area Update | failedTAUReason |  |
| failed bearer activation reason | This field gives information about the reason for a failed bearer activation of the target. | failedBearerActivationReason |  |
| failed attach reason | This field gives information about the reason for a failed attach attempt of the target. | failedEUTRANAttachreason, status, code (depending on the protocol) |  |
| Session modification failure reason | This field gives information about the reason for a failed session modification attempt of the target | status |  |
| EPS bearer QOS | This field indicates the Quality of Service associated with the EPS bearer procedure. | ePSBearerqOS |  |
| bearer deactivation reason | This field gives information about the reason for bearer deactivation of the target. | bearerDeactivationCause |  |
| network identifier | Operator ID plus node address. | networkIdentifier |  |
| logicalFunctionInformation | Event source logical function identifier. | logicalFunctionInformation |  |
| Failed Bearer Modification reason | The reason for failure of Bearer Modification | failedBearerModReason |  |
| ULI Timestamp | Indicates the time when the User Location Information was acquired. | uLITimestamp |
| Lifetime | Lifetime of the tunnel; it is set to a nonzero value in case of registration or lifetime extension; is set to zero in case of deregistration. | lifetime |  |
| Access technology type | Indicates the Radio Access Type | accessTechnologyType |  |
| UE address info | Includes one or more IP addresses allocated to the UE. | iPv6HomeNetworkPrefix, iPv4HomeAddress, iPv6careOfAddress, iPv4careOf Address |  |
| Additional parameters | Additional information provided by the UE, such as protocol configuration options | protConfigurationOption |  |
| serving MME address | Diameter Origin-Host and Origin-Realm of the serving MME or its IP address. | servingMME-Address |  |
| Revocation trigger | Contains the reason which triggered a PDN-GW initiated PDN-disconnection (revocation) procedure. | revocationTrigger |  |
| Home Address | Contains the UE Home IP address | homeAddress |  |
| Home Agent Address | Contains the IP address of the Home Agent | homeAgentAddress |  |
| Requested IPv6 Home Prefix | The IPv6 Home Prefix requested by the UE. | requestedIPv6HomePrefix |  |
| Care of Address | The local IP address assigned to the UE by the Access Network. | careOfAddress |  |
| HSS/AAA address | The address of the HSS/AAA triggering a pDN-GW reallocation. | hSS-AAA-address |  |
| Target PDN-GW address | The address of the PDN-GW which the UE will be reallocated to. | targetPDN-GW-Address |  |
| Foreign domain address | The relevant IP address in the foreign domain. | foreignDomainAddress |  |
| Visited network identifier | An identifier that allows the home network to identify the visited network inside the EPS Serving System Update for non 3GPP access, coded according to [53] | visitedNetworkId |  |
| DHCP v4 Address Allocation Indication | Indicates that DHCPv4 is to be used to allocate the IPv4 address to the UE | dHCPv4AddressAllocationInd |  |
| Serving Network | Identifies, for E-UTRAN access, the serving network the UE is attached to | servingNetwork |  |
| Request type | Provides the type of UE requested PDN connectivity | requestType |  |
| Failed reason | Provides the failure cause for UE requested PDN connectivity | uEReqPDNConnFailReason |  |
| destination IP address | Identifies the destination IP address of a packet. | destinationIPAddress |  |
| destination port number | Identifies the destination port number of a packet | destinationPortNumber |  |
| source IP address | Identifies the source IP address of a packet. | sourceIPAddress |  |
| source port number | Identifies the source port number of a packet. | sourcePortNumber |  |
| transport protocol | Identifies the transport protocol (i.e., Protocol Field in IPv4 or Next Header Field in IPv6. | transportProtocol |  |
| flow label | The field in the IPv6 header that is used by a source to label packets of a flow (see RFC 3697 [c]) | flowLabel |  |
| packet count | The number of packets detected and reported in a particular packet data summary report. | packetCount |  |
| packet size | The size of a packet (i.e., Total Length Field in IPv4 [a] or Payload Length Field in IPv6 [b]) | packetsize |  |
| packet direction | Identifies the direction of the intercepted packet (from target or to target) | packetDirection |  |
| packet data header copy | Provides a copy of the packet headers including IP layer and next layer, and extensions, but excluding content. | packetDataHeaderCopy |  |
| summary period | Provides the period of time during which the packets of the summary report were sent or received by the target. | summaryPeriod |  |
| sum of packet sizes | Sum of values in Total Length Fields in IPv4 packets or Payload Length Field in IPv6 packets. | sumOfPacketSizes |  |
| packet data summary reason | Provides the reason for a summary report. | packetDataSummaryReason |  |
| packet data summary | For each particular packet flow, identifies pertinent reporting information (e.g., source IP address, destination IP address, source port, destination port, transport protocol, packet count, time interval, sum of packet sizes) associated with the particular packet flow. | packetDataSummary |  |
| CSG Identity | Uniquely identifies a CSG within a PLMN. | csgIdentity |  |
| HeNB Identity | Identifies the HeNB providing access to a target UE. | heNBIdentity |  |
| HeNB IP address | Identifies the IP Address associated with an HeNB providing access to a target UE. | heNBiPAddress |  |
| HeNB Location | Identifies the location of an HeNB providing access to a target UE. | heNBLocation |  |
| Tunnel Protocol | Identifies the tunnel protocol used to transport the signalling and communications between the HeNB and the EPC. | tunnelProtocol |  |
| UE Local IP address | Identifies the UE local IP address (IP SEC terminal Point) observed by the ePDG and PDN-GW, based on local policy for Fixed Broadband access network interworking, in case the GTP based S2b interface [46] is used. | uELocalIPAddress |  |
| UE UDP Port | Identifies the UE UDP port (IP SEC terminal Point) reported on GTP based S2b interface [46] if NAT is detected and local IP address is present for Fixed Broadband access network interworking. | uEUdpPort |  |
| serving system identifier | VPLMN ID of the serving system or of the third party network interworking, included in the Diameter AVP message with the HSS | serving-System-Identifier |  |
| previous serving system identifier | Previous VPLMN id of the target (Mobile Country Code and Mobile Network Country; defined in E212 [87]) ). | previous-Serving-System-Identifier |  |
| previous serving MME address | Diameter Origin-Host and Origin-Realm of the previous serving MME. | previous-Serving-MME-Address |  |
| current visited Network Identifier | An identifier that allows the home network to identify the current visited network (Mobile Country Code and Mobile Network Country defined in E212 [87] are included in this identifier) [53]. | current-Serving-System-Identifier |  |
| current serving MME address | Provide the previous Diameter Origin-Host and Origin-Realm of the previous current MME | current-Serving-MME-Address |  |
| requesting network identifier | The requesting network identifier PLMN id (Mobile Country Code and Mobile Network Country, -- defined in E212 [87]). | requesting-Network-Identifier |  |
| requesting node type | Type of requesting node such as MSC, SMS Centre, GMLC, MME, SGSN. | requesting-Node-Type |  |
| other update | Carrier specific information related to its implementation or subscription process on its HSS. Raw data will be provided. CSP will provide to LEMF elements to understand such data. | carrierSpecificData |  |
| WLAN location information | Provides location information in form of TWAN identifier, if available at ePDG/PDN-GW, in case the GTP based s2b interface [46] is used. | tWANIdentifier |  |
| WLAN location timestamp | Provides location information timestamp in form of TWAN identifier timestamp, if available at ePDG/PDN-GW, in case the GTP based s2b interface [46] is used. | tWANIdentifierTimestamp |  |
| LALS location information | Location information provided by LALS | locationOfTheTarget |  |
| extended location parameters | Additional location information and QoS information | extendedLocParameters |  |
| LALS error code | Positioning error identification code | locationErrorCode |  |

NOTE:	LIID parameter must be present in each record sent to the LEMF.

*****  END OF CHANGE 10  *********************

************  CHANGE 11  **********************

<<< The following tables and subsequent text belong to the subclause 10.5.1.1>>>

Table 10.5.1.1.21: LALS Target Positioning REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME id |  |  |
| event date | M | Shall provide the date and time the report is created. |
| event time |  |  |
| network identifier | M | Network identifier of the LI LCS Client (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide the LALS location information, if the positioning is successful |
| extended location parameters | O | If available, additional location information and associated QoS information. |
| LALS error code | C | Provide the error identification code if the positioning is not successful. |

Table 10.5.1.1.x: LALS Enhanced Location for IRI REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed IMEI |  |  |
| event date | M | Provide the date and time the LCS Report is available at LI LCS Client. |
| event time |  |  |
| network identifier | M | Network identifier of the LI LCS Client (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| correlation number | C | Provided for correlation with the IRI records of the call, if available in the corresponding LALS triggering event. |
| location information | C | Provide the LALS location information, if the positioning is successful. |
| extended location parameters | O | If available, additional location information and associated QoS information. |
| LALS error code | C | Provide the error identification code if the positioning is not successful. |

NOTE 1: 	See the TS 33.107 [19] for a detailed description of LALS. See Annex O for information on using of the EPS ASN.1 information object for the LALS reporting.

NOTE 2: 	In some specific scenarios the amount of Enhanced Location for IRI reports data may overload the X2 and/or HI2 interfaces. To prevent the overload, a flow control for Enhanced Location for IRI Reports may be implemented, e.g. by limiting the frequency of the reports for individual target.

*****  END OF CHANGE 11  *********************

**************CHANGE 12  *********************

# B.3	Intercept related information (HI2 PS and IMS)

Declaration of ROSE operation umts-sending-of-IRI is ROSE delivery mechanism specific. When using FTP delivery mechanism, data UmtsIRIsContent must be considered.

# ASN1 description of IRI (HI2 interface)

UmtsHI2Operations {itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulintercept(2) threeGPP(4) hi2(1)  r14 (14) version-0 (0)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

IMPORTS

OPERATION,

ERROR

FROM Remote-Operations-Information-Objects

{joint-iso-itu-t(2) remote-operations(4) informationObjects(5) version1(0)}

LawfulInterceptionIdentifier,

TimeStamp,

Network-Identifier,

National-Parameters,

National-HI2-ASN1parameters,

DataNodeAddress,

IPAddress,

IP-value,

X25Address

FROM HI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulIntercept(2) hi2(1) version18(18)}; -- Imported from TS 101 671v3.12.1

-- Object Identifier Definitions

-- Security DomainId

lawfulInterceptDomainId OBJECT IDENTIFIER ::= {itu-t(0) identified-organization(4) etsi(0)

securityDomain(2) lawfulIntercept(2)}

-- Security Subdomains

threeGPPSUBDomainId OBJECT IDENTIFIER ::= {lawfulInterceptDomainId threeGPP(4)}

hi2DomainId OBJECT IDENTIFIER	::= {threeGPPSUBDomainId hi2(1)  r14 (14) version-0 (0)}

umts-sending-of-IRI  OPERATION ::=

{

ARGUMENT	UmtsIRIsContent

ERRORS		{ OperationErrors }

CODE		global:{threeGPPSUBDomainId hi2(1) opcode(1)}

}

-- Class 2 operation . The timer shall be set to a value between 3 s and 240 s.

-- The timer.default value is 60s.

-- NOTE:	The same note as for HI management operation applies.

UmtsIRIsContent		::= CHOICE

{

umtsiRIContent			UmtsIRIContent,

umtsIRISequence			UmtsIRISequence

}

UmtsIRISequence		::= SEQUENCE OF UmtsIRIContent

-- Aggregation of UmtsIRIContent is an optional feature.

-- It may be applied in cases when at a given point in time

-- several IRI records are available for delivery to the same LEA destination.

-- As a general rule, records created at any event shall be sent

-- immediately and not withheld in the DF or MF in order to

-- apply aggragation.

-- When aggregation is not to be applied,

-- UmtsIRIContent needs to be chosen.

UmtsIRIContent		::= CHOICE

{

iRI-Begin-record		[1] IRI-Parameters,	-- include at least one optional parameter

iRI-End-record			[2] IRI-Parameters,

iRI-Continue-record		[3] IRI-Parameters,	-- include at least one optional parameter

iRI-Report-record		[4] IRI-Parameters	-- include at least one optional parameter

}

unknown-version 		ERROR ::= { CODE local:0}

missing-parameter		ERROR ::= { CODE local:1}

unknown-parameter-value	ERROR ::= { CODE local:2}

unknown-parameter		ERROR ::= { CODE local:3}

OperationErrors ERROR ::=

{

unknown-version |

missing-parameter |

unknown-parameter-value |

unknown-parameter

}

-- This values may be sent by the LEMF, when an operation or a parameter is misunderstood.

-- Parameters having the same tag numbers must be identical in Rel-5 and onwards modules.

IRI-Parameters		::= SEQUENCE

{

hi2DomainId				[0]	OBJECT IDENTIFIER,  -- 3GPP HI2 domain

iRIversion				[23] ENUMERATED

{

version2 (2),

...,

version3 (3),

version4 (4),

-- note that version5 (5) cannot be used as it was missed in the version 5 of this

-- ASN.1 module.

version6 (6),

-- vesion7(7) was ommited to align with ETSI TS 101 671.

lastVersion (8)	} OPTIONAL,

-- Optional parameter "iRIversion" (tag 23) was always redundant in 33.108, because

-- the object identifier "hi2DomainId" was introduced into "IRI Parameters" in the

-- initial version of 33.108v5.0.0. In order to keep backward compatibility, even when

-- the version of the "hi2DomainId" parameter will be incremented it is recommended

-- to always send to LEMF the same: enumeration value "lastVersion(8)".

-- if not present, it means version 1 is handled

lawfulInterceptionIdentifier	[1] LawfulInterceptionIdentifier,

-- This identifier is associated to the target.

timeStamp				[3] TimeStamp,

-- date and time of the event triggering the report.)

initiator 				[4] ENUMERATED

{

not-Available		(0),

originating-Target	(1),

-- in case of GPRS, this indicates that the PDP context activation, modification

-- or deactivation is MS requested

terminating-Target	(2),

-- in case of GPRS, this indicates that the PDP context activation, modification or

-- deactivation is network initiated

...

} OPTIONAL,

locationOfTheTarget		[8] Location OPTIONAL,

-- location of the target

partyInformation 		[9] SET SIZE (1..10) OF PartyInformation OPTIONAL,

-- This parameter provides the concerned party, the identiy(ies) of the party

--)and all the information provided by the party.

serviceCenterAddress	[13] PartyInformation OPTIONAL,

-- e.g. in case of SMS message this parameter provides the address of  the relevant

-- server

sMS						[14] SMS-report OPTIONAL,

-- this parameter provides the SMS content and associated information

national-Parameters		[16] National-Parameters OPTIONAL,

gPRSCorrelationNumber	[18] GPRSCorrelationNumber OPTIONAL,

gPRSevent 				[20] GPRSEvent OPTIONAL,

-- This information is used to provide particular action of the target

-- such as attach/detach

sgsnAddress 			[21] DataNodeAddress OPTIONAL,

gPRSOperationErrorCode 	[22] GPRSOperationErrorCode OPTIONAL,

ggsnAddress 			[24] DataNodeAddress OPTIONAL,

qOS						[25] UmtsQos OPTIONAL,

networkIdentifier		[26] Network-Identifier OPTIONAL,

sMSOriginatingAddress 	[27] DataNodeAddress OPTIONAL,

sMSTerminatingAddress 	[28] DataNodeAddress OPTIONAL,

iMSevent				[29] IMSevent OPTIONAL,

sIPMessage				[30] OCTET STRING  OPTIONAL,

servingSGSN-number		[31] OCTET STRING (SIZE (1..20))	OPTIONAL,

-- Coded according to 3GPP TS 29.002 [4] and 3GPP TS 23.003 25].

servingSGSN-address		[32] OCTET STRING (SIZE (5..17)) 	OPTIONAL,

-- Octets are coded according to 3GPP TS 23.003 [25]

...,

-- Tag			[33] was taken into use by ETSI module in TS 101 671v2.13.1

ldiEvent	 			[34] LDIevent OPTIONAL,

correlation 			[35] CorrelationValues OPTIONAL,

mediaDecryption-info	[36] MediaDecryption-info OPTIONAL,

servingS4-SGSN-address	[37] OCTET STRING OPTIONAL,

-- Diameter Origin-Host and Origin-Realm of the S4-SGSN based on the TS 29.272 [59].

-- Only the data fields from the Diameter AVPs are provided concatenated

-- with a semicolon to populate this field.

sipMessageHeaderOffer   [38] OCTET STRING OPTIONAL,

sipMessageHeaderAnswer  [39] OCTET STRING OPTIONAL,

sdpOffer                [40] OCTET STRING OPTIONAL,

sdpAnswer               [41] OCTET STRING OPTIONAL,

uLITimestamp            [42] OCTET STRING (SIZE (8)) OPTIONAL,

-- Coded according to 3GPP TS 29.060 [17]; Only the ULI Timestamp value is reported.

packetDataHeaderInformation		 [43] PacketDataHeaderInformation	OPTIONAL,

mediaSecFailureIndication       [44] MediaSecFailureIndication OPTIONAL,

pANI-Header-Info		[45] SEQUENCE OF PANI-Header-Info		OPTIONAL,

-- information extracted from P-Access-Network-Info headers of SIP message;

-- described in TS 24.229 §7.2A.4 [76]

imsVoIP					[46] IMS-VoIP-Correlation	OPTIONAL,

xCAPmessage				[47] OCTET STRING OPTIONAL,

-- The entire HTTP contents of any of the target's IMS supplementary service setting

-- management or manipulation XCAP messages, mainly made through the Ut

-- interface defined in the 3GPP TS 24 623 [77].

ccUnavailableReason		[48] PrintableString	OPTIONAL,

carrierSpecificData					[49] OCTET STRING OPTIONAL,

-- Copy of raw data specified by the CSP or his vendor related to HSS.

current-Previous-Systems			[50] Current-Previous-Systems OPTIONAL,

change-Of-Target-Identity				[51] Change-Of-Target-Identity OPTIONAL,

requesting-Network-Identifier		[52] OCTET STRING OPTIONAL,

-- the requesting network identifier PLMN id (Mobile Country Code and Mobile Network Country,

-- defined in E212 [87]).

requesting-Node-Type					[53] Requesting-Node-Type OPTIONAL,

serving-System-Identifier				[54] OCTET STRING OPTIONAL,

-- the requesting network identifier (Mobile Country Code and Mobile Network Country,

-- defined in E212 [87]).

extendedLocParameters 	[55] 	ExtendedLocParameters OPTIONAL, -- LALS extended parameters

locationErrorCode		[56]	LocationErrorCode OPTIONAL,	-- LALS error code

national-HI2-ASN1parameters	[255]	National-HI2-ASN1parameters	OPTIONAL

}

-- Parameters having the same tag numbers must be identical in Rel-5 and onwards modules

*****  END OF CHANGE 12  *********************

************  CHANGE 13  **********************

<<<<<The following definitions belong to the UmtsHI2Operations ASN.1 module in Annex B.3 >>>>

ReportInterval ::= SEQUENCE

{

firstPacketTimeStamp	[0] TimeStamp,

lastPacketTimeStamp		[1] TimeStamp,

...

}

-- LALS extended location parameters are mapped from the MLP pos element parameters

-- and attributes defined in [xx], version 3.4. For details see specific [xx] clauses refered below.

ExtendedLocParameters ::= SEQUENCE

{

posMethod		[0]		PrintableString OPTIONAL, -- clause 5.3.72.1

mapData			[1] 								-- clause 5.2.2.3

CHOICE 		{base64Map 	[0] PrintableString, 	-- clause 5.3.11

url 		[1] PrintableString 	-- clause 5.3.135

} OPTIONAL,

altitude		[2]

SEQUENCE 	{alt PrintableString, -- clause 5.3.4

alt-uncertainty PrintableString OPTIONAL -- clause 5.3.6

} OPTIONAL,

speed			[3] 	PrintableString OPTIONAL, -- clause 5.3.116

direction		[4]		PrintableString OPTIONAL, -- clause 5.3.25

level-conf		[5]		PrintableString OPTIONAL, -- clause 5.3.51

qOS-not-met		[6] 	BOOLEAN OPTIONAL, -- clause 5.3.94

motionStateList [7] 								-- clause 5.2.2.3

SEQUENCE	{primaryMotionState 	[0] PrintableString, -- clause 5.3.23

secondaryMotionState 	[1] SEQUENCE OF PrintableString OPTIONAL,

confidence 				[2] PrintableString -- clause 5.3.68

} OPTIONAL,

floor    		[8]

SEQUENCE 	{floor-number PrintableString, -- clause 5.3.38

floor-number-uncertainty PrintableString OPTIONAL -- clause 5.3.39

} OPTIONAL,

additional-info	[9] PrintableString OPTIONAL, -- clause 5.3.1

-- the following parameter contains a copy of the unparsed XML code of MLP ‘pd’ element.

-- This parameter is present when the LI-LCS client cannot map the MLP ‘pd’ element into

-- an ASN.1 Location object.

-- It may also be present based on national regulations.

lALS-rawMLPPosData	[10] PrintableString OPTIONAL,

...

}

LocationErrorCode ::= INTEGER (1..699)

-- LALS location error codes are the OMA MLP result identifiers defined in [xx], Clause 5.4

END -- OF UmtsHI2Operations

*****  END OF CHANGE 13  *********************

************  CHANGE 14  **********************

## B.3a	Interception related information (HI2 CS)

For North America, the use of J-STD-25 A [23] is recommended.

Declaration of ROSE operation sending-of-IRI is ROSE delivery mechanism specific. When using FTP delivery mechanism, data IRI-Content must be considered.

ASN1 description of IRI (HI2 CS interface)

UmtsCS-HI2Operations

{itu-t (0) identified-organization (4) etsi (0) securityDomain (2) lawfulIntercept (2) threeGPP(4) hi2CS (3)  r14 (14) version-0 (0)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

IMPORTS OPERATION,

ERROR

FROM Remote-Operations-Information-Objects

{joint-iso-itu-t (2) remote-operations(4) informationObjects(5) version1(0)}

LawfulInterceptionIdentifier,

TimeStamp,

Intercepted-Call-State,

PartyInformation,

CallContentLinkCharacteristics,

CommunicationIdentifier,

CC-Link-Identifier,

National-Parameters,

National-HI2-ASN1parameters

FROM HI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulIntercept(2) hi2(1) version9(9)} -- Imported from TS 101 671v2.13.1

Location,

SMS-report,

ExtendedLocParameters,

LocationErrorCode

FROM UmtsHI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulintercept(2) threeGPP(4) hi2(1) r14(14) version-0(0)};

-- Object Identifier Definitions

-- Security DomainId

lawfulInterceptDomainId OBJECT IDENTIFIER ::= {itu-t(0) identified-organization(4) etsi(0)

securityDomain(2) lawfulIntercept(2)}

-- Security Subdomains

threeGPPSUBDomainId OBJECT IDENTIFIER ::= {lawfulInterceptDomainId threeGPP(4)}

hi2CSDomainId OBJECT IDENTIFIER ::= {threeGPPSUBDomainId hi2CS(3)  r14 (14) version-0 (0)}

umtsCS-sending-of-IRI  OPERATION ::=

{

ARGUMENT	UmtsCS-IRIsContent

ERRORS		{ OperationErrors }

CODE		global:{ threeGPPSUBDomainId hi2CS(3) opcode(1)}

}

-- Class 2 operation. The timer shall be set to a value between 3 s and 240 s.

-- The timer.default value is 60s.

-- NOTE: The same note as for HI management operation applies.

UmtsCS-IRIsContent		::= CHOICE

{

iRIContent			UmtsCS-IRIContent,

iRISequence			UmtsCS-IRISequence

}

UmtsCS-IRISequence		::= SEQUENCE OF UmtsCS-IRIContent

-- Aggregation of UmtsCS-IRIContent is an optional feature.

-- It may be applied in cases when at a given point in time several IRI records are

-- available for delivery to the same LEA destination.

-- As a general rule, records created at any event shall be sent immediately and shall

-- not held in the DF or MF in order to apply aggregation.

-- When aggregation is not to be applied, UmtsCS-IRIContent needs to be chosen.

UmtsCS-IRIContent		::= CHOICE

{

iRI-Begin-record		[1] IRI-Parameters,

--at least one optional parameter must be included within the iRI-Begin-Record

iRI-End-record			[2] IRI-Parameters,

iRI-Continue-record		[3] IRI-Parameters,

--at least one optional parameter must be included within the iRI-Continue-Record

iRI-Report-record		[4] IRI-Parameters,

--at least one optional parameter must be included within the iRI-Report-Record

...

}

unknown-version 		ERROR ::= { CODE local:0}

missing-parameter		ERROR ::= { CODE local:1}

unknown-parameter-value	ERROR ::= { CODE local:2}

unknown-parameter		ERROR ::= { CODE local:3}

OperationErrors ERROR ::=

{

unknown-version |

missing-parameter |

unknown-parameter-value |

unknown-parameter

}

--These values may be sent by the LEMF, when an operation or a parameter is misunderstood.

IRI-Parameters		::= SEQUENCE

{

hi2CSDomainId						[0]	OBJECT IDENTIFIER, -- 3GPP HI2 CS domain

iRIversion							[23] ENUMERATED

{

version1(1),

...,

version2(2),

version3(3),

-- versions 4-7 were ommited to align with UmtsHI2Operations.

lastVersion(8)

} OPTIONAL,

-- Optional parameter "iRIversion" (tag 23) was always redundant in 33.108, because

-- the object identifier "hi2CSDomainId" was introduced into "IRI Parameters" with the

-- initial HI2 CS domain module in 33.108v6.1.0. In order to keep backward compatibility,

-- even when the version of the "hi2CSDomainId" parameter will be incremented it is

-- recommended to always send to LEMF the same: enumeration value "lastVersion(8)".

-- if not present, it means version 1 is handled

lawfulInterceptionIdentifier		[1] LawfulInterceptionIdentifier,

-- This identifier is associated to the target.

communicationIdentifier				[2] CommunicationIdentifier,

-- used to uniquely identify an intercepted call.

timeStamp							[3] TimeStamp,

-- date and time of the event triggering the report.

intercepted-Call-Direct 			[4] ENUMERATED

{

not-Available(0),

originating-Target(1),

terminating-Target(2),

...

} OPTIONAL,

intercepted-Call-State 				[5] Intercepted-Call-State OPTIONAL,

-- Not required for UMTS. May be included for backwards compatibility to GSM	ringingDuration 					[6] OCTET STRING (SIZE (3)) OPTIONAL,

-- Duration in seconds. BCD coded : HHMMSS

-- Not required for UMTS. May be included for backwards compatibility to GSM	conversationDuration 				[7] OCTET STRING (SIZE (3)) OPTIONAL,

-- Duration in seconds. BCD coded : HHMMSS

-- Not required for UMTS. May be included for backwards compatibility to GSM

locationOfTheTarget					[8] Location OPTIONAL,

-- location of the target

partyInformation 					[9] SET SIZE (1..10) OF PartyInformation OPTIONAL,

-- This parameter provides the concerned party (Originating, Terminating or forwarded

-- party), the identity(ies) of the party and all the information provided by the party.

callContentLinkInformation			[10] SEQUENCE

{

cCLink1Characteristics		[1] CallContentLinkCharacteristics OPTIONAL,

-- information concerning the Content of Communication Link Tx channel established

-- toward the LEMF (or the sum signal channel, in case of mono mode).

cCLink2Characteristics		[2] CallContentLinkCharacteristics OPTIONAL,

-- information concerning the Content of Communication Link Rx channel established

-- toward the LEMF.

...

} OPTIONAL,

release-Reason-Of-Intercepted-Call	[11] OCTET STRING (SIZE (2)) OPTIONAL,

-- Release cause coded in [31] format.

-- This parameter indicates the reason why the

-- intercepted call cannot be established or why the intercepted call has been

-- released after the active phase.

nature-Of-The-intercepted-call		[12] ENUMERATED

{

--Not required for UMTS. May be included for backwards compatibility to GSM

--Nature of the intercepted "call":

gSM-ISDN-PSTN-circuit-call(0),

-- the possible UUS content is sent through the HI2 or HI3 "data" interface

-- the possible call content call is established through the HI3 „circuit„ interface

gSM-SMS-Message(1),

-- the SMS content is sent through the HI2 or HI3 "data" interface

uUS4-Messages(2),

-- the UUS content is sent through the HI2 or HI3 "data" interface

tETRA-circuit-call(3),

-- the possible call content call is established through the HI3 "circuit" interface

-- the possible data are sent through the HI3 "data" interface

teTRA-Packet-Data(4),

-- the data are sent through the HI3 "data" interface

gPRS-Packet-Data(5),

-- the data are sent through the HI3 "data" interface

...

} OPTIONAL,

serviceCenterAddress				[13] PartyInformation OPTIONAL,

-- e.g. in case of SMS message this parameter provides the address of  the relevant

-- server within the calling (if server is originating) or called

-- (if server is terminating) party address parameters

sMS									[14] SMS-report OPTIONAL,

-- this parameter provides the SMS content and associated information

cC-Link-Identifier					[15] CC-Link-Identifier OPTIONAL,

-- Depending on a network option, this parameter may be used to identify a CC link

-- in case of multiparty calls.

national-Parameters					[16] National-Parameters OPTIONAL,

...,

umts-Cs-Event						[33] Umts-Cs-Event OPTIONAL,

-- Care should be taken to ensure additional parameter numbering does not conflict with

-- ETSI TS 101 671 or Annex B.3 of this document (PS HI2).

serving-System-Identifier			[34] OCTET STRING OPTIONAL,

-- the serving network identifier PLMN id (MNC, Mobile Country Code and MNC, Mobile Network

-- Country, defined in E212 [87]) and 3GPP TR 21.905 [38].

carrierSpecificData					[35] OCTET STRING OPTIONAL,

-- Copy of raw data specified by the CSP or his vendor related to HLR.

current-Previous-Systems			[36] Current-Previous-Systems OPTIONAL,

change-Of-Target-Identity			[37] Change-Of-Target-Identity OPTIONAL,

requesting-Network-Identifier		[38] OCTET STRING OPTIONAL,

-- the requesting network identifier PLMN id (Mobile Country Code and Mobile Network Country,

-- defined in E212 [87]).

requesting-Node-Type					[39] Requesting-Node-Type OPTIONAL,

extendedLocParameters 	 [40] 	ExtendedLocParameters OPTIONAL, -- LALS extended parameters

locationErrorCode		  [41]	LocationErrorCode OPTIONAL,	-- LALS error code

national-HI2-ASN1parameters			[255]	National-HI2-ASN1parameters	OPTIONAL

}

*****  END OF CHANGE 14  *********************

**************CHANGE 15  *********************

# B.9	Intercept related information (HI2 SAE/EPS and IMS)

Declaration of ROSE operation eps-sending-of-IRI is ROSE delivery mechanism specific. When using FTP delivery mechanism, data EpsIRIsContent must be considered.

# ASN1 description of IRI (HI2 interface)

EpsHI2Operations {itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulintercept(2) threeGPP(4) hi2eps(8) r14(14) version-0 (0)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

IMPORTS

OPERATION,

ERROR

FROM Remote-Operations-Information-Objects

{joint-iso-itu-t(2) remote-operations(4) informationObjects(5) version1(0)}

LawfulInterceptionIdentifier,

TimeStamp,

Network-Identifier,

National-Parameters,

National-HI2-ASN1parameters,

DataNodeAddress,

IPAddress,

IP-value,

X25Address

FROM HI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulIntercept(2) hi2(1) version18(18)} -- Imported from TS 101 671v3.12.1

CivicAddress,

ExtendedLocParameters,

LocationErrorCode

FROM UmtsHI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulintercept(2) threeGPP(4) hi2(1)  r14 (14) version-0 (0)};

-- Imported from 3GPP TS 33.108, UMTS PS HI2

-- Object Identifier Definitions

-- Security DomainId

lawfulInterceptDomainId OBJECT IDENTIFIER ::= {itu-t(0) identified-organization(4) etsi(0)

securityDomain(2) lawfulIntercept(2)}

-- Security Subdomains

threeGPPSUBDomainId OBJECT IDENTIFIER ::= {lawfulInterceptDomainId threeGPP(4)}

hi2epsDomainId OBJECT IDENTIFIER	::= {threeGPPSUBDomainId hi2eps(8) r14(14) version-0 (0)}

eps-sending-of-IRI  OPERATION ::=

{

ARGUMENT	EpsIRIsContent

ERRORS		{ OperationErrors }

CODE		global:{threeGPPSUBDomainId hi2eps(8) opcode(1)}

}

-- Class 2 operation . The timer shall be set to a value between 3 s and 240 s.

-- The timer.default value is 60s.

-- NOTE:	The same note as for HI management operation applies.

EpsIRIsContent		::= CHOICE

{

epsiRIContent			EpsIRIContent,

epsIRISequence			EpsIRISequence

}

EpsIRISequence		::= SEQUENCE OF EpsIRIContent

-- Aggregation of EpsIRIContent is an optional feature.

-- It may be applied in cases when at a given point in time

-- several IRI records are available for delivery to the same LEA destination.

-- As a general rule, records created at any event shall be sent

-- immediately and not withheld in the DF or MF in order to

-- apply aggragation.

-- When aggregation is not to be applied,

-- EpsIRIContent needs to be chosen.

-- EpsIRIContent includes events that correspond to EPS and UMTS/GPRS.

EpsIRIContent		::= CHOICE

{

iRI-Begin-record		[1] IRI-Parameters,	-- include at least one optional parameter

iRI-End-record			[2] IRI-Parameters,

iRI-Continue-record		[3] IRI-Parameters,	-- include at least one optional parameter

iRI-Report-record		[4] IRI-Parameters	-- include at least one optional parameter

}

-- the EpsIRIContent may provide events that correspond to UMTS/GPRS as well.

unknown-version 		ERROR ::= { CODE local:0}

missing-parameter		ERROR ::= { CODE local:1}

unknown-parameter-value	ERROR ::= { CODE local:2}

unknown-parameter		ERROR ::= { CODE local:3}

OperationErrors ERROR ::=

{

unknown-version |

missing-parameter |

unknown-parameter-value |

unknown-parameter

}

-- These values may be sent by the LEMF, when an operation or a parameter is misunderstood.

-- Parameters having the same tag numbers must be identical in Rel-5 and onwards modules.

IRI-Parameters		::= SEQUENCE

{

hi2epsDomainId				[0]	OBJECT IDENTIFIER,  -- 3GPP HI2 EPS domain

lawfulInterceptionIdentifier	[1] LawfulInterceptionIdentifier,

-- This identifier is associated to the target.

timeStamp				[3] TimeStamp,

-- date and time of the event triggering the report.)

initiator 				[4] ENUMERATED

{

not-Available		(0),

originating-Target	(1),

-- in case of GPRS, this indicates that the PDP context activation, modification

-- or deactivation is MS requested

-- in case of EPS, this indicated that the EPS detach, bearer activation, modification

-- or deactivation is UE requested

terminating-Target	(2),

-- in case of GPRS, this indicates that the PDP context activation, modification or

-- deactivation is network initiated

-- in case of EPS, this indicated that the EPS detach, bearer activation, modification

-- or deactivation is network initiated

...

} OPTIONAL,

locationOfTheTarget		[8] Location OPTIONAL,

-- location of the target

partyInformation 		[9] SET SIZE (1..10) OF PartyInformation OPTIONAL,

-- This parameter provides the concerned party, the identiy(ies) of the party

--)and all the information provided by the party.

serviceCenterAddress	[13] PartyInformation OPTIONAL,

-- e.g. in case of SMS message this parameter provides the address of  the relevant

-- server

sMS						[14] SMS-report OPTIONAL,

-- this parameter provides the SMS content and associated information

national-Parameters		[16] National-Parameters OPTIONAL,

ePSCorrelationNumber	[18] EPSCorrelationNumber OPTIONAL,

-- this parameter provides GPRS Correlation number when the event corresponds to UMTS/GPRS.

ePSevent 				[20] EPSEvent OPTIONAL,

-- This information is used to provide particular action of the target

-- such as attach/detach

sgsnAddress 			[21] DataNodeAddress OPTIONAL,

gPRSOperationErrorCode 	[22] GPRSOperationErrorCode OPTIONAL,

ggsnAddress 			[24] DataNodeAddress OPTIONAL,

qOS						[25] UmtsQos OPTIONAL,

networkIdentifier		[26] Network-Identifier OPTIONAL,

sMSOriginatingAddress 	[27] DataNodeAddress OPTIONAL,

sMSTerminatingAddress 	[28] DataNodeAddress OPTIONAL,

iMSevent				[29] IMSevent OPTIONAL,

sIPMessage				[30] OCTET STRING  OPTIONAL,

servingSGSN-number		[31] OCTET STRING (SIZE (1..20))	OPTIONAL,

servingSGSN-address		[32] OCTET STRING (SIZE (5..17)) 	OPTIONAL,

-- Octets are coded according to 3GPP TS 23.003 [25]

...,

-- Tag			[33] was taken into use by ETSI module in TS 101 671v2.13.1

ldiEvent	 			[34] LDIevent OPTIONAL,

correlation 			[35] CorrelationValues OPTIONAL,

ePS-GTPV2-specificParameters   [36] EPS-GTPV2-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of GTPV2 based intercepted messages

ePS-PMIP-specificParameters    [37] EPS-PMIP-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of PMIP based intercepted messages

ePS-DSMIP-SpecificParameters   [38] EPS-DSMIP-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of DSMIP based intercepted messages

ePS-MIP-SpecificParameters     [39] EPS-MIP-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of MIP based intercepted messages

servingNodeAddress             [40] OCTET STRING               OPTIONAL,

-- this parameter is kept for backward compatibility only and should not be used

-- as it has been superseeded by parameter visitedNetworkId

visitedNetworkId               [41] UTF8String                 OPTIONAL,

-- contains the visited network identifier inside the EPS Serving System Update for

-- non 3GPP access, coded according to [53]

mediaDecryption-info	       [42] MediaDecryption-info OPTIONAL,

servingS4-SGSN-address	       [43] OCTET STRING OPTIONAL,

-- Diameter Origin-Host and Origin-Realm of the S4-SGSN based on the TS 29.272 [59].

-- Only the data fields from the Diameter AVPs are provided concatenated

-- with a semicolon to populate this field.

sipMessageHeaderOffer   [44] OCTET STRING OPTIONAL,

sipMessageHeaderAnswer  [45] OCTET STRING OPTIONAL,

sdpOffer                [46] OCTET STRING OPTIONAL,

sdpAnswer               [47] OCTET STRING OPTIONAL,

uLITimestamp            [48] OCTET STRING (SIZE (8)) OPTIONAL,

-- Coded according to 3GPP TS 29.060 [17]; Only the ULI Timestamp value is reported.

packetDataHeaderInformation		   [49] PacketDataHeaderInformation	OPTIONAL,

mediaSecFailureIndication          [50] MediaSecFailureIndication OPTIONAL,

csgIdentity			[51] OCTET STRING (SIZE (4)) OPTIONAL,  -- Octets are coded

-- according to 3GPP TS 23.003 [25].  The 27 bits specified in TS 23.003 shall be encoded as.

-- follows The most significant bit of the CSG Identity shall be encoded in the most

-- significant bit of the first octet of the octet string and the least significant bit coded

-- in bit 6 of octet 4.

heNBIdentity			[52] OCTET STRING OPTIONAL,

-- 4 or 6 octets are coded with the HNBUnique Identity

-- as specified in 3GPP TS 23.003            [25], Clause 4.10.

heNBiPAddress		[53] IPAddress  OPTIONAL,

heNBLocation			[54] HeNBLocation  OPTIONAL,

tunnelProtocol		[55] TunnelProtocol  OPTIONAL,

pANI-Header-Info		[56] SEQUENCE OF PANI-Header-Info		OPTIONAL,

-- information extracted from P-Access-Network-Info headers of SIP message;

-- described in TS 24.229 §7.2A.4 [76]

imsVoIP 				[57] IMS-VoIP-Correlation					OPTIONAL,

xCAPmessage			[58] OCTET STRING OPTIONAL,

-- The HTTP message (HTPP header and any XCAP body) of any of the target's IMS supplementary

-- service setting management or manipulation XCAP messages occuring through the Ut interface

-- defined in the 3GPP TS 24 623 [77].

logicalFunctionInformation	[59] DataNodeIdentifier OPTIONAL,

ccUnavailableReason		[60] PrintableString	OPTIONAL,

carrierSpecificData					[61] OCTET STRING OPTIONAL,

-- Copy of raw data specified by the CSP or his vendor related to HSS.

current-previous-systems			[62] Current-Previous-Systems OPTIONAL,

change-Of-Target-Identity				[63] Change-Of-Target-Identity OPTIONAL,

requesting-Network-Identifier		[64] OCTET STRING OPTIONAL,

-- the requesting network identifier PLMN id (Mobile Country Code and Mobile Network Country,

-- defined in E212 [87]).

requesting-Node-Type					[65] Requesting-Node-Type OPTIONAL,

serving-System-Identifier				[66] OCTET STRING OPTIONAL,

-- the serving network identifier PLMN id (MNC, Mobile Country Code and MNC,Mobile Network

-- Country, defined in E212 [87]) and 3GPP TR 21.905 [38], that may be included in the Diameter

-- AVP to and from the HSS.

proSeTargetType                    [67] ProSeTargetType OPTIONAL,

proSeRelayMSISDN                   [68] OCTET STRING (SIZE (1..9)) OPTIONAL,

-- coded according to 3GPP TS 29.274 [46]

proSeRelayIMSI                     [69] OCTET STRING (SIZE (3..8)) OPTIONAL,

-- coded according to 3GPP TS 29.274 [46]

proSeRelayIMEI                     [70] OCTET STRING (SIZE (8)) OPTIONAL,

-- coded according to 3GPP TS 29.274 [46]

extendedLocParameters 	[71] 	ExtendedLocParameters OPTIONAL, -- LALS extended parameters

locationErrorCode		[72]	LocationErrorCode OPTIONAL,	-- LALS error code

national-HI2-ASN1parameters	[255]	National-HI2-ASN1parameters	OPTIONAL

}

*****  END OF CHANGE 15  *********************

************  CHANGE 16  **********************

Annex O (informative):
Selection of ASN.1 HI2 sub-domain for LALS Reporting

The LALS Target Positioning reports may be encoded in any of the following HI2 ASN.1 sub-domains:

- hi2CS (for CS), specified in Annex B.3;

- hi2 (for PS), specified in Annex B.3a;

- hi2eps (for EPS), specified in Annex B.9.

The LALS Target Positioning report records and their ASN.1 parameters for each sub-domain are presented in the following Tables:

- 5.5X (for CS);

- 6.X (for PS);

- 10.5.1.1.x (for EPS).

The selection of a particular ASN.1 sub-domain for LALS reporting is implementation-specific and out of scope of the present document. It may depend on the ability of the LI LCS Client to identify the network domain serving the UE, e.g. from the domain specific parameters in the LCS Server response, like the RAI for PS. It is also possible that a default sub-domain is used, e.g. by the operators that have a single network domain deployed.

For the LALS Enhanced Location for IRI reporting the ASN.1 HI2 sub-domain is the same one that is used for reporting of the corresponding triggering event.

*****  END OF CHANGE 16  *********************