3GPP TSG-SA3 Meeting #65	s3i170133

# West Palm Beach, Florida, United States, 25th Apr 2017 - 28th Apr 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.108 | CR | 0359 | rev | 0 | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MMS Correlation Correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3-LI |  |  |  |  |  |  |  |  |  |
| Work item code: | LI14 |  |  |  |  |  | Date: |  |  | 2017-04-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | MMS Correlation is broken. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution fixes mis-named correlation parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MMS interception is not implementable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** FIRST MODIFICATION ***

Table 15.3.6.2.2.8: MMS Upload REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed IMEI |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed MSISDN |  |  |
| observed IMPU/IMPI |  |  |
| observed E.164 |  |  |
| observed MMS Address/MMbox ID |  |  |
| observed IPv4/IPv6 Address |  |  |
| event type | M | Provide MMS event type (i.e., MMS Upload). |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided including the network element identifier. |
| lawful intercept identifier | M | Shall be provided. |
| MMS Version | M | Shall be provided. |
| Transaction ID | M | Shall be provided. |
| MM State | C | If sent by the target, shall be included. |
| MM Flags | C | If sent by the target, shall be included. |
| Content Type | M | Shall be provided. |
| Content Location | C | If available, shall be included. |
| Store Status | M | Shall be provided. |
| Store Status Text | C | If sent to the target, shall be included. |
| MMBox Description pdu | M | Shall be provided. The MMBox description PDU (as described in Table 15.3.6.2.2.16) corresponds to the particular MM being uploaded. |

*** END FIRST MODIFICATION ***

*** SECOND MODIFICATION ***

Table 15.3.6.2.2.15: MMS View Confirm REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed IMEI |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed MSISDN |  |  |
| observed IMPU/IMPI |  |  |
| observed E.164 |  |  |
| observed MMS Address/MMbox ID |  |  |
| observed IPv4/IPv6 Address |  |  |
| event type | M | Provide MMS event type (i.e., MMS View Confirm). |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided including the network element identifier. |
| lawful intercept identifier | M | Shall be provided. |
| MMS Version | M | Shall be provided. |
| Transaction id | M | Shall be provided. |
| MM State | C | Provide if sent to the target. |
| MM Flags | C | Provide if sent to the target. |
| Content Location | C | Provide if sent to the target. |
| MMS Start | C | Provide if sent to the target. |
| MMS Limit | C | Provide if sent to the target. |
| MMS Attributes | C | Provide if sent to the target. |
| MMS Totals | C | Provide if sent to the target. |
| MMS Quotas | C | Provide if sent to the target. |
| Response Status | M | Shall be provided. |
| Response Status Text | C | Provide if sent to the target. |
| MMS Message Count | C | Provide if sent to the target. |
| Content type | M | Shall be provided. |
| MMBox Description pdu | M | Shall provide one or more MMBox description PDUs where each MMBox description PDU (as described in Table 15.3.6.2.2.16) corresponds to a particular MM. |

*** END SECOND MODIFICATION ***

*** THIRD MODIFICATION ***

# B.15	Intercept related information (HI2 MMS)

Declaration of ROSE operation mms-sending-of-IRI is ROSE delivery mechanism specific. When using FTP delivery mechanism, data MmsIRIsContent must be considered.

ASN1 description of IRI (HI2 interface)

MmsHI2Operations {itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulintercept(2) threeGPP(4) hi2mms(16) r14(14) version-1 (1)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

IMPORTS

OPERATION,

ERROR

FROM Remote-Operations-Information-Objects

{joint-iso-itu-t(2) remote-operations(4) informationObjects(5) version1(0)}

LawfulInterceptionIdentifier,

TimeStamp,

Network-Identifier,

National-Parameters,

National-HI2-ASN1parameters,

DataNodeAddress,

IPAddress,

IP-value,

X25Address

FROM HI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulIntercept(2) hi2(1) version18(18)} -- Imported from TS 101 671v3.12.1

Location

FROM UmtsHI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulintercept(2) threeGPP(4) hi2(1)  r13 (13) version-0 (0)};

-- Imported from 3GPP TS 33.108, UMTS PS HI2

-- Object Identifier Definitions

-- Security DomainId

lawfulInterceptDomainId OBJECT IDENTIFIER ::= {itu-t(0) identified-organization(4) etsi(0)

securityDomain(2) lawfulIntercept(2)}

-- Security Subdomains

threeGPPSUBDomainId OBJECT IDENTIFIER ::= {lawfulInterceptDomainId threeGPP(4)}

hi2mmsDomainId OBJECT IDENTIFIER	::= {threeGPPSUBDomainId hi2mms(16) r14(14) version-1 (1)}

mms-sending-of-IRI  OPERATION ::=

{

ARGUMENT	MmsIRIsContent

ERRORS		{ OperationErrors }

CODE		global:{threeGPPSUBDomainId hi2mms(16) opcode(1)}

}

-- Class 2 operation . The timer shall be set to a value between 3 s and 240 s.

-- The timer.default value is 60s.

-- NOTE:	The same note as for HI management operation applies.

MmsIRIsContent		::= CHOICE

{

mmsiRIContent			MmsIRIContent,

mmsIRISequence			MmsIRISequence

}

MmsIRISequence		::= SEQUENCE OF MmsIRIContent

-- Aggregation of MmsIRIContent is an optional feature.

-- It may be applied in cases when at a given point in time

-- several IRI records are available for delivery to the same LEA destination.

-- As a general rule, records created at any event shall be sent

-- immediately and not withheld in the DF or MF in order to

-- apply aggragation.

-- When aggregation is not to be applied,

-- MmsIRIContent needs to be chosen.

-- MmsIRIContent includes events that correspond to MMS.

MmsIRIContent		::= CHOICE

{

iRI-Begin-record		[1] IRI-Parameters,	-- not applicable for the present document

iRI-End-record		[2] IRI-Parameters, -- not applicable for the present document

iRI-Continue-record		[3] IRI-Parameters,	-- not applicable for the present document

iRI-Report-record		[4] IRI-Parameters	-- include at least one optional parameter

}

-- the MmsIRIContent may provide events that correspond to UMTS/GPRS as well as EPS.

unknown-version 		ERROR ::= { CODE local:0}

missing-parameter		ERROR ::= { CODE local:1}

unknown-parameter-value	ERROR ::= { CODE local:2}

unknown-parameter		ERROR ::= { CODE local:3}

OperationErrors ERROR ::=

{

unknown-version |

missing-parameter |

unknown-parameter-value |

unknown-parameter

}

-- These values may be sent by the LEMF, when an operation or a parameter is misunderstood.

-- Parameters having the same tag numbers must be identical in Rel-14 and onwards modules.

IRI-Parameters		::= SEQUENCE

{

hi2mmsDomainId				[0]	OBJECT IDENTIFIER,  -- 3GPP HI2 MMS domain

lawfulInterceptionIdentifier			[1] LawfulInterceptionIdentifier,

-- This identifier is associated to the target.

timeStamp					[3] TimeStamp,

-- date and time of the event triggering the report.

locationOfTheTarget				[4] Location OPTIONAL,

-- location of the target

partyInformation 				[5] SET SIZE (1..10) OF PartyInformation OPTIONAL,

-- This parameter provides the concerned party, the identiy(ies) of the party

-- and all the information provided by the party.

mMSevent 				[7] MMSEvent OPTIONAL,

serviceCenterAddress				[8] PartyInformation OPTIONAL,

-- this parameter provides the address of the relevant MMS server

mMSParties				[9] MMSParties OPTIONAL,

-- this parameter provides the MMS parties (To, CC, BCC, and From) in the communication.

mMSVersion				[10] INTEGER OPTIONAL,

transactionID				[11] UTF8String OPTIONAL,

messageID					[12] UTF8String OPTIONAL,

-- In accordance with [90] it is encoded as in email address as per [RFC2822]. The characters

-- "<" and ">" are not included.

mMSDateTime				[13] DateTime OPTIONAL,

messageClass				[14] MessageClass OPTIONAL,

expiry					[15] DateTime OPTIONAL,

distributionIndicator				[16] YesNo OPTIONAL,

elementDescriptor				[17] ElementDescriptor OPTIONAL,

retrievalMode				[18] YesNo OPTIONAL,

-- if retrievalMode is included, it must be coded to Yes indicating Manual retreival mode

-- recommended.

retrievalModeText				[19] EncodedString OPTIONAL,

senderVisibility				[20] YesNo OPTIONAL,

-- Yes indicates Show and No indicates Do Not Show.

deliveryReport				[21] YesNo OPTIONAL,

readReport				[22] YesNo OPTIONAL,

applicID					[23] UTF8String OPTIONAL,

replyApplicID				[24] UTF8String OPTIONAL,

auxApplicInfo				[25] UTF8String OPTIONAL,

contentClass				[26] ContentClass OPTIONAL,

dRMContent				[27] YesNo OPTIONAL,

replaceID					[28] UTF8String OPTIONAL,

contentLocation				[29] ContentLocation OPTIONAL,

mMSStatus				[30] MMSStatus OPTIONAL,

reportAllowed				[31] YesNo OPTIONAL,

previouslySentBy				[32] PreviouslySentBy OPTIONAL,

previouslySentByDateTime 			[33] PreviouslySentByDateTime OPTIONAL,

mMState					[34] MMSState OPTIONAL,

desiredDeliveryTime				[35] DateTime OPTIONAL,

deliveryReportAllowed			[36] YesNo OPTIONAL,

store					[37] YesNo OPTIONAL,

responseStatus				[38] ResponseStatus OPTIONAL,

responseStatusText				[39] ResponseStatusText OPTIONAL,

storeStatus				[40] StoreStatus OPTIONAL,

storeStatusText				[41] EncodedString OPTIONAL,

-- mMState				[42] MMSState OPTIONAL,

mMFlags					[43] MMFlags OPTIONAL,

mMBoxDescriptionPdus			[44] SEQUENCE OF MMBoxDescriptionPdus  OPTIONAL,

cancelID					[45] UTF8String OPTIONAL,

cancelStatus				[46] YesNo OPTIONAL,

-- Yes indicates cancel successfully received and No indicates cancel request corrupted.

mMSStart					[47] INTEGER OPTIONAL,

mMSLimit					[48] INTEGER OPTIONAL,

mMSAttributes				[49] MMSAttributes OPTIONAL,

mMSTotals				[50] YesNo OPTIONAL,

mMSQuotas				[51] YesNo OPTIONAL,

mMSMessageCount				[52] INTEGER OPTIONAL,

messageSize				[53] INTEGER OPTIONAL,

mMSForwardReqDateTime			[54] DateTime OPTIONAL,

adaptationAllowed				[55] YesNo OPTIONAL,

priority					[56] Priority OPTIONAL,

mMSCorrelationNumber			[57] MMSCorrelationNumber OPTIONAL,

-- this parameter provides MMS Correlation number when the event will also provide CC.

contentType				[58] OCTET STRING OPTIONAL,

national-Parameters				[59] National-Parameters OPTIONAL

}

-- Parameters having the same tag numbers must be identical in Rel-14 and onwards modules

-- PARAMETERS FORMATS

Address::= EncodedString

Addresses::= SEQUENCE OF Address

ClassIdentifier ::= ENUMERATED

{

personal 			(0),

advertisement	 	(1),

informational		(2),

auto				(3),

...

}

ContentClass ::= ENUMERATED

{

text 			(0),

image-basic 	(1),

image-rich		(2),

video-basic		(3),

video-rich		(4),

megapixel		(5),

content-basic	(6),

content-rich	(7),

...

}

ContentLocation ::= SEQUENCE

{

contentLocationURI		[1] OCTET STRING,

-- See Clause 7.3.10 of [90] for the coding of the contentLocationURI.

statusCount				[2] INTEGER OPTIONAL,

-- the statusCount is included only for the MMS Delete event.

...

}

ElementDescriptor ::= SEQUENCE

{

contentReferenceValue [1] UTF8String,

parameterName		  [2] ParameterName,

parameterValue		  [3] ParameterValue,

...

}

EncodedString::= CHOICE

{

text	[1] UTF8String,

encodedTextString	[2] EncodedTextString,

...

}

EncodedTextString::= SEQUENCE

{

stringType	[1] OCTET STRING,

-- stringType shall be encoded with MIBEnum values as registered with IANA as defined in [90].

actualString	[2] OCTET STRING,

...

}

From ::= SEQUENCE OF FromAddresses

FromAddresses ::= CHOICE

{

actualAddress	[1] EncodedString,

insertToken		[2] NULL,

...

}

MessageClass ::= CHOICE

{

classIdentifier	[1] ClassIdentifier,

tokenText		[2] OCTET STRING,

...

}

MMBoxDescriptionPdus ::= SEQUENCE

{

mMSCorrelation		[1] MMSCorrelationNumber OPTIONAL,

toAddresses			[2] Addresses,

cCAddresses			[3] Addresses OPTIONAL,

bCCAddresses		[4] Addresses OPTIONAL,

fromAddress			[5] From,

messageID			[6] UTF8String,

mMSDateTime			[7] DateTime OPTIONAL,

previouslySentBy		[8] PreviouslySentBy OPTIONAL,

previouslySentByDateTime	[9] PreviouslySentByDateTime OPTIONAL,

mMState			[10] MMState OPTIONAL,

mMFlags			[11] MMFlags OPTIONAL,

messageClass		[12] MessageClass OPTIONAL,

priority			[13] Priority OPTIONAL,

deliveryTime		[14] DateTime OPTIONAL,

expiry			[15] DateTime OPTIONAL,

deliveryReport		[16] YesNo OPTIONAL,

readReport			[17] YesNo OPTIONAL,

messageSize			[18] INTEGER OPTIONAL,

contentLocation		[19] ContentLocation OPTIONAL,

contentType			[20] OCTET STRING OPTIONAL,

...

}

MMFlags ::= SEQUENCE

{

tokenAction		[1] TokenAction,

mmFlagkeywords	[2] EncodedString

}

MMSAttributes ::= CHOICE

{

attributeApplicID		[1] UTF8String,

attributeAuxApplicInfo		[2] UTF8String,

attributeBCC		[3] Address,

attributeCC			[4] Address,

attributeContent		[5] OCTET STRING,

attributeContentType		[6] OCTET STRING,

attributeDate		[7] DateTime,

attributeDeliveryReport		[8] YesNo,

attributeDeliveryTime		[9] DateTime,

attributeExpiry		[10] DateTime,

attributeFrom		[11] From,

attributeMessageClass		[12] MessageClass,

attributeMessageID		[13] UTF8String,

attributeMessageSize		[14] INTEGER,

attributePriority		[15] Priority,

attributeReadReport		[16] YesNo,

attributeTo			[17] Address,

attributeReplyApplicID		[18] UTF8String,

attributePreviouslySentBy	[19] PreviouslySentBy,

attributePreviouslySentByDateTime	[20] PreviouslySentByDateTime,

attributeAdditionalHeaders	[21] OCTET STRING,

...

}

MMSCorrelationNumber ::= OCTET STRING

MMSEvent ::= ENUMERATED

{

send 		(0),

notification	(1),

notificationResponse	(2),

retrieval		(3),

retrievalAcknowledgement(4),

forwarding		(5),

store		(6),

upload		(7),

delete		(8),

delivery		(9),

readReplyFromTarget	(10),

readReplyToTarget	(11),

cancel		(12),

viewRequest		(13),

viewConfirm		(14),

...

}

MMSParties::= SEQUENCE

{

toAddresses		[1] Addresses OPTIONAL,

cCAddresses		[2]	Addresses OPTIONAL,

bCCAddresses	[3] Addresses OPTIONAL,

fromAddresses	[4] From OPTIONAL,

...

}

MMSState::= ENUMERATED

{

draft 		(0),

sent		(1),

new			(2),

retreived	(3),

forwarded	(4),

...

}

MMSStatus::= ENUMERATED

{

expired 		(0),

retrieved		(1),

rejected		(2),

deferred		(3),

unrecognised	(4),

indeterminate	(5),

forwarded		(6),

unreachable		(7),

...

}

ParameterName::= CHOICE

{

integername [1] INTEGER,

textName	[2] UTF8String,

...

}

ParameterValue::= CHOICE

{

intValue  [1] OCTET STRING,

textValue [2] UTF8String,

...

}

PreviouslySentby::= SEQUENCE

{

forwardedCount		[1] INTEGER,

forwardedPartyID	[2] EncodedString,

...

}

PreviouslySentbyDateTime::= SEQUENCE

{

forwardedCount		[1] INTEGER,

forwardedDateTime	[2] DateTime,

...

}

Priority ::= ENUMERATED

{

low 	(0),

normal 	(1),

high	(2),

...

}

ResponseStatus::= SEQUENCE

{

statusCount	[1]	EncodedString OPTIONAL,

-- the statusCount shall only be included for the Delete event.

actualResponseStatus	[2] ActualResponseStatus,

...

}

ResponseStatusText::= SEQUENCE

{

statusCount	[1]	EncodedString OPTIONAL,

-- the statusCount shall only be included for the Delete event.

actualResponseStatusText	[2] EncodedString,

...

}

ActualResponseStatus ::= ENUMERATED

{

ok 										(0),

errorUnspecified						(1),

errorServiceDenied						(2),

errorMessageFormatCorrupt				(3),

errorSendingAddressUnresolved			(4),

errorMessageNotFound					(5),

errorNetworkProblem						(6),

errorContentNotAccepted					(7),

errorUnsuportedMessage					(8),

errorTransientFailure					(9),

errorTransientSendingAddressUnresolved	(10),

errorTransientMessageNotFound			(11),

errorTransientNetworkProblem			(12),

errorTransientPartialSucess				(13),

errorPermanentFailure					(14),

errorPermanentServiceDenied				(15),

errorPermanentMessageFormatCorrupt		(16),

errorPermanentSendingAddressUnresolved	(17),

errorPermanentMessageNotFound			(18),

errorPermanentContentNotAccepted		(19),

errorPermanentReplyChargingLimitationsNotMet	(20),

errorPermanentReplyChargingRequestNotAccepted	(21),

errorPermanentReplyChargingForwardingDenied		(22),

errorPermanentReplyChargingNotSupported			(23),

errorPermanentAddressHidingNotSupported			(24),

errorPermanentLackOfPrepaid						(25),

...

}

StoreStatus ::= ENUMERATED

{

success 		(0),

errorTransient 	(1),

high			(2),

...

}

TokenAction::= ENUMERATED

{

addToken 		(0),

removeToken 	(1),

filterToken		(2),

...

}

YesNo::= BOOLEAN

-- TRUE indicates Yes and FALSE indicates No.

END -- OF MmsHI2Operations

*** END THIRD MODIFICATION ***