LI(19)P50011r3

TC LI#50

| ETSI/TC LI |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  | TS103280CR009 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CR status: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | TS 103 280 |  |  |  |  |  |  |  |  | CR |  |  | 009 |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  | 2.2.1 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Affected version of the standard  |  |  |  |  |  |  |  |  |  |  |
| ETSI number of TC LI  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by TO of ETSI/TC LI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  | TC LI#50 |  |  | for approval |  |  |  |  |  | X |  | strategic |  |  |  |  |  |  |  |  |  |  |  | editorial |  |  |  |  |
|  |  |  |  | for information |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  | Tencastle, UK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  | 07/02/2019 |  |  |  |
| Subject: |  | Addition of 5G identifiers to common parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: |  | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (only one category |  | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| shall be marked |  | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| with an X) |  | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | 3GPP SA3-LI has elected to adopt a number of ETSI TC LI standards as part of its work on 5G LI. In order to support their work, both TS 103 221-1 (X1) and TS 103 120 (HI-1) need to support 5G identifiers (SUPI, SUCI, PEI and GPSI), and therefore we propose including them in the dictionary of common parameters. This necessarily means including NAI and IMEISV. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2.1, 3, 6, Annex A, Annex B. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other docs affected: |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| version number to be included |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  | Adding SUPI and PEI also requires adding NAI and IMEISV, as well as ASN.1 representations of IMEI. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Referenced documents which are not found to be publicly available in the expected location might be found at https://docbox.etsi.org/Reference.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are necessary for the application of the present document.

[1]	ETSI TS 102 232-1: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 1: Handover specification for IP delivery".

[2]	W3C Recommendation 5 April 2012: "W3C XML Schema Definition Language (XSD) 1.1 Part 2: Datatypes".

[3]	Recommendation ITU-T X.680: "Information technology - Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[4]	Recommendation ITU-T E.164: "The international public telecommunication numbering plan".

[5]	Recommendation ITU-T E.212: "The international identification plan for public networks and subscriptions".

[6]	ETSI TS 123 003: "Digital cellular telecommunications system (Phase 2+) (GSM); Universal Mobile Telecommunications System (UMTS); Numbering, addressing and identification (3GPP TS 23.003)".

[7]	ETSI TS 102 657: "Lawful Interception (LI); Retained data handling; Handover interface for the request and delivery of retained data".

[8]	IETF RFC 791: "Internet Protocol".

[9]	IETF RFC 4632: "Classless Inter-domain Routing (CIDR): The Internet Address Assignment and Aggregation Plan".

[10]	IETF RFC 8200: "Internet Protocol, Version 6 (IPv6) Specification".

[11]	IETF RFC 4291: "IP Version 6 Addressing Architecture".

[12]	IETF RFC 793: "Transmission Control Protocol".

[13]	IETF RFC 768: "User Datagram Protocol".

[14]	IEEE 802.3 TM: "IEEE Standard for Ethernet".

[15]	IETF RFC 5322: "Internet Message Format".

[16]	W3C Recommendation 28 October 2014: "HTML5 A vocabulary and associated APIs for HTML and XHTML".

[17]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace".

[18]	ISO 3166-1: "Codes for the representation of names of countries and their subdivisions -- Part 1: Country codes".

[19]	IEEE Std 1003.1TM-2008: "IEEE Standard for Information Technology - Portable Operating System Interface (POSIX(R))".

[20]	ISO/IEC 7812-1:2015: "Identification cards -- Identification of issuers -- Part 1: Numbering system".

[21]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[22]	IETF RFC 3966: "The tel URI for Telephone Numbers".

[23]	NIMA Technical Report 8350.2: "Department of Defense World Geodetic System 1984, Its Definition and Relationships With Local Geodetic Systems".

[24]	3GPP TS 23.501: "System Architecture for the 5G System".

[25]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[26]	IETF RFC 4282: "The Network Access Identifier".

[27]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

SECOND CHANGE

# 6	Parameter dictionary

LIID

| Name | LIID |
| --- | --- |
| Description | For each target identity related to an interception measure, the authorized CSP operator shall assign a special Lawful Interception IDentifier (LIID), which has been agreed between the LEA and the CSP. It is used within parameters of all HI interface ports. Using an indirect identification, pointing to a target identity makes it easier to keep the knowledge about a specific interception target limited within the authorized CSP operators and the handling agents at the LEA. The Lawful Interception IDentifier LIID is a component of the CC delivery procedure and of the IRI records. It shall be used within any information exchanged at the Handover Interfaces HI2 and HI3 for identification and correlation purposes. The LIID format shall consist of alphanumeric characters. It might for example, among other information, contain a lawful authorization reference number, and the date, when the lawful authorization was issued. The authorized CSP shall either enter a unique LIID for each target identity of the interception subject or as a national option a single LIID for multiple target identities all pertaining to the same interception subject. Example: The interception subject has a telephony service with three telephone numbers. The CSP enters for each telephone number an own LIID, or optionally enters one LIID for all three telephone numbers. If more than one LEA intercepts the same target identity, there shall be unique LIIDs assigned, relating to each LEA. |
| Usage guidance | The LIID is defined as an OCTET STRING in ASN.1. This means it is possible to use binary octets or ASCII printable characters to express the LIID. To correctly handle this, the parameter accepts both variations. |
| References | ETSI TS 102 232-1 [1], clause 5.2.2. |
| Example | ZZZ123 (ASCII printable LIID) 46565527098f6bcd4621d373cade4e832627b4f6ff00ff00ff (Binary LIID, represented in HEX) |
| Regular expression | ^([!-~]{1,25})¦([0-9a-f]{26,50})$ |
| XSD | LIID, simpleType |
| ASN.1 | LIID, OCTET STRING |

UTCDateTime

| Name | UTCDateTime |
| --- | --- |
| Description | A UTC timestamp with second precision. |
| Usage guidance | - |
| References | W3C XML Schema Definition Language [2], section 3.3.7 |
| Example | 2015-12-27T13:37:00Z |
| Regular expression | ^[0-9]{4}-[0-9]{2}-[0-9]{2}T[0-9]{2}:[0-9]{2}:[0-9]{2}Z$ |
| XSD | UTCDateTime, simpleType |
| ASN.1 | Not defined |

UTCMicrosecondDateTime

| Name | UTCMicrosecondDateTime |
| --- | --- |
| Description | A UTC timestamp with microsecond precision. |
| Usage guidance | - |
| References | W3C XML Schema Definition Language [2], section 3.3.7 |
| Example | 2015-12-27T13:37:00.012345Z |
| Regular expression | ^[0-9]{4}-[0-9]{2}-[0-9]{2}T[0-9]{2}:[0-9]{2}:[0-9]{2}\.[0-9]{6}Z$ |
| XSD | UTCMicrosecondDateTime, simpleType |
| ASN.1 | Not defined |

QualifiedDateTime

| Name | QualifiedDateTime |
| --- | --- |
| Description | A timestamp with second precision and timezone qualifier. |
| Usage guidance | - |
| References | W3C XML Schema Definition Language 1.1 Part 2: Datatypes [2], section 3.3.7 |
| Example | 2015-12-27T13:37:00+02:00 |
| Regular expression | ^[0-9]{4}-[0-9]{2}-[0-9]{2}T[0-9]{2}:[0-9]{2}:[0-9]{2}(Z¦[+-] [0-9]{2}:[0-9]{2})$ |
| XSD | QualifiedDateTime, simpleType |
| ASN.1 | QualifiedDateTime, GeneralizedTime Timestamps shall be provided with a timezone qualifier. The fractional part of a second shall not be present. Local time format shall not be used. |

QualifiedMicrosecondDateTime

| Name | QualifiedMicrosecondDateTime |
| --- | --- |
| Description | A timestamp with microsecond precision and timezone qualifier. |
| Usage guidance | - |
| References | W3C XML Schema Definition Language 1.1 Part 2: Datatypes [2], section 3.3.7 |
| Example | 2015-12-27T13:37:00.012345+02:00 |
| Regular expression | ^[0-9]{4}-[0-9]{2}-[0-9]{2}T[0-9]{2}:[0-9]{2}:[0-9]{2}\.[0-9]{6}(Z¦ [+-][0-9]{2}:[0-9]{2})$ |
| XSD | QualifiedMicrosecondDateTime, simpleType |
| ASN.1 | QualifiedMicrosecondDateTime, GeneralizedTime Timestamps shall be provided with a timezone qualifier. The fractional part of a second with no more than 6 digits shall be present. Local time format shall not be used. |

InternationalE164

| Name | InternationalE164 |
| --- | --- |
| Description | E.164 Number in fully international format, written as decimal digits. |
| Usage guidance | - |
| References | Recommendation ITU-T E.164 [4], clause 6 |
| Example | 31612345678 |
| Regular expression | ^[0-9]{1,15}$ |
| XSD | InternationalE164, simpleType |
| ASN.1 | NumericString (SIZE(1..15)) |

IMSI

| Name | IMSI |
| --- | --- |
| Description | International Mobile Subscriber Identity, written as decimal digits. |
| Usage guidance | - |
| References | Recommendation ITU-T E.212 [5], clause 6.1 3GPP TS 23.003 [6], clause 2.2 and clause 2.3 |
| Example | 204081234567890 |
| Regular expression | ^[0-9]{6,15}$ |
| XSD | IMSI, simpleType |
| ASN.1 | NumericString (SIZE(6..15)) |

IMEI

| Name | IMEI |
| --- | --- |
| Description | International Mobile station Equipment Identity, written as decimal digits without the Luhn check digit, annex B of ISO/IEC 7812-1 [20]. |
| Usage guidance | To avoid implementation issues, the IMEI parameter explicitly excludes the Luhn check digit, annex B of ISO/IEC 7812-1 [20]. (See notes 1 and 2) |
| References | 3GPP TS 23.003 [6], clause 6 |
| Example | 35395803121326 |
| Regular expression | ^[0-9]{14}$ |
| XSD | IMEI, simpleType |
| ASN.1 | NumericString (SIZE(14)) |
| NOTE 1: ETSI TS 102 657 [7], clause E.3 identifies potential issues with the inclusion/exclusion of the check digit. As such, the IMEI parameter is explicitly specified without the check digit. NOTE 2: The IMEICheckDigit parameter can be used when the check digit is explicitly required. |  |

IMEICheckDigit

| Name | IMEICheckDigit |
| --- | --- |
| Description | International Mobile station Equipment Identity, written as decimal digits with the Luhn check digit, annex B of ISO/IEC 7812-1 [20]. |
| Usage guidance |  |
| References | 3GPP TS 23.003 [6], clause 6 |
| Example | 353958031213269 |
| Regular expression | ^[0-9]{15}$ |
| XSD | IMEICheckDigit, simpleType |
| ASN.1 | NumericString (SIZE(15)) |

IMEISV

| Name | IMEISV |
| --- | --- |
| Description | International Mobile station Equipment Identity and Software Version Number as defined in TS 23.003 [6] clause 6.2.2, written as decimal digits including a software version number instead of a Luhn check digit. |
| Usage guidance | - |
| References | ETSI TS 123.003 [6], clause 6.2.2 |
| Example | 3539580312132601 |
| Regular expression | ^[0-9]{16}$ |
| XSD | IMEISV, simpleType |
| ASN.1 | NumericString (SIZE(16)) |

IPv4Address

| Name | IPv4Address |
| --- | --- |
| Description | IPv4 address, written in dotted decimal notation. |
| Usage guidance | - |
| References | IETF RFC 791 [8] |
| Example | 192.0.2.1 |
| Regular expression | ^((25[0-5]¦2[0-4][0-9]¦[01]?[0-9]?[0-9])\.){3}(25[0-5]¦2[0-4] [0-9]¦[01]?[0-9]?[0-9])$ |
| XSD | IPv4Address, simpleType |
| ASN.1 | Not defined |

IPv4CIDR

| Name | IPv4CIDR |
| --- | --- |
| Description | IPv4 CIDR, written in dotted decimal notation followed by CIDR notation. |
| Usage guidance | - |
| References | IETF RFC 791 [8] and IETF RFC 4632 [9] |
| Example | 192.0.2.0/24 |
| Regular expression | ^((25[0-5]¦2[0-4][0-9]¦[01]?[0-9]?[0-9])\.){3}(25[0-5]¦2[0-4] [0-9]¦[01]?[0-9]?[0-9])/([1-2]?[0-9]¦3[0-2])$ |
| XSD | IPv4CIDR, simpleType |
| ASN.1 | Not defined |

IPv6Address

| Name | IPv6Address |
| --- | --- |
| Description | IPv6 address, written as eight groups of four hexadecimal digits separated by a colon. |
| Usage guidance | It is recognized that IPv6 address formatting has various options. To reduce complexity in technical implementations, the IPv6Address parameter restricts the address to the fully uncompressed representation of the IPv6 address. |
| References | IETF RFC 8200 [10] |
| Example | 2001:0db8:0000:0000:0000:0000:0000:0001 |
| Regular expression | ^([0-9a-f]{4}:){7}([0-9a-f]{4})$ |
| XSD | IPv6Address, simpleType |
| ASN.1 | Not defined |

IPv6CIDR

| Name | IPv6CIDR |
| --- | --- |
| Description | IPv6 CIDR, written as eight groups of four hexadecimal digits separated by a colon followed by CIDR notation. |
| Usage guidance | See IPv6Address parameter for usage guidance. |
| References | IETF RFC 8200 [10], IETF RFC 4632 [9] and IETF RFC 4291 [11] |
| Example | 2001:0db8:0000:0000:0000:0000:0000:0000/48 |
| Regular expression | ^([0-9a-f]{4}:){7}([0-9a-f]{4})/(([1-9][0-9]?)¦(1[0-1][0-9])¦ (12[0-8]))$ |
| XSD | IPv6CIDR, simpleType |
| ASN.1 | Not defined |

IPAddress

| Name | IPAddress |
| --- | --- |
| Description | Either a IPv4Address parameter or IPv6Address parameter. |
| Usage guidance | - |
| References | - |
| Example | XSD <IPAddress> <IPv4Address>192.0.2.1</IPv4Address> </IPAddress> or <IPAddress> <IPv6Address>2001:0db8:0000:0000:0000:0000:0000:0001</IPv6Address> </IPAddress> |
| Regular expression | - |
| XSD | IPAddress, complexType |
| ASN.1 | Not defined |

IPCIDR

| Name | IPCIDR |
| --- | --- |
| Description | Either a IPv4CIDR parameter or IPv6CIDR parameter. |
| Usage guidance | - |
| References | - |
| Example | XSD <IPCIDR> <IPv4CIDR>192.0.2.0/24</IPv4CIDR> </IPCIDR> or <IPAddress> <IPv6CIDR> 2001:0db8:0000:0000:0000:0000:0000:0000/48</IPv6CIDR> </IPAddress> |
| Regular expression | - |
| XSD | IPCIDR, complexType |
| ASN.1 | Not defined |

TCPPort

| Name | TCPPort |
| --- | --- |
| Description | TCP port, written in decimal notation. |
| Usage guidance | - |
| References | IETF RFC 793 [12] |
| Example | 22 |
| Regular expression | ^([1-9][0-9]{0,3}¦[1-5][0-9]{4}¦6[0-4][0-9]{3}¦65[0-4][0-9]{2}¦ 655[0-2][0-9]¦6553[0-5])$ |
| XSD | TCPPort, simpleType |
| ASN.1 | TCPPort, INTEGER |

TCPPortRange

| Name | TCPPortRange |
| --- | --- |
| Description | TCP port range, consists of a 'start' TCPPort parameter and an 'end' TCPPort parameter. |
| Usage guidance | The start and end values are inclusive. |
| References | - |
| Example | Regular expression 1024-2048 XSD <TCPPortRange> <start>1024</start> <end>2048</end> </TCPPortRange> |
| Regular expression | ^([1-9][0-9]{0,3}¦[1-5][0-9]{4}¦6[0-4][0-9]{3}¦65[0-4][0-9]{2}¦ 655[0-2][0-9]¦6553[0-5])-([1-9][0-9]{0,3}¦[1-5][0-9]{4}¦6[0-4] [0-9]{3}¦65[0-4][0-9]{2}¦655[0-2][0-9]¦6553[0-5])$ |
| XSD | TCPPortRange, complexType |
| ASN.1 | TCPPortRange, SEQUENCE |

UDPPort

| Name | UDPPort |
| --- | --- |
| Description | UDP port, written in decimal notation. |
| Usage guidance | - |
| References | IETF RFC 768 [13] |
| Example | 53 |
| Regular expression | ^([0-9]{1,4}¦[1-5][0-9]{4}¦6[0-4][0-9]{3}¦65[0-4][0-9]{2}¦655[0-2] [0-9]¦6553[0-5])$ |
| XSD | UDPPort, simpleType |
| ASN.1 | UDPPort, INTEGER |

UDPPortRange

| Name | UDPPortRange |
| --- | --- |
| Description | UDP port range, consists of a 'start' UDPPort parameter and an 'end' UDPPort parameter. |
| Usage guidance | The start and end values are inclusive. |
| References | - |
| Example | Regular expression 2048-4096 XSD <UDPPortRange> <start>2048</start> <end>4096</end> </UDPPortRange> |
| Regular expression | ^([0-9]{1,4}¦[1-5][0-9]{4}¦6[0-4][0-9]{3}¦65[0-4][0-9]{2}¦655[0-2] [0-9]¦6553[0-5])-([0-9]{1,4}¦[1-5][0-9]{4}¦6[0-4][0-9]{3}¦65[0-4] [0-9]{2}¦655[0-2][0-9]¦6553[0-5])$ |
| XSD | UDPPortRange, complexType |
| ASN.1 | UDPPortRange, SEQUENCE |

Port

| Name | Port |
| --- | --- |
| Description | Either a TCPPort parameter or a UDPPort parameter. |
| Usage guidance | - |
| References | - |
| Example | XSD <Port> <TCPPort>22</TCPPort> </Port> |
| Regular expression | ^([0-9]{1,4}¦[1-5][0-9]{4}¦6[0-4][0-9]{3}¦65[0-4][0-9]{2}¦655[0-2] [0-9]¦6553[0-5])$ |
| XSD | Port, complexType |
| ASN.1 | Port, CHOICE |

PortRange

| Name | PortRange |
| --- | --- |
| Description | Either a TCPPortRange parameter or a UDPPortRange parameter. |
| Usage guidance | The start and end values are inclusive. |
| References | - |
| Example | XSD <PortRange> <TCPPortRange> <start>2048</start> <end>4096</end> </TCPPortRange> </PortRange> |
| Regular expression | - |
| XSD | PortRange, complexType |
| ASN.1 | PortRange, CHOICE |

IPAddressPort

| Name | IPAddressPort |
| --- | --- |
| Description | Combination of an IPAddress parameter and a Port parameter. |
| Usage guidance | - |
| References | - |
| Example | XSD <IPAddressPort> <address> <IPv4Address>192.0.2.1</IPv4Address> </address> <port> <TCPPort>22</TCPPort> </port> </IPAddressPort> |
| Regular expression | - |
| XSD | IPAddressPort, complexType |
| ASN.1 | Not defined |

IPAddressPortRange

| Name | IPAddressPortRange |
| --- | --- |
| Description | Combination of an IPAddress parameter and a PortRange parameter. |
| Usage guidance | - |
| References | - |
| Example | XSD <IPAddressPortRange> <address> <IPv4Address>192.0.2.1</IPv4Address> </address> <portRange> <TCPPortRange> <start>2048</start> <end>4096</end> </TCPPortRange> </portRange> </IPAddressPortRange> |
| Regular expression | - |
| XSD | IPAddressPortRange, complexType |
| ASN.1 | Not defined |

MACAddress

| Name | MACAddress |
| --- | --- |
| Description | MAC address, written as six groups of two hexadecimal digits separated by a colon. |
| Usage guidance | - |
| References | IEEE 802.3 [14] |
| Example | c0:ff:ee:c0:ff:ee |
| Regular expression | ^([a-f0-9]{2}:){5}[a-f0-9]{2}$ |
| XSD | MACAddress, simpleType |
| ASN.1 | Not defined |

EmailAddress

| Name | EmailAddress |
| --- | --- |
| Description | E-mail address |
| Usage guidance | - |
| References | IETF RFC 5322 [15] |
| Example | john.doe@example.com |
| Regular expression | ^[a-zA-Z0-9.!#$%&'*+\/=?^_`{¦}~-]+@[a-zA-Z0-9]([a-zA-Z0-9-]{0,61} [a-zA-Z0-9])?(\.[a-zA-Z0-9]([a-zA-Z0-9-]{0,61}[a-zA-Z0-9])?)*$ See note. |
| XSD | EmailAddress, simpleType |
| ASN.1 | Not defined |
| NOTE: The regular expression above is sourced from the W3C HTML5 Recommendation [16]. |  |

UUID

| Name | UUID |
| --- | --- |
| Description | UUID |
| Usage guidance | - |
| References | IETF RFC 4122 [17] |
| Example | de305d54-75b4-431b-adb2-eb6b9e546013 |
| Regular expression | ^[a-f0-9]{8}-[a-f0-9]{4}-[a-f0-9]{4}-[a-f0-9]{4}-[a-f0-9]{12}$ |
| XSD | UUID, simpleType |
| ASN.1 | Not defined |

ISOCountryCode

| Name | ISOCountryCode |
| --- | --- |
| Description | An ISO 3166-1 [18] alpha-2 two-letter country code. |
| Usage guidance | - |
| References | ISO 3166-1 [18] alpha-2 |
| Example | "NL" |
| Regular expression | ^[A-Z]{2}$ |
| XSD | ISOCountryCode, simpleType |
| ASN.1 | Not defined |

ShortString

| Name | ShortString |
| --- | --- |
| Description | A string with a maximum length of 255 characters. |
| Usage guidance | - |
| References | - |
| Example | string |
| Regular expression | - |
| XSD | ShortString, simpleType |
| ASN.1 | Not defined |

LongString

| Name | LongString |
| --- | --- |
| Description | A string with a maximum length of 65 535 characters. |
| Usage guidance | - |
| References | - |
| Example | string |
| Regular expression | - |
| XSD | LongString, simpleType |
| ASN.1 | Not defined |

SIPURI

| Name | SIPURI |
| --- | --- |
| Description | SIP URI |
| Usage guidance | - |
| References | IETF RFC 3261 [21], section 19.1 |
| Example | sip:user@example.com |
| Regular expression | ^sips?:[a-zA-Z0-9!#$&-;=?-\[\]_~%]+$ |
| XSD | SIPURI, simple type |
| ASN.1 | Not defined |

TELURI

| Name | TELURI |
| --- | --- |
| Description | TEL URI |
| Usage guidance | - |
| References | IETF RFC 3966 [22] |
| Example | tel:+447700900000 |
| Regular expression | ^tel:[a-zA-Z0-9!#$&-;=?-\[\]_~%]+$ |
| XSD | TELURI, simple type |
| ASN.1 | Not defined |

WGS84CoordinateDecimal

| Name | WGS84CoordinateDecimal |
| --- | --- |
| Description | A geographical latitude-longitude coordinate, referring to the WGS84 reference ellipsoid, given in decimal notation |
| Usage guidance | - |
| References | NIMA Technical Report 8350.2 [23] (for WGS84 definition itself, not for the syntax defined here) |
| Example | XSD <WGS84CoordinateDecimal> <latitude>N43.616000</latitude> <longitude>E007.053000</longitude> </WGS84CoordinateDecimal> |
| Regular expression | - |
| XSD | WGS84CoordinateDecimal, complexType |
| ASN.1 | WGS84CoordinateDecimal, SEQUENCE |

WGS84LatitudeDecimal

| Name | WGS84LatitudeDecimal |
| --- | --- |
| Description | A geographical latitude, referring to the WGS84 reference ellipsoid, given in decimal notation |
| Usage guidance | The latitude is given as two digits before the decimal point, left-padded with zero where necessary. The latitude is specific to six decimal places, right-padded with zero where necessary. |
| References | NIMA Technical Report 8350.2 [23] (for WGS84 definition itself, not for the syntax defined here) |
| Example | N43.616000 |
| Regular expression | ^[NS][0-9]{2}\.[0-9]{6}$ |
| XSD | WGS84LatitudeDecimal, simpleType |
| ASN.1 | WGS84LatitudeDecimal, OCTET STRING |

WGS84LongitudeDecimal

| Name | WGS84LongitudeDecimal |
| --- | --- |
| Description | A geographical longitude, referring to the WGS84 reference ellipsoid, given in decimal notation |
| Usage guidance | The longitude is given as three digits before the decimal point, left-padded with zero where necessary. The longitude is specific to six decimal places, right-padded with zero where necessary. |
| References | NIMA Technical Report 8350.2 [23] (for WGS84 definition itself, not for the syntax defined here) |
| Example | E007.053000 |
| Regular expression | ^[EW][0-9]{3}\.[0-9]{6}$ |
| XSD | WGS84LongitudeDecimal, simpleType |
| ASN.1 | WGS84LongitudeDecimal, OCTET STRING |

WGS84CoordinateAngular

| Name | WGS84CoordinateAngular |
| --- | --- |
| Description | A geographical latitude-longitude coordinate, referring to the WGS84 reference ellipsoid, given in angular notation |
| Usage guidance | - |
| References | NIMA Technical Report 8350.2 [23] (for WGS84 definition itself, not for the syntax defined here) |
| Example | XSD <WGS84CoordinateAngular> <latitude>N433700.62</latitude> <longitude>E0070310.42</longitude> </WGS84CoordinateAngular> |
| Regular expression | - |
| XSD | WGS84CoordinateAngular, complexType |
| ASN.1 | WGS84CoordinateAngular, SEQUENCE |

WGS84LatitudeAngular

| Name | WGS84LatitudeAngular |
| --- | --- |
| Description | A geographical latitude, referring to the WGS84 reference ellipsoid, given in angular notation |
| Usage guidance | Values are specified as "XDDMMSS.ss", i.e. a concatenation of the following fixed-length values, each padded with zeroes where necessary: A one-character hemisphere indicator, "N" or "S". A two-digit value indicating degrees A two-digit value indicating arc-minutes A two-digit value indicating whole arc-seconds A decimal point A two-digit value indicating fractional arc-seconds |
| References | NIMA Technical Report 8350.2 [23] (for WGS84 definition itself, not for the syntax defined here) |
| Example | N433700.62 |
| Regular expression | ^[NS][0-9]{6}\.[0-9]{2}$ |
| XSD | WGS84LatitudeAngular, simpleType |
| ASN.1 | WGS84LatitudeAngular, OCTET STRING |

WGS84LongitudeAngular

| Name | WGS84LongitudeAngular |
| --- | --- |
| Description | A geographical longitude, referring to the WGS84 reference ellipsoid, given in angular notation. |
| Usage guidance | Values are specified as "XDDDMMSS.ss" i.e. a concatenation of the following fixed-length values, each padded with zeroes where necessary: A one-character hemisphere indicator, "E" or "W". A three-digit value indicating degrees A two-digit value indicating arc-minutes A two-digit value indicating whole arc-seconds A decimal point A two-digit value indicating fractional arc-seconds |
| References | NIMA Technical Report 8350.2 [23] (for WGS84 definition itself, not for the syntax defined here) |
| Example | E0070310.42 |
| Regular expression | ^[EW][0-9]{7}\.[0-9]{2}$ |
| XSD | WGS84LongitudeAngular, simpleType |
| ASN.1 | WGS84LongitudeAngular, OCTET STRING |

SUPIIMSI

| Name | SUPIIMSI |
| --- | --- |
| Description | Subscription Permanent Identifier as defined in 3GPP TS 23.501 [24] clause 5.9.2 in IMSI representation. |
| Usage guidance | In 3GPP Release 15 a SUPI may contain either an IMSI or an NAI, as defined in 3GPP TS 23.501 [24]. This representation is used for a SUPI in IMSI format. |
| References | 3GPP TS 23.501 [24] ETSI TS 123 003 [6] clause 2.2 |
| Example | See definition of IMSI |
| Regular expression |  |
| XSD |  |
| ASN.1 |  |

SUPINAI

| Name | SUPINAI |
| --- | --- |
| Description | Subscription Permanent Identifier as defined in 3GPP 23.501 [24] clause 5.9.2 in NAI representation. |
| Usage guidance | In 3GPP Release 15 a SUPI may contain either an IMSI or an NAI, as defined in 3GPP TS 23.501 [24]. This representation is used for a SUPI in NAI format. |
| References | 3GPP TS 23.501 [24] IETF RFC 4282 [26] |
| Example | See definition of NAI |
| Regular expression |  |
| XSD |  |
| ASN.1 |  |

SUCI

| Name | SUCI |
| --- | --- |
| Description | Subscription Concealed Identifier as defined in 3GPP 33.501 [25] clause 6.12.2. |
| Usage guidance | The structure of a SUCI is given in TS 23.003 [6] clause 2.2B, and the IE encoding format is given in TS 24.501 [27]. When the ASN.1 representation is used, the octets of the SUCI are provided as defined in TS 24.501 [27] clause 9.11.3.4, with the 5GS Mobile identity IEI and length fields (i.e. octets 1 and 2 in Figure 9.11.3.4.2) omitted. When XSD or string representations are used, the same octets are provided but in hex-binary representation. |
| References | 3GPP TS 33.501 [25] 3GPP TS 24.501 [27] |
| Example |  |
| Regular expression | ^[0-9a-f]+$ |
| XSD | SUCI, simpleType |
| ASN.1 | SUCI, OCTET STRING |

PEIIMEI

| Name | PEIIMEI |
| --- | --- |
| Description | Permanent Equipment Identifier as defined in 3GPP 23.501 [24] clause 5.9.3 in IMEI representation, without the final check / spare digit. |
| Usage guidance | In 3GPP Release 15 a PEI may contain either an IMEI or an IMEISV as defined in 3GPP TS 23.501 [24]. This representation is used for IMEI format without the final check / spare digit. |
| References | 3GPP TS 23.501 [24] clause 5.9.3 ETSI TS 123.003 [6] clause 6.2.1 |
| Example | See definition of IMEI |
| Regular expression |  |
| XSD |  |
| ASN.1 |  |

PEIIMEICheckDigit

| Name | PEIIMEICheckDigit |
| --- | --- |
| Description | Permanent Equipment Identifier as defined in 3GPP 23.501 [24] clause 5.9.3 in IMEI representation with Luhn check digit. |
| Usage guidance | In 3GPP Release 15 a PEI may contain either an IMEI or an IMEISV as defined in 3GPP TS 23.501 [24]. This representation is used for IMEI format including the Luhn check digit. |
| References | 3GPP TS 23.501 [24] clause 5.9.3 ETSI TS 123.003 [6] clause 6.2.1 |
| Example | See definition of IMEICheckDigit |
| Regular expression |  |
| XSD |  |
| ASN.1 |  |

PEIIMEISV

| Name | PEIIMEISV |
| --- | --- |
| Description | Permanent Equipment Identifier as defined in 3GPP 23.501 [24] clause 5.9.3 in IMEISV representation. |
| Usage guidance | In 3GPP Release 15 a PEI may contain either an IMEI or an IMEISV as defined in 3GPP TS 23.501 [24]. This representation is used for IMEISV format written as decimal digits including a software version number instead of a Luhn check digit. |
| References | 3GPP TS 23.501 [24] ETSI TS 123.003 [6] clause 6.2.2 |
| Example | See definition of IMEISV |
| Regular expression |  |
| XSD |  |
| ASN.1 |  |

GPSIMSISDN

| Name | GPSIMSISDN |
| --- | --- |
| Description | Generic Public Subscription Identifier as defined in 3GPP 23.501 [24] clause 5.9.8 in MSISDN representation (see TS 23.003 [6] clause 3.2). |
| Usage guidance | In 3GPP Release 15 a GPSI may contain either a MSISDN, or an External Identifier given as a an NAI following the rules given in 3GPP TS 23.003 [6] clause 19.7.2. This representation is used for MSISDN following the format given in ETSI TS 123.003 [6] clause 3.2. |
| References | 3GPP TS 23.501 [24] ETSI TS 123.003 [6] |
| Example | 31612345678 |
| Regular expression | ^[0-9]{1,15}$ |
| XSD | simpleType |
| ASN.1 | NumericString (SIZE(1..15)) |

GPSINAI

| Name | GPSINAI |
| --- | --- |
| Description | Generic Public Subscription Identifier as defined in 3GPP 23.501 [24] clause 5.9.8 in NAI representation. |
| Usage guidance | In 3GPP Release 15 a GPSI may contain either a MSISDN, or an External Identifier given as a an NAI following the rules given in 3GPP TS 23.003 [6] clause 19.7.2. This representation is used for NAI format. |
| References | 3GPP TS 23.501 [24] IETF RFC 4282 [26] |
| Example | See definition of NAI |
| Regular expression |  |
| XSD |  |
| ASN.1 |  |

NAI

| Name | NAI |
| --- | --- |
| Description | Network Access Identifier following the format given in IETF RFC 4282 [26]. |
| Usage guidance | In general an NAI will take the form “username@realm”. |
| References | IETF RFC 4282 [26] |
| Example | user@homerealm.example.net |
| Regular expression | - |
| XSD | NAI, simpleType |
| ASN.1 | NAI, UTF8String |

THIRD CHANGE

Annex A (normative):
XSD definition

<?xml version="1.0" encoding="utf-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema" xmlns="http://uri.etsi.org/03280/common/2019/02" targetNamespace="http://uri.etsi.org/03280/common/2019/02" version="2.3.1" elementFormDefault="qualified">

<xs:simpleType name="ShortString">

<xs:restriction base="xs:string">

<xs:maxLength value="255"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="LongString">

<xs:restriction base="xs:string">

<xs:maxLength value="65535"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="LIID">

<xs:restriction base="xs:normalizedString">

<xs:pattern value="([!-~]{1,25})|([0-9a-f]{26,50})"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="UTCDateTime">

<xs:restriction base="xs:dateTime">

<xs:pattern value="[0-9]{4}-[0-9]{2}-[0-9]{2}T[0-9]{2}:[0-9]{2}:[0-9]{2}Z"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="UTCMicrosecondDateTime">

<xs:restriction base="xs:dateTime">

<xs:pattern value="[0-9]{4}-[0-9]{2}-[0-9]{2}T[0-9]{2}:[0-9]{2}:[0-9]{2}\.[0-9]{6}Z"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="QualifiedDateTime">

<xs:restriction base="xs:dateTime">

<xs:pattern value="[0-9]{4}-[0-9]{2}-[0-9]{2}T[0-9]{2}:[0-9]{2}:[0-9]{2}(Z|[+-][0-9]{2}:[0-9]{2})"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="QualifiedMicrosecondDateTime">

<xs:restriction base="xs:dateTime">

<xs:pattern value="[0-9]{4}-[0-9]{2}-[0-9]{2}T[0-9]{2}:[0-9]{2}:[0-9]{2}\.[0-9]{6}(Z|[+-][0-9]{2}:[0-9]{2})"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="InternationalE164">

<xs:restriction base="xs:token">

<xs:pattern value="[0-9]{1,15}"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="IMSI">

<xs:restriction base="xs:token">

<xs:pattern value="[0-9]{6,15}"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="IMEI">

<xs:restriction base="xs:token">

<xs:pattern value="[0-9]{14}"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="IMEICheckDigit">

<xs:restriction base="xs:token">

<xs:pattern value="[0-9]{15}"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="IMEISV">

<xs:restriction base="xs:token">

<xs:pattern value="[0-9]{16}"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="IPv4Address">

<xs:restriction base="xs:token">

<xs:pattern value="((25[0-5]|2[0-4][0-9]|[01]?[0-9]?[0-9])\.){3}(25[0-5]|2[0-4][0-9]|[01]?[0-9]?[0-9])"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="IPv4CIDR">

<xs:restriction base="xs:token">

<xs:pattern value="((25[0-5]|2[0-4][0-9]|[01]?[0-9]?[0-9])\.){3}(25[0-5]|2[0-4][0-9]|[01]?[0-9]?[0-9])/([1-2]?[0-9]|3[0-2])"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="IPv6Address">

<xs:restriction base="xs:token">

<xs:pattern value="([0-9a-f]{4}:){7}([0-9a-f]{4})"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="IPv6CIDR">

<xs:restriction base="xs:token">

<xs:pattern value="([0-9a-f]{4}:){7}([0-9a-f]{4})/(([1-9][0-9]?)|(1[0-1][0-9])|(12[0-8]))"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="IPAddress">

<xs:choice>

<xs:element name="IPv4Address" type="IPv4Address"/>

<xs:element name="IPv6Address" type="IPv6Address"/>

</xs:choice>

</xs:complexType>

<xs:complexType name="IPCIDR">

<xs:choice>

<xs:element name="IPv4CIDR" type="IPv4CIDR"/>

<xs:element name="IPv6CIDR" type="IPv6CIDR"/>

</xs:choice>

</xs:complexType>

<xs:simpleType name="TCPPort">

<xs:restriction base="xs:integer">

<xs:minExclusive value="1"/>

<xs:maxInclusive value="65535"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="TCPPortRange">

<xs:sequence>

<xs:element name="start" type="TCPPort"/>

<xs:element name="end" type="TCPPort"/>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="UDPPort">

<xs:restriction base="xs:integer">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="65535"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="UDPPortRange">

<xs:sequence>

<xs:element name="start" type="UDPPort"/>

<xs:element name="end" type="UDPPort"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="Port">

<xs:choice>

<xs:element name="TCPPort" type="TCPPort"/>

<xs:element name="UDPPort" type="UDPPort"/>

</xs:choice>

</xs:complexType>

<xs:complexType name="PortRange">

<xs:choice>

<xs:element name="TCPPortRange" type="TCPPortRange"/>

<xs:element name="UDPPortRange" type="UDPPortRange"/>

</xs:choice>

</xs:complexType>

<xs:complexType name="IPAddressPort">

<xs:sequence>

<xs:element name="address" type="IPAddress"/>

<xs:element name="port" type="Port"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="IPAddressPortRange">

<xs:sequence>

<xs:element name="address" type="IPAddress"/>

<xs:element name="portRange" type="PortRange"/>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="MACAddress">

<xs:restriction base="xs:token">

<xs:pattern value="([a-f0-9]{2}:){5}[a-f0-9]{2}"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="EmailAddress">

<xs:restriction base="ShortString">

<xs:pattern value="[a-zA-Z0-9\.!#$%&amp;'\*\+\\/=\?\^_`\{\|\}~\-]+@[a-zA-Z0-9]([a-zA-Z0-9-]{0,61}[a-zA-Z0-9])?(\.[a-zA-Z0-9]([a-zA-Z0-9-]{0,61}[a-zA-Z0-9])?)*"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="UUID">

<xs:restriction base="xs:token">

<xs:pattern value="[a-f0-9]{8}-[a-f0-9]{4}-[a-f0-9]{4}-[a-f0-9]{4}-[a-f0-9]{12}"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="ISOCountryCode">

<xs:restriction base="xs:token">

<xs:pattern value="[A-Z]{2}"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="SIPURI">

<xs:restriction base="xs:anyURI">

<xs:pattern value="sips?:[a-zA-Z0-9!#$&amp;-;=?-\[\]_~%]+"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="TELURI">

<xs:restriction base="xs:anyURI">

<xs:pattern value="tel:[a-zA-Z0-9!#$&amp;-;=?-\[\]_~%]+"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="WGS84CoordinateDecimal">

<xs:sequence>

<xs:element name="latitude" type="WGS84LatitudeDecimal"/>

<xs:element name="longitude" type="WGS84LongitudeDecimal"/>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="WGS84LatitudeDecimal">

<xs:restriction base="xs:string">

<xs:pattern value="[NS][0-9]{2}\.[0-9]{6}"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="WGS84LongitudeDecimal">

<xs:restriction base="xs:string">

<xs:pattern value="[EW][0-9]{3}\.[0-9]{6}"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="WGS84CoordinateAngular">

<xs:sequence>

<xs:element name="latitude" type="WGS84LatitudeAngular"/>

<xs:element name="longitude" type="WGS84LongitudeAngular"/>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="WGS84LatitudeAngular">

<xs:restriction base="xs:string">

<xs:pattern value="[NS][0-9]{6}\.[0-9]{2}"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="WGS84LongitudeAngular">

<xs:restriction base="xs:string">

<xs:pattern value="[EW][0-9]{7}\.[0-9]{2}"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="SUPIIMSI">

<xs:restriction base="IMSI">

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="SUPINAI">

<xs:restriction base="NAI">

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="SUCI">

<xs:restriction base="xs:hexBinary"/>

</xs:simpleType>

<xs:simpleType name="PEIIMEI">

<xs:restriction base="IMEI">

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="PEIIMEICheckDigit">

<xs:restriction base="IMEICheckDigit">

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="PEIIMEISV">

<xs:restriction base="IMEISV">

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="GPSIMSISDN">

<xs:restriction base="xs:token">

<xs:pattern value="[0-9]{1,15}"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="GPSINAI">

<xs:restriction base="NAI">

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="NAI">

<xs:restriction base="xs:string">

</xs:restriction>

</xs:simpleType>

</xs:schema>

FOURTH CHANGE

Annex B (normative):
ASN.1 definition

Common-Parameters

{itu-t(0) identified-organization(4) etsi(0) common-parameters(3280) version231(231)}

DEFINITIONS IMPLICIT TAGS EXTENSIBILITY IMPLIED ::= BEGIN

-- Object Identifier definitions

commonParameterDomainId OBJECT IDENTIFIER ::= {itu-t(0) identified-organization(4) etsi(0) common-parameters(3280) version231(231)}

-- Common Parameter below are as defined in clause 6

LIID ::= OCTET STRING (SIZE (1..25))

TCPPort ::= INTEGER (1..65535)

TCPPortRange ::= SEQUENCE

{

start [0] TCPPort,

end   [1] TCPPort

}

UDPPort ::= INTEGER (0..65535)

UDPPortRange ::= SEQUENCE

{

start [0] UDPPort,

end   [1] UDPPort

}

Port ::= CHOICE

{

tCPPort [0] TCPPort,

uDPPort [1] UDPPort

}

PortRange ::= CHOICE

{

tCPPortRange [0] TCPPortRange,

uDPPortRange [1] UDPPortRange

}

QualifiedDateTime ::= GeneralizedTime

QualifiedMicrosecondDateTime ::= GeneralizedTime

WGS84CoordinateDecimal ::= SEQUENCE

{

latitude  [0] WGS84LatitudeDecimal,

longitude [1] WGS84LongitudeDecimal

}

WGS84LatitudeDecimal ::= OCTET STRING (SIZE (10))

WGS84LongitudeDecimal ::= OCTET STRING (SIZE (11))

WGS84CoordinateAngular ::= SEQUENCE

{

latitude  [0] WGS84LatitudeAngular,

longitude [1] WGS84LongitudeAngular

}

WGS84LatitudeAngular ::= OCTET STRING (SIZE (10))

WGS84LongitudeAngular ::= OCTET STRING (SIZE (11))

InternationalE164 ::= NumericString (SIZE(1..15))

IMSI ::= NumericString (SIZE(6..15))

IMEI ::= NumericString (SIZE(14))

IMEICheckDigit ::= NumericString (SIZE(15))

IMEISV ::= NumericString (SIZE(16))

SUPIIMSI ::= IMSI

SUPINAI ::= NAI

SUCI ::= OCTET STRING

PEIIMEI ::= IMEI

PEIIMEICheckDigit ::= IMEICheckDigit

PEIIMEISV ::= IMEISV

GPSIMSISDN ::= NumericString (SIZE(1..15))

GPSINAI ::= NAI

NAI ::= UTF8String

END

FIFTH CHANGE

# 3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

3GPP	3rd Generation Partnership Project

ASCII	American Standard Code for Information Interchange

ASN.1	Abstract Syntax Notation One

CC	Content of Communication

CIDR	Classless Inter-Domain Routing

CSP	Communications Service Provider

GPSI	Generic Public Subscription Identifier

HEX	HEXadecimal

HI	Handover Interface

HI1	Handover Interface port 1 (for administrative information)

HI2	Handover Interface port 2 (for Intercept Related Information)

HI3	Handover Interface port 3 (for Content of Communication)

IEC	International Electrotechnical Commission

IEEE	Institute of Electrical and Electronics Engineers

IETF	Internet Engineering Task Force

IMEI	International Mobile station Equipment Identity

IMEISV	International Mobile station Equipment Identity and Software Version number

IMSI	International Mobile Subscriber Identity

IP	Internet Protocol

IPv4	Internet Protocol version 4

IPv6	Internet Protocol version 6

IRI	Intercept Related Information

ISO	International Organisation for Standardisation

ITU-T	International Telecommunication Union - Telecommunication

LEA	Law Enforcement Agency

LIID	Lawful Interception Identifier

MAC	Media Access Control

NIMA	National Imagery and Mapping Agency

NAI	Network Access Identifier

PEI	Permanent Equipment Identifier

POSIX	Portable Operating System Interface

RFC	Request For Comments

SIP	Session Initialization Protocol

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

TCP	Transmission Control Protocol

UDP	User Datagram Protocol

URI	Uniform Resource Identifier

UTC	Coordinated Universal Time

UUID	Universally Unique Identifier

WGS84	World Geodetic System 1984

XML	eXtended Markup Language

XSD	XML Schema Definition