LI(19)50014r2

TC LI#50

| ETSI/TC LI |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  | TS102232-7CR007 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CR status: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | TS 102 232-7 |  |  |  |  |  |  |  |  | CR |  |  | 007 |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  | 3.18.1 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Affected version of the standard  |  |  |  |  |  |  |  |  |  |  |
| ETSI number of TC LI  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by TO of ETSI/TC LI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  | TC LI#50 |  |  | for approval |  |  |  |  |  | X |  | strategic |  |  |  |  |  |  |  |  |  |  |  | editorial |  |  |  |  |
|  |  |  |  | for information |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  | Tencastle, UK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  | 07/02/2019 |  |  |  |
| Subject: |  | Inclusion of 33.128 services in part 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: |  | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (only one category |  | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| shall be marked |  | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| with an X) |  | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | 3GPP SA3-LI has elected to adopt a number of ETSI TC LI standards as part of its work on 5G LI. In order to support their work, TS 102 232-1 needs fields to transport SA3-LI defined IRI and CC PDUs. It is assumed that, due to Rel15 timescales, the most sensible option is to include 5G details in Part 7. This approach simply requires us to add a new option to each of the CCContents and IRIContents fields to allow SA3-LI-defined PDUs to be transported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2.1, 5, 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other docs affected: |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| version number to be included |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (Section 1 - Scope)

# 1	Scope

## Introduction

The present document specifies an approach for the handover of the lawfully-intercepted information that is defined in the two standards: 3GPP TS 33.108 [3] and ANSI/J-STD-025-B [4]. The present document uses the handover techniques defined in ETSI TS 102 232-1 [2]. In this way, the present document allows additional services to be delivered through a common interface.

## UMTS/GPRS and EPS - 3GPP TS 33.108

The scope of the present document includes the handover of lawfully-intercepted information from the following parts of 3GPP TS 33.108 [3]:

- Intercept Related Information (IRI) and the Content of Communication (CC) from the mobile circuit-switched domain (3GPP TS 33.108 [3], clause 5).

- IRI and CC from the mobile packet-switched domain (3GPP TS 33.108 [3], clause 6).

- IRI and CC from the multi-media domain (3GPP TS 33.108 [3], clause 7).

- IRI and CC from the EPS domain (3GPP TS 33.108 [3], clause 10).

- IRI and CC from the IMS Conference domain (3GPP TS 33.108 [3], clause 11).

- IRI and CC from the IMS-based VoIP domain (3GPP TS 33.108 [3], clause 12).

- IRI from the Proximity Services domain (3GPP TS 33.108 [3], clause 13).

- IRI and CC from the Group Communication System Enablers domain (3GPP TS 33.108 [3], clause 14).

The present document does not override or supersede any specifications or requirements in 3GPP TS 33.108 [3].

## CDMA2000 - ANSI/J-STD-025-B

The scope of the present document includes the handover of lawfully-intercepted information from the following parts of ANSI/J-STD-025-B [4]:

- Call identifying information from the mobile circuit-switched domain (ANSI/J-STD-025-B [4], clause 5.4).
Call Content (CC) from the mobile circuit-switched domain is not covered by the present document.

- Communication identifying information (CII) and CC from the mobile packet-switched domain (ANSI/J-STD-025-B [4], clause 5.5).

The present document does not override or supersede any specifications or requirements in ANSI/J-STD-025-B [4].

3GPP TS 33.128

The scope of the present document includes the handover of lawfully-intercepted information in accordance with 3GPP TS 33.128 [6]

SECOND CHANGE (Section 2 - References)

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Referenced documents which are not found to be publicly available in the expected location might be found at https://docbox.etsi.org/Reference/.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are necessary for the application of the present document.

[1]	ETSI TS 101 671: "Lawful Interception (LI); Handover interface for the lawful interception of telecommunications traffic".

NOTE:	Periodically ETSI TS 101 671 is published as ETSI ES 201 671. A reference to the latest version of the TS as above reflects the latest stable content from ETSI/TC LI.

[2]	ETSI TS 102 232-1: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 1: Handover specification for IP delivery".

[3]	ETSI TS 133 108: "Universal Mobile Telecommunications System (UMTS); LTE; 3G security; Handover interface for Lawful Interception (LI) (3GPP TS 33.108)".

[4]	TIA/ATIS ANSI/J-STD-025-B (July 2006): "Lawfully Authorized Electronic Surveillance", as amended by ANSI/J-STD-025-B-1 (September 2006): "Lawfully Authorized Electronic Surveillance (LAES) Addendum 1 - Addition of Mobile Equipment IDentifier (MEID)" and by ANSI/J-STD-025-B-2 (April 2007): "Lawfully Authorized Electronic Surveillance (LAES) - Addendum 2 - Support for Carrier Identity".

[5]	Public Law 103-414: "Communications Assistance for Law Enforcement Act (CALEA)", US 103rd Congress, 108 STAT. 4279 (October 25, 1994).

[6]	3GPP TS 33.128: "Handover interface for Lawful Interception (LI)".

THIRD CHANGE (Section 5)

# 5	3GPP handover Headers, data exchange and networks

## 5.1	Approach

ETSI TS 102 232-1 [2] describes a technique for data exchange, and specifies the headers that shall be associated with the results of interception. The present document follows ETSI TS 102 232-1 [2] regarding headers, data exchange and networks demonstrates how the header fields in ETSI TS 102 232-1 [2] can be populated in a direct and straightforward manner using the interception information available in 3GPP TS 33.108 [3] and 3GPP TS 33.128 [6].

## 5.2	Structures

IRI events from 3GPP TS 33.108 [3], for both circuit and packet switched services, are sent using the uMTSIRI element of IRIContents.

CC from CS domain delivery in IP are sent using the CSvoice-CC-PDU element of CCContents, which is the CSvoice-CC-PDU from 3GPP TS 33.108 [3], containing cSvoiceLIC-header and payload.

CC from packet switched services are sent using the uMTSCC element CCContents, which is an OCTET STRING.

Alternatively, subject to national agreement, CC from packet switched services are sent using the uMTSCC-CC-PDU element of CCContents, which is the CC-PDU from 3GPP TS 33.108 [3], containing the uLIC-header and payload.

CC and IRI PDUs formatted according to 3GPP TS 33.128 [6] are sent using the threeGPP33128DefinedCC and threeGPP33128DefinedIRI elements of CCContents and IRIContents respectively.

FOURTH CHANGE (New Section)

# 15		 IRI and CC for services defined in 3GPP TS 33.128

## 15.1	Definition of IRI events and CC

IRI and CC events are defined as per 3GPP TS 33.128 [6].

## 15.2	IRI and CC format

3GPP TS 33.128 [6] defines the format for the contents of the threeGPP33128DefinedIRI and threeGPP33128DefinedCC elements for each IRI and CC event.