LI(19)P50015r3

TC LI#50

| ETSI/TC LI |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  | TS102232-1CR003 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CR status: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | TS 103 221-1 |  |  |  |  |  |  |  |  | CR |  |  | 003 |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  | 1.3.1 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Affected version of the standard  |  |  |  |  |  |  |  |  |  |  |
| ETSI number of TC LI  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by TO of ETSI/TC LI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  | TC LI#50 |  |  | for approval |  |  |  |  |  | X |  | strategic |  |  |  |  |  |  |  |  |  |  |  | editorial |  |  |  |  |
|  |  |  |  | for information |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  | Tencastle, UK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  | 07/02/2019 |  |  |  |
| Subject: |  | Support for 5G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: |  | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (only one category |  | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| shall be marked |  | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| with an X) |  | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | 3GPP SA3-LI has elected to adopt a number of ETSI TC LI standards as part of its work on 5G LI. In order to support their work, TS 103 22-1 needs to include a number of additional fields, including; - 5G-specific identifiers such as SUPI from TS 103 280 (added via a separate CR). - Support for ADMF-MDF interface (i.e. inclusion of LIID/XID mapping) - Add the option to specify a correlation number for a Task to support triggering scenarios (where e.g. a CC-TF must specify what correlation number to assign to traffic intercepted at a CC-POI) - Addition of an “implicitDeactivationAllowed” flag to allow Tasks to “delete” themselves (requested by 3GPP SA3-LI for one-shot triggering scenarios such certain LALS use cases) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, 5.1.1, 6.2.1.2, 6.3.1.2, 6.5.2.2, Annex D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other docs affected: |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| version number to be included |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (Section 4)

# 4	Overview

## 4.1	Reference model

## 4.1.1		Introduction

The X1 interface is based on communication between two entities; the controlling function (e.g. a CSP Administration Function (ADMF)), and the controlled function (e.g. a Network Element (NE) performing interception or mediation). The X1 reference model is shown in figure 1.

Figure 1: X1 reference model

While this document uses the terms “NE”, the term is intended to represent any given network function (NF) which is intended to be given information regarding interception or mediation. Similarly, the term “ADMF” is intended to represent any given network function that controls interception or mediation in other functions.

## 4.1.2	ADMF deployment model

## Figure 2 shows a deployment model for X1 where a CSP ADMF uses X1 to provision a number of NEs to perform interception.

Figure 2: X1 reference model

Only one ADMF shall make changes by X1 to a given NE. This is called the ADMF which is "responsible" for that NE.

Onward delivery of information from the NE is called X2 (for IRI) and X3 (for CC). The choice of protocols for X2 and X3 are out of scope of the present document.

Some deployments may involve multiple ADMFs for redundancy or other purposes; where multiple ADMFs are required, the NE shall be implemented such that it presents itself as a separate NE to each ADMF.

ADMF and NE shall implement time synchronization where possible; in situations where it is not possible, the ADMF shall maintain knowledge of the timing offset between the ADMF and NE.

NOTE 1:	The present document may be used in direct delivery scenarios, in which the NE delivers directly to the LEMF. Any consequences of using direct delivery are out of scope of the present document.

## 4.1.3	Triggering deployment model

Figure 3 shows another possible deployment model for X1, where the X1 protocol is used to trigger interception by one in a second network function. In this deployment model, the “Triggering Function” takes on the role of the ADMF in the previous deployment model, while the “Triggered Function” takes on the role of the NE.

Figure 3: X1 deployment model for Triggering Functions

If this deployment model is used, then in the following clauses references to the ADMF should be interpreted as applying to the Triggering Function, while references to the NE should be interpreted as references to the Triggered Function.

## 4.1.4	Mediation function deployment model

Figure 4 shows another possible deployment model for X1, where the X1 protocol is used to manage a CSP mediation function. In this deployment model, the MF takes on the role of the NE in the previous deployment model.

Figure 4: X1 deployment model for Mediation Functions

If this deployment model is used, then in the following clauses references to the NE should be interpreted as applying to the MF.

SECOND CHANGE (Section 5.1.1)

### 5.1.1	Start and end of a Task

A Task relates to a single target identifier, and goes from the point an ActivateTask Request is sent by the ADMF to the time a DeactivateTask Request is sent by the ADMF, a "terminating fault" occurs, or (for Tasks with the 
“ImplicitDeactivationAllowed” flag set) the NE determines that it has completed.

The present document does not define which situations are categorized as “terminating faults”. Local recovery procedures should be followed before a Task is ended with a “terminating fault”. In general, irrecoverable failures with an interception, or major security issues at an NE should be considered terminating faults, and certain outcomes with keepalives are also terminating faults (where defined in clause 6.6.2).

THIRD CHANGE (Section 6.2.1.2)

#### 6.2.1.2	TaskDetails

The TaskDetails structure shall include the following.

Table 4: TaskDetails

| Field | Description | Format | M/C/O |
| --- | --- | --- | --- |
| XID | Uniquely identifies the Task. There may be more than one different Task relating to the same target identifier (two distinct XIDs). The X1 interface supports delivery of this situation (i.e. it is not considered an error on the X1 interface). | UUIDv4 (see clause 5.1). | M |
| TargetIdentifiers | List of criteria which are used to identify the traffic to be intercepted. Where multiple criteria are present, all criteria are required to be matched. If an NE cannot target based on the criteria specified (e.g. due to an unsupported format or inappropriate combination of identifiers) the NE shall reject the request with an appropriate error. It is an implementation decision which identifiers and combinations of identifiers are supported. | Each TargetIdentifier given follows one of the formats given in table 5. | M |
| DeliveryType | Statement of whether to deliver X2 and/or X3. An MF shall ignore the contents of the field, and use the DeliveryType value given in the relevant MediationDetails structure (see Annex D) | Enumerated value - one of "X2Only", "X3Only" and "X2andX3". | M |
| ListOfDIDs | Details of where to send the intercepted traffic. It is an implementation decision for the NE to determine how to duplicate traffic if multiple destinations are specified, or if multiple destinations are supported. | List of Destination Identifiers (DID) referencing the desired delivery destination records. | M |
| ListOfMediationDetails | Set of details for use by an NE that is performing mediation (i.e. a mediation function) | See Annex D | O |
| CorrelationID | Correlation identifier to assign to intercepted material for this Task. Intended for use in triggering scenarios, and shall be ignored by non-mediation function NEs. | Unsigned integer | O |
| ImplicitDeactivationAllowed | Indication that a Task may implicitly deactivate itself once the NE has determined that it has completed. On deactivation of the Task, the NE shall issue a ReportTaskIssue message with the appropriate TaskReportType (see clause 6.5.2) | Boolean | O |
| TaskDetailsExtensions | One or more extension placeholders; each may be populated by a list of elements defined by external specifications. | See annex B. | O |

If a Task is specific with an invalid combination of DeliveryType and Destinations (e.g. "X2andX3" delivery specified, but only an X2 Destination given), then the NE shall reject the ActivateTaskRequest with an appropriate error.

The list of permissible TargetIdentifier formats is given in table 5.

Table 5: TargetIdentifier Formats

| Format Name | Description | Format |
| --- | --- | --- |
| E164Number | E.164 Number in fully international format, written as decimal digits | Given in ETSI TS 103 280 [4] InternationalE164 format |
| IMSI | International Mobile Subscriber Identity, following the Recommendation ITU-T E.212 [5] numbering scheme, written as decimal digits | Given in ETSI TS 103 280 [4] IMSI format |
| IMEI | International Mobile station Equipment Identity, following the numbering plan defined in ETSI TS 123 003 [6], written as decimal digits without the (Luhn) check digit | Given in ETSI TS 103 280 [4] IMEI format |
| MACAddress | A MAC address | Given in ETSI TS 103 280 [4] MACAddress format |
| IPv4Address | An IPv4 address | Given in ETSI TS 103 280 [4] IPv4Address format |
| IPv6Address | IPv6 address | Given in ETSI TS 103 280 [4] IPv6Address format |
| IPv4CIDR | IPv4CIDR, written in dotted decimal notation followed by CIDR notation | Given in ETSI TS 103 280 [4] IPv4CIDR format |
| IPv6CIDR | IPv6CIDR written as eight groups of four hexadecimal digits separated by a colon, followed by CIDR notation | Given in ETSI TS 103 280 [4] IPv6CIDR format |
| TCPPort | TCP Port number, written in decimal notation | Given in ETSI TS 103 280 [4] TCPPort format |
| TCPPortRange | Range of TCP Ports, written as decimal numbers separated by a colon | Given in ETSI TS 103 280 [4] TCPPortRange format |
| UDPPort | UDP Port number, written in decimal notation | Given in ETSI TS 103 280 [4] UDPPort format |
| UDPPortRange | Range of UDP Ports, written as decimal numbers separated by a colon | Given in ETSI TS 103 280 [4] UDPPortRange format |
| EmailAddress | Email address following W3C HTML 5 Recommendation | Given in ETSI TS 103 280 [4] EmailAddress format |
| SIP-URI | SIP-URI according to the SIP URI scheme given in IETF RFC 3261 [7] | Given in ETSI TS 103 280 [4] SIPURI format |
| TEL-URI | TEL-URI according to the TEL URI scheme (see IETF RFC 3966 [8]) Implementers should consider whether the value could be sent as an E.164 number (or one of the related types) instead | Given in ETSI TS 103 280 [4] TELURI format |
| H323-URI | H323 URI according to the H323 URI scheme (see IETF RFC 3508 [9]) | Given in H323Uri format (see XSD schema) |
| IMPU | IP Multimedia Public Identity, as per ETSI TS 123 003 [6] | Given in IMPU format (see XSD schema) |
| IMPI | IP Multimedia Private Identity, as per ETSI TS 123 003 [6] | Given in IMPI format (see XSD schema) |
| NAI | Network Access Identifier following IETF RFC 7542 [10] format | Given in ETSI TS 103 280 [4] NAI format. |
| RADIUS | Any Radius attribute that uniquely identifies the subscriber within the specific CSP (see note 1) | Given as binary octets containing RADIUS AVP following IETF RFC 2865 [11] clause 5 (see note 2) |
| GTPUTunnelId | GTP-U Tunnel Identifier | Given as a 32-bit integer |
| GTPCTunnelId | GTP-C Tunnel Identifier | Given as a 32-bit integer |
| CallPartyRole | Identifies the role of a party in a call. Intended for use in conjunction with e.g. E164Number | One of the values "Originating", "Terminating", "ForwardedTo" |
| NonLocalIdentifier | Identifies whether the identifier is local or non-local. Intended for use in conjunction with e.g. E164Number | One of the values "Local" or "NonLocal" |
| SUPIIMSI | Subscription Permanent Identifier in IMSI format | Given in ETSI TS 103 280 [4] SUPIIMSI format |
| SUPINAI | Subscription Permanent Identifier in NAI format | Given in ETSI TS 103 280 [4] SUPINAI format |
| SUCI | Subscription Concealed identifier | Given in ETSI TS 103 280 [4] SUCI format |
| PEIIMEI | Permanent Equipment Identifier in IMEI format | Given in ETSI TS 103 280 [4] PEIIMEI format |
| PEIIMEICheckDigit | Permanent Equipment Identifier in IMEICheckDigit format | Given in ETSI TS 103 280 [4] PEIIMEICheckDigit format |
| PEIIMEISV | Permanent Equipment Identifier in IMEISV format | Given in ETSI TS 103 280 [4] PEIIMEISV format |
| GPSIMSISDN | General Purpose Subscription Identifier in MSISDN format | Given in ETSI TS 103 280 [4] GPSIMSISDN format |
| GPSINAI | General Purpose Subscription Identifier in NAI format | Given in ETSI TS 103 280 [4] GPSINAI format |
| TargetIdentifierExtension | Identifier defined by an external specification | See annex B |
| NOTE 1: Future versions of the present document may need to consider temporary identifiers including pseudonyms or short-term identifiers which have been derived from the permanent identifiers. NOTE 2: Depending on NE implementation, this may not be exactly the same binary representation used to match traffic e.g. for case-insensitive matching. |  |  |

FOURTH CHANGE (Section 6.3.1.2)

#### 6.3.1.2	DestinationDetails

DestinationDetails relate to the delivery of information from the NE to a Destination..

FIFTH CHANGE (Section 6.5.2.2)

#### 6.5.2.2	Task report types

The TaskReportType shall be one of the following:

- All clear: non-terminating fault resolved.

- Warning: not traffic-affecting.

- Non-terminating fault (currently unable to collect traffic but not terminating).

- Terminating fault. The message is used by the NE to indicate that the Task has experiences a terminating fault and has been deactivated.

- Implicit Deactivation: A Task with the “ImplicitDeactivationAllowed” flag has been deactivated.

- Actioned: Request has been fully actioned and was successful (to follow up on "OK-Acknowledged" response from clause 5.2).

- Failed: Request has been fully actioned but was unsuccessful (to follow up on "OK-Acknowledged" response from clause 5.2). This is a terminating fault.

SIXTH CHANGE (New Annex)

Annex D (normative):
Using Task Object at Mediation Functions

# D.1	Introduction

An ADMF may use X1 messages to provision a mediation function instead of a point of interception, following the deployment model given in clause 4.3. This annex describes how the usage and meaning of the messages defined in clause 6 differ when used for this purpose. Unless otherwise specified, the messages are used as for any other NE.

# D.2	TaskDetails

### D.2.1		General

The TaskDetails structure used in the ActivateTask and ModifyTask messages are used as for an NE with the differences described in the following sections.

# D.2.2	MediationDetails structure

The MediationDetails structure provides additional details for a Task, specific to Mediation Functions.

Table D.1: Mediation Details structure

| Field |  | Description |  |  | Format |  |  | M/C/O |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LIID |  | Lawful Interception Identifier associated with the Task (multiple instances of this structure may be used to indicate that multiple LIIDs are associated with the Task) |  |  | LIID as defined in TS 103 280 [4] |  |  | M |  |  |
| DeliveryType | Statement of whether to deliver HI2 and/or HI3 for this LIID. | Enumerated value – one of “HI2Only”, “HI3Only” or “HI2andHI3” | M |
| StartTime |  |  | Start time associated with the activation of interception or mediation for the Task (which may be in the future) |  |  | Timestamp |  |  | O |  |  |
| EndTime |  |  | End time associated with the deactivation of interception or mediation for the Task |  |  | Timestamp |  |  | O |  |  |
| ListOfDIDs | Details of where to send the intercepted traffic for this LIID. If specified, the details shall be used instead of any delivery destinations specified in the ListOfDIDs field in the TaskDetails structure. | List of Destination Identifiers (DID) referencing the desired delivery destination records. | O |
| MediationDetailsExtension |  |  | One or more extension placeholders; each may be populated by a list of elements defined by external specifications. |  |  | See annex B. |  |  | O |  |  |

NOTE:	This version of the specification does not specify in standard way how delivery restrictions that may be applicable on a per-LIID basis may be controlled over X1. However, this shortfall may be addressed via proprietary extensions or a standard mechanism in the future.