3GPP SA3LI#72 BIS	s3i190106

Dusseldorf, DE (Germany), 26-28 February 2019

Source:	OTD

Title:	pCR on Cell Supplemental Information Reporting

Document for:	Agreement

Agenda Item:	 10

Work Item / Release:	LI15

Abstract of the contribution: Stage 3 description of Cell Supplemental Information Reporting  (clause 7.3.4. of TS 33.128).

*************** Start of changes ******************

### 7.3.4	Cell Database Information Reporting

#### 7.3.4.1	General Description

When a Cell Site Identity is provided in a xIRI event for location, the CSP may provide specific Cell Site Supplemental Information (CSI) records from their Cell Site database. This information should be placed in the events location parameter if the xIRI message is still available in the MDF before its delivery or use a separate Cell Site Report (CSR) capability as described in TS 33.127 [5] Clause 7.3.4 Cell database information reporting. If the Cell Site Supplemental information for a Cell Site Identity has already been sent for an intercept and it has not changed, the CSP is not required to send the record again.

#### 7.3.4.2	Cell Site Reporting in xIRI event

If a xIRI event record from the network contains a Cell Site Identity (e.g. Cell Global Identifier) provided for location information of the target, the MF should enrich the xIRI Record with Cell Site Supplemental Information retrieved from the CSP's Cell Site Database prior to delivery of the xIRI event to the LEMF. The parameters as shown in Table 7.3.4.2-1 shall be included within the location parameter. However, if the MF cannot insert this information without undue delay into the xIRI event Record, the MF should create and deliver a separate CSR as shown in 7.3.4.3 Table 7.3.4.3-1.

Table 7.3.4.2-1: Cell Site Supplemental information Parameters

| Field name | Description | M/C/O |
| --- | --- | --- |
| location | Shall provide Cell Supplemental Information for location of the Cell Identity, if available. | C |
| operatorSpecificInfo | If provided, other Carrier-specific information whose format is not standardized inserted here. | C |

#### 7.3.4.3	xIRI Cell Site Report

A dedicated Cell Site Report Record shall be created when the MF could not enrich the xIRI event message as described in clause 7.3.4.2.

If the Cell Site Supplemental information for a Cell Identity has already been sent for an intercept and it has not changed, the CSP is not required to send the record again.

- The type of this xIRI report shall use the IRI-Report-record:

Table 7.3.4.3-1: Cell Site Report record

| Parameter | Description/Conditions | M/C/O |
| --- | --- | --- |
| eventType | Shall indicate a CSR report. | M |
| timeStamp | Shall provide the date and time the event is detected. | M |
| networkIdentifier | Shall be provided. | M |
| lawfulInterceptionIdentifier | The Lawful Interception Identifier. | M |
| location | Shall provide Cell Supplemental Information for location of the Cell Identity, if available. The Location parameter shall also include the identity of the cell location (in globalCellID for instance). | C |
| operatorSpecificInfo | If provided, other Carrier-specific information whose format is not standardized inserted here. | C |

ASN

XIRICellSiteReportMessage ::= SEQUENCE

{

-- See clause 7.3.4 for details of this structure

eventType					[1]	EventType, -- Cell Site report (CSR)

timestamp					[2]	Timestamp,

networkIdentifier			[3] Network-Identifier,

lIID						[4]	LawfulInterceptionIdentifier,

location					[5]	LocationInformation

}

Note: within 33.108 operatorSpecificInfo is part of Location sequence, this is TBD for 33.128.

*************** End of all changes ******************