3GPP TSG-SA WG3 LI Meeting #73	s3i190250

April 9-12, 2019; La Jolla, CA

Source:	Ministère de l’économie et des finances (France)

Title:	discussion paper on MSRP in TS 33108 or TS 33128 as parameter to decode RCS services

Document for:	Agreement/discussion

Agenda Item:	9.1

Work Item / Release:	LI16

Abstract of the contribution: In 3GPP meetings in 2017 and 2018, discussions were held related to add TS 102 32-5 parameters related to IMS and eRCS that were not in TS 33.108 and indirectly TS 102 232-7. Decision was made to agree on the concept related to eRCS but to wait for full interception for all IMS based services in TS 33.128 and in TS 33.127. It seems that eIMS5G study has been recently completed by SA2 that may be the basis for IMS development for SBA/slices issues for R16.

Guidance is requested to SA3LI related to eRCS with LTE access and indirectly IMS development with NR access.

## Context

In New Dehli meeting (3GPP SA3LI#68), Telefonica and Ministery of Economy and Finances, proposed discussion papers (revised in s3i180060, s3i180032) with draft CRs at the same time to additional fields to make migration from ETSI TS 102 232-5 to TS 33.108 possible, specially related to eRCS services based on IMS plateform and specially deliver MSRP frametype in HI3.

- For s3i180032, “It was agreed to generate a CR (s3i180064) based on this draft CR and some of the content discussed in s3i180060.”

- For S3i180064 (and s3i180060), “It was pointed out that a CR (S3i180064) will also be required for TS 33.107 to add SIP IP addresses to Stage2. For such reason this CR was not pursued in order to be able to align two CRs to be generated at the same time for TSs 33.107&108, for the next meeting, SA3LI#69.

During SA3LI#69 (Newport Beach), Telefonica made two related CR on TS 33.107 and TS 33.108. Telefonica; made:

- s3i180124 (Stage 2 Interception of IMS-based multimedia services), on TS 33107,

- s3i180125 (Stage 3 Interception of IMS-based multimedia services), on TS 33108.

SA3LI agreed with the concept however decided not to implement this change now and instead implement full interception for all IMS based services in TS 33.128 and in TS 33.127.

## IMS based services and 5GC

It has to be noted that

- TS 33.108 and TS 33.107 are quite frozen standards for R15.

- TS 33.128 and TS 33.127 do cover 5GC development for R15, for data access.

Full interception for all IMS based services in TS 33.128 and in TS 33.127 haven’t been developped, waiting for IMS development on R16 by other 3GPP groups ((5GS) (such equivalent to the SRVCC operation defined for 4G networks (EPS).

According to GSMA’s ‘Road to 5G Introduction and Migration”, with recommendation to MNOs; it seems that VoLTE in case option 3X but also option 2 (5GC if NR coverage is continuous)):

https://www.gsma.com/futurenetworks/wp-content/uploads/2018/04/Road-to-5G-Introduction-and-Migration_FINAL.pdf

3GPP SA has specified that 5G must provide voice/video communication services based on the IMS. TR 23.794, “study on Enhanced IMS to 5GC Integration” of SA2 has been approved at the last SA#83. This TR is to study and specify potential enhancements to the IMS architecture to enable IMS FE to integrate with the 5GC network functions to enable IMS applications to directly leverage the features and capabilities of 5GC. It cover issue related to SBA, HSS’s SBI , and slice impacts related to IMS services (Release 16).

## eRCS?

In the following meetings of SA3LI#69 (Newport Beach Meeting), many CRs were made in meetings during 2018 and beginning of 2019 related to TS 33.108 and TS 33.107 (i.e.: S8HR LI: Location reporting corrections, Reporting Media Bearer information for S8HR, Cell Supplemental Information Reporting, Addition of PTC References, and recently LI function on SCEP for IoT)

Unfortunately, MNOs are working to develop GSMA’s e RCS services with LTE access.

## Guidances and decisions from SA3LI

- Do SA3LI confirm to wait for full interception of IMS based services, in R16 in order to improve TS 33.127 and TS 33.128, including LTE access and eRCS?

  - If yes, shall SA3LI wait for SA2 development based on its TR 23.794 on enhanced IMS?

  - If not, how as TS 33.107 and TS 33.108 are quite frozen, but there are Telefonica’s s3i180124 and s3i180125?