3GPP SA3LI#73	s3i190223

# La Jolla (California), US, 9-12 April 2019

Source:	CISCO Systems

Title:	cc-POI/LI-Aggregator for handling Multiple UPFs

Document for:	Discussion, towards CR for 33.127

Agenda Item:			  8.1 - Release 15 and earlier

Work Item / Release:	LI15/rel 15

Abstract of the contribution:

In the CUPS model, typically there are many UPs in a data-center/EPCs/remote RAN deployments. 5GC demands higher performance requirements of both UP and LI functionality. There are aspects of Performance, Security, Reliability, Manageability and Compliance all of which have to be met and work in concert to ensure the disaggregated model delivers unconstrained forwarding and high-speed intercept througputs. In order to support better deployment flexibility, deliver faster LI, and highspeed PgW functions at the UPs, we propose the option to split some of intercept functionality between the (internal)cc-POI at the UP and an (external) cc-POI aggregator function keeping all the semantics as standardized with minimal changes. The idea also helps minimizes a large fan out of connections between the cc-POIs at the UPs and MDF-3, just for handling intercepts from multiple UPs. The proposal extends Li15 and discusses applicability of the external cc-POI as an LI aggregator function for UPs for providing better manageabilty, scale out, and delivering performance benefits when a large number of UPs are deployed in 5GC.

We aim to collaborate with other vendors and operators, to make the CR part of 33.127.

- Introduction

In the 5GC network, user plane functions are separated from the control plane functions. The SMF that handles control plane actions (e.g. establishing, modifying, deleting) for the PDU sessions shall include an IRI-POI that has the LI capability to generate the related xIRI. The UPF that handles the user plane data shall include a CC-POI that has have the capability to duplicate the user plane packets from the PDU sessions based on the interception rules received from the SMF. Figure 6.2-4 shows the LI architecture for SMF/UPF based interception.

Figure 6.2-4: LI architecture showing LI at SMF/UPF

When interception of communication contents is required, the CC-TF present in the SMF sends a trigger to the CC-POI present in the UPF over the LI_T3 interface which can be based on N4 functionalities (between SMF and UPF) with LI specific security measures applied.

The trigger sent from the CC-TF to CC-POI includes the following information:

- User plane packet detection rules.

- Target identity.

- Correlation number.

- MDF3 address.

NOTE 2:	When LI_T3 is used, the LI_X1 between LIPF and CC-POI present in the UPF is used to monitor the user plane data.

The CC-POI present in the UPF generates the xCC from the user plane packets, and delivers the xCC (that includes the correlation number and the target identity) to the MDF3. The MDF3 delivers the CC to the LEMF over LI_HI3.

- Proposed Enhancement

An alternative implementation with an additional external CC-POI is shown in Figure 6.2-5 (also see CR)

Figure 6.2-5: LI architecture showing LI at SMF/UPF with additional external CC-POI.

For purposes of our discussion, we can call the cc-POI at the UPF as an internal cc-POI, and the new entity as an external cc-POI. The UPF duplicates the user plane packets of the traffic to be intercepted (identified by the packet detection rules) as instructed by the SMF and then sends the duplicated user plane packets to the external CC-POI over the X3u reference point. The SMF also provides the forwarding action rules to the UPF over the LI_T3 interface which enables the UPF to determine how to send the duplicated user plane packets over the X3u reference point to the external CC-POI. The CP function provides the intercept control information (such as correlation identifier, target identity, and intercepted packet identification rules) to the external CC-POI over the LI_X3 reference point. The external CC-POI receives the user plane packets from the UP function (over the secure X3u reference point), associates the user plane packets to the target interception based on the intercept related information that it received from the CP function (over the LI_X3 reference point) and then delivers the CC to the MDF3 over the LI_X3 reference point.

NOTE 1:	The external CC-POI can or can be a standalone point or be co-located with a UPF. In the latter case, the architecture reduces to that shown in Figure 6.2-4.

NOTE 2:	The X3u interface is same as that specified in TS 29.244 [x] for EPC and used between CUPS User plane and SX3LIF functionality specified in TS 33.107 [11]. The X3u interface needs to be a secure interface.

- Functional Components

Figure 6.2-6: LI functional components showing external CC-POI.

In figure 6.2-6:, we break down the functional components of our proposal, to demonstrate the interactions possible between the UP/internal cc-POI and the external cc-POI/LI. The external cc-POIs can run on dedicated servers but colocated with the UPs for better manageability, and provisioning via the LI_T3 interfaces per standards previously described. Further the X3_u channel is dedicated and secure between the internal cc-POIs and external cc-POI. The internal cc-POI is communicating over lightweight POI performing LI without compromising on the UPs ability to sustain unimpacted dedicated forwarding performance towards the external cc-POI.

Providing a multiplexing external cc-POI works hand in hand with the internal cc-POIs for each of the UPs.  The UP identified packet detection rules will come into effect to only allow intercepted traffic on the X3_u/IN link, thereby allowing better link utilization between the UPs and external cc-POI. With the proposed solution we can minimize the number of X3 connections that would have to be provisioned at the MDF-3/LI server endpoints for each UP ie. an initial set of (say) TCP endpoints would suffice to handle the load originating from the external cc-POI.

Additionally crypto functions added to each of the UPs for LI operations can potentially constrain their ability to deliver sustained forwarding performance at the expense of server resources and CPU capacity. The internal cc-POI can therefore be a lightweight crypto engine and the external cc-POI can be a advanced server with dedicated crypto engine in hardware. By using a combination of protocols in concert, the solution can deliver higher aggregate LI throughputs with cost-benefits by splitting functions between the internal cc-POI and external cc-POI engines.

Some considerations of transport protocols, security and performance is shown below.

| Transport | X3_u | LI_X3 | Comments |
| --- | --- | --- | --- |
| UDP/rUDP | Yes | Yes | Lightweight/performant rUDP for reliability |
| TCP | Not preferred rUDP preferred | Yes | TCP on X3 interface for reliability when MDF-3 is remote and not colocated. Reliable with performance. Multiple-TCPs or M-TCP solutions towards MDF-3s to increase aggregate thruput. |
| IPSec/d-TLS/TLS | Yes | Yes | Secure. External cc-POI can have custom crypto hardware |

The table provides a representative approach to divide and conquer the options between UPs internal cc-POI and external cc-POI, and between external cc-POI and MDF-3.

In colocated deployments, we can use lightweight protocols (ex: UDP/rUDP) with software-crypto security, over the IN/X3_u interface to the external cc-POI. The external cc-POI/LI node can perform the X3encap/hw-crypto functions and deliver the payload over a reliable transport (ex:TCP or M-TCP) to the MDF-3/LI servers.

The N:1 mode of UPs cc-POI to external cc-POIs provides flexibility, reliability, security with performance. Further the external cc-POI can be considered as a Proxy entity, which enables it to balance traffic and load between faster senders (UP cc-POI) to slow receivers (MDF-3). This can be balanced by establishing an appropriate number of TCP connections on the OUT link in deployment. The external cc-POIs can be further made highly-available for redundancy purposes as necessary to provide deployment flexibility and guaranty performance at all costs, with elasticity.

Here are some of the benefits of the proposed solution:

- Ease of Configuration - more centralized

  - Since the external cc-POI is expected to be deployed as a central node, it is easier to manage and operate

- Minimizes num. of connections on Destination MDF-3 LI server. Ability for LI server to obsorb higher thruputs, while keeping minimal number of connections towards the external cc-POI aggregator

- Lesser X3 connections

  - Since the number of external cc-POIs in the network are expected to be smaller than number of UPs, the number of X3 connections could be lower with X3 on external cc-POIs

- Potential for Automation / Orchestration – controlled environment

  - Sharing the LI Admin keys can be automated so both external cc-POI and UPs internal cc-POI which could be colocated in remote sites are managed via the same LI_T3 interface

- Easier to provision intercept

  - Provisioning of X3 encap and Intercept could be done at centralized external cc-POI instead of provisioning every UPF by the LI Administrator via the same LI-T3 interface

- Don’t have to configure X3 every time a UPF spins up and down

  - Since the UPF is expected to be spun up and down frequently, if the X3 is configured on UPF, there are more frequent interactions / impacts to LI X3 setup

- Lesser impacts due to UPF mobility

  - Long term operators would be looking at moving UPFs for a subscriber and in this case, having the external cc-POI terminate X3 is easier as it would be lesser impact on LI and only a UPF terminating ID could could be included from internal cc-POI without having to change the X3 delivery point i.e. a new UPF

- Support for multiple administrative domains

  - UPs could be in different domains e.g. enterprise, consumer etc., and would reduce the impact on LI if X3 is on external cc-POI

- Simplifies transport model and deployment requirements

  - Since external cc-POI is more centralized, the transport and connectivity requirements for X3 would be relatively simpler when compared to UPs which could be distributed and even potentially deployed on Cell Site Routers in the future. As long as the X3_u is secure and reliable, it allows for deployment flexibility.

- Balance traffic between fast senders and slow receivers.

  - Depending on how the external cc-POI resources are budgeted, it can balance traffic nicely between faster senders and slow MDF-3 receivers which typically happens in a multi-vendor environment in deployments.

Some of the downsides to the solution include the following:

- Traffic dimensioning towards aggregator/external cc-POIxs

  - Traffic towards external cc-POI from UP would have to account for a percentage of traffic being intercepted and account for user plane throughput.

- X3_u towards aggregator has to be encrypted

  - The external cc-POI to UPF interface would have to have an encrypted tunnel e.g. IPSec and so would need to account for that.

- “Fake” data on N4 or aggregator to ensure detection is not possible

  - Since the detection of interception should be masked or avoided, if the UPF has only a single or small number of subscribers connected, a network administrator can detect increase of traffic volume between UPF and external cc-POI as some subscriber being intercepted.  By sending “fake” or ”simulated”  or “heartbeat” data periodically which can be discarded at the external cc-POI, detection could be avoided with obfuscation thereby ensuring end-to-end privacy of intercepted traffic flows through the system

- Potential for spinning up additional external cc-POIs when intercept traffic increases

  - Additional external cc-POIs would / could be needed depending on LI volume and its easy to spin more instances as needed to provide elasticity yet keeping all the semantics the same. Further HA/redundancy/Clustered external cc-POIs provide carrier grade deployment capability with incremental effort. External CC-POI would be another network node that will need to be managed.

5.Conclusions

There will be continued demand to increase the throughput of Li functions as 5G deployments ramp up. A fully integrated solution with an internal cc-POI functions can add a significant overhead on overall UPs forwarding performance requiring additional server CPU capacity. A more flexible yet performant solution would be to have a lightweight internal cc-POI at the UP, and a heavyweight external cc-POI acting as a multiplexer colocated with the UP. This provides certain interesting properties from a performance, reliability, security and manageability perspective. The external cc-POI can be considered as a proxy to manage unequal traffic rates between faster senders and slow receivers. When colocated with the UPs with a secure X3_u channel, the advantages are easily realised without disrupting existing architecture in Rel15. The solution as proposed is well within the boundaries of the SA3Li-Rel15 standards, with few extension as proposed, but with a lot more benefits and flexibility with future-proofed capability.

We believe this is an essential correction that the 3gpp standards body should ratify expediently.