3GPP TSG-SA3 Meeting #73-LI	s3i190261

# La Jolla (California), United States, 9th Apr 2019 - 12th Apr 2019		    was s3i190246

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.108 | CR | 0412 | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SecondaryCellGroupPSCell Reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD, NTAC, PIDS, BT, DT, Telefonica, Vodafone, Rogers, Ericsson, Nokia, Nokia Shanghai Bell) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI15 |  |  |  |  |  | Date: |  |  | 2019-04-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of Secondary Cell Group cells reporting |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds reporting of Secondary Cell Group cells reporting |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Location reporting will not be as accurate as available in the network, violating the location accuracy requirement in TS 33.126 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.5.0, 10.5.0, 13.3.1, 13.3.2.2, B.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<< START OF CHANGE 1 >>>>>>>>>>>>>>>>>>>>>>>>>>>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ETSI TS 101 331: "Lawful Interception (LI); Requirements of Law Enforcement Agencies".

[2]	ETSI ES 201 158: "Telecommunications security; Lawful Interception (LI); Requirements for network functions".

[3]	ETSI ETR 330: "Security Techniques Advisory Group (STAG); A guide to legislative and regulatory environment".

[4]	3GPP TS 29.002: "3rd Generation Partnership Project; Technical Specification Group Core Network; Mobile Application Part (MAP) specification".

[5A]	ITU-T Recommendation X.680: "Abstract Syntax Notation One (ASN.1): Specification of Basic Notation".

[5B]	ITU-T Recommendation X.681: "Abstract Syntax Notation One (ASN.1): Information Object Specification".

[5C]	ITU-T Recommendation X.681: "Abstract Syntax Notation One (ASN.1): Constraint Specification".

[5D]	ITU-T Recommendation X.681: "Abstract Syntax Notation One (ASN.1): Parameterization of ASN.1 Specifications".

[6]	ITU-T Recommendation X.690: "ASN.1 encoding rules: Specification of Basic Encoding Rules (BER), Canonical Encoding Rules (CER) and Distinguished Encoding Rules (DER)".

NOTE 1:	It is recommended that for [5A], [5B], [5C], [5D] and [6] the 2002 specific versions should be used.

[7]	ITU-T Recommendation X.880: "Information technology - Remote Operations: Concepts, model and notation".

[8]	ITU-T Recommendation X.882: "Information technology - Remote Operations: OSI realizations - Remote Operations Service Element (ROSE) protocol specification".

NOTE 2:	It is recommended that for [8] the 1994 specific versions should be used.

[9]	3GPP TS 24.008: "3GPP Technical Specification Group Core Network; Mobile radio interface Layer 3 specification, Core network protocol; Stage 3".

[10] - [12]	Void.

[13]	IETF STD 9 (RFC 0959): "File Transfer Protocol (FTP)".

[14]	3GPP TS 32.215: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication Management; Charging Management; Charging data description for the Packet Switched (PS) domain)".

[15]	IETF STD0005 (RFC 0791: "Internet Protocol".

[16]	IETF STD0007 (RFC 0793): "Transmission Control Protocol".

[17]	3GPP TS 29.060: "3rd Generation Partnership Project; Technical Specification Group Core Network; General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[18]	3GPP TS 33.106: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Lawful Interception Requirements".

[19]	3GPP TS 33.107: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Lawful interception architecture and functions".

[20]	3GPP TS 23.107: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Quality of Service QoS concepts and architecture".

[21] - [22]	Void.

[23]	ANSI/J-STD-025-A: "Lawfully Authorized Electronic Surveillance".

[24]	ETSI TS 101 671: "Handover Interface for the lawful interception of telecommunications traffic".

[25]	3GPP TS 23.003: "3rd Generation Partnership Project; Technical Specification Group Core Network; Numbering, addressing, and identification".

[26]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[27]	IETF RFC 1006: "ISO Transport Service on top of the TCP".

[28]	IETF RFC 2126: "ISO Transport Service on top of TCP (ITOT)".

[29]	ITU-T Recommendation Q.763: "Signalling System No. 7 - ISDN User Part formats and codes".

[30]	ETSI EN 300 356 (all parts): "Integrated Services Digital Network (ISDN); Signalling System No.7; ISDN User Part (ISUP) version 3 for the international interface".

[31]	ETSI EN 300 403-1 (V1.3.2): "Integrated Services Digital Network (ISDN); Digital Subscriber Signalling System No. one (DSS1) protocol; Signalling network layer for circuit-mode basic call control; Part 1: Protocol specification [ITU-T Recommendation Q.931 (1993), modified]".

NOTE 3:	Reference [31] is specific, because ASN.1 parameter "release-Reason-Of-Intercepted-Call" has the following comment: "Release cause coded in [31] format". In case later version than the given one indicated for ISDN specification ETSI EN 300 403-1 has modified format of the "release cause", keeping the reference version specific allows to take proper actions in later versions of this specification.

[32] - [33]	Void

[34]	ITU-T Recommendation Q.931: "ISDN user-network interface layer 3 specification for basic call control".

[35]	Void.

[36]	Void.

[37]	3GPP TS 23.032: "3rd Generation Partnership Project; Technical Specification Group Core Network; Universal Geographical Area Description (GAD)".

[38]	3GPP TR 21.905: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Vocabulary for 3GPP Specifications".

[39]	ISO 3166-1: "Codes for the representation of names of countries and their subdivisions - Part 1: Country codes".

[40]	3GPP TS 23.228: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS); Stage 2".

[41]	3GPP TS 29.234: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals: 3GPP System to Wireless Local Area Network (WLAN) interworking; Stage 3".

[42]	3GPP TS 23.060: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; General Packet Radio Service (GPRS); Service description".

[43]	3GPP TS 23.234: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3GPP system to Wireless Local Area Network (WLAN) Interworking; System Description".

[44]	3GPP TS 23.401: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[45]	3GPP TS 23.402: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Architecture enhancements for non-3GPP accesses".

[46]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Access (GPRS) Tunneling Protocol for Control Plane (GTPv2-C); Stage 3".

[47]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[48]	3GPP TS 29.275: "Proxy Mobile IPv6 (PMIPv6) based Mobility and Tunneling protocols; Stage 3".

[49]	3GPP TS 24.303: "Mobility management based on Dual-Stack Mobile IPv6; Stage 3".

[50]	(void)

[51]	(void)

[52]	3GPP TS 24.147: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Conferencing Using the IP Multimedia (IM) Core Network (CN) subsystem 3GPP Stage 3".

[53]	3GPP TS 29.273: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[54]	3GPP TS 33.328: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS) media plane security".

[55]	ATIS-0700005 "Lawfully Authorized Electronic Surveillance (LAES) for 3GPP IMS-based VoIP and other Multimedia Services".

[56]	3GPP TS 29.212: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Policy and Charging Control(PCC); Reference points".

[57]	Void.

[58]	IETF RFC 4217: "Securing FTP with TLS".

[59]	3GPP TS 29.272: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Evolved Packet System (EPS); Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[60]	3GPP TS 33.310: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Network Domain Security (NDS); Authentication Framework (AF)".

[61]	IETF RFC 6043: "MIKEY-TICKET: Ticket-Based Modes of Key Distribution in Multimedia Internet KEYing (MIKEY)", available at www.ietf.org

[62]	3GPP TS 25.413: "UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling".

[63]	3GPP TS 29.279: "Mobile IPv4 (MIPv4) based mobility protocols; Stage 3".

[64]	3GPP TS 29.118: "Mobility Management Entity (MME) -Visitor Location Register (VLR) SGs interface specification"

[65]	ANSI/J-STD-025-B: "Lawfully Authorized Electronic Surveillance", July 17, 2006.

[66]	3GPP TS 24.007: "Mobile Radio Interface Signalling Layer 3; General Aspects".

[67]	IETF RFC 3966: "The Tel URIs for Telephone Numbers", December, 2004.

[68]	IETF RFC 791: "Internet Protocol"

[69]	IETF RFC 2460: "Internet Protocol, Version 6 (IPv6) Specification".

[70]	IEFT RFC 3697: "IPv6 Flow Label Specification".

[71]	IETF RFC 4776: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information".

[72]	IETF RFC 5139: "Revised Civic Location Format for Presence Information Data Format Location Object (PIDF-LO)".

[73]	ISO.3166-2: International Organization for Standardization, "Codes for the representation of names of countries and their subdivisions - Part 2: Country subdivision code".

[74]	UPS SB42-4: Universal Postal Union (UPU), "International Postal Address Components and Templates".

[75]	ISO 639-1:2002: "Codes for the representation of names of languages -- Part 1: Alpha-2 code".

[76]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[77]	3GPP TS 24.623: "Technical Specification Group Core Network and Terminals; Extensible Markup Language (XML) Configuration Access Protocol (XCAP) over the Ut interface for Manipulating Supplementary Services".

[78]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[79]	3GPP TS 24.109: "Universal Mobile Telecommunications System (UMTS); Bootstrapping interface (Ub) and network application function interface (Ua); Protocol details".

[80]	IETF RFC 4825:"The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[81]	IETF RFC 7254: "A Uniform Resource Name Namespace for the Global System for Mobile Communications Association (GSMA) and the International Mobile station Equipment Identity (IMEI)"

[82]	IETF RFC 7255: "Using the International Mobile station Equipment Identity (IMEI) Uniform Resource Name (URN) as an Instance ID".

[83]	3GPP TS 22.468: "Group Communication System Enablers for LTE (GCSE_LTE)".

[84]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[85]	3GPP TS 25.321: "Medium Access Control (MAC) protocol specification".

[86]	3GPP TS 24.371: " Web Real-Time Communications (WebRTC) access to the IP Multimedia (IM) Core Network (CN) subsystem (IMS); Stage 3".

[87]	ITU-T Recommendation E.212: "The international identification plan for public networks and subscriptions".

[88]	OMA MLP TS: "Mobile Location Protocol", OMA-TS-MLP-V3_4-20150512-A.

[89]	MMS Architecture OMA-AD-MMS-V1_3-20110913-A.

[90]	Multimedia Messaging Service Encapsulation Protocol OMA-TS-MMS_ENC-V1_3-20110913-A.

[91]	3GPP TS 22.140: "Multimedia Messaging Service (MMS); Stage 1".

[92]	IETF RFC 2822: "Internet Message Format".

[93]	IETF RFC 3551: "RTP Profile for Audio and Video Conferences with Minimal Control".

[94]	IETF RFC 4566: "Session Description Protocol".

[95]	IETF RFC 3550: "Realtime Transport Protocol".

[96]	3GPP TS 29.229: "Cx and Dx interfaces based on the Diameter protocol; Protocol details".

[97]	OMA-AD-PoC-V2_1-20110802-A, Architecture Document.

[98]	OMA-TS-PoC_User Plane-V2_1-20110802-A.

[99]	3GPP TS 37.340: "Evolved Universal Radio Access (E-UTRA) and NR-Multi-connectivity; Stage 2".

[XX]	3GPP TS 36.413: "E-UTRAN – S1 Application Protocol (S1AP)".

<<<<<<<<<<<<<<<<<<<<<<<<<< END OF CHANGE 1 >>>>>>>>>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<<<<<<<< START OF CHANGE 2 >>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 4.5	HI2: Interface port for intercept related information

### 4.5.0	General

The HI2 interface port shall be used to transport all IRI, i.e. the information or data associated with the communication services of the target identity apparent to the network. It includes signalling information used to establish the telecommunication service and to control its progress, time stamps, and, if available, further information such as location information. Only information which is part of standard network signalling procedures shall be used within communication related IRI.

For all UE locations obtained, generated or reported to the LEMF, the MF/DF shall report the time at which the location was established by the location source (e.g. MME or HSS) and provide this to the MF/DF along with the location information. If this information cannot be provided to the MF/DF, then the MF/DF shall indicate that the time is not available. If the information in the MME received over S1 (TS 36.413 [XX]) includes one or more cell IDs, then all cell IDs shall be reported to the LEMF whenever location reporting is triggered at the MME.

Sending of the IRI to the LEMF shall in general take place as soon as possible, after the relevant information is available.

In exceptional cases (e.g. data link failure), the IRI may be buffered for later transmission for a specified period of time.

Within this clause only, definitions are made which apply in general for all network technologies. Additional technology specific HI2 definitions are specified in related Annexes.

<<<<<<<<<<<<<<<<<<<<<<<<<< END OF CHANGE 2 >>>>>>>>>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<<<<<<<< START OF CHANGE 3 >>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 10.5	IRI for evolved packet domain

### 10.5.0	Introduction

The IRI will in principle be available in the following phases of a data transmission:

1.	At connection attempt when the target identity becomes active, at which time packet transmission may or may not occur (set up of a bearer/tunnel, target may be the originating or terminating party);

2.	At the end of a connection, when the target identity becomes inactive (removal of a bearer/tunnel);

3.	At certain times when relevant information are available.

In addition, information on non-transmission related actions of a target constitute IRI and is sent via HI2. Also, the EPS LALS reports convey via HI2 as IRI.

The IRI may be subdivided into the following categories:

1.	Control information for HI2 (e.g. correlation information);

2.	Basic data context information, for standard data transmission between two parties.

The events defined in TS 33.107 [19] are used to generate records for the delivery via HI2.

There are several different event types received at DF2 level. According to each event, a Record is sent to the LEMF if this is required. In the case of LALS reports which are not associated with an event, a Record is sent to the LEMF without the event parameter.

The following table gives the mapping between event type received at DF2 level and record type sent to the LEMF. The applicability of the events to specific access (E-UTRAN, trusted non-3GPP access, untrusted non-3GPP access) and network protocols (GTP/PMIP S5/S8 interface) is specified in TS 33.107 [19]. Additional events and mapping with IRI Record type are applicable to EPS in case of interworking between SGSN and PDN-GW over Gn/Gp interface, as specified in this document for PS interception.

Table 10.5.1: Mapping between EPS Events and HI2 records type

| Event | IRI Record Type |
| --- | --- |
| E-UTRAN attach | REPORT |
| E-UTRAN detach | REPORT |
| Bearer activation (successful) | BEGIN |
| Bearer modification | CONTINUE |
| UE Requested bearer resource modification | REPORT |
| Bearer activation (unsuccessful) | REPORT |
| Start of interception with active bearer, NOTE 1 | BEGIN or optionally CONTINUE |
| Bearer deactivation | END |
| UE requested PDN connectivity | REPORT |
| UE requested PDN disconnection | REPORT |
| Tracking Area/EPS location update | REPORT |
| Serving Evolved Packet System | REPORT |
| PMIP attach/tunnel activation (successful) | BEGIN |
| PMIP attach/tunnel activation (unsuccessful) | REPORT |
| PMIP session modification | CONTINUE |
| PMIP detach/tunnel deactivation | END |
| Start of interception with active PMIP tunnel, NOTE 1 | BEGIN (or optionally CONTINUE) |
| PMIP PDN-GW initiated PDN disconnection | END |
| MIP registration/tunnel activation (successful) | BEGIN |
| MIP registration/tunnel activation (unsuccessful) | REPORT |
| MIP deregistration/tunnel deactivation | END |
| Start of interception with active MIP tunnel, NOTE 1 | BEGIN |
| DSMIP registration/tunnel activation (successful) | BEGIN |
| DSMIP registration/tunnel activation (unsuccessful) | REPORT |
| DSMIP session modification | CONTINUE |
| DSMIP deregistration/tunnel deactivation | END |
| Start of interception with active DSMIP tunnel, NOTE 1 | BEGIN |
| DSMIP HA Switch | REPORT |
| PMIP Resource Allocation Deactivation | END |
| MIP Resource Allocation Deactivation | END |
| Start of interception with E-UTRAN attached UE, NOTE 1 | REPORT |
| Packet Data Header Information | REPORT |
| HSS subscriber record change | REPORT |
| Cancel location | REPORT |
| Register location | REPORT |
| Location information request | REPORT |
| ProSe Remote UE Report | REPORT |
| ProSe Remote UE start of communication | BEGIN |
| ProSe Remote UE end of communication | END |
| Start of interception with ProSe Remote UE ongoing communication, NOTE 1 | BEGIN |
| Start of interception for ProSe UE-to-NW Relay, NOTE 1 | REPORT |

NOTE 1:	In some situation (e.g. during activation of second, third, etc, intercepts on the target), the MF/DF may have to detect on its own that an interception is activated on a target.

The EPS LALS records are sent to the LEMF as the REPORT IRI Records.

A set of information is used to generate the records. The records used transmit the information from mediation function to LEMF. This set of information can be extended in the network nodes or DF2 MF, if this is necessary in a specific country. The following table gives the mapping between information received per event and information sent in records.

Table 10.5.2: Mapping between Events information and IRI information

| parameter |  | description |  | HI2 ASN.1 parameter |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Observed MSISDN |  | Target Identifier with the MSISDN of the target. |  | partyInformation (partyIdentity/msISDN) |  |  |
| Observed A-MSISDN |  | Target Identifier with A-MSISDN of the target |  | partyInformation (partyIdentity/msISDN) |  |  |
| Observed IMSI |  | Target Identifier with the IMSI of the target. |  | partyInformation (partyIdentity/imsi) |  |  |
| Observed ME Id |  | Target Identifier with the ME Id of the target. |  | partyInformation (partyidentity/imei) |  |  |
| Observed MN NAI |  | Target Identifier with the NAI of the target. |  | partyInformation (partyidentity/nai) |  |  |
| Observed IMEI |  | Target Identifier with IMEI of the target |  | partyInformation (partyIdentity/imei) |  |  |
| New observed MSISDN |  | New target identifier with MSISDN of the target, when available |  | partyInformation/(partyIdentity/msISDN) |  |  |
| New observed IMSI |  | New target identifier with IMSI of the target, when available |  | partyInformation/(partyIdentity/imsi) |  |  |
| New observed IMEI |  | New target identifier with IMEI of the targert, when available |  | partyInformation (partyIdentity/imei) |  |  |
| Event type |  | Description which type of event is delivered |  | ePSevent |  |  |
| Event date |  | Date of the event generation in the node |  | timeStamp |  |  |
| Event time |  | Time of the event generation in the node |  |  |  |  |
| Access point name |  | When provided by the MME, the parameter carries the Access Point Name provided by the UE. When provided by the S-GW/PDN-GW, it is the APN used for the PDN connection |  | aPN |  |  |
| APN-AMBR |  | Contains the Aggregate Maximum Bit Rate for the APN |  | aPN-AMBR |  |  |
| PDN type |  | Indicated the used IP version (IPv4, Ipv6, Ipv4/Ipv6) |  | pDNType |  |  |
| PDN address allocation |  | Provides the IP version (Ipv4, Ipv6, Ipv4/Ipv6) and the IP address(es) allocated for the UE. |  | pDNAddressAllocation |  |  |
| Protocol Configuration Options |  | Are used to transfer parameters between the UE and the PDN-GW (e.g. address allocation preference by DHCP) |  | protConfigOptions |  |  |
| Attach type |  | Indicates the type of attach and may carry indication of handover in case of mobility with non-3GPP access. |  | attachType |  |  |
| RAT type |  | Radio Access Type |  | rATType |  |  |
| Initiator |  | This field indicates whether the procedure is UE or network initiated. |  | initiator |  |  |
| Handover indication |  | Provides information that the procedure is triggered as part of a handover |  | handoverIndication, extendedHandoverIndication |  |  |
| Procedure Transaction Identifier |  | Identifies a set of messages belonging to the same procedure; the parameter is dynamically allocated by the UE |  | procedureTransactionId |  |  |
| EPS bearer identity |  | Identifies an EPS bearer for one UE accessing via E-UTRAN. It is allocated by the MME. |  | ePSBearerIdentity |  |  |
| Bearer activation/ deactivation type |  | Indicates the type of bearer being activated/deactivated, i.e. default or dedicated. |  | bearerActivationType, bearerDeactivationType |  |  |
| Linked EPS bearer identity |  | Indicates, in case of dedicated bearer, the EPS bearer identity of the default bearer. |  | linkedEPSBearerId |  |  |
| Switch off indicator |  | Indicates whether a detach procedure is due to a switch off situation or not. |  | detachType |  |  |
| Detach type |  | Parameter sent by the network to the UE to indicate the type of detach. |  | detachType |  |  |
| Traffic Flow Template (TFT) |  | Collection of all packet filters associated with the EPS bearer. |  | tFT |  |  |
| Traffic Aggregate Description (TAD) |  | Consists of the description of the packet filter(s) for the traffic flow aggregate. |  | trafficAggregateDescription |  |  |
| Correlation number |  | Unique number for each target connection delivered to the LEMF, to help the LEA, to have a correlation between each target connection and the IRI. |  | ePSCorrelationNumber |  |  |
| Lawful interception identifier |  | Unique number for each lawful authorization. |  | lawfulInterceptionIdentifier |  |  |
| Location information |  | When authorized, this field provides the location information of the target that is present at the node at the time of event record production. |  | ePSlocationOfTheTarget |  |  |
| Time of Location |  | Date/Time of location. The time when location was obtained by the location source node. |  | ePSlocationOfTheTarget |  |  |
| Additional Cell Locations |  | The primary cell ID from each of the Master Cell (PCell) and Secondary Cell (PSCell) groups of the target, if available. |  | additionalCellLocations |  |  |
| Old location information |  | Location information of the target before Tracking Area Update. |  | ePSlocationOfTheTarget |  |  |
| Failure reason |  | The reason for the failure or rejection of the Tracking Area Update |  | failedTAUReason |  |  |
| Failed bearer activation reason |  | This field gives information about the reason for a failed bearer activation of the target. |  | failedBearerActivationReason |  |  |
| Failed attach reason |  | This field gives information about the reason for a failed attach attempt of the target. |  | failedEUTRANAttachreason, status, code (depending on the protocol) |  |  |
| Session modification failure reason |  | This field gives information about the reason for a failed session modification attempt of the target |  | status |  |  |
| EPS bearer QOS |  | This field indicates the Quality of Service associated with the EPS bearer procedure. |  | ePSBearerqOS |  |  |
| Bearer deactivation reason |  | This field gives information about the reason for bearer deactivation of the target. |  | bearerDeactivationCause |  |  |
| Network identifier |  | Operator ID plus node address. |  | networkIdentifier |  |  |
| LogicalFunctionInformation |  | Event source logical function identifier. |  | logicalFunctionInformation |  |  |
| Failed Bearer Modification reason |  | The reason for failure of Bearer Modification |  | failedBearerModReason |  |  |
| ULI Timestamp |  | Indicates the time when the User Location Information was acquired. |  | uLITimestamp |  |
| Lifetime |  | Lifetime of the tunnel; it is set to a nonzero value in case of registration or lifetime extension; is set to zero in case of deregistration. |  | lifetime |  |  |
| Access technology type |  | Indicates the Radio Access Type |  | accessTechnologyType |  |  |
| UE address info |  | Includes one or more IP addresses allocated to the UE. |  | iPv6HomeNetworkPrefix, iPv4HomeAddress, iPv6careOfAddress, iPv4careOf Address |  |  |
| Additional parameters |  | Additional information provided by the UE, such as protocol configuration options |  | protConfigurationOption |  |  |
| Serving MME address |  | Diameter Origin-Host and Origin-Realm of the serving MME or its IP address. |  | servingMME-Address |  |  |
| Revocation trigger |  | Contains the reason which triggered a PDN-GW initiated PDN-disconnection (revocation) procedure. |  | revocationTrigger |  |  |
| Home Address |  | Contains the UE Home IP address |  | homeAddress |  |  |
| Home Agent Address |  | Contains the IP address of the Home Agent |  | homeAgentAddress |  |  |
| Requested Ipv6 Home Prefix |  | The Ipv6 Home Prefix requested by the UE. |  | requestedIPv6HomePrefix |  |  |
| Care of Address |  | The local IP address assigned to the UE by the Access Network. |  | careOfAddress |  |  |
| HSS/AAA address |  | The address of the HSS/AAA triggering a pDN-GW reallocation. |  | hSS-AAA-address |  |  |
| Target PDN-GW address |  | The address of the PDN-GW which the UE will be reallocated to. |  | targetPDN-GW-Address |  |  |
| Foreign domain address |  | The relevant IP address in the foreign domain. |  | foreignDomainAddress |  |  |
| Visited network identifier |  | An identifier that allows the home network to identify the visited network inside the EPS Serving System Update for non 3GPP access, coded according to TS 29.273 [53] |  | visitedNetworkId |  |  |
| DHCP v4 Address Allocation Indication |  | Indicates that DHCPv4 is to be used to allocate the Ipv4 address to the UE |  | dHCPv4AddressAllocationInd |  |  |
| Serving Network |  | Identifies, for E-UTRAN access, the serving network the UE is attached to |  | servingNetwork |  |  |
| Request type |  | Provides the type of UE requested PDN connectivity |  | requestType |  |  |
| Failed reason |  | Provides the failure cause for UE requested PDN connectivity |  | uEReqPDNConnFailReason |  |  |
| Destination IP address |  | Identifies the destination IP address of a packet. |  | destinationIPAddress |  |  |
| Destination port number |  | Identifies the destination port number of a packet |  | destinationPortNumber |  |  |
| Source IP address |  | Identifies the source IP address of a packet. |  | sourceIPAddress |  |  |
| Source port number |  | Identifies the source port number of a packet. |  | sourcePortNumber |  |  |
| Transport protocol |  | Identifies the transport protocol (i.e., Protocol Field in Ipv4 or Next Header Field in Ipv6. |  | transportProtocol |  |  |
| Flow label |  | The field in the Ipv6 header that is used by a source to label packets of a flow (see RFC 3697 [c]) |  | flowLabel |  |  |
| Packet count |  | The number of packets detected and reported in a particular packet data summary report. |  | packetCount |  |  |
| Packet size |  | The size of a packet (i.e., Total Length Field in Ipv4 [a] or Payload Length Field in Ipv6 [b]) |  | packetsize |  |  |
| Packet direction |  | Identifies the direction of the intercepted packet (from target or to target) |  | packetDirection |  |  |
| Packet data header copy |  | Provides a copy of the packet headers including IP layer and next layer, and extensions, but excluding content. |  | packetDataHeaderCopy |  |  |
| Summary period |  | Provides the period of time during which the packets of the summary report were sent or received by the target. |  | summaryPeriod |  |  |
| Sum of packet sizes |  | Sum of values in Total Length Fields in Ipv4 packets or Payload Length Field in Ipv6 packets. |  | sumOfPacketSizes |  |  |
| Packet data summary reason |  | Provides the reason for a summary report. |  | packetDataSummaryReason |  |  |
| Packet data summary |  | For each particular packet flow, identifies pertinent reporting information (e.g. source IP address, destination IP address, source port, destination port, transport protocol, packet count, time interval, sum of packet sizes) associated with the particular packet flow. |  | packetDataSummary |  |  |
| CSG Identity |  | Uniquely identifies a CSG within a PLMN. |  | csgIdentity |  |  |
| HeNB Identity |  | Identifies the HeNB providing access to a target UE. |  | heNBIdentity |  |  |
| HeNB IP address |  | Identifies the IP Address associated with an HeNB providing access to a target UE. |  | heNBiPAddress |  |  |
| HeNB Location |  | Identifies the location of an HeNB providing access to a target UE. |  | heNBLocation |  |  |
| Tunnel Protocol |  | Identifies the tunnel protocol used to transport the signalling and communications between the HeNB and the EPC. |  | tunnelProtocol |  |  |
| UE Local IP address |  | Identifies the UE local IP address (IP SEC terminal Point) observed by the ePDG and PDN-GW, based on local policy for Fixed Broadband access network interworking, in case the GTP based S2b interface TS 29.274 [46] is used. |  | uELocalIPAddress |  |  |
| UE UDP Port |  | Identifies the UE UDP port (IP SEC terminal Point) reported on GTP based S2b interface TS 29.274 [46] if NAT is detected and local IP address is present for Fixed Broadband access network interworking. |  | uEUdpPort |  |  |
| Serving system identifier |  | VPLMN ID of the serving system or of the third party network interworking, included in the Diameter AVP message with the HSS |  | serving-System-Identifier |  |  |
| Previous serving system identifier |  | Previous VPLMN id of the target (Mobile Country Code and Mobile Network Country; defined in E212 [87]) ). |  | current-Previous-Systems/previous-Serving-System-Identifier |  |  |
| Previous serving MME address |  | Diameter Origin-Host and Origin-Realm of the previous serving MME. |  | current-Previous-Systems/previous-Serving-MME-Address |  |  |
| Current visited Network Identifier |  | An identifier that allows the home network to identify the current visited network (Mobile Country Code and Mobile Network Country defined in E212 [87] are included in this identifier) TS 29.273 [53]. |  | current-Previous-Systems/current-Serving-System-Identifier |  |  |
| Current serving MME address |  | Provide the previous Diameter Origin-Host and Origin-Realm of the previous current MME |  | current-Previous-Systems/current-Serving-MME-Address |  |  |
| Requesting network identifier |  | The requesting network identifier PLMN id (Mobile Country Code and Mobile Network Country, -- defined in E212 [87]). |  | requesting-Network-Identifier |  |  |
| Requesting node type |  | Type of requesting node such as MSC, SMS Centre, GMLC, MME, SGSN. |  | requesting-Node-Type |  |  |
| Other update |  | Carrier specific information related to its implementation or subscription process on its HSS. Raw data will be provided. CSP will provide to LEMF elements to understand such data. |  | carrierSpecificData |  |  |
| WLAN location information |  | Provides location information in form of TWAN identifier, if available at ePDG/PDN-GW, in case the GTP based s2b interface TS 29.274 [46] is used. |  | tWANIdentifier |  |  |
| WLAN location timestamp |  | Provides location information timestamp in form of TWAN identifier timestamp, if available at ePDG/PDN-GW, in case the GTP based s2b interface TS 29.274 [46] is used. |  | tWANIdentifierTimestamp |  |  |
| ProSe Remote UE Ids |  | The identities of the connected ProSe remote Ues when the ProSe UE-to-NW Relay performs a Tracking Area/EPS Location Update. |  | proSeRemoteUEContextConnected, |  |  |
| ProSe Remote UE IP info |  | The IP address(es) of the connected ProSe Remote UE(s) when the ProSe UE-to-NW Relay performs a Tracking Area/EPS Location Update. |  | proSeRemoteUEContextConnected |  |  |
| LALS location information |  | Location information provided by LALS. |  | locationOfTheTarget |  |  |
| Extended location parameters |  | Additional location information and QoS information. |  | extendedLocParameters |  |  |
| LALS error code |  | Positioning error identification code. |  | locationErrorCode |  |  |
| Changed (old/new) IMSI or MSISDN/ or IMEI |  | Provides the identity changes in Subscriber Record Change Event. |  | change-Of-Target-Identity |  |  |
| Secondary RAT Usage Indication |  | Provides an indication that the SGW has received the Secondary RAT Usage Report (see 3GPP TS 37.340 [99]) from the MME. If such information is not available, it doesn’t mean that Secondary RAT was not used. |  | secondaryRATUsageIndication |  |

NOTE 2:	LIID parameter has to be present in each record sent to the LEMF.

### 10.5.1	Events and information

#### 10.5.1.0	Introduction

This clause describes the information sent from the Delivery Function (DF) to the Law Enforcement Monitoring Facility (LEMF) to support Lawfully Authorized Electronic Surveillance (LAES). The information is described as records and information carried by a record. This focus is on describing the information being transferred to the LEMF.

The IRI events and data are encoded into records as defined in the Table 10.5.1 Mapping between EPS Events and HI2 records type and Annex B.9 Intercept related information (HI2). IRI is described in terms of a 'causing event' and information associated with that event. Within each IRI Record there is a set of events and associated information elements to support the particular service.

The communication events described in Table 10.5.1: Mapping between EPS Events and HI2 record type and Table 10.5.2: Mapping between Events information and IRI information convey the basic information for reporting the disposition of a communication. This clause describes those events and supporting information.

Each record described in this clause consists of a set of parameters. Each parameter is either:

mandatory (M)	- required for the record,

conditional (C)	- required in situations where a condition is met (the condition is given in the Description), or

optional (O)	- provided at the discretion of the implementation.

The information to be carried by each parameter is identified. Both optional and conditional parameters are considered to be OPTIONAL syntactically in ASN.1 Stage 3 descriptions. The Stage 2 inclusion takes precedence over Stage 3 syntax.

#### 10.5.1.1	REPORT record information

The REPORT record is used to report non-communication related target actions (events) and for reporting unsuccessful packet-mode communication attempts. In addition, this record is also used to report some target actions which may trigger communication attempts or modifications of an existing communication, when the communication attempt or the change of the existing communication itself is reported separately. The REPORT records are also used to deliver the LALS reports.

The REPORT record shall be triggered when:

-	the target's UE performs an E-UTRAN attach procedure (successful or unsuccessful) including via a HeNB;

-	the target's UE performs an E-UTRAN detach procedure including via a HeNB;

-	the target's UE is unsuccessful at performing an EPS bearer activation procedure;

-	the target's UE performs an UE requested bearer resource modification;

-	the target's UE performs a tracking area/EPS location update;

-	optionally when the target's UE leaves the old MME;

-	the target's UE performs an UE requested PDN connectivity procedure;

-	the target's UE performs an UE requested PDN disconnection procedure;

-	the target's UE is unsuccessful at performing a PMIP attach/tunnel activation procedure;

-	the target's UE is unsuccessful at performing a MIP registration/tunnel activation procedure;

-	the target's UE is unsuccessful at performing a DSMIP registration/tunnel activation procedure;

-	optionally when the target's UE enters or leaves IA (FFS);

-	the target's UE is ordered by the network to perform an home agent switch;

-	as a national option, a mobile terminal is authorized for service with another network operator or service provider; in that case, other related events are required as cancel location, register location, location information request from a third party's node;

--	as a national option, a REPORT record have to be generated when there is a HSS subscriber record change of IMSI or of MSISDN or IMEI triggered by a messages to or from the HSS;

-	the interception of a target is started with E-UTRAN attached target. If there are more than one PDN connections then a REPORT record is generated per PDN connection.;

-	packet data header reporting is performed on an individual intercepted packet basis and a packet is detected as it is sent or received by the target for an EPS bearer/session.;

-	when packet data summary reporting is performed on an summary basis for an EPS bearer/session associated with a particular packet flow (defined as the combination of source IP address, destination IP address, source port, destination port, and protocol and for IPv6 also include the flow label) and:

-	the packet flow starts,

-	an interim packet summary report is to be provided, or

-	packet flow ends including the case where the EPS bearer/session is deactivated.

An interim packet summary report is triggered if:

-	the expiration of a configurable Summary Timer per intercept occurs. The Summary Timer is configurable in units of seconds, or

-	a per-intercept configurable count threshold is reached;

-	when a LALS report is received from the LI LCS Client.

Packet Data Header Information is reported either on a per-packet (i.e., non-summarised) basis or in a summary report. These reports provide IRI associated with the packets detected. The packet data header information related REPORT record is used to convey packet header information during an active EPS bearer/session.

NOTE:	In the case of IP Fragments, Packet Data Header Information on a 6-tuple basis may only be available on the first packet and subsequent packets may not include such information and therefore may not be reported.

Table 10.5.1.1.1: E-UTRAN Attach REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide E-UTRAN Attach event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the target's UE, including the primary cell ID from each of the Master Cell (PCell) and Secondary Cell (PSCell) groups of the target, if available. |
| Time of Location | C | Date/Time of Location (if target location provided). |
| failed attach reason | C | Provide information about the reason for failed attach attempt of the target. |
| PDN Type | C | Indicated the used IP version (IPv4, IPv6, IPv4/IPv6), including possible reason for modification by the network |
| APN | C | Provides the Access Point Name |
| Protocol Configuration Options | C | Provides information sent from the UE to the network |
| Attach type | C | Provides the type of attach |
| EPS bearer identity | C | When the attach is successful, provides the allocated EPS bearer identity. |
| CSG Identity | C | Provide if closed/hybrid HeNB is used in the UE attachment to the network |
| HeNB Identity | C | Provide information to identify the HeNB serving the target's UE. |
| HeNB IP address | C | Provide the IP Address of the HeNB serving the target's UE used during location verification. |
| HeNB Location | C | Provide, when authorized, to identify location information for the HeNB serving the target's UE. |
| Tunnel Protocol | C | Provide to identify the tunnel protocol used to transport the signalling and communications between the HeNB and the EPC. |

Table 10.5.1.1.2: E-UTRAN Detach REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide E-UTRAN Detach event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the target's MS, including the primary cell ID from each of the Master Cell (PCell) and Secondary Cell (PSCell) groups of the target, if available. |
| Time of Location | C | Date/Time of Location (if target location provided). |
| initiator | C | Provided to indicate whether the detach is UE or network initiated |
| Switch off indicator | C | Provided to indicate whether the detach is due to a switch off |
| Detach type | C | Sent by the network to the UE to indicate the type of detach |
| CSG Identity | C | Provide if closed/hybrid HeNB is used in the UE detachment from the network |
| HeNB Identity | C | Provide information to identify the HeNB serving the target's UE. |
| HeNB IP address | C | Provide the IP Address of the HeNB serving the target's UE. |
| HeNB Location | C | Provide, when authorized, to identify location information for the HeNB serving the target's UE. |

Table 10.5.1.1.3: Bearer Activation (unsuccessful) REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| PDN address allocation | C | Provides the PDN type and PDN address(es) used by the network. |
| event type | C | Provide EPS Bearer Activation event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| access point name | C | Provide to identify the packet data network to which the attempt to connect was made; this information may be provided by the UE; the parameter is applicable only for default bearer activation. |
| RAT type | C | Provide the Radio Access Type used by the target. |
| initiator | C | Provide to indicate whether the EPS bearer activation is network-initiated, target-initiated, or not available. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the target's UE. |
| Time of Location | C | Date/Time of Location (if target location provided). |
| failed bearer activation reason | C | Provide information about the reason for failed bearer activation attempts of the target. |
| EPS bearer QOS | C | Provide to identify the QOS parameters. The parameter carries the requested EPS bearer QOS. |
| Bearer activation type | C | Provides information on default or dedicated bearer failed activation |
| APN-AMBR | C | The Aggregate Maximum Bit Rate foreseen for the APN. The parameter carries the subscribed APN-AMBR. |
| Protocol configuration options | C | Provide information about the protocol configuration options requested by the UE |
| Procedure transaction identifier | C | Used to associate the EPS bearer activation attempt to other messages triggering the procedure. |
| Linked EPS bearer identity | C | Provides, in case of failed dedicated bearer activation attempt, the EPS bearer id of the associated default bearer; not applicable in case of default bearer activation attempt. |
| Traffic Flow Template TFT | C | The TFT associated to the dedicated bearer activation attempt; not applicable in case of default bearer activation attempt |
| Handover indication | C | Provide information that the procedure is triggered as part of a handover |
| UE Local IP Address | C | The UE local IP address reported over GTP based S2b interface TS 29.274 [46]. |
| UE UDP Port | C | The UE UDP Port number provided in case of GTP based S2b interface TS 29.274 [46]. |
| WLAN location information | C | The TWAN identifier provided in case of GTP based S2b interface TS 29.274 [46]. |
| WLAN location timestamp | C | The TWAN identifier timestamp provided in case of GTP based S2b interface TS 29.274 [46]. |

Table 10.5.1.1.4: UE requested bearer resource modification REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide UE requested bearer resource modification event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the target's UE, including the primary cell ID from each of the Master Cell (PCell) and Secondary Cell (PSCell) groups of the target, if available. |
| Time of Location | C | Date/Time of Location (if target location provided). |
| failed bearer modification reason | C | Provide information about the reason for failed UE requested bearer resource modification. |
| EPS bearer QOS | C | Provide to identify the QOS parameters. |
| Procedure transaction identifier | C | Used to associate the UE requested bearer resource modification to other messages related to the procedure. |
| Linked EPS bearer identity | C | Provides the EPS bearer id of the associated default bearer. |
| EPS Bearer identity | C | Provides the EPS bearer id of the bearer which the request refers to. |
| Traffic Aggregate Description | C | Description of the packet filter(s) for the traffic flow aggregate |
| Protocol Configuration Options | C | Provide information about the protocol configuration options requested by the UE. |

Table 10.5.1.1.5: Tracking Area/EPS Location Update REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide Tracking Area/EPS Location Update event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the target's MS, including the primary cell ID from each of the Master Cell (PCell) and Secondary Cell (PSCell) groups of the target, if available. This parameter, in case of inter-MME TAU, will be sent only by the new MME. |
| Time of Location | C | Date/Time of Location (if target location provided). |
| old location information | O | Provide (only by the old MME), when authorized and if available, to identify the old location information for the target's MS, including the primary cell ID from each of the Master Cell (PCell) and Secondary Cell (PSCell) groups of the target, if available. |
| ProSe Remote UE(s) IDs | C | Applicable in case the target UE is a ProSe UE-to-NW Relay. Provided if available. |
| ProSe Remote UE(s) IP info | C | Applicable in case the target UE is a ProSe UE-to-NW Relay. Provided if available. |
| Failure reason | C | Provide, in unsuccessful case, the reason for the failure or rejection of the network procedure. |

In case of inter-MME TAU, Tracking Area/EPS Location Update REPORT Record shall be sent in the following cases:

-	when the target's UE moves to the new MME;

-	optionally when the target's UE leaves the old MME.

In addition to the case of Tracking Area Update, a Tracking Area/EPS Location Update REPORT Record shall also be sent in the following cases:

-	the target's UE performs an UE triggered service request;

-	the target's UE is involved in an X2-based handover;

-	the target's UE is involved in a S1-based handover.

Table 10.5.1.1.6: UE requested PDN connectivity REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide UE requested PDN connectivity event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| access point name | C | Provide to identify the packet data network to which the attempt to connect was made; this information may be provided by the UE (valid only for default bearer activation). |
| Request type | C | Indicates the type of request, i.e. initial request or handover |
| PDN type | C | Provide to describe the IP version requested by the target UE. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the target's UE, including the primary cell ID from each of the Master Cell (PCell) and Secondary Cell (PSCell) groups of the target, if available. |
| Time of Location | C | Date/Time of Location (if target location provided). |
| failed reason | C | Provide information about the reason for failed procedure. |
| Protocol configuration options | C | Provide information about the protocol configuration options requested by the UE |
| EPS bearer identity | C | The identity of the allocated EPS bearer |
| HeNB Identity | C | Provide information to identify the HeNB serving the target's UE. |
| HeNB IP address | C | Provide the IP Address of the HeNB serving the target's UE. |
| HeNB Location | C | Provide, when authorized, to identify location information for the HeNB serving the target's UE. |

Table 10.5.1.1.7: UE requested PDN disconnection REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide UE requested PDN disconnection event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the target's UE, including the primary cell ID from each of the Master Cell (PCell) and Secondary Cell (PSCell) groups of the target, if available. |
| Time of Location | C | Date/Time of Location (if target location provided). |
| Linked EPS bearer identity | C | The identity of the default EPS bearer associated with the PDN connection being disconnected. |
| HeNB Identity | C | Provide information to identify the HeNB serving the target's UE. |
| HeNB IP address | C | Provide the IP Address of the HeNB serving the target's UE. |
| HeNB Location | C | Provide, when authorized, to identify location information for the HeNB serving the target's UE. |

Table 10.5.1.1.8: PMIP Attach/tunnel activation (unsuccessful) REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MN NAI | C | Provide at least one and others when available |
| observed MSISDN |  |  |
| observed ME Id |  |  |
| observed IMSI |  |  |
| event type | C | Provide PMIP Attach/tunnel activation event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| lawful intercept identifier | M | Shall be provided. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| Lifetime | C | The requested lifetime for the tunnel |
| Access technology type | C | Provide the radio access type |
| failed attach reason | C | Provide information about the reason for failed attach/tunnel activation attempt of the target. |
| Handover indicator | C | Provide information that the procedure is triggered as part of the handover |
| APN | C | Provide the Access Point Name |
| UE address info | C | Includes one or more addresses allocated to the UE |
| Additional parameters | C | Provide additional parameters sent by the UE. |
| Serving Network | C | Provide to identify the serving network the UE is attached to in case of E-UTRAN access and PMIP based S5/S8 interfaces. |
| DHCPv4 Address Allocation Indication | C | Indicates that DHCPv4 is to be used to allocate the IPv4 address to the UE in case of E-UTRAN access and PMIP based S5/S8 interfaces |
| Location information | C | Provide, when authorized, to identify location information for the target's UE. |
| Time of Location | C | Date/Time of Location (if target location provided). |

Table 10.5.1.1.9: MIP registration/tunnel activation (unsuccessful) REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MN NAI | C | Provide at least one and others when available |
| observed IMSI |  |  |
| event type | C | Provide MIP registration/tunnel activation event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| lawful intercept identifier | M | Shall be provided. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| Lifetime | C | The requested lifetime for the tunnel |
| failed attach reason | C | Provide information about the reason for failed registration/tunnel activation attempt of the target. |
| Home Address | C | Provide the UE Home IP Address |
| Care of Address | C | The local IP address provided by the access network |
| Home Agent Address | C | Provide the Home Agent address |

Table 10.5.1.1.10: DSMIP registration/tunnel activation (unsuccessful) REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MN NAI | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| event type | C | Provide DSMIP registration/tunnel activation event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| lawful intercept identifier | M | Shall be provided. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| lifetime | C | The requested lifetime for the tunnel. |
| failed attach reason | C | Provide information about the reason for failed registration/tunnel activation attempt of the target. |
| Requested IPv6 Home Prefix | C | Provide the UE IPv6 Home Prefix. |
| Home address | C | Provide the assigned home address. |
| APN | C | Provides the Access Point Name. |
| Care of address | C | The local IP address provided by the access network. |

Table 10.5.1.1.11: DSMIP Home Agent Switch REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MN NAI | C | Provide at least one and others when available |
| observed IMSI |  |  |
| event type | C | Provide DSMIP Home Agent Switch event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| lawful intercept identifier | M | Shall be provided. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| HSS/AAA address | C | Provide the address of the HSS/AAA triggering the procedure |
| Target PDN-GW address | M | Provide the address of the new PDN-GW |

Table 10.5.1.1.12: Serving Evolved Packet System REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| observed ME Id |  |  |
| event type | C | Provide Serving Evolved Packet System event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Network identifier of the HSS reporting the event (Network element identifier included). |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| lawful intercept identifier | M | Shall be provided. |
| Serving MME address | C | Provide the Diameter Origin-Host and the Diameter Origin-Realm of the serving MME (in case of E-UTRAN access), or IP address of the serving MME |
| Visited Network Identifier | C | An identifier that allows the home network to identify the visited network (Mobile Country Code and Mobile Network Country defined in E.212 [87] are included in this identifier) TS 29.273 [53] |

Table 10.5.1.1.13: Start of interception with E-UTRAN attached UE REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide start of interception with PDN connection active |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| location information | C | Provide, when authorized, to identify location information for the target's UE, including the primary cell ID from each of the Master Cell (PCell) and Secondary Cell (PSCell) groups of the target, if available. |
| Time of Location | C | Date/Time of Location (if target location provided). |
| access point name | C | Provide to identify the packet data network to which the attempt to connect was made; this information may be provided by the UE (valid only for default bearer activation). |
| PDN type | C | Provide to describe the IP version requested by the target UE. |
| EPS bearer identity | C | The identity of the default EPS bearer |
| lawful intercept identifier | M | Shall be provided. |
| CSG Identity | C | Provide if closed/hybrid HeNB is used in the UE attachment to the network |
| HeNB Identity | C | Provide information to identify the HeNB serving the target's UE. |
| HeNB IP address | C | Provide the IP Address of the HeNB serving the target's UE. |
| HeNB Location | C | Provide, when authorized, to identify location information for the HeNB serving the target's UE. |
| Tunnel Protocol | C | Provide to identify the tunnel protocol used to transport the signalling and communications between the HeNB and the EPC. |

When the ICE (i.e. S-GW, PDN-GW) is not aware of the activation of multiple lawfully authorized intercepts when UE is already attached to the E-UTRAN, the MF/DF shall generate the Start of Interception with E-UTRAN attached UE REPORT record on its own using information that it has retained.

The DF2 shall not send the Start of Interception with E-UTRAN attached UE REPORT record to the LEMFs that were already intercepting the target due previous LI activation on the same target.

Table 10.5.1.1.14: Packet Data Header Information REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide Packet Data Header Information event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| initiator | C | Provide to indicate whether the EPS bearer modification is network-initiated, target-initiated, or not available. |
| network identifier | M | Shall be provided. |
| logicalFunctionInformation | O | Used to distinguish between multiple logical functions operating in a single physical network element. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the target's UE, including the primary cell ID from each of the Master Cell (PCell) and Secondary Cell (PSCell) groups of the target, if available. |
| Time of Location | C | Date/Time of Location (if target location provided). |
| EPS bearer id | C | Provides the EPS bearer id allocated by the network. |
| Handover indication | C | Provide information that the procedure is triggered as part of a handover. |
| Correlation number | M | Provide to uniquely identify the EPS bearer/tunnel delivered to the LEMF and to correlate IRI records with CC. |
| lifetime | C | The lifetime for the tunnel |
| Requested IPv6 Home Prefix | C | Provide the UE IPv6 Home Prefix |
| Home address | C | Provide the assigned home address |
| APN | C | Provides the Access Point Name |
| Care of address | C | The IP address provided by the access network |
| packet data header information | M | Shall be provided to identify the packet header information to be reported on a per-packet basis as defined in Table 10.5.1.1.15 or on a summary basis. For summary reporting includes one or more packet flow summaries where each packet flow summary is associated with a particular packet flow as defined in Table 10.5.1.1.16. |

<<<<<<<<<<<<<<<<<<<<<<<<<< END OF CHANGE 3 >>>>>>>>>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<<<<<<<< START OF CHANGE 4 >>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 13.3	ProSe Remote UE communications

### 13.3.1 General

The provisions in this clause specify the LI handover interface to report IRIs in case a ProSe Remote UE communicates through a ProSe UE-to-NW Relay.

TS 33.107 [19] specifies, see clause 17.3.3, in case the ProSe UE-to-NW Relay is a target, the conditions under which CC is required to be intercepted.

CC interception, when required, shall be reported according to annex B.10.

The IRIs will in principle be available in the following phases of ProSe Remote UE communications:

1.	When a target ProSe Remote UE connects to or disconnects from a ProSe UE-to-NW Relay;

2.	When a ProSe Remote UE connects to or disconnects from a target ProSe UE-to-NW Relay;

3.	When a target ProSe Remote UE starts communication by using a ProSe UE-to-NW Relay;

4.	When a target ProSe Remote UE ends communication by using a ProSe UE-to-NW Relay;

5.	When interception is started for a target ProSe Remote UE with ongoing communication;

6.	When interception is started for a Prose UE-to-NW Relay with connected ProSe Remote UE(s).

In addition, requirements specified in clause 10 are also applicable:

1.	When a Tracking Area/EPS Location Update occurs for a target ProSe UE-to-NW Relay;

2.	When a target ProSe UE-to-NW Relay uses one or more PDN connections for any activity/communication which is not related to relaying;

3.	To IRI and CC interception related to a target ProSe UE-to-NW Relay, when national regulations require to intercept all the CC handled by a target ProSe UE-to-NW Relay.

The IRI may be subdivided into the following categories:

1.	Control information for HI2 (e.g. correlation information);

2.	Prose Remote UE communications information.

The events defined in TS 33.107 [19] are used to generate records for the delivery via HI2.

There are multiple different event types received at DF2 level. According to each event, a Record is sent to the LEMF if this is required. The following table gives the mapping between event type received at DF2 level and record type sent to the LEMF.

Table 13.3.1-1: Mapping between ProSe Remote UE communications Events and HI2 records type

| Event | IRI Record Type |
| --- | --- |
| ProSe Remote UE Report | REPORT |
| ProSe Remote UE Start of Communication | BEGIN |
| ProSe Remote UE End of Communication | END |
| Start of interception with ProSe Remote UE ongoing communication | BEGIN or CONTINUE |
| Start of interception for ProSe UE-to-NW Relay | REPORT |

A set of information is used to generate the records. The records are used to transmit the information from the mediation function to LEMF. This set of information can be extended in the ICE or DF2 MF, if this is necessary in a specific country. The following table gives the mapping between information received per event and information sent in records.

The ASN.1 specified in Annex B.9 shall be used to provide HI2 to LEMF.

Table 13.3.1-2: Mapping between Events information and IRI information

| parameter | Description | HI2 ASN.1 parameter |
| --- | --- | --- |
| Observed IMSI | Target Identifier with the IMSI of the target. | partyInformation (party-identity) |
| Observed MSISDN | Target Identifier with the MSISDN of the target. | partyInformation (party-identity) |
| Observed IMEI | Target Identifier with the IMEI of the target. | partyInformation (party-identity) |
| Event type | Description which type of event is delivered: ProSe Remote UE Report, ProSe Remote UE start of communication, ProSe Remote UE end of communication, Start of interception with ProSe Remote UE ongoing communication, Start of interception for ProSe UE-to-NW Relay. | ePSevent |
| Event date | Date of the event generation in the ProSe Function. | timestamp |
| Event time | Time of the event generation in the ProSe Function. | timestamp |
| Network Identifier |  |  |
| Lawful interception identifier | Unique number for each lawful authorization. It shall be present in each record sent to the LEMF. | lawfulInterceptionIdentifier |
| Target type | Indicates whether the target is a ProSe Remote UE or a ProSe UE-to-NW Relay | proSeTargetType |
| ProSe Remote UE IDs | The identities of the connected or disconnected ProSe remote UEs. | proSeRemoteUeContextConnected, proSeRemoteUeContextDisconnected |
| ProSe Remote UE IP info | The IP address(es) of the connected to the ProSe Remote UE(s) provided by the ProSe UE-to-NW Relay. | proSeRemoteUeContextConnected |
| APN | The Access Point Name used by the ProSe UE-to-NW Relay for the connection | aPN |
| MSISDN of the Prose UE-to-NW Relay | MSISDN of the Prose UE-to-NW Relay (only applicable when the ProSe Remote UE is the target) | proSeRelayMSISDN |
| IMSI of the Prose UE-to-NW Relay | IMSI of the Prose UE-to-NW Relay (only applicable when the ProSe Remote UE is the target) | proSeRelayIMSI |
| IMEI of the Prose UE-to-NW Relay | IMEI of the Prose UE-to-NW Relay (only applicable when the ProSe Remote UE is the target) | proSeRelayIMEI |
| PDN address(es) | The ProSe UE-to-NW Relay IP address(es) for the PDN connection | pDNType, pDNAddressAllocation |
| Correlation number | The correlation number is used to correlate CC and IRI (in case the target is a ProSe remote UE).. | ePSCorrelationNumber |
| Location information | When authorized, this field provides the location of the ProSe UE-to-NW Relay. National regulations may require to provide the E-CGI of the ProSe UE-to-NW Relay when the target is the ProSe Remote UE | ePSlocationOfTheTarget |
| Additional Cell Locations | The primary cell ID from each of the Master Cell (PCell) and Secondary Cell (PSCell) groups of the target, if available. | additionalCellLocations |

### 13.3.2	Events and information

#### 13.3.2.1	Overview

This clause specifies the information sent from the Delivery Function (DF) to the Law Enforcement Monitoring Facility (LEMF) to support Lawful Interception (LI). The information is described as records and parameters carried by a record. The focus is on describing the information being transferred to the LEMF.

The IRI events and data are encoded into records as specified in the Table 13.3.1-1: Mapping between ProSe Remote UE communication Events and HI2 records type and Annex B.9 Intercept related information (HI2 SAE/EPS and IMS). IRI is specified in terms of a 'causing event' and parameters associated with that event. Within each IRI record there is a set of events and associated information elements to support the particular service.

The ProSe Remote UE communication events specified in Table 13.3.1-1: Mapping between Direct Discovery Events and HI2 records type and Table 13.3.1-2: Mapping between Events information and IRI information convey the basic information for reporting the ProSe Remote UE communication. This clause describes those events and supporting information.

Each record described in this clause consists of a set of parameters. Each parameter is either:

mandatory (M)	-	required for the record,

conditional (C)	-	required in situations where a condition is met (the condition is given in the Description), or

optional (O)	-	provided at the discretion of the implementation.

The information to be carried by each parameter is identified. Both optional and conditional parameters are considered to be OPTIONAL syntactically in ASN.1 Stage 3 descriptions. The Stage 2 inclusion takes precedence over Stage 3 syntax.

#### 13.3.2.2	REPORT record information

The REPORT record is used to report non-communication related target actions (events).

The REPORT record shall be triggered when:

-	The target ProSe Remote UE connects to or disconnects from a ProSe UE-to-NW Relay. See Table 13.3.2.2-1.

-	One or several UEs connect to or disconnect from a target ProSe UE-to-NW Relay. See Table 13.3.2.2-1.

-	LI is activated on a target ProSe UE-to-NW Relay with already connected ProSe Remote UEs. See Table 13.3.2.2-2.

-	A failure occurs when the target ProSe Remote UE attempts to start communication by connecting to a ProSe UE-to-NW Relay. See Table 13.3.2.2-3.

Table 13.3.2.2-1: ProSe Remote UE Report REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| Observed IMSI | C | Provide at least one and others when available |
| Observed MSISDN |  |  |
| Observed IMEI |  |  |
| Event Type | M | Provide ProSe Remote UE Report event type |
| Event Time | M | Provide the time the event is detected |
| Event Date | M | Provide the date the event is detected |
| Lawful Interception Identifier | M | Shall be provided |
| Target type | M | Shall be provided |
| Network Identifier | M | Shall be provided |
| ProSe Remote UE(s) connected IDs | C | Provided if the ProSe UE-to-NW Relay is the target |
| ProSe Remote UE(s) connected IP info | C | Provided the ProSe UE-to-NW Relay is the target |
| ProSe Remote UE(s) disconnected IDs | C | Provided if the ProSe UE-to-NW Relay is the target |
| MSISDN of the ProSe UE-to-NW Relay | C | Provided if available and if the ProSe Remote UE is the target |
| IMSI of the ProSe UE-to-NW Relay | C | Provided if available and if the ProSe Remote UE is the target |
| IMEI of the ProSe UE-to-NW Relay | C | Provided if available and if the ProSe Remote UE is the target |
| APN | C | Provided if available |
| PDN Address(es) | C | Provided if available |
| Location information | C | Provide, when authorized, to identify location information for the target UE, including the primary cell ID from each of the Master Cell (PCell) and Secondary Cell (PSCell) groups of the target, if available. |

<<<<<<<<<<<<<<<<<<<<<<<<<< END OF CHANGE 4 >>>>>>>>>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<<<<<<<< START OF CHANGE 5 >>>>>>>>>>>>>>>>>>>>>>>>>>>>

# B.9	Intercept related information (HI2 SAE/EPS and IMS)

Declaration of ROSE operation eps-sending-of-IRI is ROSE delivery mechanism specific. When using FTP delivery mechanism, data EpsIRIsContent has to be considered.

ASN1 description of IRI (HI2 interface)

EpsHI2Operations {itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulintercept(2) threeGPP(4) hi2eps(8) r15(15) version-4 (4)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

IMPORTS

OPERATION,

ERROR

FROM Remote-Operations-Information-Objects

{joint-iso-itu-t(2) remote-operations(4) informationObjects(5) version1(0)}

LawfulInterceptionIdentifier,

TimeStamp,

Network-Identifier,

National-Parameters,

National-HI2-ASN1parameters,

DataNodeAddress,

IPAddress,

IP-value,

X25Address

FROM HI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulIntercept(2) hi2(1) version18(18)} -- Imported from TS 101 671v3.14.1

CivicAddress,

ExtendedLocParameters,

LocationErrorCode

FROM UmtsHI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulintercept(2) threeGPP(4) hi2(1)  r15 (15) version-1 (1)};

-- Imported from 3GPP TS 33.108, UMTS PS HI2

-- Object Identifier Definitions

-- Security DomainId

lawfulInterceptDomainId OBJECT IDENTIFIER ::= {itu-t(0) identified-organization(4) etsi(0)

securityDomain(2) lawfulIntercept(2)}

-- Security Subdomains

threeGPPSUBDomainId OBJECT IDENTIFIER ::= {lawfulInterceptDomainId threeGPP(4)}

hi2epsDomainId OBJECT IDENTIFIER	::= {threeGPPSUBDomainId hi2eps(8) r15(15) version-4 (4)}

eps-sending-of-IRI  OPERATION ::=

{

ARGUMENT	EpsIRIsContent

ERRORS		{ OperationErrors }

CODE		global:{threeGPPSUBDomainId hi2eps(8) opcode(1)}

}

-- Class 2 operation . The timer shall be set to a value between 3 s and 240 s.

-- The timer.default value is 60s.

-- NOTE:	The same note as for HI management operation applies.

EpsIRIsContent		::= CHOICE

{

epsiRIContent			EpsIRIContent,

epsIRISequence			EpsIRISequence

}

EpsIRISequence		::= SEQUENCE OF EpsIRIContent

-- Aggregation of EpsIRIContent is an optional feature.

-- It may be applied in cases when at a given point in time

-- several IRI records are available for delivery to the same LEA destination.

-- As a general rule, records created at any event shall be sent

-- immediately and not withheld in the DF or MF in order to

-- apply aggragation.

-- When aggregation is not to be applied,

-- EpsIRIContent needs to be chosen.

-- EpsIRIContent includes events that correspond to EPS and UMTS/GPRS.

EpsIRIContent		::= CHOICE

{

iRI-Begin-record		[1] IRI-Parameters,	-- include at least one optional parameter

iRI-End-record			[2] IRI-Parameters,

iRI-Continue-record		[3] IRI-Parameters,	-- include at least one optional parameter

iRI-Report-record		[4] IRI-Parameters	-- include at least one optional parameter

}

-- the EpsIRIContent may provide events that correspond to UMTS/GPRS as well.

unknown-version 		ERROR ::= { CODE local:0}

missing-parameter		ERROR ::= { CODE local:1}

unknown-parameter-value	ERROR ::= { CODE local:2}

unknown-parameter		ERROR ::= { CODE local:3}

OperationErrors ERROR ::=

{

unknown-version |

missing-parameter |

unknown-parameter-value |

unknown-parameter

}

-- These values may be sent by the LEMF, when an operation or a parameter is misunderstood.

-- Parameters having the same tag numbers have to be identical in Rel-5 and onwards modules.

IRI-Parameters		::= SEQUENCE

{

hi2epsDomainId				[0]	OBJECT IDENTIFIER,  -- 3GPP HI2 EPS domain

lawfulInterceptionIdentifier	[1] LawfulInterceptionIdentifier,

-- This identifier is associated to the target.

timeStamp				[3] TimeStamp,

-- date and time of the event triggering the report.)

initiator 				[4] ENUMERATED

{

not-Available		(0),

originating-Target	(1),

-- in case of GPRS, this indicates that the PDP context activation, modification

-- or deactivation is MS requested

-- in case of EPS, this indicated that the EPS detach, bearer activation, modification

-- or deactivation is UE requested

terminating-Target	(2),

-- in case of GPRS, this indicates that the PDP context activation, modification or

-- deactivation is network initiated

-- in case of EPS, this indicated that the EPS detach, bearer activation, modification

-- or deactivation is network initiated

...

} OPTIONAL,

locationOfTheTarget		[8] Location OPTIONAL,

-- location of the target

-- or cell site location

partyInformation 		[9] SET SIZE (1..10) OF PartyInformation OPTIONAL,

-- This parameter provides the concerned party, the identiy(ies) of the party

--)and all the information provided by the party.

serviceCenterAddress	[13] PartyInformation OPTIONAL,

-- e.g. in case of SMS message this parameter provides the address of  the relevant

-- server

sMS						[14] SMS-report OPTIONAL,

-- this parameter provides the SMS content and associated information

national-Parameters		[16] National-Parameters OPTIONAL,

ePSCorrelationNumber	[18] EPSCorrelationNumber OPTIONAL,

-- this parameter provides GPRS Correlation number when the event corresponds to UMTS/GPRS.

ePSevent 				[20] EPSEvent OPTIONAL,

-- This information is used to provide particular action of the target

-- such as attach/detach

sgsnAddress 			[21] DataNodeAddress OPTIONAL,

gPRSOperationErrorCode 	[22] GPRSOperationErrorCode OPTIONAL,

ggsnAddress 			[24] DataNodeAddress OPTIONAL,

qOS						[25] UmtsQos OPTIONAL,

networkIdentifier		[26] Network-Identifier OPTIONAL,

sMSOriginatingAddress 	[27] DataNodeAddress OPTIONAL,

sMSTerminatingAddress 	[28] DataNodeAddress OPTIONAL,

iMSevent				[29] IMSevent OPTIONAL,

sIPMessage				[30] OCTET STRING  OPTIONAL,

servingSGSN-number		[31] OCTET STRING (SIZE (1..20))	OPTIONAL,

servingSGSN-address		[32] OCTET STRING (SIZE (5..17)) 	OPTIONAL,

-- Octets are coded according to 3GPP TS 23.003 [25]

...,

-- Tag			[33] was taken into use by ETSI module in TS 101 671v2.13.1

ldiEvent	 			[34] LDIevent OPTIONAL,

correlation 			[35] CorrelationValues OPTIONAL,

ePS-GTPV2-specificParameters   [36] EPS-GTPV2-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of GTPV2 based intercepted messages

ePS-PMIP-specificParameters    [37] EPS-PMIP-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of PMIP based intercepted messages

ePS-DSMIP-SpecificParameters   [38] EPS-DSMIP-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of DSMIP based intercepted messages

ePS-MIP-SpecificParameters     [39] EPS-MIP-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of MIP based intercepted messages

servingNodeAddress             [40] OCTET STRING               OPTIONAL,

-- this parameter is kept for backward compatibility only and should not be used

-- as it has been superseeded by parameter visitedNetworkId

visitedNetworkId               [41] UTF8String                 OPTIONAL,

-- contains the visited network identifier inside the Serving System Update for

-- non 3GPP access and IMS, coded according to [53] and 3GPP TS 29.229 [96]

mediaDecryption-info	       [42] MediaDecryption-info OPTIONAL,

servingS4-SGSN-address	       [43] OCTET STRING OPTIONAL,

-- Diameter Origin-Host and Origin-Realm of the S4-SGSN based on the TS 29.272 [59].

-- Only the data fields from the Diameter AVPs are provided concatenated

-- with a semicolon to populate this field.

sipMessageHeaderOffer   [44] OCTET STRING OPTIONAL,

sipMessageHeaderAnswer  [45] OCTET STRING OPTIONAL,

sdpOffer                [46] OCTET STRING OPTIONAL,

sdpAnswer               [47] OCTET STRING OPTIONAL,

uLITimestamp            [48] OCTET STRING (SIZE (8)) OPTIONAL,

-- Coded according to 3GPP TS 29.060 [17]; Only the ULI Timestamp value is reported.

packetDataHeaderInformation		   [49] PacketDataHeaderInformation	OPTIONAL,

mediaSecFailureIndication          [50] MediaSecFailureIndication OPTIONAL,

csgIdentity			[51] OCTET STRING (SIZE (4)) OPTIONAL,  -- Octets are coded

-- according to 3GPP TS 23.003 [25].  The 27 bits specified in TS 23.003 shall be encoded as.

-- follows The most significant bit of the CSG Identity shall be encoded in the most

-- significant bit of the first octet of the octet string and the least significant bit coded

-- in bit 6 of octet 4.

heNBIdentity			[52] OCTET STRING OPTIONAL,

-- 4 or 6 octets are coded with the HNBUnique Identity

-- as specified in 3GPP TS 23.003            [25], Clause 4.10.

heNBiPAddress		[53] IPAddress  OPTIONAL,

heNBLocation			[54] HeNBLocation  OPTIONAL,

tunnelProtocol		[55] TunnelProtocol  OPTIONAL,

pANI-Header-Info		[56] SEQUENCE OF PANI-Header-Info		OPTIONAL,

-- information extracted from P-Access-Network-Info headers of SIP message;

-- described in TS 24.229 §7.2A.4 [76]

imsVoIP 				[57] IMS-VoIP-Correlation					OPTIONAL,

xCAPmessage			[58] OCTET STRING OPTIONAL,

-- The HTTP message (HTPP header and any XCAP body) of any of the target's IMS supplementary

-- service setting management or manipulation XCAP messages occuring through the Ut interface

-- defined in the 3GPP TS 24 623 [77].

logicalFunctionInformation	[59] DataNodeIdentifier OPTIONAL,

ccUnavailableReason		[60] PrintableString	OPTIONAL,

carrierSpecificData					[61] OCTET STRING OPTIONAL,

-- Copy of raw data specified by the CSP or his vendor related to HSS.

current-previous-systems			[62] Current-Previous-Systems OPTIONAL,

change-Of-Target-Identity			[63] Change-Of-Target-Identity OPTIONAL,

requesting-Network-Identifier		[64] OCTET STRING OPTIONAL,

-- the requesting network identifier PLMN id (Mobile Country Code and Mobile Network Country,

-- defined in E212 [87]).

requesting-Node-Type				[65] Requesting-Node-Type OPTIONAL,

serving-System-Identifier			[66] OCTET STRING OPTIONAL,

-- the serving network identifier PLMN id (MNC, Mobile Country Code and MNC,Mobile Network

-- Country, defined in E212 [87]) and 3GPP TR 21.905 [38], that may be included in the Diameter

-- AVP to and from the HSS.

proSeTargetType                    [67] ProSeTargetType OPTIONAL,

proSeRelayMSISDN                   [68] OCTET STRING (SIZE (1..9)) OPTIONAL,

-- coded according to 3GPP TS 29.274 [46]

proSeRelayIMSI                     [69] OCTET STRING (SIZE (3..8)) OPTIONAL,

-- coded according to 3GPP TS 29.274 [46]

proSeRelayIMEI                     [70] OCTET STRING (SIZE (8)) OPTIONAL,

-- coded according to 3GPP TS 29.274 [46]

extendedLocParameters 	[71] 	ExtendedLocParameters OPTIONAL, -- LALS extended parameters

locationErrorCode		[72]	LocationErrorCode OPTIONAL,	-- LALS error code

otherIdentities                    [73] SEQUENCE OF PartyInformation OPTIONAL,

deregistrationReason               [74] DeregistrationReason OPTIONAL,

requesting-Node-Identifier         [75] OCTET STRING OPTIONAL,

roamingIndication                  [76] VoIPRoamingIndication 	OPTIONAL,

-- used for IMS events in the VPLMN.

cSREvent						[77]	CSREvent OPTIONAL,

ptc								[78]	PTC OPTIONAL,  -- PTC Events

ptcEncryption					[79]	PTCEncryptionInfo OPTIONAL,

additionalCellLocations			[80] SEQUENCE OF AdditionalCell OPTIONAL,

national-HI2-ASN1parameters	[255]	National-HI2-ASN1parameters	OPTIONAL

}

-- Parameters having the same tag numbers have to be identical in Rel-5 and onwards modules

-- PARAMETERS FORMATS

DataNodeIdentifier ::= SEQUENCE

{

dataNodeAddress			[1] DataNodeAddress OPTIONAL,

logicalFunctionType	[2] LogicalFunctionType OPTIONAL,

dataNodeName			[3] PrintableString(SIZE(7..25)) OPTIONAL,

--Unique identifier of a Data Node within the CSP domain. Could be a name/number combination.

...

}

LogicalFunctionType ::= ENUMERATED

{

pDNGW		(0),

mME			(1),

sGW			(2),

ePDG		(3),

hSS			(4),

...

}

PANI-Header-Info	::= SEQUENCE

{

access-Type 		[1] OCTET STRING 		OPTIONAL,

-- ASCII chain '3GPP-UTRAN-TDD', '3GPP-E-UTRAN-TDD',... : see TS 24.229 §7.2A.4 [76]

access-Class  		[2] OCTET STRING		OPTIONAL,

-- ASCII chain '3GPP-UTRAN', '3GPP-E-UTRAN',... : see TS 24.229 §7.2A.4 [76]

network-Provided	[3] NULL				OPTIONAL,

-- present if provided by the network

pANI-Location		[4] PANI-Location 		OPTIONAL,

...

}

PANI-Location  ::= SEQUENCE

{

raw-Location     [1] OCTET STRING 	OPTIONAL,

-- raw copy of the location string from the P-Access-Network-Info header

location          [2] Location      OPTIONAL,

ePSLocation       [3] EPSLocation   OPTIONAL,

...

}

PartyInformation 			::= SEQUENCE

{

party-Qualifier 	[0]  ENUMERATED

{

gPRSorEPS-Target(3),

...

},

partyIdentity 		[1] SEQUENCE

{

imei					[1] OCTET STRING (SIZE (8)) OPTIONAL,

-- See MAP format [4]

imsi					[3] OCTET STRING (SIZE (3..8)) OPTIONAL,

-- See MAP format [4] International Mobile

-- Station Identity E.212 number beginning with Mobile Country Code

msISDN					[6] OCTET STRING (SIZE (1..9)) OPTIONAL,

-- MSISDN of the target, encoded in the same format as the AddressString

-- parameters defined in MAP format document TS 29.002 [4]

e164-Format				[7] OCTET STRING    (SIZE (1 .. 25)) OPTIONAL,

-- E164 address of the node in international format. Coded in the same format as

-- the calling party number  parameter of the ISUP (parameter part:[29])

sip-uri					[8] OCTET STRING 	OPTIONAL,

-- See [26]

...,

tel-uri					[9] OCTET STRING 	OPTIONAL,

-- See [67]

nai 						 [10] OCTET STRING    OPTIONAL,

-- NAI of the target, encoded in the same format as defined by [EPS stage 3 specs]

x-3GPP-Asserted-Identity [11] OCTET STRING 	OPTIONAL,

-- X-3GPP-Asserted-Identity header (3GPP TS 24.109 [79]) of the target, used in

-- some XCAP transactions as a complement information to SIP URI or Tel URI.

xUI						[12] OCTET STRING 	OPTIONAL,

-- XCAP User Identifier (XUI)is a string, valid as a path element in an XCAP URI, that is

-- may be associated with each user served by a XCAP resource server. Defined in IETF RFC

-- 4825[80] as a complement information to SIP URI or Tel URI.

iMPI                    [13] OCTET STRING   OPTIONAL

-- Private User Identity as defined in 3GPP TS 23.003 [25]

},

services-Data-Information 	[4] Services-Data-Information OPTIONAL,

-- This parameter is used to transmit all the information concerning the

-- complementary information associated to the basic data call

...

}

Location	::= SEQUENCE

{

e164-Number         [1] OCTET STRING (SIZE (1..25)) OPTIONAL,
        -- Coded in the same format as the ISUP location number (parameter
        -- field) of the ISUP (see EN 300 356 [30]).    
	globalCellID		[2] GlobalCellID	OPTIONAL,

--see MAP format (see [4])

rAI					[4] Rai		OPTIONAL,

-- the Routeing Area Identifier in the current  SGSN is coded in accordance with the

-- § 10.5.5.15 of document [9] without the Routing Area Identification IEI

-- (only the last 6 octets are used)

gsmLocation			[5] GSMLocation OPTIONAL,

umtsLocation		[6] UMTSLocation OPTIONAL,

sAI					[7] Sai	OPTIONAL,

-- format:	PLMN-ID	3 octets (no. 1 - 3)

--			LAC		2 octets (no. 4 - 5)

--			SAC		2 octets (no. 6 - 7)

--			(according to 3GPP TS 25.413 [62])

...,

oldRAI				[8] Rai		OPTIONAL,

-- the Routeing Area Identifier in the old SGSN is coded in accordance with the

-- § 10.5.5.15 of document [9] without the Routing Area Identification IEI

-- (only the last 6 octets are used).

civicAddress		[9] CivicAddress OPTIONAL,

operatorSpecificInfo	[10]	OCTET STRING OPTIONAL,

-- other CSP specific information.

uELocationTimestamp		[11]	CHOICE

{

timestamp			[0]	TimeStamp,

timestampUnknown	[1]	NULL,

...

} OPTIONAL,

-- Date/time of the UE location

secondaryCellGroupPSCells	[12] SEQUENCE OF AdditionalCell	OPTIONAL

-- see MAP format (see [4])

}

GlobalCellID	::= OCTET STRING  (SIZE (5..7))

Rai				::= OCTET STRING (SIZE (6))

Sai				::=	OCTET STRING (SIZE (7))

AdditionalCell ::= SEQUENCE

{

globalCellID		[1] GlobalCellID	OPTIONAL,

--see MAP format (see [4])

NRCellID			[2] NRCellID OPTIONAL

gsmLocation			[3] GSMLocation OPTIONAL,

umtsLocation		[4] UMTSLocation OPTIONAL,

...

}

NRCellID ::= OCTET STRING (SIZE(8))

GSMLocation 	::= CHOICE

{

geoCoordinates 	[1] SEQUENCE

{

latitude		[1]	PrintableString (SIZE(7..10)),

-- format : 	XDDMMSS.SS

longitude		[2]	PrintableString (SIZE(8..11)),

-- format : 	XDDDMMSS.SS

mapDatum		[3]	MapDatum DEFAULT wGS84,

...,

azimuth			[4]	INTEGER (0..359) OPTIONAL

-- The azimuth is the bearing, relative to true north.

},

-- format : 	XDDDMMSS.SS

-- 				X 			: N(orth), S(outh), E(ast), W(est)

--				DD or DDD 	: degrees (numeric characters)

--				MM			: minutes (numeric characters)

--				SS.SS 		: seconds, the second part (.SS) is optionnal

-- Example :

--			latitude short form		N502312

--			longitude long form		E1122312.18

utmCoordinates	[2] SEQUENCE

{

utm-East 		[1] PrintableString (SIZE(10)),

utm-North 		[2] PrintableString (SIZE(7)),

-- example	utm-East	32U0439955

--			utm-North	5540736

mapDatum		[3]	MapDatum DEFAULT wGS84,

...,

azimuth			[4] INTEGER (0..359) OPTIONAL

-- The azimuth is the bearing, relative to true north.

},

utmRefCoordinates  	[3] SEQUENCE

{

utmref-string		PrintableString (SIZE(13)),

mapDatum			MapDatum DEFAULT wGS84,

...

},

-- example	32UPU91294045

wGS84Coordinates  	[4] OCTET STRING

-- format is as defined in [37].

}

MapDatum ::= ENUMERATED

{

wGS84,

wGS72,

eD50, 	-- European Datum 50

...

}

UMTSLocation ::= CHOICE {

point					[1]	GA-Point,

pointWithUnCertainty	[2]	GA-PointWithUnCertainty,

polygon					[3]	GA-Polygon

}

GeographicalCoordinates ::= SEQUENCE {

latitudeSign			ENUMERATED { north, south },

latitude				INTEGER (0..8388607),

longitude				INTEGER (-8388608..8388607),

...

}

GA-Point ::= SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

...

}

GA-PointWithUnCertainty ::=SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

uncertaintyCode				INTEGER (0..127)

}

maxNrOfPoints						INTEGER ::= 15

GA-Polygon ::= SEQUENCE (SIZE (1..maxNrOfPoints)) OF

SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

...

}

SMS-report		::= SEQUENCE

{

sMS-Contents	[3] SEQUENCE

{

sms-initiator		[1] ENUMERATED 	-- party which sent the  SMS

{

target			(0),

server			(1),

undefined-party	(2),

...

},

transfer-status		[2] ENUMERATED

{

succeed-transfer	(0), 		-- the transfer of the SMS message succeeds

not-succeed-transfer(1),

undefined			(2),

...

} OPTIONAL,

other-message		[3] ENUMERATED 	-- in case of terminating call, indicates if

-- the server will send other SMS

{

yes			(0),

no			(1),

undefined	(2),

...

} OPTIONAL,

content				[4] OCTET STRING (SIZE (1 .. 270)) OPTIONAL,

-- Encoded in the format defined for the SMS mobile

...

}

}

EPSCorrelationNumber ::= OCTET STRING

-- In case of PS interception, the size will be in the range (8..20)

CorrelationValues ::= CHOICE {

iri-to-CC 	[0]		IRI-to-CC-Correlation, -- correlates IRI to Content(s)

iri-to-iri	[1]		IRI-to-IRI-Correlation, -- correlates IRI to IRI

both-IRI-CC	[2]		SEQUENCE { -- correlates IRI to IRI and IRI to Content(s)

iri-CC	[0]	IRI-to-CC-Correlation,

iri-IRI	[1]	IRI-to-IRI-Correlation}

}

IMS-VoIP-Correlation ::= SET OF SEQUENCE {

ims-iri		[0]		IRI-to-IRI-Correlation,

ims-cc		[1]		IRI-to-CC-Correlation 		OPTIONAL

}

IRI-to-CC-Correlation ::= SEQUENCE { -- correlates IRI to Content

cc 		[0] SET OF 	OCTET STRING,-- correlates IRI to multiple CCs

iri 	[1] 		OCTET STRING OPTIONAL

-- correlates IRI to CC with signaling

}

IRI-to-IRI-Correlation ::= OCTET STRING -- correlates IRI to IRI

EPSEvent ::= ENUMERATED

{

pDPContextActivation 						(1),

startOfInterceptionWithPDPContextActive	(2),

pDPContextDeactivation						(4),

gPRSAttach 									(5),

gPRSDetach 									(6),

locationInfoUpdate 						(10),

sMS 										(11),

pDPContextModification						(13),

servingSystem								(14),

... ,

startOfInterceptionWithMSAttached			(15),

e-UTRANAttach								(16),

e-UTRANDetach								(17),

bearerActivatio								(18),

startOfInterceptionWithActiveBearer		(19),

bearerModificatio							(20),

bearerDeactivation							(21),

uERequestedBearerResourceModification		(22),

uERequestedPDNConnectivity					(23),

uERequestedPDNDisconnection				(24),

trackingAreaEpsLocationUpdate				(25),

servingEvolvedPacketSystem					(26),

pMIPAttachTunnelActivation					(27),

pMIPDetachTunnelDeactivation				(28),

startOfInterceptWithActivePMIPTunnel		(29),

pMIPPdnGwInitiatedPdnDisconnection			(30),

mIPRegistrationTunnelActivation			(31),

mIPDeregistrationTunnelDeactivation		(32),

startOfInterceptWithActiveMIPTunnel		(33),

dSMIPRegistrationTunnelActivation			(34),

dSMIPDeregistrationTunnelDeactivation		(35),

startOfInterceptWithActiveDsmipTunnel		(36),

dSMipHaSwitch								(37),

pMIPResourceAllocationDeactivation			(38),

mIPResourceAllocationDeactivation			(39),

pMIPsessionModification					(40),

startOfInterceptWithEUTRANAttachedUE		(41),

dSMIPSessionModification					(42),

packetDataHeaderInformation				(43),

hSS-Subscriber-Record-Change				(44),

registration-Termination					(45),

-- FFS

location-Up-Date							(46),

-- FFS

cancel-Location								(47),

register-Location							(48),

location-Information-Request				(49),

proSeRemoteUEReport						(50),

proSeRemoteUEStartOfCommunication			(51),

proSeRemoteUEEndOfCommunication			(52),

startOfLIwithProSeRemoteUEOngoingComm		(53),

startOfLIforProSeUEtoNWRelay				(54)

}

-- see [19]

CSREvent ::= ENUMERATED

{

cSREventMessage	 					(1),

...

}

IMSevent ::= ENUMERATED

{

unfilteredSIPmessage (1),

-- This value indicates to LEMF that the whole SIP message is sent , i.e. without filtering

-- CC; location information is removed by the DF2/MF if not required to be sent.

...,

sIPheaderOnly (2),

-- If warrant requires only IRI then specific content in a 'sIPMessage'

-- (e.g. 'Message', etc.) has been deleted before sending it to LEMF.

decryptionKeysAvailable (3),

-- This value indicates to LEMF that the IRI carries CC decryption keys for the session

-- under interception.

startOfInterceptionForIMSEstablishedSession (4),

-- This value indicates to LEMF that the IRI carries information related to

-- interception started on an already established IMS session.

xCAPRequest (5),

-- This value indicates to LEMF that the XCAP request is sent.

xCAPResponse (6) ,

-- This value indicates to LEMF that the XCAP response is sent.

ccUnavailable	(7),

-- This value indicates to LEMF that the media is not available for interception for intercept

-- orders that requires media interception.

sMSOverIMS	(8),

-- This value indicates to LEMF that the SMS utilized by SMS over IP (using IMS) is

-- being reported.

servingSystem(9),

-- Applicable to HSS interception

subscriberRecordChange(10),

-- Applicable to HSS interception
    registrationTermination(11),

-- Applicable to HSS interception

locationInformationRequest(12)

-- Applicable to HSS interception

}

Services-Data-Information ::= SEQUENCE

{

gPRS-parameters [1] GPRS-parameters OPTIONAL,

...

}

GPRS-parameters ::= SEQUENCE

{

pDP-address-allocated-to-the-target 	[1] DataNodeAddress OPTIONAL,

aPN 					[2] OCTET STRING (SIZE(1..100)) OPTIONAL,

-- The Access Point Name (APN) is coded in accordance with

-- 3GPP TS 24.008 [9] without the APN IEI (only the last 100 octets are used).

-- Octets are coded according to 3GPP TS 23.003 [25].

pDP-type 				[3] OCTET STRING (SIZE(2)) OPTIONAL,

-- Include either Octets 3 and 4 of the Packet Data Protocol Address information element

-- of 3GPP TS 24.008 [9] or Octets 4 and 5 of the End User Address IE of 3GPP TS 29.060 [17].

-- when PDP-type is IPv4 or IPv6, the IP address is carried by parameter

-- pDP-address-allocated-to-the-target

-- when PDP-type is IPv4v6, the additional IP address is carried by parameter

-- additionalIPaddress

...,

nSAPI					[4] OCTET STRING (SIZE (1)) OPTIONAL,

-- Include either Octet 2 of the NSAPI IE of 3GPP TS 24.008 [9]

-- or Octet 2 of the NSAPI IE of 3GPP TS 29.060 [17].

additionalIPaddress		[5] DataNodeAddress OPTIONAL

}

GPRSOperationErrorCode ::= OCTET STRING

-- The parameter shall carry the GMM cause value or the SM cause value, as defined in the

-- standard [9], without the IEI.

LDIevent ::= ENUMERATED

{

targetEntersIA 			(1),

targetLeavesIA			(2),

...

}

UmtsQos ::= CHOICE

{

qosMobileRadio [1] OCTET STRING,

-- The qosMobileRadio parameter shall be coded in accordance with the § 10.5.6.5 of

-- document [9] without the Quality of service IEI and Length of

-- quality of service IE (. That is, first

-- two octets carrying 'Quality of service IEI' and 'Length of quality of service

-- IE' shall be excluded).

qosGn [2] OCTET STRING

-- qosGn parameter shall be coded in accordance with § 7.7.34 of document [17]

}

EPS-GTPV2-SpecificParameters ::= SEQUENCE

{

pDNAddressAllocation                 [1]   OCTET STRING                       OPTIONAL,

aPN                                  [2]   OCTET STRING (SIZE (1..100))       OPTIONAL,

protConfigOptions                    [3]   ProtConfigOptions                  OPTIONAL,

attachType                           [4]   OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47]

ePSBearerIdentity                    [5]   OCTET STRING                       OPTIONAL,

detachType                           [6]   OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47], includes switch off indicator

rATType                              [7]   OCTET STRING (SIZE (1))            OPTIONAL,

failedBearerActivationReason         [8]   OCTET STRING (SIZE (1))            OPTIONAL,

ePSBearerQoS                         [9]   OCTET STRING                       OPTIONAL,

bearerActivationType                 [10]  TypeOfBearer                       OPTIONAL,

aPN-AMBR                             [11]  OCTET STRING                       OPTIONAL,

-- see 3GPP TS 29.274 [46] parameters coding rules defined for EPS-GTPV2-SpecificParameters.

procedureTransactionId               [12]  OCTET STRING                       OPTIONAL,

linkedEPSBearerId                    [13]  OCTET STRING                       OPTIONAL,

--The Linked EPS Bearer Identity shall be included and coded according to 3GPP TS 29.274 [46].

tFT                                  [14]  OCTET STRING                       OPTIONAL,

-- Only octets 3 onwards of TFT IE from 3GPP TS 24.008 [9] shall be included.

handoverIndication                   [15]  NULL                               OPTIONAL,

failedBearerModReason                [16]  OCTET STRING (SIZE (1))            OPTIONAL,

trafficAggregateDescription          [17]  OCTET STRING                       OPTIONAL,

failedTAUReason                      [18]  OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47]

failedEUTRANAttachReason             [19]  OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47]

servingMMEaddress                    [20]  OCTET STRING                       OPTIONAL,

-- Contains the data fields from the Diameter Origin-Host and Origin-Realm AVPs

-- as received in the HSS from the MME according to the TS 29.272 [59].

-- Only the data fields from the Diameter AVPs are provided concatenated

-- with a semicolon to populate this field.

bearerDeactivationType               [21]  TypeOfBearer                       OPTIONAL,

bearerDeactivationCause              [22]  OCTET STRING (SIZE (1))            OPTIONAL,
    ePSlocationOfTheTarget               [23]  EPSLocation                        OPTIONAL,
      -- the use of ePSLocationOfTheTarget is mutually exclusive with the use of locationOfTheTarget

-- ePSlocationOfTheTarget allows using the coding of the parameter according to SAE stage 3.

-- location of the target

-- or cell site location

...,

pDNType                              [24]   OCTET STRING (SIZE (1))           OPTIONAL,

-- coded according to TS 24.301 [47]

requestType                          [25]  OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47]

uEReqPDNConnFailReason               [26]  OCTET STRING (SIZE (1))            OPTIONAL,

-- coded according to TS 24.301 [47]

extendedHandoverIndication			 [27]  OCTET STRING (SIZE (1)) 			  OPTIONAL,

-- This parameter with value 1 indicates handover based on the flags in the TS 29.274 [46].

-- Otherwise set to the value 0.

-- The use of extendedHandoverIndication and handoverIndication parameters is

-- mutually exclusive and depends on the actual ASN.1 encoding method.

uLITimestamp                         [28]  OCTET STRING (SIZE (8))            OPTIONAL,

uELocalIPAddress                     [29]  OCTET STRING                       OPTIONAL,

uEUdpPort                            [30]  OCTET STRING (SIZE (2))            OPTIONAL,

tWANIdentifier                       [31]  OCTET STRING                       OPTIONAL,

tWANIdentifierTimestamp              [32]  OCTET STRING (SIZE (4))            OPTIONAL,

proSeRemoteUeContextConnected        [33]  RemoteUeContextConnected           OPTIONAL,

proSeRemoteUeContextDisconnected     [34]  RemoteUeContextDisconnected        OPTIONAL,

secondaryRATUsageIndication          [35]  NULL                               OPTIONAL
    }

-- All the parameters within EPS-GTPV2-SpecificParameters are coded as the corresponding IEs

-- without the octets containing type and length. Unless differently stated, they are coded

-- according to 3GPP TS 29.274 [46]; in this case the octet containing the instance

-- shall also be not included.

TypeOfBearer ::= ENUMERATED

{

defaultBearer          (1),
    dedicatedBearer        (2),

...

}

EPSLocation	::= SEQUENCE

{

userLocationInfo    [1] OCTET STRING (SIZE (1..39)) OPTIONAL,
        -- see 3GPP TS 29.274 [46] parameters coding rules defined for EPS-GTPV2-SpecificParameters.

gsmLocation			[2] GSMLocation OPTIONAL,

umtsLocation		[3] UMTSLocation OPTIONAL,

olduserLocationInfo	[4] OCTET STRING (SIZE (1..39))	OPTIONAL,

-- coded in the same way as userLocationInfo

lastVisitedTAI	    [5] OCTET STRING (SIZE (1..5))	OPTIONAL,

-- the Tracking Area Identity is coded in accordance with the TAI field in 3GPP TS 29.274
        -- [46].

tAIlist			    [6] OCTET STRING (SIZE (7..97))	OPTIONAL,

-- the TAI List is coded acording to 3GPP TS 24.301 [47], without the TAI list IEI

...,

threeGPP2Bsid		[7] OCTET STRING (SIZE (1..12)) OPTIONAL,

-- contains only the payload from the 3GPP2-BSID AVP described in the 3GPP TS 29.212 [56].

civicAddress		[8] CivicAddress OPTIONAL,

operatorSpecificInfo	[9]	OCTET STRING OPTIONAL,

-- other CSP specific information.

uELocationTimestamp		[10]	CHOICE

{

timestamp			[0]	TimeStamp,

timestampUnknown	[1]	NULL,

...

} OPTIONAL

-- Date/time of the UE location

}

ProtConfigOptions ::= SEQUENCE

{

ueToNetwork                           [1]  OCTET STRING (SIZE(1..251))        OPTIONAL,

-- This shall be coded with octet 3 onwards of the Protocol Configuration Options IE in

-- accordance with 3GPP TS 24.008 [9].

networkToUe                           [2]  OCTET STRING (SIZE(1..251))        OPTIONAL,

-- This shall be coded with octet 3 onwards of the Protocol Configuration Options IE in

-- accordance with 3GPP TS 24.008 [9].

...

}

RemoteUeContextConnected ::= SEQUENCE OF RemoteUEContext

RemoteUEContext ::= SEQUENCE

{

remoteUserID                          [1]    RemoteUserID,

remoteUEIPInformation                 [2]    RemoteUEIPInformation,

...

}

RemoteUserID ::= OCTET STRING

RemoteUEIPInformation ::= OCTET STRING

RemoteUeContextDisconnected ::= RemoteUserID

EPS-PMIP-SpecificParameters ::= SEQUENCE

{

lifetime                              [1]  INTEGER (0..65535)                 OPTIONAL,

accessTechnologyType                  [2]  OCTET STRING (SIZE (4))            OPTIONAL,

aPN                                   [3]  OCTET STRING (SIZE (1..100))       OPTIONAL,

iPv6HomeNetworkPrefix                 [4]  OCTET STRING (SIZE (20))           OPTIONAL,

protConfigurationOption               [5]  OCTET STRING                       OPTIONAL,

handoverIndication                    [6]  OCTET STRING (SIZE (4))            OPTIONAL,

status                                [7]  INTEGER (0..255)                   OPTIONAL,

revocationTrigger                     [8]  INTEGER (0..255)                   OPTIONAL,

iPv4HomeAddress                       [9]  OCTET STRING (SIZE (4))            OPTIONAL,

iPv6careOfAddress                     [10] OCTET STRING                       OPTIONAL,

iPv4careOfAddress                     [11] OCTET STRING                       OPTIONAL,

...,

servingNetwork                        [12] OCTET STRING (SIZE (3))            OPTIONAL,

dHCPv4AddressAllocationInd            [13] OCTET STRING (SIZE (1))            OPTIONAL,

ePSlocationOfTheTarget                [14] EPSLocation                        OPTIONAL

-- parameters coded according to 3GPP TS 29.275 [48] and RFCs specifically

-- referenced in it.

}

EPS-DSMIP-SpecificParameters ::= SEQUENCE

{

lifetime                              [1]   INTEGER (0..65535)                OPTIONAL,

requestedIPv6HomePrefix               [2]   OCTET STRING (SIZE (25))          OPTIONAL,

-- coded according to RFC 5026

homeAddress                           [3]   OCTET STRING (SIZE (8))           OPTIONAL,

iPv4careOfAddress                     [4]   OCTET STRING (SIZE (8))           OPTIONAL,

iPv6careOfAddress                     [5]   OCTET STRING (SIZE(16))           OPTIONAL,

aPN                                   [6]   OCTET STRING (SIZE (1..100))      OPTIONAL,

status                                [7]   INTEGER (0..255)                  OPTIONAL,

hSS-AAA-address                       [8]   OCTET STRING                      OPTIONAL,

targetPDN-GW-Address                  [9]   OCTET STRING                      OPTIONAL,

...

-- parameters coded according to 3GPP TS 24.303 [49] and RFCs specifically

-- referenced in it.

}

EPS-MIP-SpecificParameters ::= SEQUENCE

{

lifetime                              [1]   INTEGER (0.. 65535)               OPTIONAL,

homeAddress                           [2]   OCTET STRING (SIZE (4))           OPTIONAL,

careOfAddress                         [3]   OCTET STRING (SIZE (4))           OPTIONAL,

homeAgentAddress                      [4]   OCTET STRING (SIZE (4))           OPTIONAL,

code                                  [5]   INTEGER (0..255)                  OPTIONAL,

foreignDomainAddress                  [7]   OCTET STRING (SIZE (4))           OPTIONAL,

...

-- parameters coded according to 3GPP TS 29.279 [63] and RFCs specifically

-- referenced in it.

}

MediaDecryption-info ::= SEQUENCE OF CCKeyInfo

-- One or more key can be available for decryption, one for each media streams of the

-- intercepted session.

CCKeyInfo ::= SEQUENCE

{

cCCSID	 [1]	OCTET STRING,

-- the parameter uniquely mapping the key to the encrypted stream.

cCDecKey [2]	OCTET STRING,

cCSalt   [3]    OCTET STRING OPTIONAL,

-- The field reports the value from the CS_ID field in the ticket exchange headers as

-- defined in IETF RFC 6043 [61].

...

}

MediaSecFailureIndication  ::= ENUMERATED

{

genericFailure  (0),

...

}

PacketDataHeaderInformation ::= CHOICE

{

packetDataHeader		[1]	PacketDataHeaderReport,

packetDataSummary	[2]	PacketDataSummaryReport,

...

}

PacketDataHeaderReport ::= CHOICE

{

packetDataHeaderMapped	[1]	PacketDataHeaderMapped,

packetDataHeaderCopy	[2]	PacketDataHeaderCopy,

...

}

PacketDataHeaderMapped ::= SEQUENCE

{

sourceIPAddress			[1] IPAddress,

sourcePortNumber		[2] INTEGER (0..65535) OPTIONAL,

destinationIPAddress	[3] IPAddress,

destinationPortNumber	[4] INTEGER (0..65535) OPTIONAL,

transportProtocol		[5] INTEGER,

-- For IPv4, report the "Protocol" field and for IPv6 report "Next Header" field.

-- Assigned Internet Protocol Numbers can be found at

-- http://www.iana.org/assignments/protocol-numbers/protocol-numbers.xml

packetsize				[6] INTEGER OPTIONAL,

flowLabel				[7] INTEGER OPTIONAL,

packetCount				[8] INTEGER OPTIONAL,

direction				[9] TPDU-direction,

...

}

TPDU-direction ::= ENUMERATED

{

from-target 	(1),

to-target 		(2),

unknown 		(3)

}

PacketDataHeaderCopy ::= SEQUENCE

{

direction				[1] TPDU-direction,

headerCopy				[2] OCTET STRING,	-- includes a copy of the packet header at the IP

-- network layer and above including extension headers, but excluding contents.

...

}

PacketDataSummaryReport ::= SEQUENCE OF PacketFlowSummary

PacketFlowSummary ::= SEQUENCE

{

sourceIPAddress			[1] IPAddress,

sourcePortNumber		[2] INTEGER (0..65535) OPTIONAL,

destinationIPAddress	[3] IPAddress,

destinationPortNumber	[4] INTEGER (0..65535) OPTIONAL,

transportProtocol		[5] INTEGER,

-- For IPv4, report the "Protocol" field and for IPv6 report "Next Header" field.

-- Assigned Internet Protocol Numbers can be found at

-- http://www.iana.org/assignments/protocol-numbers/protocol-numbers.xml

flowLabel				[6] INTEGER OPTIONAL,

summaryPeriod			[7] ReportInterval,

packetCount				[8] INTEGER,

sumOfPacketSizes		[9] INTEGER,

packetDataSummaryReason	[10] ReportReason,

...

}

ReportReason ::= ENUMERATED

{

timerExpired			(0),

countThresholdHit		(1),

pDPComtextDeactivated   (2),

pDPContextModification	(3),

otherOrUnknown			(4),

...

}

ReportInterval ::= SEQUENCE

{

firstPacketTimeStamp	[0] TimeStamp,

lastPacketTimeStamp		[1] TimeStamp,

...

}

TunnelProtocol ::= CHOICE

{

rfc2868ValueField [0] OCTET STRING, -- coded to indicate the type of tunnel established between

-- the HeNB and the SeGW as specified in TS 33.320.  The actual coding is provided in 3 octets

-- with the Value field of the Tunnel Type RADIUS attribute as specified in IETF RFC 2868.

-- This corresponds to the outer layer tunnel between the HeNB and the SeGW as viewed by the

-- SeGW

nativeIPSec       [1] NULL, -- if native IPSec is required by TS 33.320 between HeNB and SeGW

...

}

HeNBLocation ::= EPSLocation

Requesting-Node-Type ::= ENUMERATED

{

mSC				(1),

sMS-Centre		(2),

gMLC			(3),

mME				(4),

sGSN			(5),

...

}

Change-Of-Target-Identity ::= SEQUENCE

{

new-MSISDN								[1] PartyInformation OPTIONAL,

-- new MSISDN of the target, encoded in the same format as the AddressString

-- parameters defined in MAP format document TS 29.002 [4]

new-A-MSISDN				[2] PartyInformation OPTIONAL,

-- new A-MSISDN of the target, encoded in the same format as the AddressString

-- parameters defined in TS 23.003 [25]

old-MSISDN				[3] PartyInformation OPTIONAL,

-- old MSISDN of the target, encoded in the same format as the AddressString

-- parameters defined in MAP format document TS 29.002 [4]

old-A-MSISDN				[4] PartyInformation OPTIONAL,

-- old A-MSISDN of the target, encoded in the same format as the AddressString

-- parameters defined in TS 23.003 [25]

new-IMSI 								[5] PartyInformation OPTIONAL,

-- See MAP format [4] International Mobile

-- Station Identity E.212 number beginning with Mobile Country Code

old-IMSI 								[6] PartyInformation OPTIONAL,

-- See MAP format [4] International Mobile

-- Station Identity E.212 number beginning with Mobile Country Code

new-IMEI 								[7] PartyInformation OPTIONAL,

-- See MAP format [4] International Mobile

-- Equipement Identity defined in MAP format document TS 29.002 [4]

old-IMEI 								[8] PartyInformation OPTIONAL,

-- See MAP format [4] International Mobile

-- Equipement Identity defined in MAP format document TS 29.002 [4]

...,

new-IMPI							[9] PartyInformation OPTIONAL,
	old-IMPI							[10] PartyInformation OPTIONAL,

new-SIP-URI							[11] PartyInformation OPTIONAL,

old-SIP-URI							[12] PartyInformation OPTIONAL,

new-TEL-URI							[13] PartyInformation OPTIONAL,

old-TEL-URI							[14] PartyInformation OPTIONAL

}

Current-Previous-Systems ::= SEQUENCE

{

serving-System-Identifier			[1] OCTET STRING OPTIONAL,

-- VPLMN id (Mobile Country Code and Mobile Network Country, E. 212 number [87]).

current-Serving-MME-Address			[2] DataNodeIdentifier OPTIONAL,

-- The IP address of the current serving MME or its the Diameter Origin-Host and Origin-Realm.

previous-Serving-System-Identifier		[3] OCTET STRING OPTIONAL,

-- VPLMN id (Mobile Country Code and Mobile Network Country, defined in E212 [87]).

previous-Serving-MME-Address			[4] DataNodeIdentifier OPTIONAL,

-- The IP address of the previous serving MME or its Diameter Origin-Host and Origin-Realm.

...

}

ProSeTargetType ::= ENUMERATED

{

pRoSeRemoteUE		(1),

pRoSeUEtoNwRelay	(2),

...

}

VoIPRoamingIndication ::= ENUMERATED {

roamingLBO			(1), -- used in IMS events sent by VPLMN with LBO as roaming

roamingS8HR			(2), -- used in IMS events sent by VPLMN with S8HR as roaming

...

}

DeregistrationReason ::= CHOICE

{

reason-CodeAVP			[1] INTEGER,

server-AssignmentType	[2] INTEGER,

-- Coded according to 3GPP TS 29.229 [96]

...

}

PTCEncryptionInfo ::= SEQUENCE {

cipher					[1]	UTF8String,

cryptoContext			[2] UTF8String OPTIONAL,

key						[3] UTF8String,

keyEncoding				[4] UTF8String,

salt					[5] UTF8String OPTIONAL,

pTCOther				[6] UTF8String OPTIONAL,

...

}

PTC ::= SEQUENCE {

abandonCause				[1] UTF8String,

accessPolicyFailure		[2] UTF8String	OPTIONAL,

accessPolicyType			[3] AccessPolicyType,

alertIndicator				[5] AlertIndicator,

associatePresenceStatus	[6] AssociatePresenceStatus,

bearer-capability			[7] UTF8String OPTIONAL,

-- identifies the Bearer capability information element (value part)

broadcastIndicator			[8] BOOLEAN	OPTIONAL,

-- default False, true indicates this is a braodcast to a group

contactID  					[9] UTF8String,

emergency 					[10] Emergency OPTIONAL,

emergencyGroupState		[11] EmergencyGroupState OPTIONAL,

timeStamp					[12] TimeStamp,

pTCType						[13] PTCType OPTIONAL,

failureCode					[14] UTF8String	OPTIONAL,

floorActivity				[15] FloorActivity OPTIONAL,

floorSpeakerID				[16] PTCAddress,

groupAdSender				[17] UTF8String,

-- Identifies the group administrator who was the originator of the group call.

groupID						[18] UTF8String,

groupAuthRule				[19] GroupAuthRule OPTIONAL,

groupCharacteristics		[20] UTF8String,

holdRetrieveInd				[21] BOOLEAN,

-- true indicates target is placed on hold, false indicates target was retrived from hold.

holdRetUser					[22] UTF8String,

-- the name of the associate who removes the target off hold.

imminentPerilInd 			[23] ImminentPerilInd OPTIONAL,

implicitFloorReq			[24] ImplicitFloorReq OPTIONAL,

initiationCause				[25] InitiationCause OPTIONAL,

invitationCause				[26] UTF8String,

iPAPartyID					[27] UTF8String,

iPADirection				[28] IPADirection OPTIONAL,

listManagementAction		[29] ListManagementAction OPTIONAL,

listManagementFailure		[30] UTF8String,

listManagementType			[31] ListManagementType OPTIONAL,

maxTBTime					[32] UTF8String,  -- defined in seconds.

mCPTTGroupID				[33] UTF8String,

mCPTTID						[34] UTF8String OPTIONAL,

mCPTTInd					[35] BOOLEAN OPTIONAL,

-- default False indicates to associate from target, true indicates to the target.

location					[36] Location OPTIONAL,

mCPTTOrganizationName		[37] UTF8String,

mediaStreamAvail 			[38] BOOLEAN,

-- True indicates available for media, false indicates not able to accept media.

priority-Level				[40] Priority-Level OPTIONAL,

preEstSessionID				[41] UTF8String,

preEstStatus				[42] PreEstStatus OPTIONAL,

pTCGroupID					[43] UTF8String,

pTCIDList					[44] UTF8String OPTIONAL,

pTCMediaCapability			[45] UTF8String OPTIONAL,

pTCOriginatingId   		[46] UTF8String OPTIONAL,

pTCOther					[47] UTF8String OPTIONAL,

pTCParticipants  			[48] UTF8String OPTIONAL,

pTCParty					[49] UTF8String,

pTCPartyDrop				[50] UTF8String,

pTCSessionInfo   			[51] UTF8String,

pTCServerURI				[52] UTF8String,

pTCUserAccessPolicy		[53] UTF8String,

pTCAddress					[54] PTCAddress OPTIONAL,

queuedFloorControl			[55] BOOLEAN OPTIONAL,

--Default FALSE,send TRUE if Queued floor control is used.

queuedPosition				[56] UTF8String OPTIONAL,

-- indicates the queued position of the Speaker (Target or associate) who has the

-- right to speak.

registrationRequest		[57] RegistrationRequest OPTIONAL,

registrationOutcome		[58] RegistrationOutcome OPTIONAL,

retrieveID					[59] UTF8String,

rTPSetting					[60] RTPSetting OPTIONAL,

talkBurstPriority			[61] Priority-Level	OPTIONAL,

talkBurstReason				[62] Talk-burst-reason-code OPTIONAL,

-- Talk-burst-reason-code Defined according to the rules and procedures

-- in (OMA-PoC-AD [97])

talkburstControlSetting	[63] TalkburstControlSetting OPTIONAL,

targetPresenceStatus		[64] UTF8String,

port-Number					[65] INTEGER (0..65535),

...

}

AccessPolicyType  ::= SEQUENCE

{

userAccessPolicyAttempt 			[1] BOOLEAN,

-- default False, true indicates Target has accessed.

groupAuthorizationRulesAttempt		[2] BOOLEAN,

-- default False, true indicates Target has accessed.

userAccessPolicyQuery				[3] BOOLEAN,

-- default False, true indicates Target has accessed.

groupAuthorizationRulesQuery		[4] BOOLEAN,

-- default False, true indicates Target has accessed.

userAccessPolicyResult				[5] UTF8String,

groupAuthorizationRulesResult		[6] UTF8String,

...

}

AlertIndicator ::= ENUMERATED

{

-- indicates the group call alert condition.

sent	(1),

received	(2),

cancelled	(3),

...

}

AssociatePresenceStatus ::=	SEQUENCE

{

presenceID		[1] UTF8String,

-- identity of PTC Client(s)or the PTC group

presenceType	[2] PresenceType,

presenceStatus	[3] BOOLEAN,

-- default false, true indicates connected.

...

}

PresenceType	::= ENUMERATED

{

pTCClient		(1),

pTCGroup		(2),

-- identifies the type of presenceID given [PTC Client(s) or PTC group].

...

}

Emergency 		::= ENUMERATED

{

-- MCPTT services indication of peril condition.

imminent  (1),

peril	  (2),

cancel	  (3),

...

}

EmergencyGroupState		::= SEQUENCE

{

-- indicates the state of the call, at least one of these information

-- elements shall be present.

clientEmergencyState [1] ENUMERATED

{

-- in case of MCPTT call, indicates the response for the client

inform			 (1),

response		 (2),

cancelInform 	 (3),

cancelResponse	 (4),

...

} OPTIONAL,

groupEmergencyState	 [2] ENUMERATED

{

-- in case of MCPTT group call, indicates if there is a group emergency or

-- a response from the Target to indicate current Client state of emergency.

inForm			(1),

reSponse		(2),

cancelInform 	(3),

cancelResponse	(4),

...

},

...

}

PTCType					::= ENUMERATED

{

pTCStartofInterception  					(1),

pTCServinSystem								(2),

pTCSessionInitiation						(3),

pTCSessionAbandonEndRecord					(4),

pTCSessionStartContinueRecord				(5),

pTCSessionEndRecord 						(6),

pTCPre-EstablishedSessionSessionRecord		(7),

pTCInstantPersonalAlert						(8),

pTCPartyJoin								(9),

pTCPartyDrop								(10),

pTCPartyHold-RetrieveRecord					(11),

pTCMediaModification						(12),

pTCGroupAdvertizement						(13),

pTCFloorConttrol							(14),

pTCTargetPressence							(15),

pTCAssociatePressence						(16),

pTCListManagementEvents						(17),

pTCAccessPolicyEvents						(18),

pTCMediaTypeNotification					(19),

pTCGroupCallRequest							(20),

pTCGroupCallCancel							(21),

pTCGroupCallResponse						(22),

pTCGroupCallInterrogate						(23),

pTCMCPTTImminentGroupCall					(24),

pTCCC										(25),

...

}

FloorActivity	::= SEQUENCE

{

tBCP-Request	[1] BOOLEAN,

-- default False, true indicates Granted.

tBCP-Granted	[2] BOOLEAN,

-- default False, true indicates Granted permission to talk.

tBCP-Deny		[3] BOOLEAN,

-- default True, False indicates permission granted.

tBCP-Queued 	[4] BOOLEAN,

-- default False, true indicates the request to talk is in queue.

tBCP-Release	[5] BOOLEAN,

-- default True, true indicates the Request to talk is completed,

-- False indicates PTC Client has the request to talk.

tBCP-Revoke		[6] BOOLEAN,

-- default False, true indicates the privilege to talk is canceld from the

-- PTC server.

tBCP-Taken		[7] BOOLEAN,

-- default True, false indicates another PTC Client has the permission to talk.

tBCP-Idle		[8]	BOOLEAN,

-- default True, False indicates the Talk Burst Protocol is taken.

...

}

GroupAuthRule ::= ENUMERATED

{

allow-Initiating-PtcSession			(0),

block-Initiating-PtcSession			(1),

allow-Joining-PtcSession				(2),

block-Joining-PtcSession				(3),

allow-Add-Participants					(4),

block-Add-Participants					(5),

allow-Subscription-PtcSession-State	(6),

block-Subscription-PtcSession-State	(7),

allow-Anonymity							(8),

forbid-Anonymity						(9),

...

}

ImminentPerilInd 		::= ENUMERATED

{

request			(1),

response		(2),

cancel			(3),

-- when the MCPTT Imminent Peril Group Call Request, Response or Cancel is detected

...

}

ImplicitFloorReq		::= ENUMERATED

{

join			(1),

rejoin			(2),

release			(3),

-- group Call request to join, rejoin, or release of the group call

...

}

InitiationCause			::= ENUMERATED

{

requests				(1),

received				(2),

pTCOriginatingId		(3),

-- requests or receives a session initiation from the network or another

-- party to initiate a PTC session. Identify the originating PTC party, if known.

...

}

IPADirection			::= ENUMERATED

{

toTarget		(0),

fromTarget		(1),

...

}

ListManagementAction	::= ENUMERATED

{

create 		(1),

modify 		(2),

retrieve 	(3),

delete 		(4),

notify 		(5),

...

}

ListManagementType		::= ENUMERATED

{

contactListManagementAttempt	(1),

groupListManagementAttempt		(2),

contactListManagementResult	(3),

groupListManagementResult		(4),

requestSuccessful				(5),

...

}

Priority-Level			::= ENUMERATED

{

pre-emptive			(0),

high-priority		(1),

normal-priority		(2),

listen-only			(3),

...

}

PreEstStatus			::= ENUMERATED

{

established		(1),

modify			(2),

released		(3),

...

}

PTCAddress				::= SEQUENCE

{

uri					[0] UTF8String,

-- The set of URIs defined in [RFC3261] and related SIP RFCs.

privacy-setting		[1] BOOLEAN,

-- Default FALSE, send TRUE if privacy is used.

privacy-alias		[2] VisibleString	OPTIONAL,

-- if privacy is used, the PTC Server creates an anonymous PTC Address of the form

-- <sip:anonymous@anonymous.invalid>. In addition to  anonymity, the anonymous PTC

-- Addresses SHALL be unique within a PTC Session. In case more than one anonymous

-- PTC Addresses are used in the same PTC Session, for the second Anonymous PTC

-- Session and thereafter, the PTC Server SHOULD use the form

-- sip:anonymous-n@anonymous.invalid where n is an integer number.

nickname 			[3] UTF8String 	OPTIONAL,

...

}

RegistrationRequest		::= ENUMERATED

{

register		(1),

re-register		(2),

de-register		(3),

...

}

RegistrationOutcome		::= ENUMERATED

{

success		(0),

failure		(1),

...

}

RTPSetting				::= SEQUENCE

{

ip-address  [0] IPAddress,

port-number [1] Port-Number,

-- the IP address and port number at the PTC Server for the RTP Session

...

}

Port-Number ::= INTEGER (0..65535)

TalkburstControlSetting	::= SEQUENCE

{

talk-BurstControlProtocol	[1] UTF8String,

talk-Burst-parameters		[2] SET OF VisibleString,

-- selected by the PTC Server from those contained in the original SDP offer in the

-- incoming SIP INVITE request from the PTC Client

tBCP-PortNumber 			[3] INTEGER (0..65535),

-- PTC Server's port number to be used for the Talk Burst Control Protocol

...

}

Talk-burst-reason-code ::= VisibleString

END -- OF EpsHI2Operations

<<<<<<<<<<<<<<<<<<<<<<<<<< END OF CHANGE 5 >>>>>>>>>>>>>>>>>>>>>>>>>>>>