3GPP TSG-SA3 Meeting #80-LI-e-b	s3i210104

# Online, 02 – 04 March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0161 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of FiveGGUTI ASN1 in LI_XER |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (NTAC) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI16 |  |  |  |  |  | Date: |  |  | 2021-02-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The ASN.1 definition of the FiveGGUTI field in LI_XER (and OCTET STRING of length 14) is inconsistent with the definition given in Table 6.2.2A-1 and -2 which says that the first four octets of the IE defined in TS 24.501 figure 9.11.3.4.1 should be omitted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The length of the ASN.1 definition is corrected to 10 octets. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is inconsistent and cannot be implemented reliably. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | See contribution s3i210105 for supporting discussion. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

Annex F (normative): ASN.1 schema for LI_XER messages

TS33128IdentityAssociation

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulIntercept(2) threeGPP(4) tS33128IdentityAssociation(20) r16(16) version1(1)}

DEFINITIONS IMPLICIT TAGS EXTENSIBILITY IMPLIED ::=

BEGIN

tS33128IdentityAssociationOID RELATIVE-OID ::= {threeGPP(4) tS33128IdentityAssociation(20) r16(16) version1(1)}

iEFRecordOID RELATIVE-OID ::= {tS33128IdentityAssociationOID iEF(1)}

IEFMessage ::= SEQUENCE

{

iEFRecordOID        [1] RELATIVE-OID,

record              [2] IEFRecord

}

IEFRecord ::= CHOICE

{

associationRecord   [1] IEFAssociationRecord,

deassociationRecord [2] IEFDeassociationRecord,

keepalive           [3] IEFKeepaliveMessage,

keepaliveResponse   [4] IEFKeepaliveMessage

}

IEFAssociationRecord ::= SEQUENCE

{

sUPI            [1] SUPI,

fiveGGUTI       [2] FiveGGUTI,

timestamp       [3] GeneralizedTime,

tAI             [4] TAI,

nCGI            [5] NCGI,

nCGITime        [6] GeneralizedTime,

sUCI            [7] SUCI OPTIONAL,

pEI             [8] PEI OPTIONAL,

fiveGSTAIList   [9] FiveGSTAIList OPTIONAL

}

IEFDeassociationRecord ::= SEQUENCE

{

sUPI            [1] SUPI,

fiveGGUTI       [2] FiveGGUTI,

timestamp       [3] GeneralizedTime,

nCGI            [4] NCGI,

nCGITime        [5] GeneralizedTime

}

IEFKeepaliveMessage ::= SEQUENCE

{

sequenceNumber  [1] INTEGER

}

FiveGGUTI ::= OCTET STRING (SIZE(10))

NCGI ::= SEQUENCE

{

pLMNID [1] PLMNID,

nCI [2] NCI

}

PLMNID ::= OCTET STRING (SIZE(3))

NCI ::= BIT STRING (SIZE(36))

TAI ::= OCTET STRING (SIZE(6))

SUPI ::= CHOICE

{

iMSI        [1] IMSI,

nAI         [2] NAI

}

IMSI ::= NumericString (SIZE(6..15))

NAI ::= UTF8String

FiveGSTAIList ::= SEQUENCE OF TAI

PEI ::= CHOICE

{

iMEI        [1] IMEI,

iMEISV      [2] IMEISV,

mACAddress  [3] MACAddress,

eUI64       [4] EUI64
}

IMEI ::= NumericString (SIZE(14))

IMEISV ::= NumericString (SIZE(16))

MACAddress ::= OCTET STRING (SIZE(6))

EUI64 ::= OCTET STRING (SIZE(8))

SUCI ::= OCTET STRING (SIZE(8..3008))

END

END OF CHANGES