3GPP TSG-SA3 Meeting #80-LI-e-b	s3i210122

# Online, , 2nd Mar 2021 - 4th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0107 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on ID Mapping Location Delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3 LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI17 |  |  |  |  |  | Date: |  |  | 2021-03-04 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently, the capability to perform ID mapping from permanent to temporary identifiers is not able to support location information delivery with the identifier query return. This CR adds location information delivery capability to the procedure, enabling location return on a permanent to temporary identifier query. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1st, addition of location information to clause 5.4.14, 5.4.15, 5.4.16, 2nd , addtion of location capability to ICF response. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LEAs will not be able to derive location information in permanent to temporary identifier association request. Carriers may not be able to fully fulfill their LI obligations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.14, 5.4.15, 5.4.16, 7.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3i210106 |  |  |  |  |  |  |  |  |

*****Start of First Change*****

### 5.4.14	Interface LI_XQR

The LI_XQR interface is used by the IQF to send identifier association queries to the ICF and from the ICF to return identities associations to the IQF in response.

The following are examples of some of the information that may be passed over LI_XQR from the IQF to the ICF:

-	Information relating to the type of query.

-	Temporary or permanent identifier provided by the LEA.

-	Other information associated with identifier required for localisation provided by the LEA.

-	Cell identity.

-	Tracking area identifier.

-	Time that identifier provided by the LEA was observed by the LEA.

-	Location information request from the LEA for permanent to temporary identifier association.

The following are examples of some of the information that may be passed over LI_XQR from the ICF to the IQF:

-	Information relating to the type of query being responded to.

-	Temporary and permanent identifiers corresponding to identifier provided by LEA.

-	Identifier association validity start and end times.

-	Location information.

NOTE:	The location information returned in the IQF response is the information associated at the time of the specific identifier association caching at the ICF.

### 5.4.15	Interface LI_HIQR

The LI_HIQR interface is used by the LEA to send identifier association queries to the IQF and from the IQF to return identities associations to the LEA in response.

The following are examples of some of the information that may be passed over LI_HIQR from LEA to the IQF:

-	Information relating to the type of query.

- 	Warrant/authorisation identifier.

-	Temporary or permanent identifier provided by the LEA.

-	Other information associated with identifier required for localisation provided by LEA.

-	Cell identity.

-	Tracking area identifier.

-	Time that identifier provided by LEA was observed by the LEA.

-	Location information request for permanent to temporary identifier association.

The following are examples of some of the information that may be passed over LI_HIQR from IQF to the LEA:

-	Information relating to the type of query being responded to.

-	Warrant/authorisation identifier.

-	Temporary and permanent identifiers corresponding to identifier provided by LEA.

-	Identifier association validity start and end times.

-	Location information.

### 5.4.16	Interface LI_XER

The LI_XER interface is used by the IEF to send identifier association events to the ICF.

The following are examples of some of the information that may be passed over LI_XER from the IEF to the ICF:

-	Permanent identifier and temporary identifier association.

-	Permanent identifier and temporary identifier excommunication / de-association.

-	Time stamp of association observation.

-	Location information.

*****Start of Second Change*****

### 7.7.3	ICF Response parameters

The list of event parameters is specified in TS 33.128 [15]. Each event shall include at the minimum the following information:

-	Subscription permanent identifier.

-	Related temporary identifier(s).

-	Start of validity timestamp(s).

-	End of validity timestamp(s).

The following additional information shall be included if it was available in the IEF records provided to the ICF:

-	Permanent equipment identifier.

The following additional information shall be included when available and if requested in the IQF to ICF query:

-	Location information (i.e. Cell identity and tracking area identifier).