3GPP SA3#81-LI-e-a	S3i210257

# eMeeting, 12-16 April 2021

Source:	NDRE, Security Service

Title:	VPLMN LI for Roaming Scenarios with Encryption: Introduction

Document for:	Discussion

Agenda Item:	7.2

Work Item / Release:	LI17

Abstract of the contribution:

At SA3LI#73, a CR to TS 33.126 (agreed as [11]) added the requirement that

"[when roaming]…the home CSP shall provide to the visited CSP, the means to decrypt user services which are encrypted between the ME and an entity outside the visited CSP and using cryptographic parameters established in the home CSP."

3GPP SA3 has defined (or is about to define) several services which are incompatible with the above requirement in roaming situations (some may even exhibit difficulty for LI also in the non-roaming case).

No details on a possible Stage-2 implementation of the above requirement have yet been discussed in SA3LI. The present document is a first introductory document in a suite of five contributions, initiating a discussion on different components necessary for defining a Stage-2 implementation of the above requirement.

Two main types of LI-approaches can be envisioned. The first approach is largely transparent to the non-LI domain of the network but requires explicit assistance from the HPLMN, as well as a somewhat complex technical solution in the LI domain of the VPLMN. The second approach is based on adaptation of the current network security architectures through introduction of an explicit middlebox in the UP path in the VPLMN. This second approach has several advantages in terms of simplicity, robustness, and visibility to the ME/subscriber.

1	Background

1.1	Example use-cases

There exist some standards developed (or in development) by 3GPP that causes conflicts between the need for security and regulatory LI compliance, in particular for roaming situations.

EXAMPLE 1: The battery efficiency improvements of BEST [4] may not generally be utilized in roaming since the specified security solutions prevents Lawful Intercept in the VPLMN. As stated in [4], Annex A: "Due to regulatory requirements, operators may have to disable the BEST service for UEs roaming in their network".

EXAMPLE 2: A similar situation exists for S8HR solution [3] where SIP-signalling and media encryption currently cannot be enabled. From [3], clause 2.4.3: 

"The HPMN must ensure based on the roaming agreement that IMS layer signalling and media confidentiality protection is not activated in order to enable the VPMN to meet the local regulatory requirements". 

The 5G-equivalent of S8HR is the N9HR solution [12] whose usage has similar caveats.

EXAMPLE 3: The Generic Authentication/Bootstrapping Architecture (GAA/GBA) defined in [5] lacks a solution for LI in roaming situations. Extract from [8], clause 14.2: 

"NOTE 1: The details of LI capabilities for GBA in a roaming scenario is for further study".

EXAMPLE 4: SA3 is also defining a new, 5G-specific counterpart of GAA/GBA, called AKMA [7]. The potential forthcoming regulatory compliance aspects are captured in the pre-study work: 

"AKMA service shall be made such that it can comply with rules and regulations of the serving network; AKMA service shall be able to signal if services are not available under the regulations of the serving network", 

see [6], clause 4.10.3, but are not addressed in the normative TS [7].

Regarding Example 4, SA3LI has previously sent an LS to SA3 on this topic [13], to which SA3 responded in [14] that "roaming aspects are not considered" and that the service is currently "HPLMN limited". There is however no normative text in [7] supporting this. In principle, AKMA, as specified, appears to only require the ME to have IP-connectivity in order to be used; there is no "blocking mechanism" defined for the roaming case.

It is not clear if there today exist widespread deployments of BEST and GBA. A problem is however that these solutions, if/when starting to be deployed in some market, can make LI difficult in a jurisdiction outside that market, when subscribers/devices from that market roams into a foreign network.

It needs to be noted that legal requirements may vary across different jurisdictions. The present document assumes there are legal and/or contractual agreements in place such that services added by the HPLMN (or VPLMN) is required to not interfere with law enforcement operations in the jurisdiction of the VPLMN. Further, it is assumed that adding encryption where it currently isn’t present is desirable.

1.2	A note on the non-roaming case

While the focus here lies on roaming situations, LI-obligations obvsiously also apply to the non-roaming case. Note that for BEST, GAA/GBA and AKMA, use of the defined encryption solutions could in all these cases create an encrypted tunnel between the ME and a point outside the HPLMN (an enterprise or some over-the-top service) even in a non-roaming case. As will be apparent from the analysis in the companion contributions [15-17], LI in the non-roaming case could therefore still be non-trivial to implement. In short, this holds because decryption of the service requires the PLMN to have knowledge about "security state information", which, without new explicit functionality, would not be available to the PLMN even in non-roaming situations.

1.3	Rationale and scope

The rationale behind the aforementioned CR, [11] is to traverse conflicts of interest: to generally ensure HPLMN-enabled encryption can be activated in roaming scenarios, while still having compatibility with regulatory LI requirements of the VPLMN. The new requirement added to [10] via the CR [11] is reproduced here for the purpose of self-containment:

R6.4 – 175  	CSP provided cryptographic parameters in roaming – When a home CSP’s subscriber is roaming, independently of whether or not the subscriber is an LI Target in the VPLMN, the home CSP shall provide to the visited CSP the means to decrypt user services which are encrypted between the ME and an entity outside the visited CSP and using cryptographic parameters established in the home CSP.

It may be of interest to note that the above requirement does not apply to keys for non-encryption services such as authentication, nor to encryption services terminating inside the USIM/eSIM, e.g. SIM-OTA, potential USIM-based e-payment services, etc.

Other requirements from [10] that are relevant include: R - 6.3.20, 6.3.110, 6.3.130, 6.3.420, 6.4.160, 6.4.170.

The term HPLMN-enabled encryption is in the sequel (and in the companion contributions [15-18]) used to denote traffic characterized according to the conditions of R6.4 – 175, i.e. pertaining to

- user services,

- encrypted between the ME and an entity outside the visited CSP, and

- using cryptographic parameters established in the home CSP,

which would apply to all of the Examples 1-4 above (if encryption was/is enabled).

1.4	Requirements outside 3GPP

SA3LI is not the only source of requirements such as R6.4 – 175 of [11].

The European Commission recently released the 2021 version of the “Rolling plan for Standardization”, [1]. This includes the following text:

“…challenges have been identified so far that could benefit from an appropriate standardisation activity, namely:

   ...

   Being able to benefit from a clear copy of intercepted communications (only 5G native encryption is concerned, OTTs encryption is out of scope), even if the target is an inbound roamer (encrypted S8HR and N9HR roaming agreement) or a user of encrypted new mobile messaging services such RCS.”

Further, the document ENFOPOL 55 [2] states:

“If network operators/service providers initiate encoding, compression or encryption of telecommunications traffic, law enforcement agencies require the network operators/service providers to provide intercepted communications en clair."

2	Contribution overview

As mentioned, the present document is the first in a suite of five contributions, initiating a discussion on different components necessary for defining a Stage-2 implementation of requirement R6.4-175 of [10], the contributions being structured as follow:

- s3i210257: introduction and background (the present document).

- s3i210258 [15]: problem statement, e.g. identification of architectural- and soultion components. Here, a common architecture is identified for the services [3,4,5,7] in a roaming case (with encryption enabled). Three main parts of the problem statement are identified:

  - the problem of detecting (in the VPLMN) encrypted service usage, (this discussion is self-contained in [15]),

  - the problem of retrieving (by the VPLMN) the necessary decryption key, and,

  - the problem of retrieving other parameters necessary for traffic decryption in the VPLMN.

- s3i210259 [16]: key retrieval, discusses how to obtain necessary decryption key(s) to the VPLMN, i.e. potential solutions to 2b.

- s3i210260 [17]: auxiliary security parameter retrieval, discusses how to obtain other parameters necessary for decrypting traffic associated with an LI target in the VPLMN, i.e. potential solutions 
to 2c.

- s3i210261 [18] provides a summary. Additionally, s3i210262 [19] contains companion presentation material in a (perhaps) more accessible style.

Two main solution approaches are identified in [15-17]:

- a transparent approach in which only minimal additions to the non-LI part of the existing Rel-16 architecture are needed. The LI functions of the VPLMN here operate as an add-on in way resembling the current LMISF/BBIFF used in (unencrypted) S8HR/N9HR.

- an explicit middlebox approach in which the VPLMN takes a more active part in security signalling in the UP between ME and the (encrypted) service. This requires adaptation of new/additional security protocols as part of [3,4,5,7], but as a pay-off shows advantages in terms of greater simplicity, robustness, and end-user visibility.

Due to non-detectability and mid-session LI requirements, in both (i) and (ii), the VPLMN engages in exactly the same way for all subscribers, regardless of whether they are known LI targets or not.

Readers who are not interested in technical details (and readers who are impatient) may skip directly to the summary of [18,19].

4	References

[1] EU Rolling plan for standardization, https://joinup.ec.europa.eu/collection/rolling-plan-ict-standardisation/5g

[2] ENFOPOL 55.

[3] GSMA IR.65, "IMS Roaming, Interconnection and Interworking Guidelines".

[4] TS 33.163, "Battery Efficient Security for very low Throughput Machine Type Communication (MTC) devices (BEST)".

[5] TS 33.220, "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[6] TR 33.835, "Study on authentication and key management for applications; based on 3GPP credential in 5G".

[7] TS 33.535, "Authentication and key management for applications based on 3GPP credential in 5G".

[8] TS 33.107, "3G security; Lawful interception architecture and functions".

[10] TS 33.126, "Lawful Interception requirements", Rel-16.

[11] s3i190258, CR to 33.126, "CSP provided keys in roaming".

[12] S3-193957, "N9HR Regulatory Obligations".

[13] S3-202848 / S3i200477, "Regulatory LI compliance of AKMA"

[14] S3-202950, "Reply LS on Regulatory LI compliance of AKMA"

[15] s3i210258, "VPLMN LI for Roaming Scenarios with Encryption: Problem Statement"

[16] s3i210259, "VPLMN LI for Roaming Scenarios with Encryption: Key Retrieval"

[17] s3i210260, "VPLMN LI for Roaming Scenarios with Encryption: Auxilliary Parameter Retrieval"

[18] s3i210261, "VPLMN LI for Roaming Scenarios with Encryption: Summary and Architecture Options"

[19] s3i210262, “VPLMN LI for Roaming Scenarios with Encryption: Overview Presentation Material”