3GPP SA3#81-LI-e-a	S3i210259

# eMeeting, 12-16 April 2021

Source:	NDRE, Security Service

Title:	VPLMN LI for Roaming Scenarios with Encryption: Key Retrieval

Document for:	Discussion

Agenda Item:	x.y

Work Item / Release:	LI17

Abstract of the contribution:

At SA3LI#73, a CR to TS 33.126 (agreed as [10]) added the requirement that

"[when roaming]…the home CSP shall provide to the visited CSP, the means to decrypt user services which are encrypted between the ME and an entity outside the visited CSP and using cryptographic parameters established in the home CSP."

The setting in which this requirement is defined is denoted "HPLMN-enabled encryption". No details on a possible Stage-2 implementation of the above requirement have yet been discussed in SA3LI.

The present document is the third in a suite of five contributions, initiating a discussion on different components necessary for defining a Stage-2 implementation. Specifically, the present document identifies possible approaches to the key retrieval problem as defined in the accompanying contribution [11].

1	Background

In [11], the security architectures for BEST [1], GAA/GBA [2,3,4], AKMA [5] and S8HR/N9HR (with encryption enabled according to [8,9] for CP and UP) are compared from the viewpoint for providing VPLMN-LI for HPLMN-enabled encrypted serviced, i.e. where the encryption/decryption keys are by default not known to the VPLMN. The present document discusses options for enabling the VPLMN to retrieve a decryption key, suitable for VPLMN LI.

Also in [11], the following general framework, common to [1-5,8-9]  is identified

Figure 1: Generic architecture for HPLMN-enabled encryption (without LI support added).

In the figure, encryption is as defined in [11] assumed provided between the Security Terminating Function (STF) and the ME, based on a service specific key SK, derived from a master key MK by a Key Server Function (KSF).

For LI purposes, a key retrieval problem arises, where a key KLI needs to be retrieved from the HPLMN to VPLMN, enabling VPLMN decryption of traffic associated with targeted subscribers. The present document analyses this key retrieval problem in more depth.

The key KLI could be identical to SK, or it could be a key derived from SK, as long as knowledge of KLI enables decryption. Due to the common requirements of

- undetectability of LI activation across PLMNs, and

- enabling activation of mid-session intercept in the VPLMN,

it can immediately be noted that the key KLI needs to be available to the VPLMN for all inbound roamers in the VPLMN, regardless of whether (at the time of initial attach) the subscriber is known to be (or become) an LI target.

The table below summarizes the analysis done in [11] in mapping the network functions (KSF and STF) to a network function of the respective architecture.

| Functionality | BEST [1] | GAA/GBA [2,3,4] | AKMA [5] | S8HR/N9HR (with encryption [8,9]) |
| --- | --- | --- | --- | --- |
| KSF | HSE | BSF | AAnF | CP: HSS or S-CSCF UP: depends on end-to-accessedge, end-to-middle, or end-to-end encryption, may also depend on KMS (Key Mgmt Server) implementation. |
| STF | CP: HSE UP: HSE and possibly also EAS (if encryption configured as "end-to-end") | NAF | AF | CP: P-CSCF UP: see above |

Table 1: Mapping of security functionality to the respective architecture of each use case.

The next table maps the keys MK, SK and KLI to specific keys of the different architectures as analysed in [11].

| Key | BEST [1] | GAA/GBA [2,3,4] | AKMA [5] | S8HR/N9HR (with encryption [8,9]) |
| --- | --- | --- | --- | --- |
| MK | CK, IK | Ks | KAKMA | CP: (CKIM, IKIM) UP: same consideration as in Table 1 |
| SK | KE2Menc (and KEAS_PSK for end-to-end case) | KsNAF | KAF | See above. |
| KLI | KE2Menc (and KEAS_PSK for end-to-end case) | KsNAF | KAF | CP: CKESP UP: See above |

Table 2: Mapping of security keys to keys defined the respective architecture of each use case.

NOTE: Due to the hierarchical dependency between the keys MK (known to KSF)  SK (known to KSF and STF)  KLI (known to STF and possibly also KSF), the key KLI could, depending on the use case, be retrievable from STF and possibly also from KSF. As observed in [11], it is desirable to assign KLI as a key which minimizes risk of over-collection and unnecessary privacy exposure of the subscribers.

In the sequel, options for providing KLI from HPLMN to VPLMN are discussed. As discussed in [11] this can be done through a

- transparent approach: by introduction of a stand-alone, inter-PLMN key-transfer function, the key transfer becomes transparent to the rest of the network (i.e. no changes to the non-LI parts of the network are needed, beyond adding the key-transfer function), see clause 2.2, or,

- explicit approach: the (non-LI) roaming network architecture is modified by the presence of an entity in the VPLMN which takes an active role in security signalling with the ME and obtains keys automatically, through its role, see clause 2.3.

2	Key retrieval approach

2.1	Transparent approach

2.1.1	BEST, GAA/GBA and AKMA

Recall that by the analysis of [11], detection of usage of BEST or GAA/GBA, can be based on

- BEST: use of specific APN/DNN (see [1], annex A);

- GBA: an ME connection to a KSF with distinguishable server name (the so-called BSF, see [6] clause 16.2).

This is however not the case in AKMA [5], since there is no explicit signalling at all between ME and KSF (the AAnF). With this in mind, and since the transparent approach seeks to minimize changes to the existing security architecture, the only generic and feasible way to ensure that necessary keys are always retrievable (also for mid-session LI) in the VPLMN for BEST, GAA/GBA and AKMA is to place the responsibility on the HPLMN to always "push" keys, rather than having the VPLMN "pull" them.

Here, the service-based architecture of 5G seems to provide a way forward. Assuming that relevant KSF services of the HPLMN have already registered in the hNRF (not shown in the diagram below), an outline of an approach could be as follows.

- Some (to-be-determined) NF in the VPLMN discovers the KSF (there may be several) of the HPLMN, and specifically an associated key-retrieval SBI service(s), herein by example denoted Nksf_RoamingKeyMgmt. (This discovery is by [7] performed via the two NRF:s, details omitted.)

- An inbound roamer from the HPLMN registers in the VPLMN. Step 3 and 4 are performed for all such inbound roamers, regardless of whether they are, or might become, LI targets in the VPLMN. This is crucial to avoid detectability of LI and to ensure possibility of a later mid-session intercept. Details of the registration signalling is assumed well-known and is therefore omitted.

- The NF of step 1 is notified about the registration thorough some kind of indication, including the SUPI. 

NOTE 1: The SEAF might be a suitable candidate for this NF, in which case this is done over an AMF-internal interface. Alternatively, an LI-specific function could take this role, though this appears unattractive as it would imply an interface from the HPLMN into the LI-domain of the VPLMN.  (Additionally, there may be non-LI reason for having access to the key in the VPLMN, see NOTE 2.)

- The NF of the VPLMN now subscribes to Nksf_RoamingKeyMgmt for relevant key management events (e.g. for GBA, this could correspond to bootstrapping events) from the KSF, including the subscriber identity (SUPI). Again, this step is by [7] carried out via the vNRF and may include authorization of the service subscription.

NOTE 2: The keys obtained through the Nksf_RoamingKeyMgmt service need not only be useful for LI. Knowledge of these keys would also allow in-network content services in the VPLMN which would otherwise be impossible when encryption keys are not known.

- A key establishment event occurs in the KSF of the HPLMN for the subscriber.

- The KSF sends a notification, including the subscriber identity as well as the relevant KLI-key to the NF of the VPLMN. By [7], this would be done directly from the KSF.

This is conceptually outlined in the diagram below, based on the 5G subscription/notification procedures as currently defined in [7]. (While all inter-PLMN traffic below is routed via the two SEPP:s, processing and adding security to such traffic, details of this is omitted for the sake of simplicity.)

Figure 2: Possible key retrieval approach via 5G SBA.

A candidate for the VPLMN NF might as mentioned be the SEAF: the SEAF has the role of storing keys and other security parameters for subscribers and it also happens to be collocated with the AMF, which already has an IRI-POI by TS 33.127. Alternative placements are discussed in [13].

The 5G system has a security feature that can serve an important role in the above outlined procedure. In earlier systems (up to and including 4G) there was no means for the HPLMN to actually verify that the VPLMN is not fraudulent when it claims to have an inbound roamer from the HPLMN. 5G includes an enhancement to the authentication procedure, by which the HPLMN can obtain a "cryptographic proof" that the claimed subscriber indeed is accessing through the VPLMN. This reduces the risk that an untrusted VPLMN obtains keys for subscribers who are actually not present in the VPLMN.

2.1.2	S8HR/N9HR (with encryption)

In this case, the situation is in one aspect simpler, in another it is more complicated. While the request for an IMS/Vo5GS session with encrypted media plane is from [11] in principle detectable through dedicated DNN/QFIs and through inspection of explicit the SIP-signalling, there can be a "Catch-22" if the SIP-signalling is also encrypted. This is additionally problematic if one also assumes that the so called "SDES" option for media plane encryption might be used (which carries the media plane encryption key in-band in the SIP signalling). If mid-session start of intercept is to be supported under these conditions, it requires that some network function of the VPLMN is already able to routinely decrypt all SIP-signalling, also for subscribers who are (yet) not known to be LI targets, and to record the media plane keys for a potential future media plane (CC) LI-activation. While this may at first appear unattractive, it has to be noted that an ability to decrypt in the VPLMN (i.e. having access to certain keys), also for subscribers who at the time of attach are not known to be LI-targets, is an unavoidable consequence of obligation to provide mid-session intercept of encrypted communication. Furthermore, the current S8HR/N9HR LI-solution (relying on LMISF and BBIFF) would simply not work if SIP-signalling was encrypted in a way which prevents VPLMN inspection.

In any case, CP (SIP) encryption keys could be pushed by the P-CSCF to some NF of the VPLMN (perhaps LMISF or BBIFF) which would allow LI of the encrypted IMS control plane, and also enable retrieval of keys and other security parameters present in the SIP-signalling. Still, the media plane encryption could also be configured in other ways (e.g. via a KMS) which would require more in-depth, case by case analysis. How to address the key retrival problem for media plane more in general is therefore omitted from the present document.

2.2	Explicit (middlebox) approach

With this approach, an explicit middlebox is introduced into each of the architectures (BEST, GBA, AKMA, S8HR/N9HR). All the key management and key exchange operations between the ME and the STF explicitly involves the middlebox in-path, i.e. the ME will authenticate the middlebox and its presence in the signalling and traffic plane and authorizes it. The middlebox obviously needs to be present for all inbound roamers, even those who are not known to be LI targets. This is necessary to avoid detectability of LI and to enable mid-session intercept. Nevertheless, the fact that the middlebox presents itself explicitly to the ME adds an important visibility aspect toward the ME and the subscriber. Further, the middlebox could then also perform other value added services to the subscribers (antivirus, parental control, etc). The big advantage is however clearly the great simplification it provides in terms of a simpler, more robust approach to the key retrieval problem. In terms of defining a suitable middlebox protocol, it is noted ETSI (TC Cyber) is in the process of defining a suite of such protocols for different purposes.

Figure 3: Possible key retrieval approach via an explicit middlebox. The middlebox authenticates to the ME and participates in key establishment between ME and STF, thereby automatically ensuring it has access to keys.

3	Summary

Two solutions to the key retrieval problem supporting VPLMN LI for HPLMN-enabled encryption has been presented for BEST, GAA/GBA and AKMA, based on (i) the 5G SBA, or, (ii) an explicit middlebox. (A middlebox might be useful also as a complement in the 5G SBA approach). While details are omitted, a similar approach is potentially viable also for S8HR/N9HR (with encryption), though the location of KSF will be different for CP and UP. Further, the location of the KSF for UP is also dependent on the chosen encryption approach (end-to-accessedge, end-to-middle or end-to-end).

An explicit middlebox appears to provide several advantages in terms of simplicity, robustness and visibility to the ME/subscriber, in comparison to the transparent, SBA approach.

Additional details (auxiliary security parameter retrieval) are discussed in the accompanying contribution [12]. An overall LI-architecture outline and topics for further study is provided in [13].

While privacy minded individuals and organizations might find the idea of sharing keys across VPLMNs somewhat "hard to digest" it can be observed that such key sharing has occurred since the dawn of 2G, every time a subscriber of a HPLMN attaches via a foreign VPLMN (these keys are provided as part of authentication vectors, and are used to encrypt over the radio access network). Further, since LI obligation remains a fact, the only other viable option is to not enable encryption at all, which is a strictly worse solution.

4	References

[1] TS 33.163, "Battery Efficient Security for very low Throughput Machine Type Communication (MTC) devices (BEST)".

[2] TS 33.220, "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[3] TS 33.221, "Generic Authentication Architecture (GAA); Support for subscriber certificates"

[4] TS 33.222, "Generic Authentication Architecture (GAA); Access to network application functions using Hypertext Transfer Protocol over Transport Layer Security (HTTPS)"

[5] TS 33.535, "Authentication and key management for applications based on 3GPP credential in 5G".

[6] TS 23.003, "Numbering, addressing and identification".

[7] TS 29.510, "5G System; Network Function Repository Services".

[8] TS 33.203, "Access security for IP-based services".

[9] TS 33.828, "IP Multimedia Subsystem (IMS) media plane security".

[10] s3i190258, CR to 33.126, "CSP provided keys in roaming", NDRE.

[11] s3i210258, "VPLMN LI for Roaming Scenarios with Encryption: Problem Statement"

[12] s3i210260, "VPLMN LI for Roaming Scenarios with Encryption: Auxilliary Parameter Retrieval"

[13] s3i210261, "VPLMN LI for Roaming Scenarios with Encryption: Summary and Architecture Options"