3GPP TSG-SA3 Meeting #83-LI-e-b	s3i210838r1

- Online, , 1st Nov 2021 - 5th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.126 | CR | 0020 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Serving CSP change / inbound |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI(OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI17 |  |  |  |  |  | Date: |  |  | 2021-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Missing requirement |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds missing requirement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement would continue to be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3 Detect and Capture |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | s3i210838 |  |  |  |  |  |  |  |  |

## 6.3	Detect and Capture

R6.3 - 10	Access Level Interception - The CSP shall be able to perform network access level interception in both the core and on the edge of the network (e.g. IP-CAN level interception).

R6.3 - 20	Service Level Interception - The CSP shall be able to perform service level interception in both the core and on the edge of the network (e.g. IMS based VoIP).

R6.3 - 30	Multi Party Service Interception - CSP shall be able to report the multi-party service Interception Product of targeted group communications and its users.

R6.3 - 40	Third Party Assisted Services - If a CSP uses Third Parties as part of its service provision, the CSP shall be responsible for ensuring that the overall service complies with applicable LI regulations and requirements.

R6.3 - 50	Third Party ME or UE Interception - To the extent that a CSP manages a Third Party ME or UE, the CSP shall be able to report communications of such Third Party ME or UE (e.g. status of devices with a relay or forward function).

R6.3 - 60	Third Party ME or UE Users Interception - To the extent that a CSP manages Third Party ME or UE, the CSP shall be able to report communications of the end users connected to the CSP network via a Third Party ME or UE that is managed by the CSP (e.g. status of users communicating via ME or UE with a relay or forward function).

R6.3 - 70	Modification of services – Any change to any target service settings, as known to the CSP, shall be able to be reported.

R6.3 - 80	Multiple Services Per target - The CSP shall be able to simultaneously perform LI for multiple services for a given target.

R6.3 - 90	Multiple Targets - The CSP shall be able to simultaneously perform intercepts on multiple independent targets.

R6.3 - 100	Multiple LEAs - The CSP shall be able to simultaneously perform independent intercepts for any given target under different warrants.

R6.3 - 110	Roaming Targets - The visited CSP shall be able to perform interception of inbound roaming targets.

R6.3 - 120	Roaming – Outbound - The CSP shall be able to notify the LEA whenever the CSP becomes aware that the target has left, or entered, a visited network.

R6.3 - 130	Roaming – Inbound - The CSP shall be able to notify the LEA whenever the CSP becomes aware that the inbound roaming target has entered, or has left, the network.

R6.3 - 140	Serving CSP change / outbound - When the target changes serving CSP, the CSP that is served the warrant shall be able to provide the LEA with the identity of the new CSP if known.

R6.3 - 141	Serving CSP change / returning to HPLMN - When the target returns to the HPLMN, the home CSP that is served the warrant shall be able to provide the LEA with the identity of the CSP from which the target has returned if known.

R6.3 - 150	Roaming Identifiers Visited CSP - The visited CSP shall be able to obtain and validate the long term 3GPP identifiers of all inbound roamers from the home CSP regardless of the use of privacy mechanisms (based on roaming agreements).

R6.3 - 160	Roaming Identifiers Home CSP - The home CSP shall provide the long term 3GPP identifiers to the visited CSP for outbound roamers (based on roaming agreements).

R6.3 - 170	Outbound Roaming Home Network - CSPs shall be able to intercept its outbound roamers, if the communication pass through the home CSP's network.

R6.3 – 180	Access Network Identity - The CSP shall provide the LEA the identity of the 3GPP or non 3GPP Access Network as known by the CSP.

R6.3 - 190	Location - The CSP shall be able to obtain and report the location of the target.

R6.3 - 200	Location Triggers - The CSP shall be able to obtain and report the target location at certain network events associated with the target.

R6.3 - 210	Communication Location Reporting - The CSP shall be able to obtain and report the target location at start and end of communication, as well as during the communication including periodically and per event.

R6.3 - 220	Location Reporting - The CSP shall be able to obtain and report the target location for both active and idle MEs or UEs triggered either by UE-Action (e.g. UE cell site change) or on a periodic basis or on demand by the LEA.

R6.3 - 230	Location Reporting Independency - Location information may be reported as part of interception of a service (e.g. VoLTE, RCS), or independently.

R6.3 - 240	Location Accuracy - The CSP shall report the most accurate target location available to the CSP.

R6.3 - 245	Radiolocation Assistance - The CSP shall be able to provide information to assist the LEA to perform radiolocation of target UEs.

R6.3 - 250	Multiple Location Sources - The CSP shall be able to report the source of each location information report provided to the LEMF (e.g. cell site identifier, GPS).

R6.3 - 260	Location Positioning Methods - The CSP shall be able to report the positioning method used to obtain location information (e.g. network-based, UE-based, access-based).

R6.3 - 270	Additional Location Information - If the CSP has additional location information of the target beyond cell site identifier (e.g. altitude, civic address, geo-coordinates), the CSP shall be able to provide this.

R6.3 - 280	Location Senescence - The CSP shall provide information that indicates when the location was determined (e.g. age of location, timestamp).

R6.3 - 290	Trusted/Untrusted Location - The location information reported to the LEMF shall be location information trusted by the 3GPP network (i.e. the location information is either 3GPP network derived or verified), if available. The CSP shall also be able to report target location information from untrusted sources (e.g. user provided) in addition to or in absence of the trusted location information.

R6.3 - 300	Location Trust Indication - The CSP shall be able to indicate to the LEA whether the location information is trusted or untrusted.

R6.3 - 310	Projected Location - The CSP shall be able to indicate to the LEA whether the location information of the target is measured or possible.

R6.3 - 320	Non 3GPP access - For non 3GPP access the CSP shall be able to provide the identity and location of the non 3GPP access function serving the UE as known by the CSP.

R6.3 - 330	Roaming Location - In the case of inbound roaming, the visited CSP that was served a warrant shall be able to provide location information without assistance from the home CSP.

R6.3 - 340	Location Changes in the Visited Network - In the case of roaming, the home CSP that was served a warrant shall be able to provide location information as visible in the home network.

R6.3 - 350	Location Requests - The home CSP shall be able to provide notification of target-related location information requests received from outside the home network when these requests are visible to the home network as part of normal network operations.

R6.3 – 360	LCS Use - The CSP shall be able to use LCS, if available, in support of LALS for an LCS-targetable UE (with or without target LCS subscription).

R6.3 – 370	LALS Reporting – The CSP shall be able to provide on-demand and periodic LALS reports of the target's location independent of the target's communication state.

R6.3 - 380	Up-to-date LALS location - LALS shall report either the current (updated) location, or if the current location is unavailable the last known location of a target's UE.

R6.3 - 390	LALS failure notification - If the location is unavailable, LALS shall be able to report a failure reason, as to why the location is unavailable.

R6.3 - 400	Target specificity - The CSP shall ensure no communications are intercepted other than those of, or associated with, the target's equipment, facilities or services.

R6.3 - 410	Service specificity - The CSP shall ensure that only the communication services specified by the warrant are intercepted.

R6.3 – 420	Service Scope - All CSP based services shall be in scope of LI including mission critical services and non-mission critical services.

R6.3 - 430	Service Activation - The CSP shall report service activation.

R6.3 - 440	Service Invocation - The CSP shall report service invocation.

R6.3 - 450	Service Modification - The CSP shall report service modifications (e.g., changes to content, content descriptors, timing descriptors, group participation, copy of service content).

R6.3 - 460	Service Deactivation - The CSP shall report service deactivation.

R6.3 - 470	Service Up/Download - The CSP shall report service related uploading or downloading.

R6.3 - 480	Service Access Method - The CSP shall report the access method used by the target to interact with the service (e.g., via ME, UE or web).

R6.3 - 490	Early media - The CSP shall be able to intercept early media (e.g., CAT, CRS).

R6.3 - 500	Context Comprehensibility - The CSP shall include in Interception Product information that allows the LEA to establish the Context of Communications.

R6.3 - 510	Service Indication - The CSP shall include in Interception Product an indication of the communication service as known by the CSP network.

R6.3 - 520	Interdependency of IRI and CC - The CSP shall ensure IRI containing CC metadata is delivered in a timely and accurate manner such that it shall be possible to decode CC in real time.

R6.3 - 530	Reporting Post Session Established Digits - The CSP shall support extracting and reporting dialled digits after the session is established (e.g. user dialled, signalled) via the CSP services, on a per-warrant basis.

R6.3 - 540	Post Session Established Digit Reporting for IRI and CC Intercepts - The CSP shall be able to support extracting and reporting digits after the session is established for IRI-only intercepts, as well as for intercepts that report both IRI and CC.

R6.3 - 550	Toggle for Post Session Established Digit Extraction - The CSP shall support the Post Session Established Digit Extraction capability with a toggle feature that can activate/deactivate this capability, per warrant.

R6.3 - 560	Charging - The 3GPP system shall be able to generate LI charging event records.