SA3LI#84-e-a	 S3i220052

January 24-28, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	IMS LI_T3 trigger illustrations

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI17

Abstract of the contribution: This provides input to the TR 33.929 on IMS LI_T3 illustrations

PROPOSAL

During the meeting on January 26, 2022, when the document S3i220027 was presented, SA3 LI agreed to add the illustrative diagrams shown in S3i220027 to the TR 33.929 instead of the proposed Annex to TS 33.929.

This documents basically ports the text from S3i220027 with corrections made to the diagrams to present the UDP port numbers separated from the IP address with ":" instead of "/". Also, within the diagrams, Option 1 and Option 2 were added to the triggers since the previous method could be mis-interpreted by those who just read the diagrams, not the text.

References to TS 33.128 [4] clauses were changed to refer to TS 33.128 [4] itself.

Eight (8) more diagrams are added here to illustrate the timing of two LI_T3 triggers when used with asymmetric media stream case.

All the proposed text is new and hence, shown in the pCR without the change marks.

PROPOSAL

Add this to the TR 33.929.

# X	Illustrations of IMS CC-POI triggering scenarios

## X.1	Symmetric media streams

### X.1.1	General

This clause illustrates the LI_T3 scenarios when the media streams in an IMS session are symmetric where the source IP address and UDP port number of the IP layer of a media stream sent from a media node is same as the destination IP address and IP address of the IP layer for that media stream that it has given to the remote end.

In this case, an implementation may choose to use only one LI_T3 ActivationTask trigger (as illustrated here) with the local IP address and UDP port number of the IMS media function that intercepts the media.

If two LI_T3 ActivationTask triggers are used for symmetric media stream case, then the illustrations shown in clause 2 apply.

### X.1.2	SDP offer in the forward message – originating session

The figure X.1-1 illustrates the LI_T3 trigger sent from a CC-TF present in the P-CSCF to the CC-POI present in the IMS-AGW as per the 2 options described in TS 33.128 [4] for an IMS originating session from a target when the SDP offer is included in a forward SIP message (e.g. SIP INVITE).

Figure X.1-1: ActivateTask for target originated sessions (SDP offer in forward SIP)

The ActivateTask that can be sent after step 3 has the following:

When the media interception is performed at the core network side:

-	Target Identifiers: IP address 198.51.100.20.

-	UDP port number 10060.

-	Media Direction For Target: ToTarget.

When the media interception is performed at the access side:

-	Target Identifiers: IP address 198.51.100.10.

-	UDP port number 10030.

-	Media Direction For Target: FromTarget.

The IP Address 198.51.100.20 and the UDP port number 10060 are also present in the SDP offer sent toward the terminating end of the session.

The IP Address 198.51.100.10 and the UDP port number 10030 are also present in the SDP answer sent toward the originating end of the session (i.e. target UE).

The media streams received at the IP address 198.51.100.20 and UDP port 10060 are to the target (core network side media interception) and the media streams received at the IP address 198.51.100.10 and the UDP port 10030 (access side media interception) are from the target.

### X.1.3	SDP offer in the response message – originating session

The figure X.1-2 illustrates the LI_T3 trigger sent from a CC-TF present in the P-CSCF to the CC-POI present in the IMS-AGW as per the 2 options described TS 33.128 [4] for an IMS originating session from a target when the SDP offer is included in a response SIP message (e.g. SIP 200 OK).

Figure X.1-2: ActivateTask for target originated sessions (SDP offer in response SIP)

The ActivateTask that can be sent after step 5 has the following:

When the media interception is performed at the core network side:

-	Target Identifiers: IP address 198.51.100.20.

-	UDP port number 10060.

-	Media Direction For Target: ToTarget.

When the media interception is performed at the access side:

-	Target Identifiers: IP address 198.51.100.10.

-	UDP port number 10030.

-	Media Direction For Target: FromTarget.

The IP Address 198.51.100.20 and the UDP port number 10060 are also present in the SDP answer sent toward the terminating end of the session.

The IP Address 198.51.100.10 and the UDP port number 10030 are also present in the SDP offer sent toward the originating end of the session (i.e. target UE).

The media streams received at the IP address 198.51.100.20 and UDP port 10060 (core network side media interception) are to the target and the media streams received at the IP address 198.51.100.10 and the UDP port 10030 (access side media interception) are from the target.

### X.1.4	SDP offer in the forward message – terminating session

The figure X.1-3 illustrates the LI_T3 trigger sent from a CC-TF present in the P-CSCF to the CC-POI present in the IMS-AGW as per the 2 options described in TS 33.128 [4] for an IMS terminating session from a target when the SDP offer is included in a forward SIP message (e.g. SIP INVITE).

Figure X.1-3: ActivateTask for target terminating sessions (SDP offer in forward SIP)

The ActivateTask that can be sent after step 5 has the following:

When the media interception is performed at the core network side:

-	Target Identifiers: IP address 203.0.113.10.

-	UDP port number 20040.

-	Media Direction For Target: ToTarget.

When the media interception is performed at the access side:

-	Target Identifiers: IP address 203.0.113.20.

-	UDP port number 20060.

-	Media Direction For Target: FromTarget.

The IP Address 203.0.113.10 and the UDP port number 20040 are also present in the SDP answer sent toward the originating end of the session.

The IP Address 203.0.113.20 and the UDP port number 20060 are also present in the SDP offer sent toward the terminating end of the session (i.e. target UE).

The media streams received at the IP address 203.0.113.10 and UDP port 20040 (core network side media interception) are to the target and media streams received at the IP address 203.0.1113.20 and the UDP port 20060 (access side media interception) are from the target.

### X.1.5	SDP offer in the response message – terminating session

The figure X.1-4 illustrates the LI_T3 trigger sent from a CC-TF present in the P-CSCF to the CC-POI present in the IMS-AGW as per the 2 options described in TS 33.128 [4] for an IMS terminating session from a target when the SDP offer is included in a response SIP message (e.g. SIP 200 OK).

Figure X.1-4: ActivateTask for target terminating sessions (SDP offer in response SIP)

The ActivateTask that can be sent after step 3 has the following:

When the media interception is performed at the core network side:

-	Target Identifiers: IP address 203.0.113.10.

-	UDP port number 20040.

-	Media Direction For Target: ToTarget.

When the media interception is performed at the access side:

-	Target Identifiers: IP address 203.0.113.20.

-	UDP port number 20060.

-	Media Direction For Target: FromTarget.

The IP Address 203.0.113.10 and the UDP port number 20040 are also present in the SDP offer sent toward the originating end of the session.

The IP Address 203.0.113.20 and the UDP port number 20060 are also present in the SDP answer sent toward the terminating end of the session (i.e. target UE).

The media streams received at the IP address 203.0.113.10 and UDP port 20040 (core network side media interception) are to the target and media streams received at the IP address 203.0.1113.20 and the UDP port 20060 (access side media interception) are from the target.

### X.1.6	SDP offer in the forward message – session to target non-local ID (IP)

The figure X.1-5 illustrates the LI_T3 trigger sent from a CC-TF present in the IBCF to the CC-POI present in the TrGW as described in TS 33.128 [4] for an IMS sessions to a target non-local ID (in the IP domain) when the SDP offer is included in a forward SIP message (e.g. SIP INVITE).

Figure X.1-5: ActivateTask for sessions to target non-local ID in IP domain (SDP offer in forward SIP)

The ActivateTask that can be sent after step 5 has the following:

The media interception is performed at the core network side:

-	Target Identifiers: IP address 203.0.113.10.

-	UDP port number 20040.

-	Media Direction For Target: ToTarget.

When the media interception is performed at the peer network side:

-	Target Identifiers: IP address 203.0.113.20.

-	UDP port number 20060.

-	Media Direction For Target: FromTarget.

The IP Address 203.0.113.10 and the UDP port number 20040 are also present in the SDP answer sent toward the originating end of the session.

The IP Address 203.0.113.20 and the UDP port number 20060 are also present in the SDP offer sent toward the terminating end of the session (i.e. toward the network that has the target non-local ID).

The media streams received at the IP address 203.0.113.10 and UDP port 20040 (core network side media interception) are to the target and media streams received at the IP address 203.0.1113.20 and the UDP port 20060 (peer network side media interception) are from the target.

### X.1.7	SDP offer in the forward message – session from target non-local ID (IP)

The figure X.1-6 illustrates the LI_T3 trigger sent from a CC-TF present in the IBCF to the CC-POI present in the TrGW as described in TS 33.128 [4] for an IMS sessions from a target non-local ID (IP domain) when the SDP offer is included in a forward SIP message (e.g. SIP INVITE).

Figure X.1-6: ActivateTask for sessions from target non-local ID in IP domain (SDP offer in forward SIP)

The ActivateTask that can be sent after step 3 has the following:

The media interception is performed at the core network side:

-	Target Identifiers: IP address 198.51.100.20.

-	UDP port number 10060.

-	Media Direction For Target: ToTarget.

When the media interception is performed at the peer network side:

-	Target Identifiers: IP address 198.51.100.10.

-	UDP port number 10040.

-	Media Direction For Target: FromTarget.

The IP Address 198.51.100.20 and the UDP port number 10060 are also present in the SDP offer sent toward the terminating end of the session.

The IP Address 198.51.100.10 and the UDP port number 10040 are also present in the SDP answer sent toward the originating end of the session.

The media streams received at the IP address 198.51.100.20 and UDP port 10060 (core network side media interception) are to the target and media streams received at the IP address 198.51.100.10 and the UDP port 10040 (peer network side media interception) are from the target.

### X.1.8	SDP offer in the forward message – session to target non-local ID (CS)

The figure X.1-7 illustrates the LI_T3 trigger sent from a CC-TF present in the MGCF to the CC-POI present in the IM-MGW as described in TS 33.128 [4] for an IMS sessions to a target non-local ID (CS domain) when the SDP offer is included in a forward SIP message (e.g. SIP INVITE).

Figure X.1-7: ActivateTask for sessions to target non-local ID in CS domain (SDP offer in forward SIP)

The ActivateTask that can be sent after step 5 has the following:

The media interception is performed at the core network side:

-	Target Identifiers: IP address 203.0.113.10.

-	UDP port number 20040.

-	Media Direction For Target: ToTarget.

The IP Address 203.0.113.10 and the UDP port number 20040 are also present in the SDP answer sent toward the originating end of the session.

The inter-PLMN signaling CS-based (e.g. ISUP). The details of this inter-PLMN signaling are not shown in the figure X.1-7.

The media streams received at the IP address 203.0.113.10 and UDP port 20040 are to the target.

### X.1.9	SDP offer in the forward message – session from target non-local ID (CS)

The figure X.1-8 illustrates the LI_T3 trigger sent from a CC-TF present in the MGCF to the CC-POI present in the IM-MGW as described in TS 33.128 [4] for an IMS sessions from a target non-local ID (CS domain) when the SDP offer is included in a forward SIP message (e.g. SIP INVITE).

Figure X.1-8: ActivateTask for sessions from target non-local ID in CS domain (SDP offer in forward SIP)

The ActivateTask that can be sent after step 3 has the following:

The media interception is performed at the core network side:

-	Target Identifiers: IP address 198.51.100.10.

-	UDP port number 10060.

-	Media Direction For Target: ToTarget.

The IP Address 198.51.100.10 and the UDP port number 10060 are also present in the SDP offer sent toward the terminating end of the session.

The inter-PLMN signaling is CS-based (e.g. ISUP). The details of this inter-PLMN signaling are not shown in the figure X.1-8.

The media streams received at the IP address 198.51.100.10 and UDP port 10060 are to the target.

## X.2	Asymmetric media streams

### X.2.1	General

This clause illustrates the LI_T3 scenarios when the media streams in an IMS session are asymmetric where the source IP address and UDP port number of the IP layer of a media stream sent from a media node is different from the destination IP address and IP address of the IP layer that it has given to the remote end.

In this case, a separate LI_T3 ActivationTask trigger is used to intercept the media for each of the media direction. For a simple session involving two parties (as illustrated here), two LI_T3 ActivationTask triggers are used.

### X.2.2	SDP offer in the forward message – originating session (core)

The figure X.2-1 illustrates the LI_T3 triggers sent from a CC-TF present in the P-CSCF to the CC-POI present in the IMS-AGW when the media interception is done at the core network side of the IMS-AGW as per the two options described in TS 33.128 [4] for an IMS originating session from a target when the SDP offer is included in a forward SIP message (e.g. SIP INVITE).

Figure X.2-1: ActivateTasks for target originated sessions (SDP offer in forward SIP, core)

The first ActivateTask that can be sent after step 3 has the following:

-	Target Identifiers: IP address 198.51.100.20.

-	UDP port number 10060.

-	Media Direction For Target: ToTarget.

The second ActivateTask that can be sent after the step 8 Ack (not shown) has the following:

-	Target Identifiers: IP address 203.0.113.10.

-	UDP port number 20040.

-	Media Direction For Target: FromTarget.

The IP Address 198.51.100.20 and the UDP port number 10060 are also present in the SDP offer sent toward the terminating end of the session.

The IP Address 203.0.113.10 and the UDP port number 20040 are also present in the SDP answer received from the terminating end of the session.

The media streams received at the IP address 198.51.100.20 and UDP port 10060 are to the target and the media streams sent to the IP address 203.0.113.10 and the UDP port 20040 are from the target.

### X.2.3	SDP offer in the forward message – originating session (access)

The figure X.2-2 illustrates the LI_T3 triggers sent from a CC-TF present in the P-CSCF to the CC-POI present in the IMS-AGW when the media interception is done at the access side of the IMS-AGW as per the two options described in TS 33.128 [4] for an IMS originating session from a target when the SDP offer is included in a forward SIP message (e.g. SIP INVITE).

Figure X.2-2: ActivateTasks for target originated sessions (SDP offer in forward SIP, access)

The first ActivateTask that can be sent after step 3 has the following:

-	Target Identifiers: IP address 198.51.100.20.

-	UDP port number 10030.

-	Media Direction For Target: FromTarget.

The second ActivateTask that can be sent after the step 3 has the following:

-	Target Identifiers: IP address 192.0.2.10.

-	UDP port number 30010.

-	Media Direction For Target: ToTarget.

The IP Address 198.51.100.20 and the UDP port number 10030 are also present in the SDP answer sent toward the originating end of the session (i.e. target).

The IP Address 192.0.2.10 and the UDP port number 30010 are also present in the SDP offer received from the originating end of the session (i.e. target).

The media streams received at the IP address 198.51.100.20 and UDP port 10030 are from the target and the media streams sent to the IP address 192.0.2.10 and the UDP port 30010 are to the target.

### X.2.4	SDP offer in the response message – originating session (core)

The figure X.2-3 illustrates the LI_T3 triggers sent from a CC-TF present in the P-CSCF to the CC-POI present in the IMS-AGW when the media interception is done at the core network side of the IMS-AGW as per the two options described in TS 33.128 [4] for an IMS originating session from a target when the SDP offer is included in a response SIP message (e.g. SIP 200 OK).

Figure X.2-3: ActivateTask for target originated sessions (SDP offer in response SIP) (core)

The first ActivateTask that can be sent after step 5 has the following:

-	Target Identifiers: IP address 198.51.100.20.

-	UDP port number 10060.

-	Media Direction For Target: ToTarget.

The second ActivateTask that can be sent after step 5 has the following:

-	Target Identifiers: IP address 203.0.113.10.

-	UDP port number 20040.

-	Media Direction For Target: FromTarget.

The IP Address 198.51.100.20 and the UDP port number 10060 are also present in the SDP answer sent toward the terminating end of the session.

The IP Address 203.0.113.10 and the UDP port number 20040 are also present in the SDP offer received from the terminating end of the session.

The media streams received at the IP address 198.51.100.20 and UDP port 10060 are to the target and the media streams sent to the IP address 203.0.113.10 and the UDP port 20040 are from the target.

### X.2.5	SDP offer in the response message – originating session (access)

The figure X.2-4 illustrates the LI_T3 triggers sent from a CC-TF present in the P-CSCF to the CC-POI present in the IMS-AGW when the media interception is done at the access side of the IMS-AGW as per the two options described in TS 33.128 [4] for an IMS originating session from a target when the SDP offer is included in a response SIP message (e.g. SIP 200 OK).

Figure X.2-4: ActivateTask for target originated sessions (SDP offer in response SIP) (access)

The first ActivateTask that can be sent after step 5 has the following:

-	Target Identifiers: IP address 198.51.100.20.

-	UDP port number 10030.

-	Media Direction For Target: fromTarget.

The second ActivateTask that can be sent after step 8 Ack (not shown) has the following:

-	Target Identifiers: IP address 192.0.2.10.

-	UDP port number 30010.

-	Media Direction For Target: ToTarget.

The IP Address 198.51.100.20 and the UDP port number 10030 are also present in the SDP offer sent toward the originating end of the session (i.e. target)

The IP Address 192.0.2.10 and the UDP port number 30010 are also present in the SDP answer received from the originating end of the session.

The media streams received at the IP address 198.51.100.20 and UDP port 10030 are from the target and the media streams sent to the IP address 192.0.2.10 and the UDP port 30010 are to the target.

### X.2.6	SDP offer in the forward message – terminating session (core)

The figure X.2-5 illustrates the LI_T3 triggers sent from a CC-TF present in the P-CSCF to the CC-POI present in the IMS-AGW when the media interception is done at the core network side of the IMS-AGW as per the two options described in TS 33.128 [4] for an IMS terminating session to a target when the SDP offer is included in a forward SIP message (e.g. SIP INVITE).

Figure X.2-5: ActivateTask for target terminating sessions (SDP offer in forward SIP) (core)

The first ActivateTask that can be sent after step 5 has the following:

-	Target Identifiers: IP address 203.0.113.10.

-	UDP port number 20040.

-	Media Direction For Target: ToTarget.

The second ActivateTask that can be sent after step 5 has the following:

-	Target Identifiers: IP address 198.51.100.10.

-	UDP port number 10060.

-	Media Direction For Target: FromTarget.

The IP Address 203.0.113.10 and the UDP port number 20040 are also present in the SDP answer sent toward the originating end of the session.

The IP Address 198.51.100.10 and the UDP port number 10060 are also present in the SDP offer received from the originating end of the session.

The media streams received at the IP address 203.0.113.10 and UDP port 20040 are to the target and media streams sent to the IP address 198.51.100.10 and the UDP port 10060 are from the target.

### X.2.7	SDP offer in the forward message – terminating session (access)

The figure X.2-6 illustrates the LI_T3 triggers sent from a CC-TF present in the P-CSCF to the CC-POI present in the IMS-AGW when the media interception is done at the access side of the IMS-AGW as per the two options described in TS 33.128 [4] for an IMS terminating session to a target when the SDP offer is included in a forward SIP message (e.g. SIP INVITE).

Figure X.2-6: ActivateTask for target terminating sessions (SDP offer in forward SIP) (access)

The first ActivateTask that can be sent after step 5 has the following:

-	Target Identifiers: IP address 203.0.113.20.

-	UDP port number 20060.

-	Media Direction For Target: FromTarget.

The second ActivateTask that can be sent after step 8 Ack (not shown) has the following:

-	Target Identifiers: IP address 198.0.2.20.

-	UDP port number 30010.

-	Media Direction For Target: ToTarget.

The IP Address 203.0.113.10 and the UDP port number 20060 are also present in the SDP offer sent toward the terminating end of the session (i.e. target).

The IP Address 198.0.2.20 and the UDP port number 30010 are also present in the SDP answer received from the terminating end of the session (i.e. target).

The media streams received at the IP address 203.0.113.10 and UDP port 20060 are from the target and media streams sent to the IP address 198.0.2.20 and the UDP port 30010 are to the target.

### X.2.8	SDP offer in the response message – terminating session (core)

The figure X.2-7 illustrates the LI_T3 triggers sent from a CC-TF present in the P-CSCF to the CC-POI present in the IMS-AGW when the media interception is done at the core network side of the IMS-AGW as per the two options described in TS 33.128 [4] for an IMS terminating session to a target when the SDP offer is included in a response SIP message (e.g. SIP 200 OK).

Figure X.2-7: ActivateTask for target terminating sessions (SDP offer in response SIP) (core)

The first ActivateTask that can be sent after step 3 has the following:

-	Target Identifiers: IP address 203.0.113.10.

-	UDP port number 20040.

-	Media Direction For Target: ToTarget.

The second ActivateTask that can be sent after step 8 Ack (not shown) has the following:

-	Target Identifiers: IP address 198.51.100.10.

-	UDP port number 10060.

-	Media Direction For Target: FromTarget.

The IP Address 203.0.113.10 and the UDP port number 20040 are also present in the SDP offer sent toward the originating end of the session.

The IP Address 198.51.100.10 and the UDP port number 10060 are also present in the SDP answer received from the originating end of the session.

The media streams received at the IP address 203.0.113.10 and UDP port 20040 are to the target and media streams sent to the IP address 198.51.100.10 and the UDP port 10060 are from the target.

### X.2.9	SDP offer in the response message – terminating session (core)

The figure X.2-8 illustrates the LI_T3 triggers sent from a CC-TF present in the P-CSCF to the CC-POI present in the IMS-AGW when the media interception is done at the access side of the IMS-AGW as per the two options described in TS 33.128 [4] for an IMS terminating session to a target when the SDP offer is included in a response SIP message (e.g. SIP 200 OK).

Figure X.2-8: ActivateTask for target terminating sessions (SDP offer in response SIP) (access)

The first ActivateTask that can be sent after step 3 has the following:

-	Target Identifiers: IP address 203.0.113.10.

-	UDP port number 20060.

-	Media Direction For Target: FromTarget.

The second ActivateTask that can be sent after step 3 has the following:

-	Target Identifiers: IP address 192.0.20.10.

-	UDP port number 30010.

-	Media Direction For Target: ToTarget.

The IP Address 203.0.113.10 and the UDP port number 20060 are also present in the SDP answer sent toward the terminating end of the session (i.e. target).

The IP Address 192.0.20.10 and the UDP port number 30010 are also present in the SDP offer received from the terminating end of the session.

The media streams received at the IP address 203.0.113.10 and UDP port 20060 are from the target and media streams sent to the IP address 192.0.20.10 and the UDP port 30010 are to the target.