3GPP SA3LI#88e-b	s3i230150

- 21-23 February 2023, electronic

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.108 | CR | 0428 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reporting MT-SM originating UE address in alphanumeric format |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Ericsson, Softel Systems, OTD_US) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI17 |  |  |  |  |  | Date: |  |  | 2023-02-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current version of the TS, there is no explicit parameter to report the originating UE address of the SMS, which is included in the SMS header (TP-OA), especially when this is provided in alphanumeric format. Based on the current specification, the header of the SMS is reported only in case the whole SMS content is authorized to be reported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When delivery of SMS content is not authorized, only the TP-User-Data is removed from the SMS content, so ensuring that other parameters are reported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In some scenarios, the TS would not allow reporting of a MT/SM originating address in a standard way, especially in alphanumeric format. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2; 5.2.3; 6.5.1.1; 7.5.0; 15.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3i230136 |  |  |  |  |  |  |  |  |

*** FIRST CHANGE ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ETSI TS 101 331: "Lawful Interception (LI); Requirements of Law Enforcement Agencies".

[2]	ETSI ES 201 158: "Telecommunications security; Lawful Interception (LI); Requirements for network functions".

[3]	ETSI ETR 330: "Security Techniques Advisory Group (STAG); A guide to legislative and regulatory environment".

[4]	3GPP TS 29.002: "3rd Generation Partnership Project; Technical Specification Group Core Network; Mobile Application Part (MAP) specification".

[5A]	ITU-T Recommendation X.680: "Abstract Syntax Notation One (ASN.1): Specification of Basic Notation".

[5B]	ITU-T Recommendation X.681: "Abstract Syntax Notation One (ASN.1): Information Object Specification".

[5C]	ITU-T Recommendation X.681: "Abstract Syntax Notation One (ASN.1): Constraint Specification".

[5D]	ITU-T Recommendation X.681: "Abstract Syntax Notation One (ASN.1): Parameterization of ASN.1 Specifications".

[6]	ITU-T Recommendation X.690: "ASN.1 encoding rules: Specification of Basic Encoding Rules (BER), Canonical Encoding Rules (CER) and Distinguished Encoding Rules (DER)".

NOTE 1:	It is recommended that for [5A], [5B], [5C], [5D] and [6] the 2002 specific versions should be used.

[7]	ITU-T Recommendation X.880: "Information technology - Remote Operations: Concepts, model and notation".

[8]	Void.

[9]	3GPP TS 24.008: "3GPP Technical Specification Group Core Network; Mobile radio interface Layer 3 specification, Core network protocol; Stage 3".

[10] - [12]	Void.

[13]	IETF STD 9 (RFC 0959): "File Transfer Protocol (FTP)".

[14]	3GPP TS 32.215: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication Management; Charging Management; Charging data description for the Packet Switched (PS) domain)".

[15]	IETF STD0005 (RFC 0791: "Internet Protocol".

[16]	IETF STD0007 (RFC 0793): "Transmission Control Protocol".

[17]	3GPP TS 29.060: "3rd Generation Partnership Project; Technical Specification Group Core Network; General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[18]	3GPP TS 33.106: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Lawful Interception Requirements".

[19]	3GPP TS 33.107: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Lawful interception architecture and functions".

[20]	3GPP TS 23.107: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Quality of Service QoS concepts and architecture".

[21] - [22]	Void.

[23]	ANSI/J-STD-025-A: "Lawfully Authorized Electronic Surveillance".

[24]	ETSI TS 101 671: "Handover Interface for the lawful interception of telecommunications traffic".

[25]	3GPP TS 23.003: "3rd Generation Partnership Project; Technical Specification Group Core Network; Numbering, addressing, and identification".

[26]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[27]	IETF RFC 1006: "ISO Transport Service on top of the TCP".

[28]	IETF RFC 2126: "ISO Transport Service on top of TCP (ITOT)".

[29]	ITU-T Recommendation Q.763: "Signalling System No. 7 - ISDN User Part formats and codes".

[30]	ETSI EN 300 356 (all parts): "Integrated Services Digital Network (ISDN); Signalling System No.7; ISDN User Part (ISUP) version 3 for the international interface".

[31]	ETSI EN 300 403-1 (V1.3.2): "Integrated Services Digital Network (ISDN); Digital Subscriber Signalling System No. one (DSS1) protocol; Signalling network layer for circuit-mode basic call control; Part 1: Protocol specification [ITU-T Recommendation Q.931 (1993), modified]".

NOTE 3:	Reference [31] is specific, because ASN.1 parameter "release-Reason-Of-Intercepted-Call" has the following comment: "Release cause coded in [31] format". In case later version than the given one indicated for ISDN specification ETSI EN 300 403-1 has modified format of the "release cause", keeping the reference version specific allows to take proper actions in later versions of this specification.

[32] - [33]	Void

[34]	ITU-T Recommendation Q.931: "ISDN user-network interface layer 3 specification for basic call control".

[35]	Void.

[36]	Void.

[37]	3GPP TS 23.032: "3rd Generation Partnership Project; Technical Specification Group Core Network; Universal Geographical Area Description (GAD)".

[38]	3GPP TR 21.905: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Vocabulary for 3GPP Specifications".

[39]	ISO 3166-1: "Codes for the representation of names of countries and their subdivisions - Part 1: Country codes".

[40]	3GPP TS 23.228: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS); Stage 2".

[41]	3GPP TS 29.234: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals: 3GPP System to Wireless Local Area Network (WLAN) interworking; Stage 3".

[42]	3GPP TS 23.060: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; General Packet Radio Service (GPRS); Service description".

[43]	3GPP TS 23.234: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3GPP system to Wireless Local Area Network (WLAN) Interworking; System Description".

[44]	3GPP TS 23.401: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[45]	3GPP TS 23.402: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Architecture enhancements for non-3GPP accesses".

[46]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Access (GPRS) Tunneling Protocol for Control Plane (GTPv2-C); Stage 3".

[47]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[48]	3GPP TS 29.275: "Proxy Mobile IPv6 (PMIPv6) based Mobility and Tunneling protocols; Stage 3".

[49]	3GPP TS 24.303: "Mobility management based on Dual-Stack Mobile IPv6; Stage 3".

[50]	(void)

[51]	(void)

[52]	3GPP TS 24.147: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Conferencing Using the IP Multimedia (IM) Core Network (CN) subsystem 3GPP Stage 3".

[53]	3GPP TS 29.273: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[54]	3GPP TS 33.328: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS) media plane security".

[55]	ATIS-0700005 "Lawfully Authorized Electronic Surveillance (LAES) for 3GPP IMS-based VoIP and other Multimedia Services".

[56]	3GPP TS 29.212: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Policy and Charging Control(PCC); Reference points".

[57]	Void.

[58]	IETF RFC 4217: "Securing FTP with TLS".

[59]	3GPP TS 29.272: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Evolved Packet System (EPS); Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[60]	3GPP TS 33.310: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Network Domain Security (NDS); Authentication Framework (AF)".

[61]	IETF RFC 6043: "MIKEY-TICKET: Ticket-Based Modes of Key Distribution in Multimedia Internet KEYing (MIKEY)", available at www.ietf.org

[62]	3GPP TS 25.413: "UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling".

[63]	3GPP TS 29.279: "Mobile IPv4 (MIPv4) based mobility protocols; Stage 3".

[64]	3GPP TS 29.118: "Mobility Management Entity (MME) -Visitor Location Register (VLR) SGs interface specification"

[65]	ANSI/J-STD-025-B: "Lawfully Authorized Electronic Surveillance", July 17, 2006.

[66]	3GPP TS 24.007: "Mobile Radio Interface Signalling Layer 3; General Aspects".

[67]	IETF RFC 3966: "The Tel URIs for Telephone Numbers", December, 2004.

[68]	IETF RFC 791: "Internet Protocol"

[69]	IETF RFC 2460: "Internet Protocol, Version 6 (IPv6) Specification".

[70]	IEFT RFC 3697: "IPv6 Flow Label Specification".

[71]	IETF RFC 4776: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information".

[72]	IETF RFC 5139: "Revised Civic Location Format for Presence Information Data Format Location Object (PIDF-LO)".

[73]	ISO.3166-2: International Organization for Standardization, "Codes for the representation of names of countries and their subdivisions - Part 2: Country subdivision code".

[74]	UPS SB42-4: Universal Postal Union (UPU), "International Postal Address Components and Templates".

[75]	ISO 639-1:2002: "Codes for the representation of names of languages -- Part 1: Alpha-2 code".

[76]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[77]	3GPP TS 24.623: "Technical Specification Group Core Network and Terminals; Extensible Markup Language (XML) Configuration Access Protocol (XCAP) over the Ut interface for Manipulating Supplementary Services".

[78]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[79]	3GPP TS 24.109: "Universal Mobile Telecommunications System (UMTS); Bootstrapping interface (Ub) and network application function interface (Ua); Protocol details".

[80]	IETF RFC 4825:"The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[81]	IETF RFC 7254: "A Uniform Resource Name Namespace for the Global System for Mobile Communications Association (GSMA) and the International Mobile station Equipment Identity (IMEI)"

[82]	IETF RFC 7255: "Using the International Mobile station Equipment Identity (IMEI) Uniform Resource Name (URN) as an Instance ID".

[83]	3GPP TS 22.468: "Group Communication System Enablers for LTE (GCSE_LTE)".

[84]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[85]	3GPP TS 25.321: "Medium Access Control (MAC) protocol specification".

[86]	3GPP TS 24.371: " Web Real-Time Communications (WebRTC) access to the IP Multimedia (IM) Core Network (CN) subsystem (IMS); Stage 3".

[87]	ITU-T Recommendation E.212: "The international identification plan for public networks and subscriptions".

[88]	OMA MLP TS: "Mobile Location Protocol", OMA-TS-MLP-V3_5-20181211-C.

[89]	MMS Architecture OMA-AD-MMS-V1_3-20110913-A.

[90]	Multimedia Messaging Service Encapsulation Protocol OMA-TS-MMS_ENC-V1_3-20110913-A.

[91]	3GPP TS 22.140: "Multimedia Messaging Service (MMS); Stage 1".

[92]	IETF RFC 2822: "Internet Message Format".

[93]	IETF RFC 3551: "RTP Profile for Audio and Video Conferences with Minimal Control".

[94]	IETF RFC 4566: "Session Description Protocol".

[95]	IETF RFC 3550: "Realtime Transport Protocol".

[96]	3GPP TS 29.229: "Cx and Dx interfaces based on the Diameter protocol; Protocol details".

[97]	OMA-AD-PoC-V2_1-20110802-A, Architecture Document.

[98]	OMA-TS-PoC_User Plane-V2_1-20110802-A.

[99]	3GPP TS 37.340: "Evolved Universal Radio Access (E-UTRA) and NR-Multi-connectivity; Stage 2".

[100]	3GPP TS 36.413: "E-UTRAN – S1 Application Protocol (S1AP)".

[101]	3GPP TS 29.336: "Home Subscriber Server (HSS) diameter interfaces for interworking with packet data networks and applications".

[102]	IETF RFC 3588: "Diameter Base Protocol".

[103]	IETF RFC 4282: "The Network Access Identifier".

[104]	ETSI TS 102 232-1: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 1: Handover specification for IP delivery".

[105]	ETSI TS 102 232-7: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 7: Service-specific details for Mobile Services".

[106]	3GPP TS 33.126: " Lawful interception requirements ".

[XX]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

*** NEXT CHANGE ***

### 5.2.3	Delivery of IRI

The events defined in TS 33.107 [19] are used to generate Records for the delivery via HI2. The LALS reports defined in TS 33.107 [19] are delivered via HI2, as well.

There are thirteen different events type received at DF2 level. According to each event, a Record is sent to the LEMF if this is required. In the case of LALS reports, which are not associated with an event, a Record is sent to the LEMF without the event parameter.

The following table gives the mapping between event type received at DF2 level and record type sent to the LEMF.

It is an implementation option if the redundant information will be sent for each further event.

Table 5.4: Structure of the records for UMTS (CS)

| Event | IRI Record Type |
| --- | --- |
| Call establishment | BEGIN |
| Answer | CONTINUE |
| Supplementary service | CONTINUE |
| Handover | CONTINUE |
| Release | END |
| Location update | REPORT |
| Subscriber controlled input | REPORT |
| SMS | REPORT |
| Serving system | REPORT |
| HLR subscriber record change | REPORT |
| Cancel location | REPORT |
| Register location | REPORT |
| Location information request | REPORT |

The LALS report records are sent to the LEMF with the REPORT IRI Record Type.

NOTE 1:	Void.

A set of information is used to generate the records. The records used transmit the information from mediation function to LEMF. This set of information can be extended in 3G MSC server or 3G GMSC server or DF2/MF, if this is necessary in a specific country. The following table gives the mapping between information received per event or report and information sent in records.

Table 5.5: Description of parameters

| Parameter | Definition | ASN.1 parameter |
| --- | --- | --- |
| Observed MSISDN | Target Identifier with the MSISDN of the target | PartyInformation/msISDN |
| Observed IMSI | Target Identifier with the IMSI of the target | PartyInformation/imsi |
| Observed IMEI | Target Identifier with the IMEI of the target, it has to be checked for each call over the radio interface | PartyInformation/imei |
| Observed Non-Local ID | Target Identifier with the E.164 number of Non-Local ID target | Partyinformation/e164-Format |
| New observed MSISDN | New target identifier with MSISDN of the target, when available | PartyInformation/msISDN |
| New observed IMSI | New target identifier with IMSI of the target, when available | PartyInformation/imsi |
| New observed IMEI | New target identifier with IMEI of the target, when available | PartyInformation/imei |
| Event type | Description of which type of event is delivered: Establishment, Answer, Supplementary service, Handover, Release, SMS, Location update, Subscriber controlled input, HLR subscriber record change, Serving system, Cancel location, Register location, Location information request | umts-CS-Event. In case this parameter is not sent over the HI2 interface, the presence of other parameters on HI2 indicates the event type (e.g. sMS or sciData parameter presence) |
| Event date | Date of the event generation in the 3G MSC server or 3G GMSC server or in the HLR | timeStamp |
| Event time | Time of the event generation in the 3G MSC server or 3G GMSC server or in the HLR |  |
| Dialled number | Dialled number before digit modification, IN-modification, etc. | PartyInformation (= originating)/DSS1-parameters/calledpartynumber |
| Connected number | Number of the answering party | PartyInformation/supplementary-Services-Info |
| Other party address | Directory number of the other party for originating calls Calling party for terminating calls (See NOTE 1) | PartyInformation (= terminating)/calledpartynumber PartyInformation/callingpartynumber |
| Call direction | Information if the target is calling or called e.g. MOC/MTC or originating/terminating in or/out | intercepted-Call-Direct |
| CID | Unique number for each call sent to the DF, to help the LEA, to have a correlation between each call and the IRI (combination of Interception Node ID and the correlation number) | communicationIdentifier |
| Lawful interception identifier | Unique number for each surveillance lawful authorization | lawfulInterceptionIdentifier |
| CGI/SAI | CGI or SAI of the target; for the location information | locationOfTheTarget |
| Location area code | Location-area-code of the target defines the Location Area in a PLMN |  |
| Location Information | LALS location information |  |
| Time of Location | Date/Time of location. The time when location was obtained by the location source node. |  |
| Serving system identifier | VPLMN ID of the serving system or of the third party network interworking with the HLR | serving-System-Identifier |
| Basic service | Information about Tele service or bearer service | PartyInformation/DSS1-parameters-codeset-0 |
| Supplementary service | Supplementary services used by the target e.g. Call forwarding, CW, ECT | PartyInformation/Supplementary-Services |
| Forwarded to number | Forwarded to number at call forwarding | PartyInformation/calledPartyNumber (party-Qualifier indicating forwarded-to-party) |
| Call release reason | Call release reason of the target call | release-Reason-Of-intercepted-Call |
| SMS | The SMS content with header which is sent with the SMS-service (see NOTE 2) | sMS |
| SCI | Non-call related Subscriber Controlled Input (SCI) which the 3G MSC server receives from the ME | PartyInformation/sciData |
| Other update | Carrier specific information related to its implementation or subscription process on its HLR | carrierSpecificData |
| Changed (old/new) IMSI or MSISDN or IMEI | Provides the identity changes in Subscriber Record Change Event. | change-Of-Target-Identity |
| Previous serving system identifier | Previous VPLMN Id of the target | current-Previous-Systems/previous-Serving-System-Identifier |
| Previous serving MSC-number | An E.164 number of the previous serving MSC included in the intercepted MAP message | current-Previous-Systems/previous-Serving-MSC-Number |
| Previous serving MSC-address | An IP address of the previous serving MSC, included in the intercepted MAP message | current-Previous-Systems/previous-Serving-MSC- Address |
| NOTE: LIID parameter has to be present in each record sent to the LEMF. |  |  |

NOTE 1:	In case of SMS, the information about the other party is included in the SMS TPDU (TS 23.040 [XX] clause 9.2); in such case, the use of this parameter is implementation dependent. In case of terminating SMS in which the originating address is in alphanumeric format, this parameter shall not be used to carry information about the originating party.

NOTE 2:	 For intercepts requiring IRI only delivery, depending on national regulations, SMS content may be excluded by replacing all the octets of the TP-User-Data (TS 23.040 [XX] clause 9.2.3.24) with octets of value zero.

Table 5.5A: Serving System REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed IMEI |  |  |
| event type | C | Provide Serving System event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Network identifier of the HLR reporting the event (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| serving system identifier | C | Provide the VPLMN id (Mobile Country Code and Mobile Network Country, E. 212 number [87]). |

Table 5.5B: HLR subscriber record change REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| new observed MSISDN | C | Provide at least one and others when available. |
| new observed IMSI |  |  |
| New observed IMEI |  |  |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| observed IMEI |  |  |
| event type | C | Provide HLR subscriber record change event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Network identifier of the HLR reporting the event (Network element identifier included). |
| changed (old/new) IMSI or MSISDN or IMEI | M | Shall provide what was changed (old/new MSISDN, old/new IMSI or old/new IMEI) |
| lawful intercept identifier | M | Shall be provided. |
| carrier specific data | C | Provide to raw data of this specific update related to HLR. |

Table 5.5C: Cancel location REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| event type | C | Provide cancel Location change event type. (purge from HLR sent to SGSN included). |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Network identifier of the HLR reporting the event (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| previous serving system identifier | C | Provide the previous VPLMN id (Mobile Country Code and Mobile Network Country, defined in E212 [87])). |
| previous serving MSC-number | C | Provide to identify the E.164 number of the previous serving MSC. |
| previous serving MSC-address | C | Provide to identify the IP address of the previous serving MSC. |

Table 5.5D: Register location REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| event type | C | Provide register location event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Network identifier of the HLR reporting the event (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| previous serving system identifier | C | Provide the previous VPLMN id (Mobile Country Code and Mobile Network Country; defined in E212 [87]) ). |
| previous serving MSC number | C | Provide to identify the E.164 number of the previous serving MSC. |
| previous serving MSC address | C | Provide to identify the IP address of the previous serving MSC. |
| current serving system identifier | C | Provide the previous VPLMN id (Mobile Country Code and Mobile Network Country, defined in E212 [87])). |
| current serving MSC number | C | Provide to identify the E.164 number of the current serving MSC. |
| current serving MSC address | C | Provide to identify the IP address of the current serving MSC. |

Table 5.5E: Location information request REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| event type | C | Provide location information request event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Network identifier of the HLR reporting the event (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| requesting network identifier | C | Provide the requesting network identifier PLMN id (Mobile Country Code and Mobile Network Country, defined in E212 [87]). |
| requesting node type | C | Provide the requesting node type (GMSC; SMS Centre; GMLC, MME, SGSN). |

Table 5.5F: LALS Target Positioning REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed IMEI |  |  |
| event date | M | Provide the date and time the LCS Report is available at LI LCS Client. |
| event time |  |  |
| network identifier | M | Network identifier of the LI LCS Client (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide the LALS location information, if the positioning is successful |
| extended location parameters | O | If available, additional location information and associated QoS information. |
| Time of Location | C | Date/Time of Location. (if target location provided). |
| LALS error code | C | Provide the error identification code if the positioning is not successful. |

Table 5.5G: LALS Enhanced Location for IRI REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed IMEI |  |  |
| event date | M | Provide the date and time the LCS Report is available at LI LCS Client. |
| event time |  |  |
| network identifier | M | Network identifier of the LI LCS Client (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| communication identity number | C | Provided for correlation with the IRI records of the call, if available in the corresponding LALS triggering event. |
| location information | C | Provide the LALS location information, if the positioning is successful. |
| extended location parameters | O | If available, additional location information and associated QoS information. |
| Time of Location | C | Date/Time of Location. (if target location provided). |
| LALS error code | C | Provide the error identification code if the positioning is not successful. |

NOTE 2:	See the TS 33.107 [19] for a detailed description of LALS. See Annex O for information on using of the CS ASN.1 information object for the LALS reporting.

NOTE 3: 	In some specific scenarios the amount of Enhanced Location for IRI reports data may overload the X2 and/or HI2 interfaces. To prevent the overload, a flow control for Enhanced Location for IRI Reports may be implemented, e.g. by limiting the frequency of the reports for individual target.

*** NEXT CHANGE ***

#### 6.5.1.1	REPORT record information

The REPORT record is used to report non-communication related subscriber actions (events) and for reporting unsuccessful packet-mode communication attempts.

The REPORT record is also used for the PS LALS reports.

The REPORT record shall be triggered when:

-	the target's mobile station performs a GPRS attach procedure (successful or unsuccessful);

-	the target's mobile station performs a GPRS detach procedure;

-	the target's mobile station is unsuccessful at performing a PDP context activation procedure;

-	the target's mobile station performs a cell, routing area, or combined cell and routing area update;

-	the interception is activated after target's mobile station has successfully performed GPRS attach procedure;

-	optionally when the target's mobile station leaves the old SGSN;

-	optionally when the target's mobile station enters or leaves IA;

-	the target's mobile station sends an SMS-Mobile Originated (MO) communication. Dependent on national requirements, the triggering for the REPORT record event shall occur either when the 3G SGSN receives the SMS from the target MS or, when the 3G SGSN receives notification that the SMS-Centre successfully received the SMS;

a mobile station sends an SMS-Mobile Originated (MO) communication to a Non-Local ID target. Dependent on national requirements, the triggering event for the REPORT record shall occur either when the 3G SGSN receives the SMS from a MS for a Non-Local ID target or, when the 3G SGSN receives notification that the SMS-Centre successfully received the SMS for the Non-Local ID target;

-	the target's mobile station receives a SMS Mobile-Terminated (MT) communication. Dependent on national requirements, the triggering event for the REPORT record shall occur either when the 3G SGSN receives the SMS from the SMS-Centre or, when the 3G SGSN receives notification that the target MS successfully received the SMS;

a mobile station receives a SMS Mobile-Terminated (MT) communication from a Non-Local ID target. Dependent on national requirements, the triggering event for the REPORT record shall occur either when the 3G SGSN receives the SMS from the SMS-Centre originating from a Non-Local ID target or, when the 3G SGSN receives notification that the MS successfully received the SMS originating from a Non-Local ID target;

-	as a national option, a mobile terminal is authorized for service with another network operator or service provider; in that case, other related events are required as cancel location, register location, location information request from a third party's node;

-	as a national option, a REPORT record have to be generated when there is a HLR subscriber record change of IMSI or of MSISDN triggered by a messages to or from the HLR;

-	packet data header reporting is performed on an individual intercepted packet basis and a packet is detected as it is sent or received by the target for a packet-data communication PDP Context.;

-	when packet data summary reporting is performed on a summary basis for a packet-data communication PDP Context.associated with a particular packet flow (defined as the combination of source IP address, destination IP address, source port, destination port, and protocol and for IPv6 also include the flow label) and:

-	the packet flow starts,

-	an interim packet summary report is to be provided, or

-	packet flow ends including the case where PDP Context is deactivated.

An interim packet data summary report is triggered if:

-	the expiration of a configurable Summary Timer per intercept occurs. The Summary Timer is configurable in units of seconds. Or

-	a per-intercept configurable count threshold is reached.

-	when a LALS report information is received from the LI LCS Client.

Packet Data Header Information is reported either on a per-packet (i.e. non-summarised) basis or in a summary report. These reports provide IRI associated with the packets detected. The packet data header information related REPORT record is used to convey packet data header information during an active packet-data communication PDP Context.

NOTE:	In the case of IP Fragments, Packet Data Header Information on a 6-tuple basis may only be available on the first packet and subsequent packets may not include such information and therefore may not be reported.

Table 6.3: GPRS Attach REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed IMEI |  |  |
| event type | M | Shall provide GPRS Attach event type. |
| event date | M | Shall provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the target's MS. |
| Time of Location | C | Date/Time of Location. (if target location provided). |
| failed attach reason | C | For failed attach attempts of the target, provide information about the reason for the failed attach attempt. |

Table 6.4: GPRS Detach REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed IMEI |  |  |
| event type | M | Shall provide GPRS Detach event type. |
| event date | M | Shall provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the target's MS. |
| Time of Location | C | Date/Time of Location. (if target location provided). |

Table 6.5: PDP Context Activation (unsuccessful) REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed IMEI |  |  |
| observed PDP address | C | When a: - static address requested by the target's MS in association with a target-initiated PDP context activation request is unsuccessful; or - address offered by the network in association with a network-initiated PDP context activation request and the target's MS rejects the network-initiated PDP context activation, The address requested or offered shall be reported. |
| iP assignment | C | When an observed PDP address is reported, shall provide to indicate observed PDP address is statically or dynamically assigned. |
| event type | M | Shall provide PDP Context Activation event type. |
| event date | M | Shall provide the date and time the event is detected. |
| event time |  |  |
| access point name | C | If available (i.e., provided by the UE) shall identify either the: - packet data network to which the target requested to be connected when the target's mobile station is unsuccessful at performing a PDP context activation procedure (MS to Network); or - access point of the packet data network that requested to be connected to the MS when the target's mobile station rejects a network-initiated PDP context activation (Network to MS). |
| PDP type | C | When an observed PDP address is reported, provide to describe the PDP type of the observed PDP address. The PDP Type defines the end user protocol to be used between the external packet data network and the MS. |
| initiator | M | Shall be provided to indicate whether the PDP context activation is network-initiated, target-initiated, or not available. |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the target's MS. |
| Time of Location | C | Date/Time of Location. (if target location provided). |
| failed context activation reason | M | Information about the reason for failed context activation attempts of the target shall be provided. |
| umts QOS | C | Provide to identify the QOS parameters. |

Table 6.6: Location Information Update REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed IMEI |  |  |
| event type | M | Shall provide Location Information Update event type. |
| event date | M | Shall provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the target's MS. This parameter, in case of inter-SGSN RAU, will be sent only by the new SGSN. |
| Time of Location | C | Date/Time of Location. (if target location provided). |
| old location information | O | Provide (only by the old SGSN), when authorized and if available, to identify the old location information for the target's MS. |
| ldi event | O | Provide, when authorized, to indicate whether the target is entering or leaving the interception area (only applicable for location dependant interception). |

Location Information Update REPORT Record shall be sent in the following cases:

-	when the target's mobile station moves to the new SGSN;

-	optionally when the target's mobile station leaves the old SGSN.

Table 6.7: SMS-MO and SMS-MT Communication REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed IMEI |  |  |
| observed Non-Local ID |  |  |
| event type | M | Shall provide SMS event type. |
| event date | M | Shall provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| SMS originating address | O | Provide to identify the originating and destination address of the |
| SMS destination address |  | SMS message |
| location information | C | Provide, when authorized, to identify location information for the target's MS. |
| Time of Location | C | Date/Time of Location. (if target location provided). |
| SMS | C | Shall be provided, to deliver SMS content in case of IRI+CC interception, including header which is sent with the SMS-service. For intercepts requiring IRI only delivery, depending on national regulations, SMS content may be excluded by replacing all the octets of the TP-User-Data (TS 23.040 [XX] clause 9.2.3.24) with octets of value zero. The information about the other party involved in the SMS is included in the SMS TPDU (TS 23.040 [XX] clause 9.2). The parameter is conditional for backward compatibility reasons. |
| service centre address | C | If SMS content is not provided, shall be provided to identify the address of the relevant SMS-C server. If SMS content is provided, this parameter is optional. |
| SMS initiator | M | Shall be provided to indicate whether the SMS is MO, MT, or Undefined. |

Table 6.8: Serving System REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| observed IMEI |  |  |
| event type | M | Shall provide Serving System event type. |
| event date | M | Shall provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Network identifier of the HLR reporting the event (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| serving system identifier | M | Shall provide the VPLMN id (Mobile Country Code and Mobile Network Country, E. 212 number [87]). |

Table 6.9: Start Of Interception with mobile station attached REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed IMEI |  |  |
| event type | M | Shall provide Start Of Interception with mobile station attached event type. |
| event date | M | Shall provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the target's MS. |
| Time of Location | C | Date/Time of Location. (if target location provided). |

Start Of Interception with mobile station attached REPORT Record shall be sent in the following case:

-	the interception is activated any time after target's mobile station has successfully performed GPRS attach procedure.

When the ICE (i.e. SGSN, GGSN) is not aware of the activation of multiple lawfully authorized intercepts when the mobile station has already completed the GPRS attach procedures, the MF/DF shall generate the Start of Interception with mobile station attached REPORT record on its own using information that it has retained.

When the REPORT-record is used to convey the start of interception with mobile station attached, the DF2 shall not send the Start of Interception with mobile station attached REPORT record to the LEMFs that were already intercepting the target due previous LI activation on the same target.

Table 6.9A: Packet Data Header Information REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed IMEI |  |  |
| observed PDP address | C | If available, shall be provided in the following cases to identify the: - static address requested by the target's MS, and allocated by the Network for a successful PDP context activation. - address allocated dynamically by the network to the target MS in association with a PDP context activation (i.e. address is sent by the Network in an Activate PDP Context Accept) for a successful PDP context activation procedure when the PDP Context activation request does not contain a static PDP address. - address offered by the network in association with a network-initiated PDP context activation request when the target's MS accepts the network-initiated PDP context activation request. |
| event type | M | Shall provide the Packet Data Header Information event type. |
| event date | M | Shall provide the date and time the event is detected. |
| event time |  |  |
| access point name | C | If available, shall be provided to identify the packet data network to which the target is connected. |
| PDP type | C | When a PDP address is provided, shall provide the PDP type of the observed PDP address. The PDP Type defines the end user protocol to be used between the external packet data network and the MS. |
| network identifier | M | Shall be provided. |
| correlation number | M | Shall provide to uniquely identify the PDP context delivered to the LEMF used to correlate IRI records with CC. |
| lawful intercept identifier | M | Shall be provided. |
| packet data header information | M | Shall be provided to identify the packet header information to be reported on a per-packet basis as defined in Table 6.9B or on a summary basis. For summary reporting includes one or more packet flow summaries where each packet flow summary is associated with a particular packet flow as defined in Table 6.9C. |
| NSAPI | O | Provided for additional information. |

Table 6.9B: Contents of per-packet, packet data header information parameter

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| source IP address | C | Provide when mapping packet header information to identify the source IP address for a particular packet flow. |
| source port number | C | Provide when mapping packet header information to report the source port number for a particular packet flow when the transport protocol supports port numbers. |
| destination IP address | C | Provide when mapping packet header information to Identify the destination IP address for a particular packet flow. |
| destination port number | C | Provide when mapping packet header information to report the destination port number for a particular packet flow when the transport protocol supports port numbers. |
| transport protocol | C | Provide when mapping packet header information to identify the transport protocol (e.g. TCP) for a particular packet flow. |
| flow label | C | Provide when mapping packet header information for IPv6 only for a particular packet flow. |
| direction | M | Shall be provided. Identifies the direction of the packet (from target or to target). |
| packet size | O | Provide when mapping packet header information to convey the value contained in Total Length Fields of the IPv4 packets or the value contained in the Payload Length fields of the IPv6 packets. |
| packet data header copy | C | Provide when reporting a copy of the entire packet header information rather than mapping individual information and so it is alternative to the individual information. |

Table 6.9C: Contents of a single summary flow packet data header information parameter

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| source IP address | M | Shall be provided. Identifies the source IP address for a particular packet flow. |
| source port number | C | Provide to report the source port number for a particular packet flow when the transport protocol supports port numbers. |
| destination IP address | M | Shall be provided. Identifies the destination IP address for a particular packet flow. |
| destination port number | C | Provide to report the destination port number for a particular packet flow when the transport protocol supports port numbers. |
| transport protocol | M | Identifies the transport protocol (e.g. TCP) for a particular packet flow. |
| flow label | C | Provide for IPv6 only for a particular packet flow. |
| summary period | M | Shall provide the period of time during which the packets of a particular packet flow of the summary report were sent or received by the target and defined by specifying the time when the first packet and the last packet of the reporting period were detected. |
| packet count | M | Shall provide the number of packets detected for a particular packet flow. |
| sum of packet sizes | O | Provides the sum of values contained in Total Length Fields of the IPv4 packets or the sum of the values contained in the Payload Length fields of the IPv6 packets. |
| packet data summary reason | M | Shall provide the reason for the report being delivered to the LEMF (i.e. timeout, count limit, end of session). |

Table 6.9D: HLR subscriber record change REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| new observed MSISDN | C | Provide at least one and others when available. |
| new observed IMSI |  |  |
| new Observed IMEI |  |  |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| observed IMEI |  |  |
| event type | M | Shall provide HLR subscriber record change event type. |
| event date | M | Shall provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall provide the network identifier of the HLR reporting the event (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| changed (old/new) IMSI or MSISDN or IMEI | M | Shall provide what was changed (old/new MSISDN, old/new IMSI or old/new IMEI) |
| carrier Specific Data | O | Provide raw data of this specific update. |

Table 6.9E: Cancel location REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| event type | M | Shall provide cancel Location change event type. (purge from HLR sent to SGSN included). |
| event date | M | Shall provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall provide the network identifier of the HLR reporting the event (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| previous serving system identifier | C | If available, shall provide the previous VPLMN id (Mobile Country Code and Mobile Network Country, defined in E.212 [87]). |
| previous serving SGSN-Number | C | Provide the E.164 number of the previous serving SGSN, if available. |
| previous serving SGSN-Address | C | Provide the IP address of the previous serving SGSN, if available. |
| previous serving S4-SGSN-address | C | Provide the Diameter Origin-Host and Origin-Realm of the previous serving S4-SGSN, if available. |

Table 6.9F: Register location REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| event type | M | Shall provide register location event type. |
| event date | M | Shall provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall provide the network identifier of the HLR reporting the event (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| previous serving system identifier | C | If available, shall provide the previous VPLMN id (Mobile Country Code and Mobile Network Country; defined in E.212 [87]). |
| previous serving SGSN-Number | C | Provide the E.164 number of the previous serving SGSN, if available. |
| previous serving SGSN-Address | C | Provide the IP address of the previous serving SGSN, if available. |
| previous serving S4-SGSN-address | C | Provide the Diameter Origin-Host and Origin-Realm of the previous serving S4-SGSN, if available. |
| current serving system identifier | M | Shall provide the current VPLMN id (Mobile Country Code and Mobile Network Country, defined in E.212 [87]). |
| current serving SGSN-Number | C | Provide the E.164 number of the current serving SGSN, if available. |
| current serving SGSN-Address | C | Provide the IP address of the current serving SGSN, if available. |
| current serving S4-SGSN-address | C | Provide the Diameter Origin-Host and Origin-Realm of the current serving S4-SGSN, if available. |

Table 6.9G: Location information request REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| event type | M | Shall provide location information request event type. |
| event date | M | Shall provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall provide the network identifier of the HLR reporting the event (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| requesting network identifier | C | Provide the requesting network identifier PLMN id (Mobile Country Code and Mobile Network Country, defined in E.212 [87]). Editor's Note: The specific condition for this parameter needs clarification and consideration can be given to reporting whatever requesting network identifier is observed regardless of whether that identifier is a PLMN id in accordance with E212. |
| requesting node type | C | For GMSC; SMS Centre; GMLC, MME, SGSN, shall provide the requesting node type (GMSC; SMS Centre; GMLC, MME, SGSN) |

Table 6.9H: LALS Target Positioning REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed IMEI |  |  |
| event date | M | Shall provide the date and time the report is created. |
| event time |  |  |
| network identifier | M | Network identifier of the LI LCS Client (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide the LALS location information, if the positioning is successful. |
| Time of Location | C | Date/Time of Location. (if target location provided). |
| extended location parameters | O | If available, additional location information and associated QoS information. |
| LALS error code | C | Provide the error identification code, if the positioning is not successful. |

Table 6.9I: LALS Enhanced Location for IRI REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed IMEI |  |  |
| event date | M | Provide the date and time the LCS Report is available at LI LCS Client. |
| event time |  |  |
| network identifier | M | Network identifier of the LI LCS Client (Network element identifier included). |
| lawful intercept identifier | M | Shall be provided. |
| correlation number | C | Provided for correlation with the IRI records of the call, if available in the corresponding LALS triggering event. |
| location information | C | Provide the LALS location information, if the positioning is successful. |
| Time of Location | C | Date/Time of Location. (if target location provided). |
| extended location parameters | O | If available, additional location information and associated QoS information. |
| LALS error code | C | Provide the error identification code if the positioning is not successful. |

In the present document, the LALS Target Positioning reports are only sent when the target is successfully located. If the location information is not available, e.g. when the target is not attached to the network, no record is sent to the LEMF.

NOTE 1:	See the TS 33.107 [19] for a detailed description of LALS. See Annex O for information on using of the PS ASN.1 information object for the LALS reporting.

NOTE 2: 	In some specific scenarios the amount of Enhanced Location for IRI reports data may overload the X2 and/or HI2 interfaces. To prevent the overload, a flow control for Enhanced Location for IRI Reports may be implemented, e.g. by limiting the frequency of the reports for individual target.

*** NEXT CHANGE ***

## 7.5	IRI for IMS

### 7.5.0	Introduction

In addition, information on non-transmission related actions of a target constitute IRI and is sent via HI2, e.g. information on SIP message with call forwarding configuration information.

The IRI may be subdivided into the following categories:

1.	Control information for HI2 (e.g. correlation information).

2.	Basic data context information, for standard data transmission between two parties (e.g. SIP- or XCAP-message).

3.	Information needed to decrypt media traffic between the parties.

For each event, a Record is sent to the LEMF, if this is required. The following table gives the mapping between event type received at DF2 level and record type sent to the LEMF.

Table 7.1: Mapping between IMS Events and HI2 Records Type

| Event | IRI Record Type |
| --- | --- |
| SIP-message | REPORT |
| XCAP-request | REPORT |
| XCAP response | REPORT |
| Media decryption keys available | REPORT |
| Start of interception for already established IMS session | REPORT |
| Serving System | REPORT |
| Subscriber record change | REPORT |
| Registration Termination | REPORT |
| Location Information Request | REPORT |

A set of information is used to generate the record. The records used transmit the information from mediation function to LEMF. This set of information can be extended in the CSCF or DF2 MF, if new IEs are available and if this is necessary in a specific country. The following table gives the mapping between information received per event and information sent in records.

Once IRI only interception is underway, LEMF receives IMS specific IRI only (SIP IRI) from CSCF or IRI only (XCAP Message IRI) from the XCAP server managing the XCAP resource associated with the IMS supplementary service setting, or IRI only from the HSS. LEMF does not receive CC, and therefore it is not possible to correlate IMS specific IRI with CC.

Once IRI and CC interception is underway, LEMF receives IMS specific IRI both from a GSN and from a CSCF. LEMF receives SIP messages also from a GSN within CC. LEMF receives IRI of XCAP events from functions such as XCAP authentication and resource management function. In certain cases, however, SIP messages may be encrypted between UE and CSCF. XCAP message between the UE and the AS managing the target's IMS supplementary service settings may be encrypted. In these cases LEMF needs to receive unencrypted SIP or XCAP messages in IMS specific IRI provided from CSCF, or from the XCAP server managing the target's IMS supplementary service settings. The LI service delivery of XCAP events related to XCAP authentication process is for further study.

In some cases the CC is encrypted according to one of the IMS media security solutions specified in TS 33.328 [54]. In these cases the LEMF receives encrypted CC and decrypts it based on the decryption information received over the HI2 interface.

NOTE 0:	CC interception is not applicable at the HSS.

When the InstanceID is present in IMS signalling TS 24.229 [76], and contains an IMEI URN [81], [82], the IMEI shall be extracted and converted to the reporting format defined for partyInformation (imei).

NOTE 1:	Delivery of decrypted CC in the above scenario is FFS.

NOTE 1a:	GSN has no possibility to decrypt SIP messages based on the IMS security architecture.

NOTE 2:	Security mechanisms for protecting delivery of key material over the HI2 in line with TS 33.328 [54] are FFS.

NOTE 2a: When the CSCF is not aware of the activation of multiple lawfully authorized intercepts on a single target, the MF/DF needs to generate the REPORT with Start of Interception on an already established IMS session on its own using information that it has retained.

The DF2 shall not send the REPORT with Start of Interception with an already established IMS session to the LEMFs that were already intercepting the session due to a previous LI activation on the same target.

Table 7.2: Mapping between IMS Events Information and IRI Information

| Parameter | Description | HI2 ASN.1 parameter |
| --- | --- | --- |
| Observed SIP URI | Observed SIP URI | partyInformation (partyIdentity(sip-uri)) |
| Observed TEL URI | Observed TEL URI | partyInformation (partyIdentity(tel-uri)) |
| Observed IMEI | Observed IMEI | partyInformation (partyIdentity(imei)) |
| Observed IMPI | Observed IMPI (NOTE 12) | partyInformation (partyIdentity(impi)) |
| Observed IMSI | Observed IMSI (NOTE 12) | partyInformation partyIdentity( (imsi)) |
| Observed MSISDN | Observed MSISDN (NOTE 12) | partyInformation (partyIdentity(msISDN)) |
| Event type | IMS Event It indicates whether the IRI contains a CC unfiltered SIP message, a CC filtered SIP message, an XCAP request, an XCAP response, or the media decryption keys. For interception at the HSS, it indicates whether the IRI contains a Serving system, a Subscriber Record Change, a Registration Termination or a Location Information Request. | iMSevent |
| Event date | Date of the event generation in the CSCF or in the XCAP server managing the target's IMS supplementary service setting(s). | timeStamp |
| Event time | Time of the event generation in the CSCF or in the XCAP server managing the target's IMS supplementary service setting(s). |  |
| Network identifier | Unique number of the intercepting CSCF or the XCAP server managing the target's IMS supplementary service setting(s). | networkIdentifier |
| Correlation number | Unique number for each PDP context/Bearer delivered to the LEMF, to help the LEA, to have a correlation between each PDP Context/Bearer and the IRI. | gPRSCorrelationNumber |
| Correlation | Correlation number; unique number for each PDP context/Bearer delivered to the LEMF, to help the LEA, to have a correlation between each PDP Context/Bearer and the IRI. ASN.1 as: iri-to-CC Signalling PDP context/Bearer correlation number; unique number for signalling PDP context/Bearer delivered to the LEMF, to help the LEA, to have a correlation between each PDP Context/Bearer and the IRI. Used in the case two PDP contexts/Bearers are used. ASN.1 as: iri-to-CC SIP correlation number; either Call-id or some implementation dependent number that uniquely identify SIP messages of the same SIP session. ASN.1 as: iri-to-iri XCAP transaction correlation number: It correlates the XCAP request and reponse. | correlation |
| Lawful interception identifier | Unique number for each lawful authorization. | lawfulInterceptionIdentifier |
| SIP message | Either whole SIP message, or SIP message header (plus SDP body, if any). SIP message header (plus SIP message body part conveying IRI such as SDP) is used if warrant requires only IRI. In such cases, specific content in the SIP Message (e.g. 'Message', etc.) has to be deleted; unknown headers shall not be deleted. For intercepts requiring IRI only delivery, depending on national regulations, SMS content may be excluded by replacing all the octets of the TP-User-Data (TS 23.040 [XX] clause 9.2.3.24) with octets of value zero, while SMS headers (which convey information including originating and destination addresses, SMS centre address) are included, if available. Location information that the service provider is aware of (e.g. location in PANI header) is removed when delivery of such information is not lawfully authorized. | sIPMessage |
| Media-decryption-info | Session keys and additional info for the decryption of the CC streams belonging to the intercepted session. This field is present if available at the DF/MF | mediaDecryption-info Contain for each key the follow triplet: cCCSID, cCDecKey, cCSalt (optionally) |
| SIP message header offer | Header of the SIP message carrying the SDP offer (NOTE 10). | sipMessageHeaderOffer |
| SIP message header answer | Header of the SIP message carrying the SDP answer (NOTE 10). | sipMessageHeaderAnswer |
| SDP offer | SDP offer used for the establishment of the IMS session (NOTE 10). | sdpOffer |
| SDP answer | SDP answer used for the establishment of the IMS session (NOTE 10). | sdpAnswer |
| MediaSec key retrieval failure indication | Provides the information that the procedure to get encryption keys from the KMS failed. | mediaSecFailureIndication |
| PANI header information | Elements of P-Access-Network-Info headers in SIP message; defined in TS 24.229 [76] §7.2A.4. | pANI-Header-Info |
| XCAP message | XCAP message (i.e. to report separately the XCAP request and XCAP response between the UE and the XCAP server managing the XCAP resources of the target's IMS supplementary service setting(s); based on TS 24.623 [77]). | xCAPMessage |
| VoIP Roaming Indication | Applicable to IMS events related to VoLTE only. Indicates the roaming architecture in the VPLMN: Local Breakout (LBO) or S8HR (S8-reference point based home routing). | roamingIndication |
| Changed (old/new) IMSI or MSISDN/TEL URI/SIP URI/IMPI or IMEI | Provides the identity changes in Subscriber Record Change Event. | change-Of-Target-Identity |
| Other User Identities | Provides other IMPU or IMPI that was allocated to the Target being deregistered in HSS. | otherIdentities |
| Deregistration Reason | Provides the reason of de-registration in HSS Coded according to 3GPP TS 29.229 [96], values would be coded according to Reason-Code AVP when deregistration is initiated by HSS, and to Server-Assignment-Type AVP when indicated by SCSF. | deregistrationReason |
| Previous serving system identifier | Provides an identifier as defined in 3GPP TS 29.229 [96] that allows the home network to identify the previous visited network when deregistration is done. | visitedNetworkId |
| Current Serving System Identifier | Provides an identifier as defined in 3GPP TS 29.229 [96] that allows the home network to identify the current visited network. | visitedNetworkId |
| Other update | Carrier specific information related to implementation or subscription process on HSS. Raw data will be provided. CSP will provide to LEMF elements to understand such data. | carrierSpecificData |
| Requesting network identifier | The requesting network identifier PLMN id (Mobile Country Code and Mobile Network Code, defined in E.212 [87]). | requesting-Network-Identifier |
| Requesting node identifier | The requesting node identifier | requesting-Node-Identifier |
| Requesting node type | Type of requesting node such as MSC, SMS Centre, GMLC, MME, SGSN. | requesting-Node-Type |
| Location information | In case of S8HR, this parameter carries the UE location information that the LMISF receives from the MME through the S-GW/BBIFF. | ePSlocationOfTheTarget |
| Time of Location | Date/Time of location. The time when location was obtained by the location source node. | ePSlocationOfTheTarget |

NOTE 3:	Void.

NOTE 4: Void.

NOTE 5:	Void.

NOTE 6:	Void.

NOTE 7:	LIID parameter has to be present in each record sent to the LEMF.

NOTE 8:	Details for the parameter SIP message. If the warrant requires only signaling information, specific content in the parameter 'SIP message' like IMS (Immediate Messaging) has to be deleted/filtered. It should be noted that SDP content within SIP messages is reported even for warrants requiring only IRI.

NOTE 9:	In case of IMS event reporting involving the correlation number parameter, the gPRSCorrelationNumber HI2 ASN.1 parameter, which is also used in the IRIs coming from UMTS PS nodes, is used as container.

NOTE 10:	This parameter is applicable only in case of start of interception for an already established IMS session.

NOTE 11:	For separated IMS VoIP, the imsVoIP (as defined in clause 12) may be used instead of Correlation Number or Correlation shown in table 7.2.

NOTE 12: Applicable to HSS only.

pANI-header-info parameter includes elements present in the P-Access-Network-Info (PANI) header in intercepted SIP messages originated by the target's UE and handled by the CSCFs. The mediation function shall parse these intercepted SIP messages and copy from the PANI header the type/class of access and, if required by the warrant, location information in the related parameters specified in Annexes B.3 and B.9. In such case, the SIP messages carrying the PANI header shall also be sent to the LEMF unmodified.

In case the warrant does not require providing target's location information, any location information shall be filtered from the intercepted raw SIP messages, prior that these are delivered to the LEMF. In such case, as an implementation option, location information may be masked (e.g. filled with blanks or other characters) instead of filtered.

*** NEXT CHANGE ***

### 15.2.3	SMS over IMS

For separate delivery of SMS when SMS over IMS (using IMS SIP signalling handled by the core network) is used, the following REPORT Records shall be reported by DF2 to the LEMF:

1)	SMS over IP REPORT Record (see Table 15.2.3.1).

2)	HSS related REPORT Records:

a.	Serving System (6.5.1.1) or Serving Evolved Packet System (10.5.1.1) for use when roaming

b.	Registration termination or Cancel Location (6.1.1.1 or 10.5.1.1);

c.	Register location REPORT Record (6.5.1.1 or 10.5.1.1);

d.	Location information request REPORT Record (6.5.1.1. or 10.5.1.1).

The above REPORT Records shall be able to be reported from DF2 to the LEMF independent of any other services that may or may not be intercepted.

Table 15.2.3.1: SMS over IMS REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed SIP-URI | C | SIP URI of the target (if available). |
| observed TEL-URI | C | TEL URI of the target (if available). |
| observed IMEI | C | IMEI of the target (if available). |
| event type | M | Provide SMS over IP event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| PANI header information | C | Elements of P-Access-Network-Info header information in SIP messages; described in TS 24.229 [76] §7.2A.4. Provided if available and applicable. |
| SMS originating address | M | Shall be provided to identify the origination address for the SMS. |
| SMS destination address | M | Shall be provided to identify the destination address for the SMS. |
| SMS | C | Shall be provided, to deliver SMS content in case of IRI+CC interception, including header which is sent with the SMS-service. For intercepts requiring IRI only delivery, depending on national regulations, SMS content may be excluded by replacing all the octets of the TP-User-Data (TS 23.040 [XX] clause 9.2.3.24) with octets of value zero. The information about the other party involved in the SMS is included in the SMS TPDU (TS 23.040 [XX] clause 9.2). The parameter is conditional for backward compatibility reasons. |
| service centre address | M | Shall be provided. |
| SMS Initiator | M | Shall be provided to indicate whether the SMS is MO, MT, or Undefined. |
| location information | C | Provide, when authorized, to identify location information for the target's MS. |
| Time of Location | C | Date/Time of UE Location (if target location provided). |

*** END OF CHANGES ***