SA3LI#93	 S3i240241

Washington DC, April 16-19, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	pCR to TR 33.929-2: STIR/SHAKEN related LI reporting – Part-3-1 (signing required for intra-network IMS sessions, intra-network sessions, basic scenarios)

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR adds STIR/SHAKEN related LI reporting to illustrate xIRIs generated from the IRI-POIs of IMS LI and STIR/SHAKEN LI for basic intra-network sessions when signing is required for intra-network sessions.

BACKGROUND

This pCR illustrates the STIR/SHAKEN related LI reporting basic scenarios for intra-network sessions when signing is not required for intra-network sessions.

The clause numbers are adjusted according to the recommendations of split TR 33.929 (i.e. TR 33.929-2).

PROPOSAL

Incorporate the following to the TR 33.929-2.

*** The following (only clause headings are shown here) are in the TR 33.929-2 and other pCRs***

# 4 	STIR/SHAKEN related LI reporting

## 4.1	General

Text in TR 33.929-2.v001.

## 4.2	Background

Text in TR 33.929-2.v001.

## 4.3	IMS LI and STIR/SHAKEN LI

Text in TR 33.929-2.v001.

## 4.4	LI reporting details

### 4.4.1	General

### 4.4.2	General concept Overview

### 4.4.3	General principles of LI reporting

*** End of clause headings from TR 33.929-2 and other pCRs***

**** New text****

### 4.4.4	Signing required for intra-network IMS sessions

#### 4.4.4.1	General

The subsequent sub-clauses illustrate the scenarios when signing is required for intra-network IMS sessions.

When the intra-network signing is required, it is always the Telephony AS that interacts with the Signing AS and with Verification AS unless it is an emergency call (see clause 4.4.7).

The Telephony AS at the originating end of the call (i.e. Telephony AS associated with the calling party) interacts with the Signing AS and the Telephony AS at the terminating end (i.e. Telephony AS associated with the called party) of the call interacts with the Verification AS. The validation result is forwarded to the called party.

The IMS NFs that provide the STIR/SHAKEN related LI reporting are dependent on call scenario and the target.

The illustrations show the IMS LI reporting and STIR/SHAKEN related LI reporting as defined in TS 33.127 [3], TS 33.128 [4]. The TR 33.929-1 illustrates various scenarios for IMS LI with the default option.

The clause 4.3 of the present document indicate the conditions for an IMS NF to have the IRI-POI for the STIR/SHAKEN related reporting.

The Telephony AS on both ends of the call provide the LI functions for STIR/SHAKEN related reporting. It is assumed that all the PASSporTs that are validated are reported in the STIRSHAKENSignatureValdiation message to the LEMFs.

#### 4.4.4.2	Intra-network IMS sessions, basic scenarios

#### 4.4.4.2.1	General

The subsequent sub-clauses illustrate the STIR/SHAKEN related LI reporting along with the IMS LI for basic call scenarios (i.e. no redirections).

The calling party's identity is signed and included in the "shaken" PASSporT with calling party's identity in the origination claim and the called party's identity in the destination claim.

##### 4.4.4.2.2	Scenario 1: Party A (target) calls Party B

###### 4.4.4.2.2.1	The scenario

In this scenario, Party A calls Party B. Party A is the target.

Figure 4.4.4.2.2.1-1: Party A (target) calls Party B.

The clause 4.4.4.2.2.2 and 4.4.4.2.2.3 illustrate the LI aspects for this scenario.

###### 4.4.4.2.2.2	With default option for IMS LI

The illustration shown in figure 4.4.4.2.2.2-1 assumes the default option (see TS 33.127 [3]) for IMS LI and therefore, the S-CSCF of A provides the IRI-POI functions for IMS LI.

Figure 4.4.4.2.2.2-1: The LI aspects of scenario 1 (default option for IMS LI)

For target (Party A)

The IRI-POI present in the S-CSCF (of A) generates and delivers the xIRIIMSMessage to the MDF2 and the MDF2 forwards it as an IRI message IMSMessage to the LEMF.

The Telephony AS (of A) interacts with the Signing AS to authenticate/sign A’s identity and the IRI-POI present in the same Telephony AS generates and delivers the xIRISTIRSHAKENSignatureGeneration record with the "shaken" PASSporT for Party A to the MDF2. The MDF2 forwards it as an IRI message STIRSHAKENSignatureGeneration to the LEMF.

The IRI-POI present in the S-CSCF (of A) uses the P-Asserted ID or the From headers (pointing to Party A) present in the incoming SIP INVITE to perform a target match.

The IRI-POI present in the Telephony AS (of A) uses the P-Asserted ID or the From headers (pointing to Party A) present in the incoming SIP INVITE to perform a target match.

###### 4.4.4.2.2.3	With alternate option for IMS LI

The illustration shown in 4.4.4.2.2.3-1 assumes that the alternate option (see TS 33.127 [3]) for IMS LI and therefore, the P-CSCF of A provides the IRI-POI functions for IMS LI.

NOTE:	The alternate option for IMS LI is not shown in every illustration. It is shown here so as to illustrate influence it may have on the STIR/SHAKEN related LI reporting.

Figure 4.4.4.2.2.3-1: The LI aspects for scenario 1 (alternate option for IMS LI)

For target (Party A)

The IRI-POI present in the P-CSCF (of A) generates and delivers the xIRIIMSMessage to the MDF2 and the MDF2 forwards it as an IRI message IMSMessage to the LEMF.

The Telephony AS (of A) interacts with the Signing AS to authenticate/sign A’s identity and the IRI-POI present in the same Telephony AS generates and delivers the xIRISTIRSHAKENSignatureGeneration record with the "shaken" PASSporT for Party A to the MDF2. The MDF2 forwards it as an IRI message STIRSHAKENSignatureGeneration to the LEMF.

The IRI-POI present in the P-CSCF (of A) uses the P-Asserted ID or the From headers (pointing to Party A) present in the incoming SIP INVITE to perform a target match.

The IRI-POI present in the Telephony AS (of A) uses the P-Asserted ID or the From headers (pointing to Party A) present in the incoming SIP INVITE to perform a target match.

##### 4.4.4.2.3	Scenario 2: Party A calls Party B (target)

###### 4.4.4.2.3.1	The scenario

In this scenario, Party A calls Party B. Party B is the target.

Figure 4.4.4.2.3.1-1: Party A calls Party B (target )

The figures 4.4.4.2.3.2-1 and 4.4.4.2.3.3-1 illustrate the LI aspects for this scenario.

###### 4.4.4.2.3.2	With default option for IMS LI

The illustration shown in figure 4.4.4.2.3.2-1 assumes the default option (see TS 33.127 [3]) for IMS LI and therefore, the S-CSCF of B provides the IRI-POI functions for IMS LI.

Figure 4.4.4.2.3.2-1: The LI aspects for scenario 2 (default option for IMS LI)

For target (Party B)

The IRI-POI present in the S-CSCF (of B) generates and delivers the xIRIIMSMessage to the MDF2 and the MDF2 forwards it as an IRI message IMSMessage to the LEMF.

The Telephony AS (of B) interacts with the Verification AS to verify the received "shaken" PASSporT and the IRI-POI present in the same Telephony AS generates and delivers the xIRISTIRSHAKENSignatureValidation record to the MDF2. The MDF2 forwards it as an IRI message STIRSHAKENSignatureValidation to the LEMF.

The IRI-POI present in the S-CSCF (of B) uses the Request URI or the To headers (pointing to Party B) present in the incoming SIP INVITE to perform a target match.

The IRI-POI present in the Telephony AS (of B) uses the Request URI or the To headers (pointing to Party B) headers present in the incoming SIP INVITE to perform a target match.

###### 4.4.4.2.3.3	With alternate option for IMS LI

The illustration shown in 4.4.4.2.3.3-1 assumes that the alternate option (see TS 33.127 [3]) for IMS LI and therefore, the P-CSCF of B provides the IRI-POI functions.

NOTE:	The alternate option for IMS LI is not shown in every illustration. It is shown here so as to illustrate influence it may have on the STIR/SHAKEN related LI reporting.

Figure 4.4.4.2.3.3-1: The LI aspects for scenario 2 (alternate option for IMS LI

For target (Party B)

The IRI-POI present in the P-CSCF (of B) generates and delivers the xIRIIMSMessage to the MDF2 and the MDF2 forwards it as an IRI message IMSMessage to the LEMF.

The Telephony AS (of B) interacts with the Verification AS to verify the received "shaken" PASSporT and the IRI-POI present in the same Telephony AS generates and delivers the xIRISTIRSHAKENSignatureValidation record to the MDF2. The MDF2 forwards it as an IRI message STIRSHAKENSignatureValidation to the LEMF.

The IRI-POI present in the P-CSCF (of B) uses the Request URI or the To headers (pointing to Party B) present in the incoming SIP INVITE to perform a target match.

The IRI-POI present in the Telephony AS (of B) uses the Request URI or the To headers (pointing to Party B) headers present in the incoming SIP INVITE to perform a target match.

**** End of new text ****