3GPP SA3LI#93	s3i240219

16-19 April 2024, Washington, DC (US)

Source:	NDRE

Title:	AKMA LI status (and conclusion?)

Document for:	Discussion

Agenda Item:	4.2

Work Item / Release:	Release 18

Abstract of the contribution: This discussion (and informational) paper summarizes the AKMA LI results from SA3#115. It also suggests some considerations going forward.

1	Introduction and background

As is well known, with the Rel-18 addition of support for AKMA in roaming, there has been an intense exchange between SA3 and SA3LI to address the implied regulatory concerns, culminating in the LS s3i240084 from SA3LI#92 asking SA3 to, in lieu of other near-term solutions, add an AKMA usage control mechanism. SA3LI has now received a response LS 
(S3-241043/s3i240207) with an attached CR (S3-240915), for discussion at this meeting.

Behind the resulting SA3 LS/CR mentioned above, there were actually a surprisingly large number of contributions on the topic, which seems of interest to summarize. The discussions leading up to this point also provides a valuable basis for discussion, going forward (Rel-19+, 6G).

2.	Summary of AKMA LI-related inputs to SA3#115

The following documents were input to, and discussed at, SA3#115. Except possibly for the first, all the remaining eight are directly relevant for LI.

LS S3-240257 "LS on AKMA service restrictions in Rel-17" (CT3). Although not primarily focusing on LI, CT3 here asks about the need to restrict AKMA in roaming for Rel-17. The SA3 response S3-241042 (based on ZTE's S3-240453 and China Mobile's S3-240726) is that roaming is not "considered" in Rel-17.

LS S3-240264 "LS on AKMA service restrictions in roaming" (SA3LI). This was SA3LI's aforementioned LS which also contained a concrete proposal for a restriction mechanism: AUSF informs the AKMA AAnF via some form of "roaming indicator", and AAnF then applies policy to determine whether to restrict or not.

CR S3-240356 and mini-WID S3-240357, "AKMA service mid session disabling in roaming" (Nokia, Nokia Shanghai Bell, NDRE). These lift the issue that AKMA over non-3GPP access could initially falsely be treated as non-roaming and only later (when also 3GPP access is activated) be recognized as roaming. The CR proposes that AAnF, as soon roaming is detected, notifies those AFs that already received AKMA keys. Since enforcement is in the hands of the AF, the mechanism needs to assume the AF is trustworthy to meet LI requirements.

CR S3-240365 "AKMA service restriction in roaming" (NDRE, Ministère Economie et Finances, NTAC, Nokia, OTD_US, Security Service). This was a concrete CR by SA3LI participants implementing the ideas from the SA3LI LS above. Concretely, the AUSF informs ("pushes") the AAnF about the serving network name in conjunction with AKMA anchor key establishment. This adds some flexibility over a mere "roaming indicator" since it allows per-VPLMN authorization of AKMA usage. It also ensures AAnF has all needed information as early as possible.

CR S3-240454 and discussion S3-240455 "AKMA service restrictions" (ZTE). In this proposal, the AF requests to be notified about PLMN ID changes by the PCF/PCRF, akin to S3-240356. A major difference is however that S3-240356 only addressed mid-session concerns which seemingly cannot be handled in any other way, whereas here, all AKMA roaming control is delegated to the AF (which might even be outside 3GPP domain). Not acceptable from LI point of view.

CR S3-240708 "AKMA roaming policy control in AAnF" (China Mobile). This was similar in spirit to S3-240365, but rather than having the AUSF push the roaming information to AAnF in conjunction with AKMA anchor key establishment, the AAnF instead, in conjunction with key requests from AF, requests to subscribe to roaming status reports from the UDM. More details about a comparison of these two approaches are provided below.

Discussion S3-240757 "Proposal for a way forward on AKMA restrictions" (Ericsson). This contribution discussed and compared the push approach of S3-240365 and the subscribe/pull approach of S3-240708. However, Ericsson proposes to use another SBA service than the one in S3-240708.

3	Summary of discussions at SA3#115

Since it was quite clear that an entirely AF-centric approach such as that in S3-240454 is not viable to support LI concerns, a basis for a working roaming restriction mechanism at AAnF had to be based on S3-240365 (push) or S3-240708 (pull).

Ericsson's comparative analysis in S3-240757 suggested that reusing existing SBIs as-is would be preferred since it limits the need to affect CT3 specification, but is still not free of CT3 impact. For example,  in case of a reject to an external AKMA key request via NEF, the NEF most likely needs to handle a new error indication. Also, an advantage of the push approach is that when the AAnF receives a key request, it already has the information in needs in order to authorize the request—using on-demand SBI-call when the key request is received incurs some delay. However, this delay is not correlated with whether or not the request is related an LI target, so in that regard, both options seem acceptable. During the discussions, the majority of SA3 delegates seemed to prefer reusing the pre-defined SBA services in the pull fashion.

The remaining discussion focused on which SBA service to use, the choice lying between Nudm_UECM_Get (S3-240757) and Nudm_EventExposure_Subscribe (S3-240365). The majority of delegates preferred the latter and became the basis for the final CR.

It also seemed straight forward to merge the Nokia mid-session control solution from S3-240356 into the same CR, so this was also done. This seems to add some value to handle some additional corner-cases.

All in all, most delegates (including those also active in SA3LI) seemed satisfied with the overall outcome. There were some indications from a few delegates that there could be need to polish the solution by some small future CRs. It was unclear what remaining issues this referred to.

4	Going forward

It would of course have been even better if a solution could be found that actually permits AKMA encryption in roaming (like the one for the non-roaming case already specified in TS 33.127/128). But from LI perspective, the AKMA solution is now at least not causing regulatory compliance issues.

Going forward, it seems unlikely to be feasible to define a more satisfactory solution within Rel-19 since the work prioritization is done. Handling a more complete solution within "AKMA maintenance" also seems difficult, since according to the studies already done in SA3LI, a complete solution is rather complex, requiring at least

- defining protocols for key- and parameter transfer from HPLMN to VPLMN,

- most likely specifying new Ua* protocol cipher-suites,

- defining a "decryption middlebox" in the VPLMN.

Most likely, a more satisfactory solution is not feasible within remaining 5G standardization time-frame.

However, not all is lost since valuable knowledge of the issues relating to handling LI for encrypted roaming traffic has been gained (solution sketches to 1-3 above have been documented in SA3LI). It is proposed that SA3LI (directly or via the individual participants) at an early stage liaise with relevant stakeholders such as SA1, SA2, and SA3 in order to ensure that the LI issues in roaming are handled more robustly and "built-in" to the overall 6G architecture from start.