3GPP TSG-SA3 Meeting #93-LI	s3i240265

# Washington DC, United States, 16th Apr 2024 - 19th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0642 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding NTN TAI Info in NRLocation as specified in TS 29.571 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Ofcom (CH)) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI18 |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR proposes to add the NTN TAI Info as a new type NtnTaiInfo in the NRLocation structure of TS 33.128 clause 7.3.3.2.6. This new NR NTN TAI Information structure is specified in NGAP as a NAS related IE in TS 38.413 clause 9.3.3.53 and available in TS 29.571 clauses 5.4.4.9 NrLocation and described in clause 5.4.4.94 as the data type NtnTaiInfo. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new type NtnTaiInfo structure in the NRLocation structure of TS 33.128 clause 7.3.3.2.6 and add the NtnTaiInfo description in a new clause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specific Non-Terrestrial Network Tracking Area Identity cannot be delivered to the LEMF within the NRLocation structure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is associated with the following changes in the Forge: Merge request: !260 Commit hash: d4281be4dc5b186af1d337054d87d0e0b8b1de88 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

** First Change – Main Document**

##### 7.3.3.2.6	Type: NRLocation

The NRLocation type is derived from the data present in the NrLocation type defined in TS 29.571 [17] clause 5.4.4.9.

Table 7.3.3.2.6-1 contains the details for the NRLocation type.

Table 7.3.3.2.6-1: Definition of type NRLocation

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| tAI | TAI | 1 | Tracking Area Identity of the target. If the TAI information is not available, the TAC of the TAI shall be set to one reserved value (e.g. 0x0000, see TS 23.003 [19] clause 19.4.2.3) and the value of the ignoreTAI parameter shall be set to TRUE. | M |
| nCGI | NCGI | 1 | NR Cell Identity for the cell where the target is located. | M |
| ageOfLocationInfo | AgeOfLocation | 0..1 | The value represents the elapsed time in minutes since the last network contact of the mobile station. Shall be present if known at the NF where the POI is located. | C |
| uELocationTimestamp | Timestamp | 0..1 | The value represents the UTC time when the NRLocation information was acquired. Shall be present if known at the NF where the POI is located. | C |
| geographicalInformation | UTF8String | 0..1 | Shall be present if known at the NF where the POI is located. If present, this parameter shall be populated with the Hexadecimal value of the location encoded as described in TS 23.032 [104] clauses 6 and 7.3. | C |
| geodeticInformation | UTF8String | 0..1 | Shall be present if known at the NF where the POI is located. If present, this parameter shall be populated with the Hexadecimal value of the location encoded as described in ITU-T Recommendation Q.763 (1999) [105] clause 3.88. | C |
| globalGNbID | GlobalRANNodeID | 0..1 | Indicates the global identity of the gNodeB in which the UE is currently located. Shall be present if known at the NF where the POI is located. | C |
| cellSiteInformation | CellSiteInformation | 0..1 | Contains location information for the cell site being reported. Shall be present if known at the NF where the POI is located or known at the MDF. | C |
| ignoreNCGI | BOOLEAN | 0..1 | This flag, when present, shall indicate if the nCGI shall be ignored. When present, it shall be set as follows: - TRUE: nCGI shall be ignored. - FALSE: nCGI shall not be ignored. | C |
| ntnTaiInfo | NtnTaiInfo | 0..1 | Contains the NR Non-Terrestrial Network Tracking Area Identity Information. When present, the tAI attribute shall be ignored, see clause 9.3.3.53 of 3GPP TS 38.413 [11]. | C |

** End of First Change – Main Document **

** Second Change – Main Document **

##### 7.3.3.2.X	Type: NtnTaiInfo

The NtnTaiInfo type is used to report the Tracking Area Identity from Non-Terrestrial Network. The NtnTaiInfo type is derived from the data present in the NtnTaiInfo type defined in TS 29.571 [17] clause 5.4.4.94.

When the NtnTaiInfo type is present in a NRLocation structure, the TAI type shall be ignored.

Table 7.3.3.2.X-1 contains the details of the NtnTaiInfo type.

Table 7.3.3.2.X-1: Structure of the NtnTaiInfo type

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| pLMNID | PLMNID | 1 | UE’s serving PLMN Identity. | M |
| tacList | TACList | 1..N | TAC list received from satellite NG-RAN without the forbidden TAIs for the UE. | M |
| derivedTac | TAC | 0..1 | May be present if the derived TAC is received from satellite NG-RAN. | C |

** End of Second Change – Main Document **

** Third Change - Attachment**

diff --git a/33128/r18/TS33128Payloads.asn b/33128/r18/TS33128Payloads.asn

index 3c66b8de17eda334c41d996c8d2229a001d4d82c..a1871068675b3a82e9f7778a777a7c3ea3c198c8 100644

--- a/33128/r18/TS33128Payloads.asn

+++ b/33128/r18/TS33128Payloads.asn

@@ -6544,7 +6544,8 @@ NRLocation ::= SEQUENCE

geodeticInformation         [6] UTF8String OPTIONAL,

globalGNbID                 [7] GlobalRANNodeID OPTIONAL,

cellSiteInformation         [8] CellSiteInformation OPTIONAL,

-    ignoreNCGI                  [9] BOOLEAN OPTIONAL

+    ignoreNCGI                  [9] BOOLEAN OPTIONAL,

+    ntnTaiInfo                  [10] NtnTaiInfo OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.10

@@ -6673,6 +6674,13 @@ RAI ::= SEQUENCE

RAC ::= OCTET STRING (SIZE(2))

+NtnTaiInfo ::= SEQUENCE

+{

+    pLMN                        [1] PLMNID,

+    tACList                     [2] TACList,

+    derivedTAC                  [3] TAC OPTIONAL

+}

+

-- TS 29.571 [17], clause 5.4.4.5

ECGI ::= SEQUENCE

{

** End of Third Change - Attachement**

** End of All Changes **