3GPP TSG-SA3 Meeting #93-LI	s3i240279

# Washington DC, United States, 16th Apr 2024 - 19th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0648 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of target identifier field to CC records |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI(OTD_US) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI18 |  |  |  |  |  | Date: |  |  | 2024-04-04 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The IRI records currently defined have a parameter to carry the target identifiers known at the MDF for the target. Additionally, this field contains provenance for the identifiers. This field provides additional insight into the various IDs that appear in a record and whether they were used to isolate the traffic or were only merely present in the record. This information is equally relevant for CC, however there is not an equivalent field in the CC records. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds a field to the CC record for Target Identifiers. Additionally, the clause describing the use of the IRI Target Identifier parameter is updated to include details for CC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may be unclear what IDs were used to isolate the traffic and how they relate to other IDs in the network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.5, TS33128Payloads.asn |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is associated with the following changes in the Forge: Merge request: !261 Commit hash: c326cc1ee7d1eed6c386c95d696360f0d1b66c0b |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### **** START OF FIRST CHANGE (MAIN DOCUMENT) ****

### 5.5.5	IRI and CC Target Identifiers

The MDF shall populate the  IRIPayload.targetIdentifiers or CCPayload.targetIdentifiers field defined in Annex A with all Target Identifiers available at the MDF.

Table 5.5.5-1 shows details for the usage of the TargetIdentifierProvenance enumerations to populate the IRIPayload.targetIdentifiers.provenance or CCPayload.targetIdentifiers.provenance fields.

Table 5.5.5-1: Details for TargetIdentifierProvenance enumeration usage

| Case | Enumeration |
| --- | --- |
| For identifiers received in the "Matched Target Identifier" conditional attribute (see clause 5.3.2) of an xIRI or xCC message | matchedOn(3) |
| For identifiers received in the "Other Target Identifier" conditional attribute (see clause 5.3.2) of an xIRI or xCC message | other(4) |
| For identifiers present in the xIRI or xCC payload | observed(2) |
| For identifiers received in the provisioning message received over LI_X1 | lEAProvided(1) |
| For any other identifier known at the MDF | other(4) |

## **** END OF CHANGES (MAIN DOCUMENT) ***

## **** START OF FIRST CHANGE (ATTACHMENTS) ****

---a/33128/r18/TS33128Payloads.asn
+++b/33128/r18/TS33128Payloads.asn

@@ -541,7 +541,8 @@ IRITargetIdentifier ::= SEQUENCE

541	541		CCPayload ::= SEQUENCE

542	542		{

543	543		    cCPayloadOID         [1] RELATIVE-OID,

544		-	    pDU                  [2] CCPDU

544	+	    pDU                  [2] CCPDU,

545	+	    targetIdentifiers    [3] SEQUENCE OF CCTargetIdentifier OPTIONAL

545	546		}

546	547

547	548		CCPDU ::= CHOICE

@@ -561,6 +562,12 @@ CCPDU ::= CHOICE

561	562		    rCSCCPDU            [7] RCSCCPDU

562	563		}

563	564

565	+	CCTargetIdentifier ::= SEQUENCE

566	+	{

567	+	    identifier                                          [1] TargetIdentifier,

568	+	    provenance                                          [2] TargetIdentifierProvenance OPTIONAL

569	+	}

570	+

564	571		-- ===========================

565	572		-- HI4 LI notification payload

566	573		-- ===========================

## **** END OF ALL CHANGES ***