3GPP TSG-SA3 Meeting #93-LI	s3i240289

# Washington DC, United States, 16th Apr 2024 - 19th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0656 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to service scoping at the LMISF-IRI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI(OTD_US) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI17 |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current text describing the service scoping flags that must be supported for Home Routed Roaming reporting has some service scoping options that do not apply. Additionally, some of the service scoping options that should be supported are not listed. Additionally, the target match logic is worded in a way that may be confusing to some LEAs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrects the list of servic scoping options the LMISF-IR must support. Additionally, the target match logic is rearranged slightly for clarity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The LMISF-IRI will have to support service scoping options that are not applicable to Home Routed IMS traffic and some service scoping options that should be supported may not be. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.10.4.2.2, 7.10.4.2.3, 7.10.4.2.4, 7.10.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## **** START OF FIRST CHANGE (MAIN DOCUMENT) ****

##### 7.10.4.2.2	Provisioning of LMISF-IRI

The LMISF-IRI shall be provisioned over LI_X1 by the LIPF for target based interception of IMS services in the VPLMN with home-routed roaming.

The target identities listed in clause 7.10.4.2.1 shall apply for the provisioning of LMISF-IRI with LMISF-IRI playing the combined role of IRI-POI and CC-TF for the interception of IMS-based services in the VPLMN with home-routed roaming.

The LMISF-IRI shall support service scoping using the following carrier provided service types (see clause 4.4.2) signaled using the structure defined in ETSI TS 103 221-1 [7]:

-	Voice

-	Messaging

-	If no service type is provisioned, the LMISF-IRI shall generate all xIRI messages for both voice and messaging.

Table 7.10.4.2-1 shows the minimum details of the LI_X1 ActivateTask message used for provisioning the LMISF-IRI for Phase-2.

Table 7.10.4.2-1: ActivateTask message for activating LMISF-IRI for Phase-2

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| XID | XID assigned by LIPF. The value used here is different from the value used in ActivateTask shown in table 7.10.3.2-4. | M |
| TargetIdentifiers | One or more of the target identifiers listed in clause 7.10.4.2.1. | M |
| DeliveryType | Set to “X2Only”, “X3Only” or “X2andX3” as needed to meet the requirements of the warrant. | M |
| ListOfDIDs | Delivery endpoints of LI_X2 or LI_X3. These delivery endpoints shall be configured using the CreateDestination message as described in ETSI TS 103 221-1 [7] clause 6.3.1 prior to first use. | M |
| ListOfServiceTypes | Indicates that the LMISF-IRI shall only deliver messages related to the indicated services. See ETS TS 103 221-1 [7] Table 4. When a single task is used to support multiple intercepts on a target identifier, the service scoping shall be the union of all of them. | C |

##### 7.10.4.2.3	Provisioning of the MDF2

The MDF2 listed as the delivery endpoint over LI_X2 for xIRI generated by the LMISF-IRI shall be provisioned over LI_X1 by the LIPF.

The target identities listed in clause 7.10.4.2.1 shall apply for the provisioning of MDF2.

The MDF2 shall support the following service scoping options (see clause 4.4) for IRI reported out of the LMISF-IRI signaled using the ServiceScopingOptions structure of the MediationDetails defined in ETSI TS 103 221-1 [7] Annex C:

-	CSP service types (signalled as ServiceType)

-	Voice.

-	Messaging.

-	If no service type is provisioned, the MDF2 shall generate all IRI messages for xIRI records it receives.

-	Location Reporting (signalled as LocationType)

-	Report location only at the beginning and end of a session (signalled as "reportBeginningAndEnd")

-	Do not report location (signalled as "doNotReport")

-	When no location reporting type is provisioned, the MDF2 shall report location every time the target location information is received.

Table 7.10.4.2-2 shows the minimum details of the LI_X1 ActivateTask message used for provisioning the MDF2.

Table 7.10.4.2-2 ActivateTask message for MDF2

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| XID | XID assigned by LIPF. | M |
| TargetIdentifiers | One or more of the target identifiers listed in clause 7.10.4.2.1. | M |
| DeliveryType | This value shall be Ignored by the MDF2. | M |
| ListOfDIDs | Delivery endpoints of LI_HI2. These delivery endpoints shall be configured using the CreateDestination message as described in ETSI TS 103 221-1 [7] clause 6.3.1 prior to first use. | M |
| ListOfMediationDetails | Sequence of Mediation Details, See table 7.10.4.2-3. | M |

Table 7.10.4.2-3: Mediation Details for MDF2

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| LIID | Lawful Intercept ID associated with the task. | M |
| DeliveryType | Set to "HI2Only". | M |
| ListOfDIDs | Details of where to send the IRI for this LIID. Shall be included if deviation from the ListofDIDs in the ActivateTask message is necessary. If included, the ListOfDIDs in the Mediation Details shall be used instead of any delivery destinations authorised by the ListOfDIDs field in the ActivateTask Message. | C |
| ServiceScopingOptions | See ETS TS 103 221-1 [7] Table 4. Shall be present if there is no default setting in the MDF or to override the default setting for this LIID. | C |

##### 7.10.4.2.4	Provisioning of the MDF3

The MDF3 listed as the delivery endpoint over LI_X3 for xCC generated by the LMISF-CC shall be provisioned over LI_X1 by the LIPF.

The target identities listed in clause 7.10.4.2.1 shall apply for the provisioning of MDF3.

The MDF3 shall support the following service scoping options (see clause 4.4) for CC reported out of the LMISF-CC signaled using the ServiceScopingOptions structure of the MediationDetails defined in ETSI TS 103 221-1 [7] Annex C:

-	CSP service types (signalled as ServiceType)

-	Voice.

-	Messaging.

-	If no service type is provisioned, the MDF3 shall generate all CC messages for xCC records it receives.

-	Location Reporting (signalled as LocationType)

-	Report location only at the beginning and end of a session (signalled as "reportBeginningAndEnd")

-	Do not report location (signalled as "doNotReport")

-	When no location reporting type is provisioned, the MDF3 shall report location every time the target location information is received.

Table 7.10.4.2-4 shows the minimum details of the LI_X1 ActivateTask message used for provisioning the MDF3.

Table 7.10.4.2-4 ActivateTask message for MDF3

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| XID | XID assigned by LIPF. | M |
| TargetIdentifiers | One or more of the target identifiers listed in clause 7.10.4.2.1. | M |
| DeliveryType | This value shall be Ignored by the MDF3. | M |
| ListOfDIDs | Delivery endpoints of LI_HI3. These delivery endpoints shall be configured using the CreateDestination message as described in ETSI TS 103 221-1 [7] clause 6.3.1 prior to first use. | M |
| ListOfMediationDetails | Sequence of Mediation Details, See table 7.10.4.2-5. | M |

Table 7.10.4.2-5: Mediation Details for MDF3

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| LIID | Lawful Intercept ID associated with the task. | M |
| DeliveryType | Set to "HI3Only". | M |
| ListOfDIDs | Details of where to send the IRI for this LIID. Shall be included if deviation from the ListofDIDs in the ActivateTask message is necessary. If included, the ListOfDIDs in the Mediation Details shall be used instead of any delivery destinations authorised by the ListOfDIDs field in the ActivateTask Message. | C |
| ServiceScopingOptionsa | See ETS TS 103 221-1 [7] Table 4. Shall be present if there is no default setting in the MDF or to override the default setting for this LIID. | C |

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

##### 7.10.4.3.2	Target match

###### 7.10.4.3.2.1	General

When the LMISF-IRI extracts a SIP REGISTER request, the LMISF-IRI examines the following for a target match:

-	From header and To header of the SIP REGISTER when the target identity is IMPU.

-	SUPI or IMSI stored in the LI_X2_LITE record when the target identity is IMPI.

-	+sip.instance-id of Contact header of the SIP REGISTER when the target identity is PEIIMEI or IMEI.

The LMISF-IRI shall store the +sip.instance-id in the LI_X2_LITE_S record for later use.

###### 7.10.4.3.2.2	LMISF-IRI provisioned for Voice

The cases below apply when the LMISF-IRI has been provisioned to deliver LI product for the CSP service type (see clause 4.4.2) of voice.

When the LMISF-IRI extracts a SIP INVITE:

-	The LMISF-IRI

examines the following to determine whether the originating party is a target match:

-	P-Preferred Identity header and From header present in the SIP INVITE when the target identity is IMPU.

-	Request URI header and To header present in the SIP INVITE when the target identity is IMPU and target is non-local ID.

-	SUPI or IMSI stored in the LI_X2_LITE record when the target identity is IMPI.

-	+sip.instance-id of Contact header received in the SIP REGISTER request when the target identity is PEIIMEI or IMEI.

-	P-Asserted-Id, From header, History Info header and Diversion header present in the SIP header when the target identity is IMPU and target is non-local ID.

-	The LMISF-IRI examines the following to determine whether the termination party is a target match:

-	Request URI header and To header present in the SIP INVITE when the target identity is IMPU and target is non-local ID.

-	Request URI and To header present in the SIP INVITE when the target identity is IMPU.

-	P-Asserted-Id, From header, History Info header and Diversion header present in the SIP header when the target identity is IMPU and target is non-local ID.

-	SUPI or IMSI stored in the LI_X2_LITE record when the target identity is IMPI.

-	+sip.instance-id of Contact header received in the SIP REGISTER request when the target identity is PEIIMEI or IMEI.

The LMISF-IRI may use the Via header or the Route header to determine whether the SIP INVITE is for an originating IMS session or a terminating IMS session. LMISF-IRI stores the SIP Call Id to associate the subsequent SIP messages received on the same session for a target match.

For subsequent SIP messages, the LMISF-IRI may use the stored LI_X3_LITE_S record to determine for a target match.

###### 7.10.4.3.2.3	LMISF-IRI provisioned for messaging

The cases below apply when the LMISF-IRI has been provisioned to deliver LI product for the CSP service type (see clause 4.4.2) of messaging.

When the LMISF-IRI extracts a SIP MESSAGE, the LMISF-IRI examines the following to determine if there is a target match:

-	For MO-SMS over IP, P-Preferred Identity header and From header present in the SIP MESSAGE when the target identity is IMPU.

-	For MO-SMS over IP, TP-DA field of SMS-SUBMIT within the Message-body of SIP MESSAGE when the target identity IMPU for target non-local ID.

-	For MT-SMS over IP, the Request URI and To header present in the SIP MESSAGE when the target identity is IMPU.

-	For MT-SMS over IP, the TP-OA field or TP-RA field of SMS-SUBMIT within the Message-body SIP MESSAGE when the target identity IMPU for target non-local ID.

-	SUPI or IMSI stored in the LI_X2_LITE record when the target identity is IMPI.

-	+sip.instance-id of Contact header received in the SIP REGISTER request when the target identity is PEIIMEI or IMEI.

LMISF-IRI may use the Via header or the Route header to determine whether the SIP MESSAGE is for MO-SMS over IP or MT-SMS over IP.

## **** END OF CHANGES (MAIN DOCUMENT) ***