3GPP TSG-SA3 Meeting #93-LI	s3i240309

# Washington DC, United States, 16th Apr 2024 - 19th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0632 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of parameter indicating the version of xIRI an IRI is generated from |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD_US, Ministère Economie et Finances) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI18 |  |  |  |  |  | Date: |  |  | 2024-04-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently, both xIRI and IRI records contain the OID of the ASN.1 module used to encode the record. In some cases, this OID is useful to determine which fields are expected to be present in a record or which events can be expected to be reported by the network. The issue is that the OID sent by the MDF in IRI is based on the version used by the MDF to encode the IRI record and does not necessarily reflect the version used at the POI to generate the xIRI. If the versions are mismatched, this may result confusion when certain fields or records that would be expected based on the MDF version are not supported by the POI. Additionally, the existing xCC did not specify that a timestamp and sequence number should be present in the xCC record. The inclusion of these values makes LI_X3 more reliable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An additional parameter is added to the IRIPayload sequence and text describing its use is added to the document. Clarifies that the timestamp and sequence number conditional attributes should be present in xCC records. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The LEA may have difficulty determining what events/parameters are to be expected resulting in LI product being misinterpreted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3, 5.5.1,5.5.2, 5.5.3, TS33128Payloads.asn |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | While the changes in this CR are mostly functionally the same as those in TDoc s3i240223 (CR 0629, Release 15), TDoc s3i240225 (CR 0630, Release 16), and TDoc s3i240225 (CR 0631, Release 17), this CR is not a mirror due to the inclusion of additonal tables and a type to carry additional mediated from types. This CR is associated with the following changes in the Forge: Merge request: !262 Commit hash: 634600a82642b09d360c8270b16a90869682cdd8 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3i240226 |  |  |  |  |  |  |  |  |

## **** START OF FIRST CHANGE (MAIN DOCUMENT) ****

## 5.3	Protocols for LI_X2 and LI_X3

### 5.3.1	General usage of ETSI TS 103 221-2

Functions having an LI_X2 or LI_X3 interface shall support the use of ETSI TS 103 221-2 [8] to realise the interface.

In the event of a conflict between ETSI TS 103 221-2 [8] and the present document, the terms of the present document shall apply.

Table 5.3.1-1 shows the minimum details of xIRI records sent over LI_X2 and xCC messages sent over LI_X3 in addition to those described in 103 221-2 [8].

Table 5.3.1-1: LI_X2/LI_X3 message details

| ETSI TS 103 221-2 [8] field name | Description |
| --- | --- |
| XID | Shall contain the appropriate XID as received in the relevant LI_X1 provisioning message (or LI_T2/LI_T3 triggering message, as appropriate), noting that the appropriate XID may be given in the ProductID field. |
| Correlation ID | Shall contain the Correlation ID generated by the IRI-POI or as received in the relevant LI_X1 provisioning message (or LI_T2/LI_T3 triggering message, as appropriate). |
| Conditional Attribute Fields | Additional details for conditional attribute fields are provided in Table 5.3.1-2. |

Table 5.3.1-2 shows the minimum details for the Conditional Attribute Fields included in xIRI records sent over LI_X2 and xCC messages sent over LI_X3 in addition to those described in 103 221-2 [8].

Table 5.3.1-2: LI_X2/LI_X3 conditional attributes

| ETSI TS 103 221-2 [8] Conditional Attribute Type | Description |
| --- | --- |
| Network Function ID (NFID) | Unless otherwise specified, the NFID conditional attribute (see ETSI TS 103 221-2 [8] clause 5.3.7) shall be set to indicate the NF that contains the POI. The NFID is defined as a unique identifier assigned to the NF by the network (e.g. FQDN) per carrier implementation and referred to in the following clauses. |
| Interception Point ID (IPID) | Unless otherwise specified, the IPID conditional attribute (see ETSI TS 103 221-2 [8] clause 5.3.8) shall be set to indicate the POI (within the NF) that generated the xIRI for the conditional attribute field. |

Additional details specific to xIRI records can be found in clause 5.3.2.

Additional details specific to xCC messages can be found in clause 5.3.3.

### 5.3.2	Usage for realising LI_X2

Table 5.3.2-1 shows the minimum details of xIRI records sent over LI_X2 in addition to those described in 103 221-2 [8] and clause 5.3.1.

Table 5.3.2-1: Additional xIRI record details

| ETSI TS 103 221-2 [8] field name | Description |
| --- | --- |
| PDU Type | The PDU Type field (see ETSI TS 103 221-2 [8] clause 5.1) shall be set to "X2 PDU" (see ETSI TS 103 221-2 [8] clause 5.2.2). |
| Payload Direction | Where a single xIRI is sent as a result of a network procedure (i.e. as result of several signalling messages exchanged between the target UE and the network), the Payload Direction field (see ETSI TS 103 221-2 [8] clause 5.2.6) shall be set based on the initiator of the network procedure. |
| Payload Format | Unless otherwise specified by the relevant clause, the payload format (see ETSI TS 103 221-2 [8] clauses 5.2.5 and 5.4) shall be set to the value 2. If the Payload does not consist of a BER-encoded @TS33128Payloads.XIRIPayload structure, the Payload Format (see ETSI TS 103 221-2 [8] clauses 5.2.5 and 5.4) shall be set according to the relevant clause of the present document. |
| Payload | Unless otherwise specified by the relevant clause, the payload shall consist of a BER-encoded @TS33128Payloads.XIRIPayload (See Table 5.3.2-3 ) structure. |
| Conditional Attribute Fields | Additional details for conditional attribute fields are provided in Table 5.3.2-2. |

Table 5.3.2-2 shows the minimum details for the Conditional Attribute Fields included in xIRI records sent over LI_X2 and xCC messages sent over LI_X3 in addition to those described in 103 221-2 [8] and Table 5.3.1-2.

Table 5.3.2-2: Additional xIRI conditional attributes

| ETSI TS 103 221-2 [8] Conditional Attribute Type | Description |
| --- | --- |
| Timestamp | Unless otherwise specified, the Timestamp (see ETSI TS 103 221-2 [8] clause 5.3.10) shall be present and set to the time at which the event occurred. |
| Sequence Number | Unless otherwise specified, the Sequence Number (see ETSI TS 103 221-2 [8] clause 5.3.9) shall be present. |
| Matched Target Identifier | Shall be set to indicate what target identity was matched to generate the xIRI (see ETSI TS 103 221-2 [8] clause 5.3.18). |
| Other Target Identifier | Shall be set with all other target identities present at the NF that contains the POI (see ETSI TS 103 221-2 [8] clause 5.3.19). |

Table 5.3.2-3 shows details for the @TS33128Payloads.XIRIPayload.

Table 5.3.2-3: Payload details for xIRIPayload

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| xIRIPayloadOID | RELATIVE-OID | 1 | Shall be populated with the value of the xIRIPayloadOID specified in the version of the ASN.1 used by the IRI-POI to generate the xIRI record. | M |
| event | XIRIEvent | 1 | Shall be populated with the event specified in the relevant clause of the present document. | M |

The TLS transport profile (see ETSI TS 103 221-2 [8] clause 6) shall be supported and used by default.

### 5.3.3	Usage for realising LI_X3

Table 5.3.3-1 shows the minimum details of xCC messages sent over LI_X3 in addition to those described in 103 221-2 [8] and clause 5.2.1.

Table 5.3.3-1: Additional xCC message details

| ETSI TS 103 221-2 [8] field name | Description |
| --- | --- |
| PDU Type | The PDU Type field (see ETSI TS 103 221-2 [8] clause 5.1) shall be set to "X3 PDU" (see ETSI TS 103 221-2 [8] clause 5.2.2). |
| Payload Direction | The payload direction shall be set as specified in the relevant clause of the present document or based on the direction of the intercepted payload. |
| Payload Format | The payload format shall be set as specified in the relevant clause of the present document. |
| Payload | The payload shall be populated as specified in the relevant clause of the present document. |
| Conditional Attribute Fields | Additional details for conditional attribute fields are provided in Table 5.3.3-2. |

Table 5.3.3-2: Additional xCC conditional attributes

| ETSI TS 103 221-2 [8] Conditional Attribute Type | Description |
| --- | --- |
| Timestamp | Unless otherwise specified, the Timestamp (see ETSI TS 103 221-2 [8] clause 5.3.10) shall be present and set to the time at which the xCC is generated. |
| Sequence Number | Unless otherwise specified, the Sequence Number (see ETSI TS 103 221-2 [8] clause 5.3.9) shall be present. |

If defined by LI for a specific 3GPP-defined-network deployment (see clause 6) or a specific 3GPP-defined service (see clause 7), the POI may use the Additional XID Related Information attributes to facilitate efficient delivery of xCC, as specified in ETSI TS 103 221-2 [8] clause 5.3.22.

NOTE:	ETSI TS 103 221-2 [8] specifies in clause 6 a default profile which is mandatory to support, but allows further profiles to be defined. In scenarios where it may not be possible to achieve the necessary LI data rates based on the default profile, alternative profiles may be considered (e.g. based on UDP, multi path TCP or other protocols). Any alternative profile needs to ensure that LI reliability, security and completeness requirements as specified in TS 33.126 [3] are met.

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

### 5.5.1	General

Functions having an LI_HI2 or LI_HI3 interface shall support the use of ETSI TS 102 232-1 [9] and ETSI TS 102 232-7 [10] to realise the interface.

In the event of a conflict between either specification and the present document, the terms of the present document shall apply.

Messages sent over LI_HI2 or LI_HI3 are structured as a header and a payload.

The header contains general information like LIID, timestamp, correlation information. Table 5.5.1-1 shows the minimum header details for IRI messages sent over LI_HI2 and CC messages sent over LI_HI3 in addition to those described in 102 232-1 [9] and ETSI TS 102 232-7 [10].

Table 5.5.1-1: LI_HI2/LI_HI3 message header details

| ETSI TS 102 232-1 [9] field name | Description |
| --- | --- |
| Lawful Interception Identifier (LIID) | Shall be set based on agreement between the network operator and the LEA. See ETSI TS 102 232-1 [9] clause 5.2.2. |
| Communication Identifier | Contains correlation information. Unless otherwise specified, shall be present and populated as described in the relevant clause of the present document. See ETSI TS 102 232-1 [9] clause 5.2.4. |
| Timestamp | Unless otherwise specified, the timestamp shall be set to the time the event was observed at the POI (i.e. the timestamp field of the xIRI or xCC). See ETSI TS 102 232-1 [9] clause 5.2.6. |
| Timestamp Qualifier | Unless otherwise specified, the timestamp qualifier (see ETSI TS 102 232-1 [9] clause 5.2.6) shall be present. |
| Network Function Identifier | The Network Function Identifier (see ETSI TS 103 232-1 [9] clause 5.2.14 and ETSI TS 102 232-7 [10] clause 15.3) shall be populated with a value mapped from the NFID conditional attribute field (see Table 5.3.1-2) if the message received over LI_X2 or LI_X3 contains the NFID conditional attribute. |
| Extended Interception Point Identifier | The Extended Interception Point Identifier (see ETSI TS 102 232-1 [9] clause 5.2.13) shall be populated with a value mapped from the IPID conditional attribute field (see Table 5.3.1-2) if the message received over LI_X2 or LI_X3 contains the IPID conditional attribute. |

The payload contains information that the MDF has received from sources in the network, such as the POI as described in clauses 6 and 7 of the present document.

### 5.5.2	Usage for realising LI_HI2

The payload of IRI mesages contains intercept related information. Details of the IRI messages can be found in Annex A of the present document.

Table 5.5.2-1 shows the minimum payload details for IRI messages sent over LI_HI2 in addition to those described in 102 232-1 [9] and ETSI TS 102 232-7 [10].

Table 5.5.2-1: LI_HI2 ETSI 102 232-1 [9] PS-PDU IRIPayload details

| ETSI TS 102 232-1 [9] field name | Description |
| --- | --- |
| IRI-Type | The IRI-Type (see ETSI TS 102 232-1 [9] clause 5.2.10) shall be populated as specified in the relevant clause of the present document. |
| Timestamp | Shall be present and populated as described in ETSI TS 102 232-1 [9] clause 6.2.3 when payload aggregation is used. |
| IRI Contents | Unless otherwise specified, the IRI Contents field shall be set to the threeGPP33128DefinedIRI choice (see TS ETSI 102 232 -7 [10] clause 15) and populated with a BER-encoded @TS33128Payloads.IRIPayload. See Table 5.5.2-2. |
| Timestamp Qualifier | Unless otherwise specified, if the timestamp field is set, the timestamp qualifier (see ETSI TS 102 232-1 [9] clause 5.2.6) shall be present and set to “timeOfInterception(1)”. |
| Network Function Identifier | The Network Function Identifier (see ETSI TS 103 232-1 [9] clause 5.2.14 and ETSI TS 102 232-7 [10] clause 15.3) shall be populated with a value mapped from the NFID conditional attribute field (see Table 5.3.1-2) if the message received over LI_X2 or LI_X3 contains the NFID conditional attribute. |
| Extended Interception Point Identifier | The Extended Interception Point Identifier (see ETSI TS 102 232-1 [9] clause 5.2.13) shall be populated with a value mapped from the IPID conditional attribute field (see Table 5.3.1-2) if the message received over LI_X2 or LI_X3 contains the IPID conditional attribute. |

Table 5.5.2-2 shows details for the @TS33128Payloads.IRIPayload.

Table 5.5.2-2: Payload details for IRIPayload

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| iRIPayloadOID | RELATIVE-OID | 1 | Shall be populated with the value of the iRIPayloadOID specified in the version of the ASN.1 used by the MDF2 to generate the IRI record. | M |
| event | IRIEvent | 1 | Unless otherwise specified, if the IRI event is generated from a XIRIPayload received over LI_X2, then MDF2 shall choose the same choice for the IRIPayload.event that was received in the xIRIPayload.event. If the IRI event is generated due to another cause, the MDF2 shall choose the IRIPayload.event appropriate for repoting the IRI event. | M |
| targetIdentifiers | SEQUENCE OF IRITargetIdentifier | 0..MAX | Shall be populated with all the Taget Identifiers available at the MDF2. See clause 5.5.5 for additional details. This parameter is conditional only for backwards compatibility. | C |
| mediatedFromIndicator | MediatedFromIndicator | 0..1 | Shall be present if the IRI is generated from an xIRIPayload received over LI_X2 and the release and version of the xIRIPayload.relativeOID is different from the release and version of the IRIPayload.relativeOID. The IRIPayload.mediatedFromIndicator.xIRIRelativeOID choice shall be used and set to the value of the xIRIPayload.relativeOID of the xIRI message used to generate the IRI message. | C |

### 5.5.3	Usage for realising LI_HI3

The payload of CC messages contains content of communication. Details of the CC can be found in Annex A of the present document.

Table 5.5.3-1 shows the minimum payload details for CC messages sent over LI_HI3 in addition to those described in 102 232-1 [9] and ETSI TS 102 232-7 [10].

Table 5.5.3-1: LI_HI3 ETSI 102 232-1 [9] PS-PDU CCPayload details

| ETSI TS 102 232-1 [9] field name | Description |
| --- | --- |
| Timestamp | Unless otherwise specified, when multiple CC records are aggregated in a single LI_HI3 message, this timestamp shall be set to the time the communication was observed at the POI (i.e. the timestamp field of the xCC). See ETSI TS 102 232-1 [9] clause 5.2.6. |
| CC Contents | Unless otherwise specified, the CC Contents field shall be set to the threeGPP33128DefinedCC choice (see TS ETSI 102 232 -7 [10] clause 15) and populated with a BER-encoded @TS33128Payloads.CCPayload. See Table 5.5.3-2. |

NOTE:	ETSI TS 102 232-1 [9] specifies in clause 6.4 a transport layer based on TCP. However, based on agreement between network operator and LEA, in scenarios where it may not be possible to achieve the necessary LI data rates based on the transport layer based on single TCP connection, alternative profiles may be considered (e.g. based on UDP, multi path TCP or other protocols). Any alternative profile needs to ensure that LI reliability, security and completeness requirements as specified in TS 33.126 [3] are met.

Table 5.5.3-2 shows details for the @TS33128Payloads.CCPayload.

Table 5.5.3-2: Payload details for CCPayload

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| cCPayloadOID | RELATIVE-OID | 1 | Shall be populated with the value of the cCPayloadOID specified in the version of the ASN.1 used by the MDF3 to generate the CC record. | M |
| pDU | CCPDU | 1 | Shall be populated as described in the relevant clause of the present document. | M |

## **** END OF CHANGES (MAIN DOCUMENT) ***

## **** START OF FIRST CHANGE (ATTACHMENTS) ****

---a/33128/r18/TS33128Payloads.asn
+++b/33128/r18/TS33128Payloads.asn

@@ -281,9 +281,10 @@ XIRIEvent ::= CHOICE

281	281

282	282		IRIPayload ::= SEQUENCE

283	283		{

284		-	    iRIPayloadOID       [1] RELATIVE-OID,

285		-	    event               [2] IRIEvent,

286		-	    targetIdentifiers   [3] SEQUENCE OF IRITargetIdentifier OPTIONAL

284	+	    iRIPayloadOID          [1] RELATIVE-OID,

285	+	    event                  [2] IRIEvent,

286	+	    targetIdentifiers      [3] SEQUENCE OF IRITargetIdentifier OPTIONAL,

287	+	    mediatedFromIndicator  [4] MediatedFromIndicator OPTIONAL

287	288		}

288	289

289	290		IRIEvent ::= CHOICE

@@ -534,6 +535,11 @@ IRITargetIdentifier ::= SEQUENCE

534	535		    provenance                                          [2] TargetIdentifierProvenance OPTIONAL

535	536		}

536	537

538	+	MediatedFromIndicator ::= CHOICE

539	+	{

540	+	    xIRIRelativeOID [1] RELATIVE-OID

541	+	}

542	+

537	543		-- ==============

538	544		-- HI3 CC payload

539	545		-- ==============

## **** END OF ALL CHANGES ***