SA3LI#94	 S3i240427

Amsterdam, July 9-13, 2024

Source:	Nokia, Nokia Shanghai Bell, Rogers Communications, OTD

Title:	Input to TR 33.929-7: LI Location Acquisition Capabilities  – Part I (Location Only LI reporting)

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19_Guide

Abstract of the contribution: This pCR provides input text to TR 33.929-7 LI for Location Only Reporting.

BACKGROUND

The TR 33.929-7 provides the implementation guidance for the LI aspects related to the location services. In this regard, the TR would cover the three services the illustrations of which are thought to be useful. Those three services are:

- Location only LI reporting.

- LALS.

- Location acquisition.

This pCR provides the input text to introduce the above and the Location only reporting related concepts and the scenarios.

PROPOSAL

Incorporate the following to the TR 33.929-7:

***** Start of the new text ****

# 4	LI for location services

## 4.1	General

The LI for location services illustrated in the subsequent clauses include the following three parts:

- Location only LI reporting.

- Lawful access location services (LALS).

- Location acquisition.

There are also other cases already illustrated in TS 33.127 [3] and TS 33.128 [4].

For example, with the case referred to as embedded location reporting as defined in TS 33.127 [3], the IRIs delivered to the LEMF as a part of interception performed for other services may include the location if the warrant authorized such location reporting. In this case, the reported location is the location of the target UE.

The location reporting does not apply when the interception is performed for a target non-local ID. When the warrant prohibits the reporting of the location, any information that conveys the location is redacted from the IRI/CC delivered to the LEMF.

In all cases, the scope of the reported location may vary as defined in TS 33.127 [3] and TS 33.128 [4].

## 4.2	Location only LI reporting

### 4.2.1	General

Location only LI reporting refers to a reporting for a warrant authorizing only the target's location information. Location only LI reporting does not apply to the target non-local ID.

The details on the scope of reported location, name of the related xIRIs/IRIs and the parameters used within those  xIRI/IRI are defined in TS 33.127 [3] and TS 33.128 [4].

The present document uses the term Location Only xIRI and Location Only IRI to refer collectively to all of  the xIRIs/IRIs used for the interceptions that deal with the location only LI reporting.  For example, SeparatedLocationReporting is a Location Only xIRI/IRI.

The present document refers to the warrants issued for interceptions other than location only LI reporting as regular warrants and the associated interceptions as regular interceptions.

### 4.2.2	Overview of the concept

The figure 4.2.2-1 provides an overview of the concepts of location only LI reporting, depicting two implementation options.  In both options, the process begins when an LEA issues a warrant to the CSP for location only LI reporting on a target UE.

All of the LI functions and LI interfaces depicted in figure 4.2.2-1 are defined in TS 33.127 [3] and TS 33.128 [4]. The NF shown in figure 4.2.2-1 can be any NF within the CSP network that has an IRI-POI with access to the target's location information. Few examples of such NFs: AMF, MME, HSS, UDM, S-CSCF, P-CSCF.

Figure 4.2.2-1: An overview of location only LI reporting

In the MDF-based implementation option, the IRI-POI is not provisioned for location only LI reporting (instead, the IRI-POI is provisioned for regular interception with location enabled). MDF2 is provisioned with location only LI reporting, so whenever the MDF2 receives an xIRI that has the location information, it constructs the Location Only IRI messages and delivers them to the LEMF.

As an example, if the MDF2 receives the IMSMessage xIRI record that includes the location information of the target, then the MDF2 would construct a SeparatedLocationReporting IRI message from the IMSMessage xIRI record and delivers the SeparatedLocationReporting IRI message to the LEMF.

In the POI-based implementation option, both the IRI-POI and the MDF2 are provisioned for location only LI reporting. In this option, the IRI-POI delivers the Location Only xIRIs to the MDF2 and the MDF2 would construct Location Only IRI messages to the LEMF.

As an example, whenever the IRI-POI in S-CSCF observes an event that would normally result in the generation of an IRIMessage xIRI record, if the IRIMessage xIRI record would have included the location information of the target, the IRI-POI would, instead, construct and deliver a SeparatedLocationReporting xIRI to the MDF2 and the MDF2 would construct the SeparatedLocationReporting IRI message and delivers it to the LEMF.

The AMFLocationUpdate xIRI/IRI which is used with the regular interception is also used as an xIRI/IRI with  location only LI reporting. Therefore, with the MDF-based implementation option, when the MDF2 receives the AMFLocationUpdate xIRI record, it constructs the AMFLocationUpdate IRI message and delivers it to the LEMF. However, if another xIRI is received from the IRI-POI present in the AMF that has the location, then the MDF2 would construct a SeparatedLocationReporting IRI message from it and deliver the same to the LEMF. The similar concepts apply for the xIRIs received from the IRI-POIs present in the MME,  UDM and HSS.

### 4.2.3	Location only LI reporting scenarios

#### 4.2.3.1	General

As described in clause 4.2.2, there are two implementation options for location only LI reporting. This clause illustrates a few scenarios that govern those options.

#### 4.2.3.2	One warrant

The figure 4.2.3.2-1 illustrates the use of POI-based implementation option for location only LI reporting. The LEA issues a warrant for authorizing only the target UE's location information (step P1).

Figure 4.2.3.2-1: Location only LI reporting – one warrant (POI-based option)

The IRI-POI and the MDF2 are provisioned for location only LI  reporting. In figure 4.2.3.2-1, the steps P2, P3 and P4 are the provisioning steps..

The IRI-POI generates the Location Only xIRIs and delivers them to the MDF2. The MDF2 constructs the Location Only IRIs and delivers them to the LEMF.  In figure 4.2.3.2-1, the steps R1 and R2 show the Location Only xIRIs and IRIs respectively, generated for the warrant from LEA.

#### 4.2.3.3	Two warrants, one target, scenario of MDF-based implementation option

The figure 4.2.3.3-1 illustrates the use of MDF-based implementation option for location only LI reporting.

LEA-1 issues a regular warrant for one of the services listed in TS 33.127 [3]/TS 33.128 [4] for the UE (step P1-1). The LEA-2 issues a location only LI reporting warrant on the same target UE (step P1-2).

The illustrated approach can be more suitable for the case where the provisioning for the regular warrant (from LEA-1) is done before the provisioning for the location only LI reporting warrant (from LEA-2).  The provisioning for regular warrant is assumed to have the location reporting enabled.

However, the illustrated approach can still be used even for the case where the provisioning for the location only LI reporting (from LEA-2) is done first or when the provisioning of regular warrant (from LEA-1) if done first, does not enable the location reporting, by modifying the existing task at the POI for regular interception with location reporting enabled.

Figure 4.2.3.3-1: Location only LI reporting – two warrants, one target UE, scenario 1

As part of the handling of the regular warrant from LEA-1, the IRI-POI and the MDF2 are provisioned for regular interception.  The MDF2 is provisioned with a new task with a new set of mediation details that include the LIID assigned LEA-1 and point to LEMF-1 as the delivery destination. In figure 4.2.3.3-1, the steps P2-1, P3 and P4 are the provisioning steps for the regular warrant from LEA-1.

The IRI-POI generates and delivers all xIRI records to the MDF2. The MDF2 constructs relevant IRI messages and delivers them to the LEMF-1. In figure 4.2.3.3-1, the steps R1 and R2-1 show the xIRIs and the IRIs respectively, generated for the warrant from LEA-1.

As part of the handling of the location only LI reporting warrant from LEA-2, the MDF2 is provisioned by updating the existing task with a new set of mediation details that include the LIID assigned to LEA-2, LocationOnly indication and point to the LEMF-2 as the delivery destination. The mediation details for LEA-1 are included as originally provisioned when the task was created.  In figure 4.2.3.3-1, the P2-2, P3 are the provisioning steps for the location only LI reporting warrant from LEA-2. The provisioning of MDF2 is an update when a second warrant is received.

The IRI-POI continues to generate and deliver all xIRI records to the MDF2. The MDF2 constructs the relevant IRI messages and delivers them to the LEMF-1. When the xIRIs received from the IRI-POI includes the location information, the MDF2 constructs the Location Only IRI messages and delivers them to the LEMF-2. In figure 4.2.3.3-1, the steps R1 and R2-1 show the xIRIs and the IRIs respectively, generated for the warrant from LEA-1, and the steps R1 and R2-2  show the xIRIs and the IRIs respectively, generated for the warrant from LEA.

As illustrated, in this scenario, the two warrants are handled by a single task in the IRI-POI and as a single task with two sets of mediation details at the MDF2.

#### 4.2.3.4	Two warrants, one target, scenario of POI-based implementation option

The figure 4.2.3.4-1 illustrates the use of POI-based implementation option for location only LI reporting.

LEA-1 issues a location only LI reporting warrant on the UE (step P1-1). The LEA-2 issues a regular warrant for one of the services listed in TS 33.127 [3]/TS 33.128 [4] for the target UE (step P1-2).

The illustrated approach can be more suitable for the case where the provisioning for the location only LI reporting warrant (from LEA-1) is done before the provisioning for the regular warrant (from LEA-2) or when the provisioning of regular warrant (from LEA-2) if done first, does not enable the location reporting.

However, the illustrated approach can still be used even for the case where the provisioning for the regular warrant (from LEA-2) with location enabled is done first even though such an implementation is not optimal.

Figure 4.2.3.4-1: Location only LI reporting – two warrants, one target UE, scenario 2

As part of the handling of the location only LI reporting warrant from LEA-1, the IRI-POI and the MDF2 are provisioned for location only LI reporting. The MDF2 is provisioned with mediation details that include the LIID assigned to LEA-1, LocationOnly indication and point to LEMF-1 as the delivery destination. In figure 4.2.3.3-1, the steps P2-1, P3-1 and P4-1 are the provisioning steps for the location only LI reporting related warrant from LEA-1.

The IRI-POI generates and delivers the Location Only xIRI records to the MDF2 and the MDF2 constructs the Location Only IRI messages and delivers the same to LEMF-1. In figure 4.2.3.4-1, the steps R1-1 and R2-1 show the Location Only xIRIs and the IRIs respectively, generated for the location only LI reporting warrant from LEA-1.

As part of the handling of the regular warrant from LEA-2 on the same target UE, the IRI-POI and the MDF2 are provisioned with a new task for regular interception. In figure 4.2.3.4-1, the steps P2-2, P3-2 and P4-2 are the provisioning steps for the regular warrant from LEA-2.

The IRI-POI generates and delivers all xIRI records to the MDF2.  The MDF2 constructs the relevant IRI messages and delivers them to the LEMF-2.  In figure 4.2.3.4-1, the steps R1-2 and R2-2 show the xIRIs and the IRIs respectively, generated for the warrant from LEA-2.

As illustrated, in this scenario, the two warrants are handled as two independent interceptions even though the two warrants are on the same target.

***** End of the new text ****