SA3LI#94	 S3i240429

Amsterdam, Jul 9-13, 2024

Source:	Nokia, Nokia Shanghai Bell, Rogers Communications

Title:	pCR to TR 33.929-7: LI Location Acquisition Capabilities – Part III (Location Acquisition)

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR provides input text to TR 33.929-7 LI for Location Acquisition.

BACKGROUND

The TR 33.929-7 provides the implementation guidance for the LI aspects related to the location services. In this regard, the TR would cover the three services the illustrations of which are thought to be useful. Those three services are:

- Location only LI reporting.

- LALS.

- Location acquisition.

This pCR provides the input text for the location acquisition related concepts.

PROPOSAL

Incorporate the following to the TR 33.929-7.

# 4	LI for location services

## 4.1	General

The LI for location services illustrated in the subsequent clauses include the following three parts:

- Location only LI reporting.

- Lawful access location services (LALS).

- Location acquisition.

There are also few other cases that may not need any more illustrations than what have been specified in TS 33.127 [3] and TS 33.128 [4].

For example, with the case referred to as embedded location reporting as defined in TS 33.127 [3], the IRIs delivered to the LEMF as a part of interception performed for other services may include the location if the warrant authorized such location reporting. In this case, the reported location is either the location of the target UE or the UE in communication with the target non-local ID. When the warrant prohibits the reporting of the location, any information that conveys the location is redacted from the IRI/CC delivered to the LEMF.

In all cases, the scope of the reported location may vary as defined in TS 33.127 [3] and TS 33.128 [4].

## 4.2	Location only LI reporting

## 4.3	LALS

***** Start of the new text ****

## 4.4	Location acquisition

### 4.4.1	General

With location acquisition, the authorized LEAs get lawful access to network provided location of target UE on demand. The authorization is done by issuing a warrant for the target UE.

The authorized LEAs send a request to the network for the target UE's location and receive the target UE's location either as a response to the request or as an IRI message, or in both ways. The different options are referred to as location delivery method.  The basic principle of location acquisition is illustrated in figure 4.4.1-1 below depicting the three methods of location delivery.

Figure 4.4.1-1: Location acquisition principles

The LEAs provide the Location Delivery Method to the CSP network (along with the warrant) prior to sending the request for the location.  The LEAs may request the network for the location as known at the time request is received or request the network for a fresh location.

The details on the scope of reported location, and the protocol (messages/parameters) used are defined in TS 33.127 [3] and TS 33.128 [4]. The location acquisition capabilities do not apply to target non-local ID. The location acquisition capabilities do not apply to the outbound roamers.

Because of the use of non-LI functions in Location Acquisition operations, the Location Acquisition comes with a higher risk of detectability and therefore, the LEA and the CSP need to take the following into consideration while employing Location Acquisition.

Two categories of detectability that the LI is concerned with are the following:

- Detectability by the target.

- Detectability by unauthorized 3rd parties.

With respect to the Location Acquisition, the detectability by the target may exist when the requested Location Acquisition involves some positioning operations. The level of detectability risk tolerance may vary by jurisdictions and the special circumstances. Similar to LALS, the use of other networks in fulfilling Location Acquisition operations should be avoided. That may restrict the use of Location Acquisition to the non-roaming scenarios only. To use LALS in a visited network a method is required to avoid any involvement of target's HPLMN.

### 4.4.2	Overview of the architecture

In order to support the location acquisition capabilities, two LI specific logical functions are introduced:

- Location Acquisition Function (LAF).

- Location Acquisition Requesting Function (LARF).

Neither the present document nor the TS 33.127 [3]/33.128 [4] identify/define the NF that can house LAF. The LARF is an LI function within the AMF for 5GS and within the MME for EPS.

The LAF receives the location acquisition request from the LEA. The LARF determines the target UE's location by interacting with the other network functions (i.e. AMF in 5GS and MME in EPS) within the CSP domain. The identities used to identify the target UE are one or more of: SUPI, GPSI, IMSI, MSISDN.

The figure 4.4.2-1 illustrates an overview of the overall architecture for location acquisition methods. All of the LI functions and LI interfaces depicted in figure 4.4.2-1 are defined in TS 33.127 [3] and TS 33.128 [4].

Figure 4.4.2-1: An overview of location acquisition

In order to realize the location acquisition capabilities, the LEAs have to first issue a warrant that identifies the target UE and location delivery method. As described in clause 4.4.1, two location delivery methods are defined for location acquisition:

- Method 1: as a response to the request.

- Method 2: as an IRI message.

- Method 3: as a response to the request and as an IRI message.

In method 1, the location is returned as a response to the request. In method 2, the location is delivered via the MDF2 to the LEMF as identified in the warrant. In method 3, the location is returned as a response to the request and delivered as an IRI message.

When the warrant is received over the LI_HI1, the LICF/LIPF provisions the LAF over LI_LAFP interface for location acquisition with the location delivery method. For the location delivery method 2 and method 3, the MDF2 is provisioned with the LEMF address for the delivery of the location as an IRI message.

The LEA sends the request for the location acquisition over the LI_HILA interface to the LAF. The LAF upon receiving the request from the LEA forwards the request to LARF over the LI_XLA interface along with the location delivery method. For the method 2 and method 3, the LAF would provide the MDF2 address to the LARF.

The LARF returns the location either to LAF (method 1, method 3) over the LI_XLA interface or to MDF2 (method 2, method3) over the LI_X2_LA interface.

With method 1, the LAF responds to the LEA request with the location over the LI_HILA interface. With method 2, the MDF2 forwards the location to the LEMF as an IRI message over the LI_HI2 interface.  With method 3, the LAF responds to the LEA request with location over the LI_HILA interface and MDF2 forwards the location to the LEMF as an IRI message over the LI_HI2 interface.

Within the TS 33.127 [3]/TS 33.128 [4], method 1 is referred to as HILADelivery. method 2 is referred to as HI2Delivery and method 3 is referred to as HILADelivery + HI2Delivery.

### 4.4.3	Overview of the concepts

#### 4.4.3.1	General

As described in clause 4.4.2, there are three methods of location delivery. This clause illustrates concepts around those three methods.

In the following illustrations, the target identity can be the SUPI or GPSI in the case of 5G and it can be  IMSI or MSISDN in the case of 4G. In the response, the SUPI (5G) or the IMSI (4G) and any available other identities of the target UE are included.

#### 4.4.3.2	Location acquisition with HILADelivery method

The illustration shown in figure 4.4.3.2-1, location acquisition concepts when HILADelivery method is used for the location delivery.

Figure 4.4.3.2-1: Location acquisition with HILADelivery for location delivery method

The LEA issues a warrant on the target UE for location acquisition over the LI_HI1 interface. In the warrant, the LEA indicate the HILADelivery as the location delivery method. The target identity and the LIID are also included in the warrant. The LIPF provisions the LAF  over the LI_LAFP interface for location acquisition. The location delivery method (HILADelivery), LIID and the target identity are included in the provisioning. In figure 4.4.3.2-1, P1, P2, and P3 are the provisioning steps.

To get the location of the target UE, the LEA sends the request over the LI_HILA interface to the LAF. The request includes the LIID and the target identity previously provided via the warrant.  The LAF identifies the AMF/MME where the target UE (represented by the target identity) is registered.

NOTE:	The identification of AMF/MME serving the target UE is not specified in the standard and left for proprietary solutions, which may include the interrogation of UDM/HSS (for non-roaming target UEs), or query of all relevant AMF/MME (for inbound roaming target UEs), or some other means.

After the AMF/MME identification, the LAF sends the request for the location determination to the LARF present in that AMF/MME over the LI_XLA interface. The request includes the target identity and the location delivery method (HILADelivery). The LARF interacts with the non-LI part of AMF/MME for the target location. Depending on the request, the AMF/MME may interact with the other network functions within the CSP domain (e.g. LMF/E-SMLC) to determine the target UE's location. Once the location is obtained, the non-LI part of AMF/MME would return the location of the target UE to the LARF. In figure 4.4.3.2-1, R1, R2, R3 and R4 are the location request steps.

The LARF returns the location over the LI_XLA interface to the LAF (since the location delivery method is HILADelivery). The LAF returns the location to the requesting LEA over the LI_HILA interface. In figure 4.4.3.2-1, D1, D2 and D3 represent the location delivery steps.

#### 4.4.3.3	Location acquisition with HI2Delivery method

The illustration shown in figure 4.4.3.3-1, location acquisition concepts when H2Delivery method is used for the location delivery.

Figure 4.4.3.3-1: Location acquisition with HI2Delivery for location delivery method

The LEA issues a warrant on the target UE for location acquisition over the LI_HI1 interface. In the warrant, the LEA indicate the HI2Delivery as the location delivery method. The target identity, the LIID  and the LEMF address are also included in the warrant. The LIPF provisions the LAF over the LI_LAFP interface for location acquisition. The location delivery method (HI2Delivery), MDF2 address and the target identity are included in the provisioning. The LIPF also provisions the MDF2 for location acquisition. The LIID, LEMF address and the target identity are included in the provisioning. Prior to the provisioning, the LIPF creates the LEMF as the destination address for the task at the MDF2. In figure 4.4.3.3-1, P1, P2, P3, P4 and P5 are the provisioning steps.

For location acquisition, the LEA sends the request over the LI_HILA interface to the LAF. The request includes the LIID and the target identifier. The LAF identifies the AMF/MME where the target UE is registered (see the NOTE in clause 4.4.3.2) and sends the request for the location determination to the LARF present in that AMF/MME over the LI_XLA interface. The request includes target identity, location delivery method (HI2Delivery) and the MDF2 address. The LARF interacts with the non-LI part of AMF/MME for the target location. Depending on the request, the AMF/MME may interact with the other network functions within the CSP domain (e.g. LMF/E-SMLC) to determine the target UE's location. Once the location is obtained, the non-LI part of AMF/MME would return the location of the target UE to the LARF. In figure 4.4.3.2-1, R1, R2, R3 and R4 are the location request steps.

The LARF forwards the location over the LI_X2_LA interface to the MDF2 (since the location delivery method is HI2Delivery). The MDF2 forwards location to the LEMF over the LI_HI2 interface.  In figure 4.4.3.2-1, D1, D2 and D3 represent the location delivery steps.

#### 4.4.3.4	Location acquisition with HILADelivery and HI2Delivery methods

The illustration shown in figure 4.4.3.4-1, location acquisition concepts when HILADelivery + H2Delivery method is used for the location delivery.

Figure 4.4.3.4-1: Location acquisition with HILADelivery + HI2Delivery for location delivery method

The LEA issues a warrant on the target UE for location acquisition over the LI_HI1 interface. In the warrant, the LEA indicate the HILADelivery + HI2Delivery as the location delivery method. The target identity, the LIID  and the LEMF address are also included in the warrant. The LIPF provisions the LAF over the LI_LAFP interface for location acquisition. The location delivery method (HILADelivery + HI2Delivery), MDF2 address and the target identity are included in the provisioning. The LIPF also provisions the MDF2 for location acquisition. The LIID, LEMF address and the target identity are included in the provisioning. Prior to the provisioning, the LIPF creates the LEMF as the destination address for the task at the MDF2. In figure 4.4.3.4-1, P1, P2, P3, P4 and P5 are the provisioning steps.

For location acquisition, the LEA sends the request over the LI_HILA interface to the LAF. The request includes the LIID and the target identifier. The LAF identifies the AMF/MME where the target UE is registered (see the NOTE in clause 4.4.3.2) and sends the request for the location determination to the LARF present in that AMF/MME over the LI_XLA interface. The request also includes the target identity, location delivery method (HILADelivery + HI2Delivery) and the MDF2 address. The LARF interacts with the non-LI part of AMF/MME for the target location. Depending on the request, the AMF/MME may interact with the other network functions within the CSP domain (e.g. LMF/E-SMLC) to determine the target UE's location. Once the location is obtained, the non-LI part of AMF/MME would return the location of the target UE to the LARF. In figure 4.4.3.2-1, R1, R2, R3 and R4 are the location request steps.

Since the HI2Delivery is one of the two location delivery methods, the LARF forwards the location over the LI_X2_LA interface to the MDF2. The MDF2 forwards location to the LEMF over the LI_HI2 interface.  In figure 4.4.3.2-1, D1, D2a and D3b represent the location delivery steps.

Since the HILADelivery is one of the two location delivery methods, the LARF also returns the location over the LI_XLA interface to the LAF. The LAF returns the location to the requesting LEA over the LI_HILA interface. In figure 4.4.3.2-1, D1, D2b and D3b represent the location delivery steps.

***** End of the new text ****