3GPP TSG-SA3 Meeting #94-LI	s3i240469

# Amsterdam, Netherlands, 9th Jul 2024 - 12th Jul 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0670 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Traffic Policies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD_US) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI18 |  |  |  |  |  | Date: |  |  | 2024-07-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 33.127 has a requirement for the support for additional filtering criteria in order to provide additional guidance for the handling of certain subsets of authorised LI product. This can be done to support needs such as bandwidth optimization. ETSI TCLI has already defined a way to signal these criteria both from the LEA to the CSP and also from the ADMF to the LI functions with Traffic Policies. This contribution introduces this concept and the usage of these standardized messages into this document. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduces ETSI TCLI defined Traffic Policies and their usage over LI_HI1 and LI_X1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no standardized solution to support the TS 33.127 requirement for filtering. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## **** START OF FIRST CHANGE (MAIN DOCUMENT) ****

## 4.X	LI product filtering

### 4.X.1	General

As described in TS 33.127 [5] Annex B, the ADMF is responsible for providing administration and local management of the warrant including filtering criteria. Some of the reasons these criteria may be applied include LEA initaiated suspend/resume (TS 33.127 [5] Annex C), as a part of location depenedent interception management, or bandwidth optimization.

This document currently defines the following methods for providing and provisioning the criteria for filtering LI product:

- Traffic Policies (see clause 4.X.2).

### 4.X.2 Traffic policies

ETSI TS 103 120 [6] clause 7.5 defines a traffic policy object which represents a policy from an LEA that is to be applied to authorised intercepted materials.

The use of Traffic Policies over LI_HI1 can be found in clause 5.4.Z.

The use of Traffic Policies over LI_X1 can be found in clause 5.2.Y.

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

### 5.2.Y	Traffic policies

The LIPF shall be able to provision the POI, TFs and the MDF2/MDF3 with Traffic Policies as described in TS 103 221-1 [7] Annex F.

Whether the traffic policies are applied on a per-task basis (at the POIs or the MDF2/MDF3) or they are applied at the

Additional information on the use of Traffic Policies in this document is described in clause 4.X.

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

### 5.4.Z	Traffic policies

Functions having an LI_HI1 interface (i.e. the ADMF) shall be able to receive traffic policies as applicable to an intercept task over the LI_HI71 interface (see clause 4.X.2).

Where TS 103 120 [6] is used to realise LI_HI1, and where the details in clause 5.4.1 apply, when the use of Traffic Policies is required, the ListOfTrafficPolicyReferences field in a given LITaskObject shall be used to identify the appropriate traffic policies for that task. The referenced TrafficPolicyObjects shall be defined as described in ETSI TS 103 120 [6] prior to being referenced in an LITaskObject.

Unless otherwise specified, traffic policies shall not be used in place of service scoping.

Additional information on the use of Traffic Policies in this document is described in clause 4.X.

## **** END OF ALL CHANGES (MAIN DOCUMENT) ****