3GPP TSG-SA3 Meeting #94-LI	s3i240483

# Amsterdam, Netherlands, 9th Jul 2024 - 12th Jul 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0241 | rev | 1 | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Location acquisition based on measurement reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (PIDS, TNO, OTD_US, NDRE, Ofcom (CH)) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2024-07-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | By adding the option to use E-CID measurements additional location related information can be obtained as part of location acquisition. The additional information potentially enhances the accuracy of the location determination of the target’s UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | E-CID measurements is added as form of location acquisition. The existing form of location acquisition is renamed to location information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Additional location related information cannot be used as potential enhancement on the existing location acquistion method. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.4.18, 5.4.19, 7.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | s3i240434 |  |  |  |  |  |  |  |  |

FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 33.126: "Lawful interception requirements".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[6]	OMA-TS-MLP-V3_5-20181211-C: "Open Mobile Alliance; Mobile Location Protocol, Candidate Version 3.5", https://www.openmobilealliance.org/release/MLS/V1_4-20181211-C/OMA-TS-MLP-V3_5-20181211-C.pdf".

[7]	ETSI TS 103 120: "Lawful Interception (LI); Interface for warrant information".

[8]	ETSI TS 103 221-1: "Lawful Interception (LI); Internal Network Interfaces; Part 1: X1 ".

[9]	3GPP TS 33.501: "Security Architecture and Procedures for the 5G System".

[10]	ETSI GR NFV-SEC 011: "Network Functions Virtualisation (NFV); Security; Report on NFV LI Architecture".

[11]	3GPP TS 33.107: "3G Security; Lawful interception architecture and functions".

[12]	3GPP TS 23.214: "Architecture enhancements for control and user plane separation of EPC nodes; Stage 2".

[13]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[14]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[15]	3GPP TS 33.128: "Protocol and Procedures for Lawful Interception; Stage 3".

[16]	ETSI TS 103 221-2: " Lawful Interception (LI); Internal Network Interfaces; Part 2: X2/X3".

[17]	MMS Architecture OMA-AD-MMS-V1_3-20110913-A.

[18]	Multimedia Messaging Service Encapsulation Protocol OMA-TS-MMS_ENC-V1_3-20110913-A.

[19]	3GPP TS 22.140: "Multimedia Messaging Service (MMS); Stage 1".

[20]	ETSI GS NFV-IFA 026: "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Architecture enhancement for Security Management Specification".

[21]	3GPP TS 33.108: "Handover Interface for Lawful Interception (LI)".

[22]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for 
Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[23]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[24]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[25]	OMA-AD-PoC-V2_1-20110802-A: "Push to talk over Cellular (PoC) Architecture".

[26]	GSMA IR.92: "IMS Profile for Voice and SMS".

[27]	GSMA NG.114: "IMS Profile for Voice, Video and Messaging over 5GS".

[28]	3GPP TS 24.147: "Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[29]	ETSI GS NFV-SEC 012: "Network Functions Virtualisation (NFV) Release 3; Security; System architecture specification for execution of sensitive NFV components".

[30]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[31]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage3".

[32]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[33]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[34]	OMA-AD-CPM-V2_2-20170926-C: "Open Mobile Alliance, OMA Converged IP Messaging System Description", http://www.openmobilealliance.org/release/CPM/V2_2-20200907-C/OMA-AD-CPM-V2_2-20170926-C.pdf.

[35]	GSMA RCC.07: "Rich Communication Suite – Advanced Communications Services and Client Specification".

[36]	IETF RFC 4975: "The Message Session Relay Protocol (MSRP)".

[37]	IETF RFC 6714: "Connection Establishment for Media Anchoring (CEMA) for the Message Session Relay Protocol (MSRP)".

[38]	IETF RFC 3862: "Common Presence and Instant Messaging (CPIM): Message Format".

[39]	3GPP TS 24.229: "IP Multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[40]	IETF RFC 8224: "Authenticated Identity Management in the Session Initiation Protocol (SIP)".

[41]	IETF RFC 8946: "Personal Assertion Token (PASSporT) Extension for Diverted Calls".

[42]	IETF draft-ietf-stir-passport-rcd-26, "PASSporT Extension for Rich Call Data".

NOTE:	The above document cannot be formally referenced until it is published as an RFC.

[43]	IETF RFC 7095: "jCard: The JSON Format for vCard".

[44]	3GPP TS 24.196: "Enhanced Calling Name (eCNAM)".

[45]	IETF RFC 8816: "Secure Telephone Identity Revisited (STIR) Out-of-Band Architecture and Use Cases".

[46]	IETF RFC 9475: "Messaging Use Cases and Extensions for Secure Telephone Identity Revisited (STIR)".

[47]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[48]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[49]	3GPP TS 33.222: "Generic Authentication Architecture (GAA); Access to network application functions using Hypertext Transfer Protocol over Transport Layer Security (HTTPS)".

[50]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[51]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[52]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[53]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[54]	3GPP TS 29.272: "Evolved Packet System (EPS); Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[55]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[xx]	3GPP TS 38.455: "NR Positioning Protocol A (NRPPa)".

[yy]	3GPP TS 36.455: "LTE Positioning Protocol A (LPPa)".

NEXT CHANGE

### 5.4.18	Interface LI_HILA

The LI_HILA interface is used by the LEA to forward the location acquisition requests to the LAF and when required to forward the location acquisition responses from the LAF to the LEA.

The following are examples of some of the information that may be passed over the LI_HILA interface from LEA to LAF:

-	Target identifier (IMSI, MSISDN, SUPI, GPSI).

-	Authorisation identifier (LIID).

-	Type of information requested (location information, E-CID measurements).

If the delivery type is via the LI_HILA, the following are examples of the essential information that may be passed over the LI_HILA interface from LAF to LEA:

-	Location information.

-	E-CID measurements information.

### 5.4.19	Interface LI_XLA

The LI_XLA interface is used by the LAF to forward the location acquisition requests to the LARF and when required to forward the location acquisition responses from the LARF to the LAF if the delivery type of location acquisition delivery is via the LI_HILA.

The following are examples of some of the information that may be passed over the LI_XLA interface from LAF to LARF:

-	Internal interception identifier.

-	Target identifier (IMSI, MSISDN, SUPI, GPSI).

-	Information relating to the type of delivery (via LI_HILA, via LI_HI2).

-	Address of MDF2.

-	Type of information requested (location information, E-CID measurements).

If the type of location acquisition delivery is via the LI_HILA, the following are examples of some of the information that may be passed over the LI_XLA interface from LARF to LAF:

-	Location information.

-	E-CID measurements information.

NEXT CHANGE

### 7.3.5	Location Acquisition

#### 7.3.5.1	General

This clause defines the location acquisition procedures, which provide lawful access to the target’s network-provided location information and E-CID measurements information of the target.

The outcome of the location information procedure is the target’s TAI, ECGI/NCGI and optionally the timestamp when the target’s location was acquired. In case of EPC, it is emulating the Insert Subscriber Data request containing the IDR-Flags with the "EPS Location Information Request" bits set at the MME and consumes the response as defined in TS 29.272 [54] clause 5.2.2.1.2. In case of 5GC, it is emulating the AMF location services request and consumes the response as defined in TS 29.518 [52] clause 5.5.2.4. The MME/AMF shall override any user consent, privacy and paging restrictions concerned with location acquisition that may apply to the target UE.

The outcome of the E-CID measurements procedure is measurement information about the target, such as Timing Advance and Angle of Arrival which can be used as additional location related information. To achieve this outcome, LARF shall generate an E-CID Measurements request based on the NRPPa protocol defined in TS 38.455 [xx] (5GC case) or the LPPa protocol defined in TS 36.455 [yy] (EPC case) and instigate the AMF/MME to send the request to RAN. The AMF/MME shall discriminate the respective RAN response and deliver it to LARF, instead of the regular processing of such responses.

For the location information procedure, the LEA shall be able to indicate in the request to the LAF whether the MME/AMF location services shall be invoked or whether the current stored value of the location as known by the MME/AMF is returned.

Prior to issuing of location acquisition requests, the LEA shall provide an authorization for these requests. This is done by issuing a warrant over the LI_HI1 interface prior to issuing the location acquisition requests. Subsequently, the LIPF will provision the MDF2 if needed.

NOTE:	The use of the location acquisition procedure may cause detectability issues due to the possibility of triggering a Network Triggered Service Request procedure for a UE. Detectability issues may also include the visibility between different LEAs using location acquisition with warrants for the same target. It is up to the LEA to decide if location acquisition is to be used.

#### 7.3.5.2	Location acquisition architecture

This clause describes the architecture for location acquisition. The architecture is based on the use of a LAF, which communicates with the LARF associated with an MME/AMF over the LI_XLA interface.

To use the location acquisition procedure, the LAF needs to determine the serving MME/AMF for the target UE.

NOTE:	The method for determining the serving MME/AMF is not defined in the present document.

The LAF is requested to perform location acquisition via the LI_HILA interface between LEA and LAF. Upon receiving the request, the LAF initiates location acquisition to the LARF via the LI_XLA interface.

The networks shall support the delivery of location acquisition results via the following methods:

-	Location acquisition results are delivered back as a response to the location acquisition request via the LAF over the LI_XLA and the LI_HILA interfaces.

-	Location acquisition results are delivered via the MDF2 over the LI_X2_LA and the LI_HI2 interfaces.

The two delivery options may be used simultaneously.

The LARF and the IRI-POI will exchange the necessary information to ensure that IRI-POI will not generate the xIRI for location acquisition requests initiated by the LARF.

The ADMF shall be able to audit the LARF over the LI_XLA interface.

##### 7.3.5.2.1	Location information delivery via the LI_HILA

The architecture for delivering location acquisition results via the LI_HILA is depicted in figure 7.3.5.2.1-1.

Figure 7.3.5.2.1-1: Delivery of the retrieved information via the LI_HILA

Upon determining the requested location acquisition information, the LARF will forward the information to the LAF via the LI_XLA interface. The retrieved information is further provided in the response to the LEA over the interface LI_HILA. The LARF and the IRI-POI will exchange the necessary information to ensure that IRI-POI will not generate the xIRI for location requests initiated by the LARF.

##### 7.3.5.2.2	Location information delivery via the LI_HI2

The architecture for delivering location acquisition information via LI_HI2 is depicted in figure 7.3.5.2.2-1.

Figure 7.3.5.2.2-1: Delivery of the retrieved information via the LI_HI2

Upon determining the requested location acquisition information, the LARF shall forward the information to the MDF2 over the LI_X2_LA interface using the information provided by the LAF in the LI_XLA request (e.g. internal intercept identifier). The retrieved information is further provided by the MDF2 to the LEMF via the LI_HI2 interface. The LARF and the IRI-POI will exchange the necessary information to ensure that IRI-POI will not generate the xIRI for location requests initiated by the LARF.

END OF CHANGES