SA3#95-LI	 S3i240631

Las Vegas; October 29-November 1, 2024

Source:	Nokia,

Title:	pCR to TR 33.929-8: MMS LI – scope of target identifier – Part VIII (with redirection, initial MSISDN to MSISDN, one MMS Relay/Server)

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR provides input text to TR 33.929-8 LI for MMS. Scope of target identifiers with redirection are illustrated in a topology view format. Here, the initial MM message is from MSISDN to MSISDN. One MMS Relay/Server is used.

BACKGROUND

The TR 33.929-8 provides the implementation guidance for the LI aspects related to the MMS. The target identities can be a phone number or an email address. Several permutations are possible when messages exchanged between two clients represented by those identities are redirected (i.e. forwarded).

The illustrations show the scenarios where originating, redirecting and redirected-to MMS User Agents are served by the same MMS Relay/Server.

In the illustrations shown here, the initial MM message is from phone number to phone number. There are two cases depending on the addressing format.

- MSISDN to MSISDN.

- MSISDN to MSISDN based email address.

This pCR illustrates of the first of those two scenarios.

PROPOSAL

Incorporate the following to the TR 33.929-8.

# 4	Illustration of LI for MMS

## 4.1	General

## 4.2	MMS background with LI scope

## 4.3	Scope of target identities

### 4.3.1	General

### 4.3.2	Topology views - without redirection

### 4.3.3	Topology views - with redirection

#### 4.3.3.1	General

#### 4.3.3.2	Multiple MMS Relay/Servers

*** New Text ***

#### 4.3.3.3	One MMS Relay Server

##### 4.3.3.3.1	General

When all the MMS User Agents are served by the same MMS Relay/Server, the identity of the originating MMS User Agent is a phone number (MSISDN, GPSI (MSISDN), or E,164 Number).

In the initial MM message, the redirecting MMS User Agent may be addressed with a phone number (MSISDN, GPSI (MSISDN), or E,164 Number) or an MSISDN based Email Address.

The redirected MMS User Agent may be address using a with a phone number (MSISDN, GPSI (MSISDN), or E,164 Number) or an MSISDN based Email Address.

The generic Email Address based identity does not apply to this scenario.

##### 4.3.3.3.2	Initial MM message from MSISDN to MSISDN

###### 4.3.3.3.2.1	Overview

MMS User Agent (A) initially sends an MM message using own phone number (MSISDN, GPSI (MSISDN), E.164 number) to MMS User Agent (B) (addressed with a phone number (MSISDN, GPSI (MSISDN), E.164 number) and the MMS User Agent (B) forwards the received MM message to destinations as illustrated.

In this illustration, it is presumed that MMS User Agent (A), MMS User Agent (B) and MMS User Agent (C) are served by the same MMS Relay Server.

Figure 4.3.3.3.2.1-1: Scope of target identities with redirection (one Relay/Server) – case 1

The path of the MM message flow from MMS User Agent (A) to MMS User Agent (B) is same as in the scenario 1 of the illustrations in clause 4.3.2.3 (figure 4.3.2.3.1-1 and 4.3.2.3.2.2-1). The MMS User Agent (A) and MMS User Agent (B) can be the targets in the MMS Relay/Server. The redirection of the MM message may happen to a phone number or to an MSDDN based Email Address.

When the MMS User Agent (B) forwards the MM message to an MSISDN based Email Address (shown as C@mmsc), the MMS Relay/Server adds the domain name for B (shown as mmsc).

The two scenarios of redirected MMS flow shown as R-1, R-2 in figure 4.3.3.3.2.1-1, are further illustrated in figure 4.3.3.3.2.1-2.

Figure 4.3.3.3.2.1-2: Two scenarios for the redirected MM message for case 1

The two redirected scenarios are described in clause 4.3.3.3.2.2.

###### 4.3.3.3.2.2	Descriptions of MM message redirected MM messages

4.3.3.3.2.2.1	Scenario R-1:

In this scenario, the redirecting MMS User Agent (B) forwards the received MM message to a phone number (MSISDN, GPSI (MSISDN), E.164 number). The MMS User Agent (C) is the recipient of that redirected MM message.

The phone number B can be a target in the MMS Relay/Server for the redirected MM message. The phone number C can be a target in the MMS Relay/Server.

4.3.3.3.2.2.2	Scenario R-2:

In this scenario, the MMS User Agent (B) forwards the received MM message to a phone number (MSISDN, GPSI (MSISDN), E.164 number) by addressing the destination using an MSISDN based Email Address (shown as C@mmsc). The MMS User Agent (C) is the recipient of that redirected MM message.

The MMS Relay/Server adds the domain name (shown as @mmsc) to the B's identity (shown as B@mmsc) before forwarding the MM message to the External Server serving the domain mmsc.

The MMS Relay/Server would remove the domain name (shown as @mmsc) from C@mmsc from C@mmsc, at the terminating end of the flow.

The phone number B can be target in the MMS Relay/Server (B) for the redirected MM message.

The MSISDN based Email Address B@mmsc can be a target non-local ID in the MMS Relay/Server.

The phone number C can be a target in the MMS Relay/Server. The MSISDN based Email Address C@mmsc- can be a target non-local ID in the MMS Relay/Server.

***** End of the new text ****