DeletSA3#95-LI	 S3i240634

Las Vegas; October 29-November 1, 2024

Source:	Nokia,

Title:	pCR to TR 33.929-8: MMS LI – scope of MMS messages – Part II (MMBox specific)

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR provides input text to TR 33.929-8 LI for MMS. Scope of MMS messages reported as xIRIs – few MMBox specific operations are illustrated here.

BACKGROUND

The TR 33.929-8 provides the implementation guidance for the LI aspects related to the MMS. MMS implementation may use MMBox to store MM messages. Few MMS LI aspects specific MMBox operations are illustrated in the pCR.

The pCR also provides a short introduction to the cardinalities of delivery reports and read reports as they apply to almost all the illustrations.

PROPOSAL

Incorporate the following to the TR 33.929-8.

# 4	Illustration of LI for MMS

## 4.1	General

## 4.2	MMS background with LI scope

## 4.3	Scope of target identities

## 4.4	Scope of MMS messages reported

### 4.4.1	General

### 4.4.2	3GPP, OMA and MMS LI messages

** New Text **

### 4.4.3	Topology views

#### 4.4.3.1	General

##### 4.4.3.1.1	Scope

The illustrations show the scope of MMS LI messages which are based on the OMA specification [19] for MM1 messages and TS 23.140 [18] for the MM4 messages because that is how the most MMS implementations are, and that is how the TS 33.128 [4] defines the stage 3 MMS LI.

A comparison of MM1 messages as defined in TS 23.140 [18] and OMA spec [19] as they apply to MMS LI is given in clause 4.4.2.

The illustrations include the following different cases:

- Target local to the MMS Relay/Server.

- Target non-local ID at the MMS Relay/Server.

- MMS origination from an MMS User Agent. The destination can be another MMS User Agent or an Email Client.

- MMS termination to an MMS User Agent. The origination can be from another MMS User Agent or an Email Client.

- Immediate retrieval.

- Deferred retrieval.

- Deletion without retrieval.

- Redirection without retrieval (same as MMS Redirection from an MMS User Agent, listed below).

- MMS redirection from an MMS User Agent. The redirection can be to an MMS User Agent or to an Email Client. Here, the initial origination can be from an MMS User Agent or an Email Client.

- When an MM message transfer involves two or more MMS User Agents, those MMS User Agents may be served by different MMS Relay/Servers, or may also be served by the same MMS Relay/Server.

As illustrated in clause 4.3 (e.g. scenario 2, clause 4.3.2.2.2.3) a MM message addressed to an MSISDN based Email Address is routed to the External Server over the MM3 interface, even though the final destination is an MMS User Agent. Likewise, an MM message originated from an MSISDN based Email Address comes to the terminating MMS Relay/Server from the External Server via the MM3 interface.

In other words, depending on the addressing scheme used, the MM messages, even if routed via the External Server, may have the MMS User Agents as the origination and the destination. However, from an MMS Relay/Server perspective, the MM messages are handled in the same way as Emails (i.e. MM1 to MM3 or vice-versa).

##### 4.4.3.1.2 	MMBox

The MMBox related steps are shown as and when an MM message transfer happens. However, the use of MMBox is optional. Therefore, all the steps along with the associated xIRIs are present if and only if MMBox is used by the target MMS User Agent.

When MMBox is not used, it is presumed that the MM messages are stored locally at the MMS/Relay Server.

When the MMBox is used, the MMS User Agent may also perform a few MMBox specific operations as illustrated in clause 4.4.3.2.2.

LI does not apply to such MMBox specific operations for target non-local ID.

##### 4.4.3.1.3	Delivery reports

When an MM message is retrieved by the terminating MM User Agent, a delivery report (with an indication of retrieval) may be sent to the originating MMS User Agent only when both of the following are true:

- The originating MMS User Agent requests for a delivery report.

- The terminating MMS User Agent permits such a delivery report.

The delivery reports are initiated by the MMS Relay/Server that serves the terminating MMS User Agent.

An intermediate delivery report may be sent by the originating MMS Relay/Server to the originating MMS User Agent as and when the MM message is forwarded to the destination, if the first condition listed above is true.

When MM message is redirected, a delivery report (with an indication of forwarding) may be sent to the originating MMS User Agent, if the above listed two conditions are met (here, the permission is from the redirecting MMS User Agent).

When the redirected-to MMS User Agent retrieves the redirected MM message, a delivery report (with an indication of retrieval) may be sent to the redirecting MMS User Agent if the above listed two conditions are met (here, the request for the delivery report is from the redirecting MM User Agent, and the permission is from the redirected-to MMS User Agent).

An illustration of the above conditions in a non-redirecting case is in figure 4.4.3.1.3-1.

Figure 4.4.3.1.3-1: Examples of delivery report conditions

The delivery report is not received if the destination is an Email Address. At the terminating end, a request for the delivery report is presumed to be not received, if the origination is an Email Address.

###### 4.4.3.1.4	Read reports

A read report may be sent to the originating MMS User Agent when both of the following are true:

- The originating MMS User Agent requests for a read report.

- The terminating MMS User Agent sends read report.

The read reports are generated by the terminating MMS User Agent.

When MM message is redirected, a read report may be sent to the redirecting MMS User Agent, if the above listed two conditions are met (here, the request is from the redirecting MMS User Agent, and read report is generated from the redirected-to MMS User Agent). When an MM message is redircted, the read-report, if generated, is sent only to the redirecting MMS User Agent.

An illustration of the above conditions in a non-redirecting case is in figure 4.4.3.1.4-1.

Figure 4.4.3.1.4-1: Examples of read report conditions

The read report is not received if the destination is an Email Address. At the terminating end, a request for report report is presumed to be not received, if the origination is from an Email Address.

#### 4.4.3.2	MMBox specific operations and LI

##### 4.4.3.2.1	Overview

The MMS User Agents may locally upload, view, retrieve, and delete the MM messages at the MMBox located within the local MMS Relay/Server. The MMS User Agents may also store an incoming MM message, update the status of a stored/uploaded MM message, in the MMBox.

This clause refers to these steps which may be taken by an MMS User Agent as MMBox specific operations.

The figure 4.4.3.2.1-1 illustrates the MMS LI messages as they relate to the MMSBox specific operations performed by a target MMS User Agent.

Figure 4.4.3.2.1-1: MMBox specific operations

From an LI perspective, the xIRIs listed are only applicable to the targets that are local to the MMS Relay/Server.

##### 4.4.3.2.2	MMBox messages and MMS LI

The figure 4.4.3.2.2-1 illustrates the generation of xIRIs in reference to the MMBox specific operations performed by the MMS User Agent as a target.

Figure 4.4.3.2.2-1: MMBox specific operations

The m-mbox-store-req is normally sent when the terminating MMS User Agent receives an incoming MM message to request the MMS Relay/Server to store the incoming MM message, if it is not stored automatically. However, m-mbox-store-req may also be used to update the state of the MM message stored in the MMBox. From that point of view, this is applicable to the originating MMS User Agent as well.

The WSP/HTTP GET is used to retrieve an MM message irrespective of it being stored in an MMBox. Accordingly, the xIRI MMSRetrieval is used to report when the target retrieves an MM message, irrespective of whether such a retrieval happens for internally stored MM message or for an MM message stored at the MMBox.

View, retrieval and deletion of MM messages with respect to the MMBox may happen for one or more MM messages.

The MMBox related operations that may also happen as and when the MM message is sent, received, or forwarded are illustrated in the subsequent clauses.

LI specifics:

The xIRI MMSMBoxUpload is generated when the MMS Relay/Server sends the m-mbox-upload-conf message to the target MMS User Agent. The xIRI also includes the information received the m-mbox-upload.req message.

The xIRI MMSMBoxStore is generated when the MMS Relay/Server sends the m-mbox-store-conf message to the target MMS User Agent. The xIRI also includes the information received the m-mbox-store.req message.

The xIRI MMSViewRequest is generated when the MMS Relay/Server receives the m-mbox-view-req message from the target MMS User Agent. The xIRI MMSViewResponse is generated when the MMS Relay/Server sends the m-mbox-view-conf message to the target MMS User Agent.

The xIRI MMSMBoxDelete is generated when the MMS Relay/Server sends the m-mbox-delete-conf message to the target MMS User Agent.

The xIRI MMSRetrieval is generated when the MMS Relay/Server sends the m-retrieve-conf to the target MMS User Agent.

***** End of the new text ****